# 3GPP TSG RAN WG4 Meeting #116	bis 														R4-2514188

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	6.10.3

Source: 	Ericsson

Title:	On performance requirement for Fast SCell activation for UE supporting Rel-18 EMR

Document for:	Discussion

# Introduction

From RAN #103 meeting the WID for RRM enhancement phase 5 is listed in [1]. The RAN4#116 WF is listed in [2].  In this paper we provide our view for the remaining open issues.

Background: WID objective #2

- Fast SCell activation for UE supporting Rel-18 EMR

- Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting

- Apply fast scell activation in FR1 and FR2-1

- Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

# Discussion

Issue 2-2-1: Test case list Agreement: - Scenario: - Normal SCell activation for FR1 and FR2 - Direct SCell activation at SCell addition for FR1 - PUCCH SCell activation for FR1 and FR2 - FFS: - Direct SCell activation at SCell addition for FR2 - UE capabilities and network configurations: - Configuration 1: UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured - Configuration 2: UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured - FFS: - Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode

The Rel-19 fast scell activation enhancement is based on the motivation that a valid EMR report can replace a normal L3 measurement report for unknown target cell. This will further enhance the unknown target cell activation requirement.

Due to the beam sweeping for FR2, any unknown target cell for activation at FR2 takes particularly long time. We see the importance of having the FR2 scenario being included since it is high gain from UE sending a valid EMR report in FR2 to save the cell search time regardless of activation signaling is MAC CE based or RRC based.

- Proposal 1: Direct Scell activation at Scell addition for FR2 shall be considered as one of the test scenarios.

From UE capabilities and network configuration point of view, the Rel-19 activation requirement will be based on a new Rel-19 UE capability. The prerequisite of the Rel-19 capability is the UE either supports Rel-18 or Rel-16 EMR capabilities.

The enhancement of the Rel-18 capability is:

- Supporting the cell reselection measurement

- Measurement results validation methods

For UE supporting Rel-16 capability only, when the timer T331 does not expire at the transition moment which indicates that the UE has continuous measurement and maintains relatively good sync with the carriers being configured, this is a valid case for the activation requirement enhancement. We think it is reasonable to support this configuration.

- Proposal 2: Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode shall be considered as one of the test configuration.

Issue 2-2-2: Details of test case

From the UE procedure aspect, performance evaluation begins when the UE receives the measurement configuration for EMR measurement and concludes when UE sent the CSI report for a successful Scell activation.

From a performance perspective, if the UE sends a valid measurement report 5 seconds prior to receiving the Scell activation command, the target cell is considered known. The known condition will apply, and specific delay activation requirements will be verified during the test. Conversely, if the UE does not send a valid measurement report before the activation command, the target cell is treated as unknown, and the unknown activation requirements will be verified during the test case.

From a test case timeline perspective, fast Scell activation for UEs supporting Rel-18 EMR requires a combination of two existing test cases, as illustrated in Figure 1.

Figure 1. Test case timelines to combine existing test cases.

- Observation 1: The EMR/Cell reselection test case and the Scell activation test case follow relatively independent timelines.

We think the T1 and T2 for the EMR test case blue timeline in Figure 1 can be merged since no UE behavior is verified during this time slot, it is just making sure UE connected to one cell and have received the EMR measurement configuration.

For legacy Scell activation test case, the green timeline in Figure 1, both activation and deactivation delay have been verified, we think it is okey to only verify the activation delay, T1 and T2 can be merged into one time duration.

The total procedure consists of 5 successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively as the figure below illustrates:

- T1: During T1, the UE is connected to Cell 1 (PCell) only and shall not have any timing information of Cell 2 (SCell to be activated). UE is configured with measurement reporting for Cell 2. The time point when UE receives RRC_Release message from the TE defines the starting point of T2.

- T2: At the beginning of T2, Cell 2 becomes detectable. Signal level of Cell 2 and the duration of T2 are set to fixed values.

- T3: At the beginning of T3, the signal level of Cell 2 is set to another value. The duration of T3 equals to measIdleValidityDuration-r18.

- T4: The time when TE sends the paging message is defined as the starting point of T4. During T4, in this test the UE shall send measurement report within the duration of T4.

- T5: At the beginning of T5, the TE sends a message for activation of the SCell (Cell 2). The activation command can be either RRC_reconfiguration or MAC CE command. The point in time at which the activation command is received at the UE antenna connector is denoted as slot #n. The UE shall be able to report valid CSI in PCell for the activated SCell at latest in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length, as defined in clause 8.3.2A.

Figure 2. Proposed test case timelines for Fast Scell activation with UE supporting Rel-18 EMR

- Proposal 3: Total test procedure consist 5 successive time periods for fast Scell activation with UE support EMR .T1: from UE camp in Pcell to receive RRC_Release configuration. T2: from Scell become detectable to signal level of Scell change to another level. T3: From Scell signal level change to end of validity duration. T4: from UE receive Paging to UE receives Scell activation command. T5: From UE receive activation command  to UE send the CSI report.

Detail UE behavior and activation delay expected can be seen from table.1

|  | Scenario | Test case configuration |  | Expected test outcome |
| --- | --- | --- | --- | --- |
| Rel-19 fast Scell activation FR1 | EMR UE report + unknown Scell activation | A6.6.9.1 | A6.5.3. 3 | Activation delay will be enhanced. |
| Rel-19 fast Scell activation FR1 | EMR UE report + unknown PUCCH Scell activation/deactivation | A6.6.9.1 | A6.5.3.7 | Activation delay will be enhanced. |
| Rel-19 fast Scell activation FR1 | Cell reselection UE report + unknown Scell activation | A6.6.9.4 | A6.5.3. 3 | Activation delay will be enhanced. |
| Rel-19 fast Scell activation FR1 | Cell reselection UE report + unknown PUCCH Scell activation/deactivation | A6.6.9.4 | A6.5.3.7 | Activation delay will be enhanced. |
| Rel-19 fast Scell activation FR2 | Cell reselection UE report + unknown Scell activation | A7.6.23.3 | A5.5.3.5 | Activation delay will be enhanced. |
| Rel-19 fast Scell activation FR2 | Cell reselection UE report + unknown PUCCH Scell activation/deactivation | A7.6.23.3 | A5.5.3.10 | Activation delay will be enhanced. |

- Proposal 4: It is preferred to have a unified test case for all Scell activation type .

#### Issue 2-2-3: Test case design

One remaining open issue is regarding how the DRX is being configured, since the Idle/Inactive measurement period will be impact if very long DRX cycle is being configured, this will not differentiate UE behaviour but only extend the test verification time. We don’t see the reason to have DRX especially long DRX cycle being configured, the key is that UE perform measurement and send EMR report so the Scell activation delay will be enhanced and verified.

- Proposal 5: There is no need to configure DRX cycle for all the test cases.

# Conclusion

	Proposal 1: Direct Scell activation at Scell addition for FR2 shall be considered as one of the test scenarios.

	Proposal 2: Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode shall be considered as one of the test configuration.

	Proposal 3: Total test procedure consist 5 successive time periods for fast Scell activation with UE support EMR .T1: from UE camp in Pcell to receive RRC_Release configuration. T2: from Scell become detectable to signal level of Scell change to another level. T3: From Scell signal level change to end of validity duration. T4: from UE receive Paging to UE receives Scell activation command. T5: From UE receive activation command  to UE send the CSI report.

	Proposal 4: It is preferred to have a unified test case for all Scell activation type .

	Proposal 5: There is no need to configure DRX cycle for all the test cases.

# References

- RP-240830 “New WID: WI resulting from discussion on RRM enhancements”, VIVO, March 2024

- R4-2512125, WF for NR_RRM_Ph5_part 2, CATT, August,2025