# 3GPP TSG RAN WG4 Meeting #116	bis 														R4-2514189

# Prague, Czech Republic, Oct. 13 -17, 2025

Agenda Item:	6.17.2.3

.3Source: 	Ericsson

Title:	Discussion on Rel-19 conditional LTM performance requirement

Document for:	Discussion

# Introduction

After RAN plenary in September 2024, the following objective has been agreed in the WID [1] for Mobility work item.

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1]

  - Specify UE evaluated conditions for triggering LTM

  - Aim to support conditional LTM including subsequent LTM

  - Limit specifying the conditional LTM to the scenario where the UE is in non-DC

  - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

- Specify RRM requirements related to the above objectives as necessary [RAN4]

In this contribution, we will address several aspects regarding the test case design for mobility RRM requirement.

# Discussion

## Intra-CU conditional LTM

#### Issue 4-3-2: CLTM test scope - intra/inter-frequency

At RAN4 # 116 meeting, the CLTM delay requirement was agreed to take the conditional handover as baseline due to the similarity between the CHO and CLTM. The CHO and CLTM are both based on the UE evaluating preconfigured handover/LTM conditions to trigger the event and UE executing the condition for target cell switch.

For conditional LTM, the candidate can be either with intra or inter-frequency measurement, we think it is good to cover both of the measurement scenario.

- Proposal 1: RAN4 test scope of conditional LTM shall cover both intra and inter-frequency measurement triggered CLTM.

#### Issue 4-3-3: CLTM test scope – L1/L3 based CLTM

The UE procedure to evaluate the candidate and execute on the target cell for CLTM is different from CHO and also different from LTM.

For example, the UE candidate evaluation for CLTM can be based on either L1 or L3 measurement while the UE only evaluates CHO candidate based on L3 measurement. The LTM test case however was designed based on both L3 reporting for candidate and L1-RSRP for the target cell.

- Proposal 2: RAN4 test scope shall verify both L1 and L3 measurement based candidate evaluation for CLTM and further discuss how to verify such as whether to reuse partial LTM test procedure.

#### Issue 4-3-4: CLTM test scope – RACH-less and RACH-based CLTM

In CLTM, the UE will receive a pre-configured execution condition for the candidate cell. When the event being triggered for certain candidate cell, the UE will execute on this candidate.

Agreement on RAN4#114,  the ending point of the CLTM will be based on whether it is RACH based or RACH-less CLTM. This is the same as the LTM procedure that both the RACH based and RACH-less cell switch are supported based on whether UE has valid TA value.  We think both RACH based and RACH-less CLTM shall be verified based on different requirements and UE behavior.

- Proposal 3: RAN4 test scope shall cover both RACH based and RACH-less CLTM.

#### Issue 4-3-5: CLTM test scope – early DL sync

One open issue from last meeting is whether the CLTM without early DL sync shall be tested. Early DL sync is part of the new function introduced from LTM, we think for CLTM is also needs to be verified. For the CLTM candidate cell that being triggered execution without DL sync partially can behave as CHO if it is triggered by L3 measurement, however, due to the fact that CLTM also support L1 measurement triggered execution, the UE can behave different as CHO.

- Proposal 4: RAN4 test scope shall cover with and without early DL sync for CLTM

#### Issue 4-3-6: CLTM test scope – PCell/PSCell

The Rel-19 CLTM discussion have been around whether to support Pcell switch to an LTM candidate what is a serving Scell, but both serving and the candidate are based on same MCG. Since the NR-DC scenario is supported, we see there is difference between Pcell swithc and Pscell switch. We think it is needed to verify Pscell as well as the Pcell switch.

- Proposal 5: The Pscell switch is needed to be covered in the test scope of CLTM.

#### Issue 4-3-7: subsequent CLTM test scope

For subsequent CLTM, we think it is important to verify intra-inter frequency measurement, RACH and RACH-less cell switch as well as the FR1 and FR2 group since the requirement will be different.

- Proposal 6: RAN4 test scope for subsequent CLTM shall cover both Intra and Inter frequency, RACH based and RACH-less scenarios.

#### Issue 4-3-8: test configuration related proposals

The conditional handover test timeline have only T1 and T2, T1 is the time from UE received the RRC reconfiguration message until the event being fulfilled while the T2 is the handover interruption time. But the LTM test timeline has 4 time periods. The T4 represents the interruption time. The difference can be seen from figure 1.

Figure 1: test timeline between CHO and LTM

- Proposal 7 : RAN4 to discuss how to verify both RACH based and RACH-less procedure for CLTM such as re-use partial LTM test procedure.

#### Issue 4-3-9: test applicability

The relationship between subsequent CLTM and CLTM is that UE have execute on one CLTM candidate and then UE execute on the next CLTM candidate. For test verification, the subsequent CLTM will naturally include a CLTM procedure. We are generally open to reduce the repeated test cases, as long as the FR group, intra-inter frequency , RACH or RACH-less, L1/L3 trigger have been verified in either of CLTM or subsequent CLTM.

- Proposal 8: CLMT and subsequent CLTM can be combined together to have full coverage of the test cases.

# Summary

The following have been proposed in the current contribution:

	Proposal 1: RAN4 test scope of conditional LTM shall cover both intra and inter-frequency measurement triggered CLTM.

	Proposal 2: RAN4 test scope shall verify both L1 and L3 measurement based candidate evaluation for CLTM and further discuss how to verify such as whether to reuse partial LTM test procedure.

	Proposal 3: RAN4 test scope shall cover both RACH based and RACH-less CLTM.

	Proposal 4: RAN4 test scope shall cover with and without early DL sync for CLTM

	Proposal 5: The Pscell switch is needed to be covered in the test scope of CLTM.

	Proposal 6: RAN4 test scope for subsequent CLTM shall cover both Intra and Inter frequency, RACH based and RACH-less scenarios.

	Proposal 7 : RAN4 to discuss how to verify both RACH based and RACH-less procedure for CLTM such as re-use partial LTM test procedure.

	Proposal 8: CLMT and subsequent CLTM can be combined together to have full coverage of the test cases.

# References

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom, 3GPP TSG RAN #105, Melbourne, Australia, Sep 9-12, 2024.