3GPP TSG-RAN WG4 Meeting #116bis                                                           R4-2514209

Prague, Czech Republic, 13th – 17th October, 2025

Source:	Samsung

Title:	Discussion on SSB adaptation SCell operation performance

Agenda item:			6.15.2.2

Document for:	Discussion

1	Introduction

In this meeting, we will continue giving some analyses on TC design including test scope and test configuration based on WF [1] agreed in last meeting.

2	Performance requirements for NES SSB adaptation

2.1 Issue 3-3-1: Test case list

Issue 3-3-1: Test case list Agreement RAN4 to at least define the following SSB adaptation test case. - L3 intra-frequency measurement RAN4 to discuss whether the following SSB adaptation test case list can be defined. - L1-RSRP - L1-SINR - CBD

L1-RSRP measurement on activated SCell were defined with the consideration of adapted SSB periodicity, which is triggered/indicated by NW signalling DCI 2_9. And for adaptation of SSB in time-domain, the UE can be configured with at most 2 additional SSB burst periodicities (from network energy saving, normal RRM measurement, low-latency SCell operation), which is different from what the periodicity can to be configured for OD-SSB. Besides, for L1-SINR, the functionality difference to legacy SSB based measurement can be verified by L1-RSRP test, so there is no need to define additional L1-SINR one/Deprioritize L1-SINR. From this, we have the following proposal:

Proposal 1: Suggest RAN4 to introduce new tests for SSB based L1-RSRP measurement requirements for NES SSB adaptation, and to deprioritize L1-SINR

- New TC is to verify the measurement procedure when SSB periodicity adaptation is activated by NW signalling DCI 2_9

2.2 Issue 3-3-2: General principle

Issue 3-3-2: General principle RAN4 to discuss whether the following principles to design the SSB adaptation test case can be agreed. - RAN4 to define the test cases for SSB adaptation in RRC_CONNECT mode only. - RAN4 to define test case for SMTC and SSB periodicity change based on DCI 2-9. - RAN4 to reuse legacy trigger events for measurement report after SSB adaptation. - Both FR1 and FR2. - RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement. - Only test non-DRX mode. - without SSB time index detection.

Based on RAN1 agreement, SFN offset (relative to SFN0) and half-frame index are configured per additional SSB periodicity value. The first SSB burst after SSB adaptation according to the SSB burst periodicity after adaptation and associated SFN offset and half-frame index, so blind detection of SSB is not needed.  Besides, although we define L1/L3 transition requirement for SSB adaptation, in legacy TC design, there is no such TC to verify requirements at transition for L1/L3 measurement. In this sense, RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement. For other principle, we’re also fine.

Proposal 2: The following principles to design the SSB adaptation test case can be accepted

- In RRC_CONNECT mode only.

- SMTC and SSB periodicity change based on DCI 2-9.

- Reuse legacy trigger events for measurement report after SSB adaptation.

- Both FR1 and FR2.

- RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement.

- Only test non-DRX mode.

- Without SSB time index detection.

2.3 SMTC and SSB configuration

- Proposals - Option 1 - TSSB and TSMTC periodicity use large deltas in the periodicity before and after SSB adaptation. - Default SSB periodicity: 20ms - Larger SSB periodicity: 80/160ms - Denser SSB periodicity: 5/10ms - Option 2 - RAN4 to use SMTC periodicity change from 80ms to 20ms and SSB periodicity change from 80ms to 20ms, and vice versa. - Recommended WF - Further discussion.

From our understanding, based on RAN1 agreement that for adaptation of SSB in time-domain, UE can be configured with X (<=2) additional SSB burst periodicities for a SCell. The following candidate values can be taken into consideration to cater to different aims, e.g., typical default SSB periodicity: 20 ms for normal RRM measurement; Larger value: 80/160 ms for energy saving gain acquisition; Denser value: 5/10 ms for fast SCell measurement or SCell activation. From this, we have the following proposal:

Proposal 3: TSSB and TSMTC periodicity use large deltas in the periodicity before and after SSB adaptation

- Typical default SSB periodicity: 20 ms

- To larger SSB periodicity: 80/160 ms; To denser SSB periodicity: 5/10 ms

3	Conclusion

In this contribution, we provided our viewpoints on RRM performance requirements for NES SSB adaptation. The following proposals are obtained:

Proposal 1: Suggest RAN4 to introduce new tests for SSB based L1-RSRP measurement requirements for NES SSB adaptation, and to deprioritize L1-SINR

- New TC is to verify the measurement procedure when SSB periodicity adaptation is activated by NW signalling DCI 2_9

Proposal 2: The following principles to design the SSB adaptation test case can be accepted

- In RRC_CONNECT mode only.

- SMTC and SSB periodicity change based on DCI 2-9.

- Reuse legacy trigger events for measurement report after SSB adaptation.

- Both FR1 and FR2.

- RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement.

- Only test non-DRX mode.

- Without SSB time index detection.

Proposal 3: TSSB and TSMTC periodicity use large deltas in the periodicity before and after SSB adaptation

- Typical default SSB periodicity: 20 ms

- To larger SSB periodicity: 80/160 ms; To denser SSB periodicity: 5/10 ms

Reference

[1] 	R4-2512327, WF on RRM requirements for Netw_Energy_NR_enh_Part1 v1