3GPP TSG-RAN WG4 Meeting #116bis                                                           R4-2514211

Prague, Czech Republic, 13th – 17th October, 2025

Source:	Samsung

Title:	Discussion on Rel-20 RRM Enhancements redcap

Agenda item:			7.4.3.2

Document for:	Discussion

1	Introduction

In RAN #109, the following objectives for NR Radio Resource Management (RRM) Phase 6 were agreed in [1] RP-252955, the details are copied as below

- Specify the following requirements for (e)RedCap UEs [RAN4] - Relaxed RLM/BFD RRM requirements - Use R17 RLM/BFD relaxation for legacy UE as a baseline - Support of less than 5 MHz channel bandwidth (15 PRB transmission bandwidth configuration and 12 PRB SSB) - RF requirements, considering example bands: n100, n106, n26, n28 and n85 - RRM requirements including relaxed RLM/BFD RRM requirements - Note 1: Use R18 3 MHz requirements for non-RedCap UE as a baseline - Note 2: Consider both 1Rx and 2Rx (e)RedCap UEs and HD-FDD - For intra-band co-located contiguous CA, specify necessary RRM requirements to support data scheduling on SCell prior to UE transmitting valid CSI report (to be started in Q4 2026) [RAN4]

In this meeting, we will give some initial views on Rel-20 RRM Enhancements redcap.

2	Relaxed RLM/BFD RRM requirements

The feasibility and performance impact of relaxing UE measurements for RLM and/or BFD was discussed in Rel-17, targeting good serving cell quality and UE low mobility state with short DRX cycle. The main aspects including criteria under which relaxed RLM and BFD can be performed, RLM and BM performance degradation during relaxation, and relaxed requirements for RLM and BFD were discussed and concluded. The relaxed requirements involving scaling factor definition captures: SSB based RLM/BFD measurement relaxation in FR1, CSI-RS based RLM/BFD measurement relaxation in FR1, SSB based RLM/BFD measurement relaxation in FR2 and CSI-RS based RLM/BFD measurement relaxation in FR2. The following Tables give an example regarding how to specify the relaxed RLM/BFD measurement requirement.

Table 8.1.2.4-1: Evaluation period TEvaluate_out_SSB_Relax for FR1

| Configuration | TEvaluate_out_SSB_Relax (ms) |
| --- | --- |
| Max(TDRX,TSSB) ≤80 ms | Max(200 K3 NOTE3, Ceil(15 K1NOTE2 P) Max(TDRX,TSSB)) |
| 80 ms < Max(TDRX,TSSB) ≤160 ms | Ceil(15 P) Max(TDRX,TSSB) |
| NOTE 1: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length and no longer than 80 ms. NOTE 2: K1 = 4 for Max(TDRX,TSSB) ≤40 ms and K1 = 2 for 40 ms<Max(TDRX,TSSB) ≤80 ms. NOTE 3: K3 = K1, if K1 ≤ 2; otherwise K3 = 1. |  |

Table 8.1.2.4-2: Evaluation period TEvaluate_out_SSB_Relax for FR2

| Configuration | TEvaluate_out_SSB_Relax (ms) |
| --- | --- |
| Max(TDRX,TSSB) ≤80 ms | Max(200 K4 NOTE3, Ceil(15 K2 NOTE2 P N) Max(TDRX,TSSB)) |
| 80 ms < Max(TDRX,TSSB) ≤160 ms | Ceil(15 P N) Max(TDRX,TSSB) |
| NOTE 1: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length and no longer than 80 ms. NOTE 2: K2 = 2. NOTE 3: K4 = K2, if K2 ≤ 2; otherwise K4 = 1. |  |

Since in the current Spec., it is observed that the following two aspects: 1) The RLM requirement (i.e., evaluation period) defined for 2Rx RedCap UE, 2) UE capability of PDCCH monitoring (i.e., PDCCH transmission parameters for OOS/IS evaluation) considered for 2Rx RedCap UE are the same as the ones specified for non-RedCap UE, if RLM and/or BFD requirements can be relaxed for non-RedCap UE under certain conditions, the corresponding requirements can be relaxed for 2Rx RedCap UE, and in theory, the same principle can be reused.

Proposal 1: To define relaxed RLM/BFD RRM requirements for 2Rx RedCap UE, the same relaxation related criteria and scaling factor defined for legacy non-Redcap UE can be reused, and such requirements are applicable to:

- UEs configured with short DRX cycle

- SSB based and CSI-RS based RLM/BFD measurement

However, for 1Rx RedCap UE, the story is a little different, one of the reasons is that # of CCE limits relaxation was considered to save UE power. In this sense, how to extend the evaluation period for RLM/BFD without sacrificing the network performance for 1Rx RedCap UEs configured with short DRX cycle may need further analysis, in this sense, we have

Proposal 2: To define relaxed RLM/BFD RRM requirements for 1Rx RedCap UE, RAN4 to discuss whether relaxation related criteria and scaling factor defined for legacy UE can be reused as a starting point

3	Conclusion

In this contribution, we provided our initial viewpoints on RRM requirements on relaxed RLM/BFD RRM requirements for (e)RedCap UEs. The following observations and proposals are obtained:

Proposal 1: To define relaxed RLM/BFD RRM requirements for 2Rx RedCap UE, the same relaxation related criteria and scaling factor defined for legacy non-Redcap UE can be reused, and such requirements are applicable to:

- UEs configured with short DRX cycle

- SSB based and CSI-RS based RLM/BFD measurement

Proposal 2: To define relaxed RLM/BFD RRM requirements for 1Rx RedCap UE, RAN4 to discuss whether relaxation related criteria and scaling factor defined for legacy UE can be reused as a starting point

Reference

[1] 	RP-252955, WID: NR Radio Resource Management (RRM) Phase 6, Apple