3GPP TSG-RAN WG4 Meeting #116 bis	R4-2514261

Prague, CZ, October 13th – October 17th, 2025

Source:	Nokia

Title:	Discussion on performance part of NR_MIMO_Ph5 work item for other RRM requirements

Agenda item:	6.13.2.2

Document for:	Discussion

Introduction

In this contribution we discuss the necessary test cases for the objectives of the NR_MIMO_Ph5 work item.

Asymmetric DL sTRP/UL mTRP

In the last RAN4 meeting for performance requirements, there were discussions about a) if to introduce test cases for asymmetric DL sTRP/UL mTRP and b) what would be the test scope. However, there were differing views on which test cases needed to be defined, as shown here below.

Issue 2-3-1: Test scope High level:; QC; Apple; Samsung; CMCC; Huawei; Nokia; MTK. Whether to define TCs to verify timing requirements?; Y; Y; Y; Y; Y. Whether to define TCs to verify TCI state switching requirements?; Y; N. FR1 and/or FR2?; both. Whether to cover Scenario#1 and Scenario#2?; Both for FR1. Scenario#2 for FR2; Both; Both; Scenarios#1; both; Scenarios#1. - Recommended WF - For the test case list, to discuss based on table above.

The main differences for asymmetric DL sTRP/UL mTRP with 2 TAs in Rel-19 MIMO Phase 5 when compared to MIMO Evo Rel-18 two TAs are that:

- Rel-19 enhancement is for single-DCI, whereas Rel-18 enhancement was for multi-DCI and

- in Rel-19 Scenario#1 there is no RS from the UL-only TRP to be used by the UE for downlink reference timing, whereas in Rel-18 the UE was supposed to have one downlink reference timing per TAG.

Furthermore, it is worth highlighting the starting point test cases that have already been defined in TS 38.133. More specifically, in TS 38.133 the following two related test cases for single TA have been specified:

- Transmit Timing Test for FR1 and FR2 (A.6.4.1.1 and A.7.4.1.1 of TS 38.133), to verify that a UE can follow frame timing, i.e., downlink reference timing, change of the connected gNB;

- Timing Advance Adjustment Accuracy Test for FR1 and FR2 (A.6.4.3.1 and A.7.4.3.1 of TS 38.133), to verify UE TA adjustment delay and accuracy requirement.

Finally, in Rel-18 enhancement for multi-TRP multi-DCI with 2 TAs, it was discussed if two additional test cases were necessary, one to verify that a UE can follow frame timing change and one to verify UE TA adjustment delay and accuracy. Eventually, it was acknowledged that with 2 TAs the most relevant and challenging aspect to be tested was the UE capability to follow two different downlink reference timings, and because of that only the Transmit Timing Test (but not the Timing Advance Adjustment Accuracy Test) was introduced for FR1 (A.6.4.1.2 of TS 38.133) and FR2 (A.7.4.1.3 of TS 38.133). Furthermore, it also worth observing that the Rel-18 Transmit Timing Test considered different configurations depending on the MRTD capabilities of the UE.

Furthermore, we also observe that a UE supporting 2 TAs for asymmetric DL sTRP/UL mTRP (Rel 19) is not required to support at the same time also 2 TAs for multi-DCI (Rel 18): that can be observed from the UE features list for Rel 19 after RAN1#121 R1-2504673, with the relevant extract reported here below (note that the prerequisite feature groups 23-1-1 and 23-1-1a refer to the Rel 17 UE capability of supporting unified TCI with joint DL/UL TCI update for intra-cell and inter-cell beam management, respectively).

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-4-4a | Support two TAs enhancement for intra-cell beam management | Support of two TAs without the restriction of multi-DCI based multi-TRP operation for intra-cell beam management | 23-1-1 | yes | n/a | Two TAs without the restriction of multi-DCI based multi-TRP operation for intra-cell beam management is not supported |
| 59. NR_MIMO_Ph5 | 59-4-4b | Support two TAs enhancement for inter-cell beam management operation | Support of two TAs without the restriction of multi-DCI based multi-TRP operation for inter-cell beam management | 23-1-1a | yes | n/a | Two TAs without the restriction of multi-DCI based multi-TRP operation for inter-cell beam management is not supported |

A UE supporting 2 TAs for asymmetric DL sTRP/UL mTRP (Rel 19) is not required to support at the same time also 2 TAs for multi-DCI (Rel 18).

In our perspective, because of that, for asymmetric DL sTRP/UL mTRP with 2 TAs we need then to introduce test cases for both Scenario#1 and Scenario#2.

Define test cases for asymmetric DL sTRP/UL mTRP with 2 TAs for both

  - Scenario#1, UL-only TRP does not transmit SSBs, and

  - Scenario#2, UL-only TRP transmits SSBs.

For asymmetric DL sTRP/UL mTRP with 2 TAs Scenario#2, for the same reasons that motivated RAN4 in Rel-18 to define only the Transmit Timing Test, we think test cases should be introduced to verify that the UE can follow downlink reference timing changes for both FR1 and FR2.

For asymmetric DL sTRP/UL mTRP with 2 TAs Scenario#2 (UL-only TRP transmits SSBs) define

- A Transmit Timing Test for FR1 extending the Rel-18 multi-TRP multi-DCI 2 TAs Transmit Timing Test defined in A.6.4.1.2 of TS 38.133 and

- A Transmit Timing Test for FR2 extending the Rel-18 multi-TRP multi-DCI 2 TAs Transmit Timing Test defined in A.7.4.1.3 of TS 38.133.

Then, also similarly to Rel-18, such Transmit Timing Test should consider different configurations depending on the different UE MRTD capabilities.

For asymmetric DL sTRP/UL mTRP with 2 TAs Scenario#2 (UL-only TRP transmits SSBs), include different configurations depending on the different UE MRTD capabilities in the Transmit Timing Tests.

Differently from Scenario#2 that has very large similarities with Rel-18 multi-TRP multi-DCI, Scenario#1 introduces major changes. In Scenario#1, the UE is required to:

- monitor one DL reference timing (implicitly meaning from the DL/UL capable TRP, as that TRP is eventually the only TRP transmitting any DL signal), which will be used for both TAGs, and

- apply the Timing Advance Commands (TACs) TA1 and TA2 to such single DL reference timing for setting the UL transmission timings for TAG1 and TAG2, respectively.

It is also worth highlighting that, at this stage of the work, MRTD and MTTD requirements have been agreed only for Scenario#2, but not yet for Scenario#1: MRTD requirements are in our view simply not possible in Scenario#1, as the UE monitors just one DL reference timing, so potentially only MTTD requirements will be introduced.

Because of the differences between Scenario#1 and Scenario#2, we think it is worthy to introduce test cases for Scenario#1 to verify both:

- that the UE follows properly the one DL reference timing and adapts the UL transmissions toward both TRPs (Transmit Timing Test) and

- that the UE applies TACs from both TRPs, i.e., UL-only TRP and DL/UL capable TRP, while monitoring a single DL reference timing from the DL/UL capable TRP (for this last test, if MTTD requirements are introduced, it is important to make sure that TACs from both TRPs are within the MTTD requirements).

For asymmetric DL sTRP/UL mTRP with 2 TAs Scenario#1 (UL-only TRP does not transmit SSBs) define

- Transmit Timing Test for FR1 and for FR2, and

- Timing Advance Adjustment Accuracy Test for FR1 and for FR2, including configurations for the TACs that depend on the MTTD requirements, if MTTD requirements are introduced.

In summary, our view about Issue 2-3-1 on the test scope for UL-Only TRP is

Summary of Nokia proposals for asymmetric DL sTRP/UL mTRP with 2 TAs test cases:

| High level: | Nokia proposal |
| --- | --- |
| Whether to define TCs to verify timing requirements? | Yes, - Transmit timing - TA adjustment accuracy |
| Whether to define TCs to verify TCI state switching requirements? | Open to discuss |
| FR1 and/or FR2? | Both |
| Whether to cover Scenario#1 and Scenario#2? | Both |

Reporting for CJT

The following was open for discussion regarding CJTC reporting mapping tables [1]:

Issue 2-2-1: Whether to define reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P? <Way forward > - Option 1: (Samsung, Apple, CMCC) - RAN4 to add reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P.

We understand that the CJTC tables are necessary for the operation using CTC. Therefore we are fine defining the CJTC tables for all the reporting cases.

RAN4 to add reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P.

Furthermore, the following was open for discussion regarding CJTC reporting accuracy requirements [1]:

Issue 2-2-2: Whether to define accuracy requirements for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P? <Way forward > - Option 1: (Samsung) - RAN4 to add accuracy requirements for CJTC reporting including CJTC-Dd, CJTC-F. - Option 2: (Apple) - RAN4 to discuss feasibility of defining accuracy requirements for time offset, frequency offset reporting based on number of samples agreed in core part. - Option 3: (Ericsson) - Develop accuracy requirements for delay, frequency and phase offset UE measurement reports for: - Different number of samples - High and low SNR - Normal and extreme conditions

As discussed in our previous papers [2], it is important that accuracy requirements cover low SNR conditions. As highlighted in [2], the CJT gains are not so significant at cell center as when compared to cell edge. Therefore, in order to take advantage of the full benefit of CJTC, RAN4 should evaluate the test cases at low SNR conditions.

Observation 7: Simulations show that CJT provides better gains in cell edge than cell center.

Use SNR=-3 dB for defining the accuracy requirements for CJTC reporting.

Lastly, the following was open with the list of test cases for CJTC reporting [1]:

Issue 2-2-3: Test scope <Way forward > - Further discuss based on the table below: High level:; QC; Nokia; Samsung; CMCC; Huawei; Ericsson. Whether to define TCs to verify CJTC-Dd, measurement period; Y; Y; Y; Y; FFS; Y. Whether to define TCs to verify CJTC-f，measurement period; Y; Y; Y; Y; FFS; Y. Whether to define TCs to verify CJTC-P， (To be verify?); Y; FFS; Y. Whether to define TCs to verify scheduling restriction; Y; FFS.

Since RAN4 already agreed not to define core requirements for CJTC-P, there is no need to define test cases for the phase requirements. Additionally, scheduling restriction is still open in the RRM core requirement discussion. We believe that scheduling restrictions are not needed for CJTC reporting, therefore, we don’t think it is needed to define test cases.

RAN4 to define test cases for CJTC reporting:

  - to verify CJTC-Dd measurement period

  - to verify CJTC-f measurement period

  - to verify CJTC-Dd measurement accuracy

  - to verify CJTC-f measurement accuracy

- Other issues: Invalid measurements impact on the test.

There is an important issue that could be discussed as part of the tests for CJT calibration, and this is the impact of invalid measurements in the test. The invalid conditions are defined as a possible reporting state for CFO and PO, however the RAN 1 agreements did not include a definition of the invalid state. UE is not configured in anyway to detect such invalid states, therefore the decision making associated with invalid state reporting is left to implementation of the device. The latter may be a problem during the test because if different UE devices under test, report invalid measurements in different amounts and by unknown reasons, the number of valid measurements out of the total of measurements is affected by that, and that could bias the test statistics.

We think that the test should be able to measure not only the accuracy of a valid measurement, but also it should be able to introduce features in the test that make it possible to include both invalid and valid measurements in the test in defined proportions, and verify the capability of the UE to determine whether a measurement is to be reported as valid or invalid. For that, it should also be needed to include in the test, for instance, a test mode feature in which the testing equipment may configure the UE with conditions that define the validity or invalidity of the measurements. Otherwise, UE may inform the threshold(s) used to determine the validity of a measurement. By knowing or configuring the conditions, the test can be configured to test the UE with a determined sequence of valid and/or invalid measurements

Define the conditions where the UE may report invalid measurements. .

Discuss about the impact of invalid measurements in the CJTC reporting test and possible ways to verify the conditions for invalid reports for CJT calibration.

Analysis of algorithm for CJT-C

From last meeting WF in RRM core requirements:

Issue 1-1: How to define measurement period and accuracy requirements for delay and frequency offset reporting for CJT calibration? Agreement: Define core requirements based on 1 sample only and further discuss the accuracy and SNR in the performance part. Companies are encouraged to discuss how to derive the accuracy performance based on the simulation results.

In order to verify potential causes for the differences on the simulation results, we present 2 algorithm options that can be used for CJTC estimation.

In the approach used in this paper, the received OFDM signal is processed by one of the estimation methods (Algorithm 1 or Algorithm 2), which estimates the frequency offset for each TRP. The quantized difference between true and estimated relative frequency offsets FOTRP1-FOTRP2 is being used for calculating statistics.

A brief description of frequency offset FOTRPN estimation algorithms is described as follows:

- Algorithm 1: correlation-based algorithm

  - Uses correlation-based search over a fine grid of possible frequency offsets.

  - For each candidate offset,

    - it generates the reference TRS symbols in time simulating the TRS signal with that offset.

    - It extracts TRS symbols from the received grid and computes the correlation (similarity) between the received TRS symbols and each reference.

  - The offset with the highest correlation is selected as the estimated frequency offset.

  - 2 settings are presented in the results:

    - Slow

      - Using 1401 offset candidates per TRP:

    - Fast

      - Using 71 offset candidates per TRP

- Algorithm 2: phase-ramp-based algorithm

  - Uses phase difference analysis on pairs of TRS symbols.

  - Extracts TRS symbols and reference symbols, computes phasors (complex ratios between pairs of TRS symbols).

  - Average the phase differences, then converts these to frequency offset estimates.

  - 2 versions are presented in the results:

    - Slow version:

      - Phasor computation is performed between all combinations of TRS pairs

      - Designed for high accuracy, handling phase ambiguities and symbol relationships in detail

    - Fast version:

      - Uses a simplified phase difference algorithm

      - Computes phasors only for neighbouring TRS symbol pairs.

      - Fewer pairs and steps than slow, so less computation.

Table 1 Simulation results with different CJTC algorithms

|  |  | Algorithm 1 – correlation based |  | Algorithm 2 – phase ramping |  |
| --- | --- | --- | --- | --- | --- |
| SCS | SNR (dB) | Slow (1401 levels) | Fast (71 levels) | Slow | fast |
| 15 | -3 | 29 | 33 | 69 | 92.5 |
|  | 0 | 16 | 18 | 27 | 48 |
|  | 3 | 11 | 11 | 16 | 27 |
| 30 | -3 | 55 | 55 | 98 | 127 |
|  | 0 | 32 | 33 | 50 | 87 |
|  | 3 | 20 | 18 | 29 | 49 |

Table 1 shows the simulation results for different types of algorithms. As pointed out by many companies in RAN4 #116 there is a big misalignment among implementation of the companies bringing simulation results. The analysis of different algorithms can be used as a way of explaining the potential causes for those differences.

In the results of Table 1 it can be observed that Algorithm 1 presents a very similar performance, even when the number of correlation vectors is reduced from 1401 levels to 71 levels.

On the other hand, algorithm 2 (based on phase ramping) presents a more significant accuracy difference when comparing the fast algorithm, i.e. comparison of consecutive TRS symbols, in relation to the slot one, i.e. comparison of all TRS symbol combination. In the case of algorithm 2, accuracy error of the slow option is at least 1.6 to 2 times larger than the error of Algorithm 1.

Considering that performance difference, it is advisable for companies to evaluate and implement one version of Algorithm 1 in their own simulations.

RAN4 to define CJT-C accuracy based on correlation algorithm described in this paper.

Conclusion

The following Observations and Proposals were made:

Observation 1: A UE supporting 2 TAs for asymmetric DL sTRP/UL mTRP (Rel 19) is not required to support at the same time also 2 TAs for multi-DCI (Rel 18).

Proposal 1: Define test cases for asymmetric DL sTRP/UL mTRP with 2 TAs for both

a.	Scenario#1, UL-only TRP does not transmit SSBs, and

b.	Scenario#2, UL-only TRP transmits SSBs.

Proposal 2: For asymmetric DL sTRP/UL mTRP with 2 TAs Scenario#2 (UL-only TRP transmits SSBs) define

a.	A Transmit Timing Test for FR1 extending the Rel-18 multi-TRP multi-DCI 2 TAs Transmit Timing Test defined in A.6.4.1.2 of TS 38.133 and

b.	A Transmit Timing Test for FR2 extending the Rel-18 multi-TRP multi-DCI 2 TAs Transmit Timing Test defined in A.7.4.1.3 of TS 38.133.

Proposal 3: For asymmetric DL sTRP/UL mTRP with 2 TAs Scenario#2 (UL-only TRP transmits SSBs), include different configurations depending on the different UE MRTD capabilities in the Transmit Timing Tests.

Proposal 4: For asymmetric DL sTRP/UL mTRP with 2 TAs Scenario#1 (UL-only TRP does not transmit SSBs) define

a.	Transmit Timing Test for FR1 and for FR2, and

b.	Timing Advance Adjustment Accuracy Test for FR1 and for FR2, including configurations for the TACs that depend on the MTTD requirements, if MTTD requirements are introduced.

Proposal 5: Summary of Nokia proposals for asymmetric DL sTRP/UL mTRP with 2 TAs test cases:

Proposal 6: RAN4 to add reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P.

Proposal 7: Use SNR=-3 dB for defining the accuracy requirements for CJTC reporting.

Proposal 8: RAN4 to define test cases for CJTC reporting:

a.	to verify CJTC-Dd measurement period

b.	to verify CJTC-f measurement period

c.	to verify CJTC-Dd measurement accuracy

d.	to verify CJTC-f measurement accuracy

Other issues: Invalid measurements impact on the test.

Proposal 9: Define the conditions where the UE may report invalid measurements.

Proposal 10: Discuss about the impact of invalid measurements in the CJTC reporting test and possible ways to verify the conditions for invalid reports for CJT calibration.

Proposal 11: RAN4 to define CJT-C accuracy based on correlation algorithm described in this paper.

- References

- R4-2509059, WF on RRM requirements for NI_MIMO_Ph5_Part1, Samsung

- R4-2508271, Other RRM requirements for MIMO ph5, Nokia