3GPP TSG-RAN WG4 Meeting #116bis	R4-2514286

Prague, Czech Republic, October 13 – 17, 2025

Source:	Nokia

Title:	On Enhancements for Air-to-Ground Network for NR

Agenda item:	6.7.4

Document for:	Discussion

Introduction

The Way Forward (WF) from RAN4#116 discussions on Rel-19 Enhancements for Air-to-ground network for NR is documented in [1]. The discussions covered 3 topics, namely, DL CA, DL MIMO and UL MIMO, with agreements as summarized below:

Topic #1 DL CA requirements Issue 1-2: Rank - Agreement - Rank 1. Issue 1-3: MCS - Agreement - 64QAM MCS22. # Topic #2 DL MIMO requirements Issue 2-2: MCS - Agreement: - 64QAM MCS 22. - FFS for 256QAM Table 2 MCS 24. # Topic #3 UL MIMO requirements Issue 3-1: Antenna configuration for incremental test - Agreement: - 2T2R with X-pol low correlation. Issue 3-2: MCS for incremental test - Agreement: - 64QAM Table 1 MCS 20. Issue 3-3: Test applicability rule for 2Tx - Agreement: - Unless otherwise stated, PUSCH requirement tests in clause 8.2.1 with 2Tx 2-layer test cases shall apply only for BS declared to be supported (see D.123 in table 4.6-1).

In this contribution, we share our views on the open issues to be discussed in RAN4#116bis.

Discussion

DL CA

The issues going to be addressed here are:

Issue 1-1: TDD pattern - Option 1: Use the legacy TDD pattern 7D1S2U as starting point. FFS for new TDD pattern 30D4S6U.

Referring to Table A.1.2-2c: TDD UL-DL configuration for SCS 30 kHz for ATG in 38.101-4 [2], the TDD pattern of 30D4S6U is mentioned. As DL CA TDD 30 kHz + TDD 30 KHz is already agreed in RAN4#115, 30D4S6U could be considered for the requirements.

Nonetheless, there was a concern raised in [3] about some potential maintenance issues to the new TDD pattern 30D4S6U introduced in Rel-18. Hence, to avoid such a concern, RAN4 shall focus on the legacy TDD pattern 7D1S2U.

For DL CA, RAN4 shall consider the legacy TDD pattern 7D1S2U only.

DL MIMO

The open issue to be addressed in this topic is:

Issue 2-1: TDD pattern - Option 1: Use the legacy TDD pattern 7D1S2U as starting point. FFS for new TDD pattern 30D4S6U. Issue 2-2: MCS - Agreement: - 64QAM MCS 22. - FFS for 256QAM Table 2 MCS 24.

Concerning the TDD pattern, as already mentioned above in the discussion on DL CA, due to the concern raised in [3] on TDD pattern 30D4S6U, for DL MIMO, RAN4 shall focus on the legacy TDD pattern 7D1S2U. Furthermore, as both MCS 13 and MCS 22 are already used in the requirements for Rel-18 ATG in TS 38.101-4 [2], for DL MIMO, RAN4 to prioritize MCS 22 to show the benefit of having MIMO.

For DL MIMO, RAN4 to consider only the legacy TDD pattern 7DS2U and 64QAM MCS 22.

Conclusion

In this contribution, we provide our views on Rel-19 Enhancements for ATG network for NR. The following Observations and Proposals were made:

Proposal 1: For DL CA, RAN4 shall consider the legacy TDD pattern 7D1S2U only.

Proposal 2: For DL MIMO, RAN4 to consider only the legacy TDD pattern 7DS2U and 64QAM MCS 22.

- References

- R4-2512602, Way Forward for [116][326] NR_ATG_enh_demod, 3GPP RAN4#116, ZTE Corporation, CMCC, Bengaluru, India, August 25-29, 2025

- 3GPP TS 38.101-4, NR - User Equipment (UE) radio transmission and reception, Part 4: Performance requirements, V18.7.0, 03-2025

- R4-2510019, Discussion on demodulation requirements for Rel-19 ATG, 3GPP RAN4#116, CMCC, Bengaluru, India, August 25-29, 2025