3GPP TSG-RAN WG4 Meeting #116bis	R4-2514288

Prague, Czech Republic, October 13 – 17, 2025

Source:	Nokia

Title:	On the Evolution of NR Duplex Operation - Sub-Band Full Duplex

Agenda item:	6.13.4

Document for:	Discussion

Introduction

The way forward from Rel-19 Evolution of NR duplex operation: Sub-band Full Duplex (SBFD) demodulation performance discussions during RAN4#116 is documented in [1]. RAN4 has agreed to not define UE demodulation requirements for Rel-19 SBFD [1]. This agreement together with the issues on BS demodulation being discussed during RAN4#116 meeting are summarized below [1]:

Topic 1 UE demodulation requirement ### Sub-topic 1-1 General Scope Issue 1-1-1 whether to introduce UE demodulation with SBFD carrier (with DL sub-band and UL sub-suband configured) - Agreement - Not to introduce UE demodulation requirements with SBFD carrier (with DL sub-band and UL sub-band) ## Topic 2 BS demodulation requirement ### Sub-topic 2-1 General Scope Issue 2-1-1 Types of interference modeling for demodulation requirement with UL muting Issue 2-1-2 how to model interference modeling for performance evaluation Issue 2-1-3 Evaluation Metrics to choose interference model and make decision on requirement introduction Issue 2-1-4 Baseline Receiver Assumption ### Sub-topic 2-2 Test setup for demodulation requirement if introduced Issue 2-2-1 Frequency Range Issue 2-2-2 Waveform for wanted signal Issue 2-2-3 sub-band pattern Issue 2-2-4 Channel Bandwidth for wanted signal Issue 2-2-5 TDD pattern Issue 2-2-6 Sub-band configuration in SBFD slot Issue 2-2-7 Number of symbol muting Issue 2-2-8 Muting symbol configurations Issue 2-2-9 PUSCH mapping type for wanted signal Issue 2-2-10 Number of DMRS symbols for wanted signal Issue 2-2-11 Channel model Issue 2-2-12 MCS Issue 2-2-13 Antenna Configuration Issue 2-2-14 Other Assumption

As RAN4 already agreed to not specify UE demodulation requirements for Rel-19 SBFD. Hence, the discussions in RAN4#116bis will mainly on open issues concerning SBFD BS demodulation performance.

This contribution describes our views on those open issues for further discussions during RAN4#116bis meeting.

Discussion

In this section, we provide our views on the open issues concerning BS demodulation performance.

General scope

There are still crucial issues that need to be discussed in RAN4#116bis. The open issues on the general scope of BS demodulation to be addressed here are highlighted as follows [1]:

Issue 2-1-1 Types of interference modeling for demodulation requirement with UL muting - Agreement - Evaluate PUSCH performance with UL muting under inter-site gNB interference modelling - FFS on self-interference and intra-site inter sector interference Issue 2-1-2 how to model interference modeling for performance evaluation - Agreement - Considering TDL+INR type of interference model for evaluation - Note: the RF assumption can be considered for interference modelling - The following candidate value for INR can be considered for evaluation as starting point - Option 1: based on link budget analysis Conditions; FR1. Interf1 > Interf2; INR1 <= 17.2dB, INR2 <= 7.2dB Trial simulation could try following sets: [INR1, INR2] = {[5, -5], [9, -1], [13, 3], [17, 7]} dB. Interf1 = Interf2; INR1 = INR2 <=11.2dB Trial simulation could try following sets: [INR1, INR2] = {[-1, -1], [3, 3], [7, 7], [11, 11]} dB. Note: The interference wave form could use PUSCH with 1+1 DM-RS and CP-OFDM to simplify simulation setup.. - Other options are not concluded - Interesting companies can provide the system level simulation to derive the INR value Issue 2-1-3 Evaluation Metrics to choose interference model and make decision on requirement introduction - agreement - RAN4 can use the following metrics to choose interference model based on evaluation result as starting point - The peak throughput can be achieved - The obvious performance gain can be shown by comparing using muting Res for estimation to not using muted REs for estimation - The target SNR (e.g., SNR at70% Tput) should be feasible for both network deployment and tests - Based on the evaluation, RAN4 can further discuss whether to introduce the PUSCH requirement with UL muting Issue 2-1-4 Baseline Receiver Assumption - agreement - MMSE-IRC with the same assumption in Rel-19 performance enhanced WI

Interference modeling

Due to different transmitter and receiver imperfections, SBFD faces challenges from cross-link interference (CLI) between UEs and CLI between gNBs. Figure 1 illustrates the CLI types studied in RAN1 and RAN4, namely:

- gNB self-interference (1),

- gNB-to-gNB inter-sector (or intra-site) interference (2),

- gNB-to-gNB inter-site interference (3),

- UE-to-UE intra-cell interference (4), and

- UE-to-UE inter-cell interference (5).

Figure 1: CLI types present in SBFD

Concerning interference signals at the gNB, the modelling of CLI assumed during the Rel-18 Duplex Evolution SI is briefly described in the following, with primary focus on FR1 (more details (and on FR2-1) can be found in TR 38.858 [3]).

- gNB Self-interference modeling

The feasibility of SBFD was assessed by breaking down the self-interference into different components, such as transmit power, frequency isolation capability, spatial isolation, TX beam nulling and RF interference cancellation. The assessment was based on a framework for calculating the residual self-interference calculation (RSIC) under the assumption that the UL sensitivity degradation at the gNB receiver due to the self-interference is 1 dB. Candidate values of RSI were provided by RAN4 to RAN1, so that the desense target was also considered in the performance evaluation of SBFD.

- gNB Inter-sector interference

The objective of gNB inter-sector interference analysis was to identify the inter-sector spatial isolation values that would result in additional desensitization at the gNB receiver. RAN4 considered different factors in both the transmitter characteristics (e.g., tx power and beam nulling) and the BS deployment (e.g., antenna isolation and absorbent materials between the different sectors). gNB inter-sector spatial isolation values between 75 dB to 103 dB for FR1 were discussed, where the latter would also result in a negligible UL received desensitization (considering the effects of ACLR/ACS on top of the spatial isolation) even for high-power FR1 macro base station deployments.

- Inter-site gNB-to-gNB interference

The co-channel inter-sub-band inter-site interference analysis considered the unwanted emission impact on the receiver selectivity. The same transmitter leakage and receiver impairment model as for investigating the gNB self-interference was used. However, the antenna isolation was replaced with the inter-site isolation which can be derived using 3GPP propagation models (based on TR 38.901 [4]). The Tx leakage baseline studied was the gNB adjacent channel leakage ration (ACLR), and the receiver impairment was the gNB adjacent channel selectivity (ACS).

PUSCH requirements

In WF [1], it is stated that MMSE-IRC as in Rel-19 performance enhanced (that is Rel-19 Demodulation Phase 5 BS MMSE-IRC) WI is to be considered for Rel-19 SBFD. Consequently, a proper SBFD interference model should be used, since MMSE-IRC in Rel-19 performance enhanced WI is also using a proper interference model.

Compared to the legacy TDD operation, the PUSCH demodulation in an SBFD-capable gNB is affected by its own self-interference as well as the gNB-to-gNB CLI (cf. CLI types 1, 2, 3 in Figure 1). Concerning the gNB self-interference, on how to accommodate such an interference into the link-level simulation is not yet clear. As the transmit antennas and the receive antennas are close to each other, propagation conditions as in [4] may not be used as the propagation condition of the self-interference signal. Nonetheless, referring to [3] self-interference can be modelled as additive white gaussian noise with fixed INR = - 6 dB targeting 1 dB desense.

Concerning the inter-site gNB-to-gNB interference, as this is appearing in SBFD multi-cell scenarios, RAN4 needs to properly model it. It can be modelled similar to how RAN4 has modeled the interference in UL inter-cell interference scenario of LTE (cf. TR 36.884 [6]). In TR 36.884 [6], the UL inter-cell interference model of LTE is obtained from system-level simulations. Such extensive system-level simulations are needed to quantify the so-called Dominant Interference Proportion (DIP), which would then be used in the link-level simulation to define the corresponding demodulation requirements. Different to TR 36.884 for LTE, in SBFD case, the system-level simulation needs to consider the other gNBs from other cells as sources of interference instead of the UEs from other cells as in the LTE case. Moreover, different allocations (e.g., DUD vs DU/UD) in SBFD symbols/slots and different SBFD BW sizes among different cells should be considered.

While in [1] it is mentioned “Interesting companies can provide the system level simulation to derive the INR value”, it is important to note here that TR 36.884 was a result of multiple companies’ system-level simulation campaigns. Alignment on the simulation assumptions and results were made ensuring reliable and accountable outcomes. Furthermore, companies involved in the system-level simulation activities came from both the operator and network vendor sides. This allowed for a more realistic deployment scenario to be considered in the system-level simulation set-up. Hence, learning from the process resulting in TR 36.884, for SBFD case, SBFD BS interference modeling should take into account a realistic SBFD deployment scenario.

Even for the work of Rel-19 NR Demodulation Phase 5 BS MMSE-IRC, it was understood that proper DIP from system level is needed. Nonetheless, since the DIP from LTE in TR 36.884 [6] can still be used (as the source of interference in TR 36.884 [6] and in Rel-19 BS MMSE-IRC is the same), Rel-19 BS MMSE-IRC was reusing LTE’s DIP for defining the requirements. This is not the case for Rel-19 SBFD which source of interference is different than in LTE case, namely, the other gNB signals instead of other UEs in other cells.

In RAN4#115 offline discussion [10], while it is known that UE-to-UE CLI is indeed a source of interference in SBFD, it was not agreed to consider it due to the difficulties of modeling the UE-to-UE interference. Hence, no UE demodulation requirements are defined.  We note here that BS demodulation has even more complex interference to consider, namely, the self-interference, intra-sector gNB and inter-site gNB interference.

We further note that there is one contribution from multiple companies in RAN#109 which support Rel-20 SBFD. They proposed to have SBFD BS to SBFD BS adjacent channel coexistence focusing on realistic deployment scenarios for Rel-20 SBFD. This shows two important issues which not yet covered in Rel-19 SBFD, namely, SBFD BS to SBFD BS adjacent channel coexistence and realistic SBFD deployment scenario. RAN Plenary has approved a new Work Item (WI) for Re-20 in which SBFD is a part of it, namely, Rel-20 NR base station (BS) RF requirement evolution for FR1 and testing phase 2 WI [8]. Hence, for Rel-19 SBFD, RAN4 needs also to ensure that for demodulation performance requirements the interference modeling must consider a realistic deployment scenario. Otherwise, RAN4 shall not define demodulation performance requirements if the SBFD interference cannot be modelled.

In SBFD, the PUSCH demodulation at SBFD-capable gNB is affected by the gNB self-interference and the gNB-to-gNB CLI.

Referring to TR 38.858, self-interference can be modelled as additive white gaussian noise with fixed INR = - 6 dB targeting 1 dB desense.

To properly model the gNB-to-gNB interference signals, similar to TR 36.884, join efforts from multiple companies are needed for the system level simulation activities, with a realistic deployment scenario and further alignment of the results, to have a reliable and accountable DIP for SBFD.

RAN Plenary has approved a WI for Rel-20 SBFD to consider SBFD BS to SBFD BS adjacent channel coexistence focusing on realistic deployment scenario.

For SBFD UE demodulation, as UE-to-UE CLI was considered difficult to model, it was agreed not to define SBFD UE demodulation requirements.

If SBFD BS interference from a realistic deployment scenario cannot be properly modeled, RAN4 shall not define BS demodulation requirements.

Inclusion in Specification

The open issue concerning BS demodulation on this subtopic is [1]:

Issue 2-3-1 Specification Impact - Way forward - Option 1: New sub-clause for demodulation requirements under same chapter with RF requirements can be considered. If RAN4 agree Not to introduce new BS demodulation requirements for SBFD, only applicability rule can be captured. - Other options are not precluded

As the discussions on the PUSCH requirements is still ongoing, this specification impact can be treated as FFS.

RAN4 to keep the specification impact discussion as FFS.

Conclusion

In this contribution, we provide our views on demodulation performance requirements for Rel-19 Evolution of NR duplex operation: Sub-band Full Duplex (SBFD). The following Observations and Proposals were made:

Observation 1: In SBFD, the PUSCH demodulation at SBFD-capable gNB is affected by the gNB self-interference and the gNB-to-gNB CLI.

Observation 2: Referring to TR 38.858, self-interference can be modelled as additive white gaussian noise with fixed INR = - 6 dB targeting 1 dB desense.

Observation 3: To properly model the gNB-to-gNB interference signals, similar to TR 36.884, join efforts from multiple companies are needed for the system level simulation activities, with a realistic deployment scenario and further alignment of the results, to have a reliable and accountable DIP for SBFD.

Observation 4: RAN Plenary has approved a WI for Rel-20 SBFD to consider SBFD BS to SBFD BS adjacent channel coexistence focusing on realistic deployment scenario.

Observation 5: For SBFD UE demodulation, as UE-to-UE CLI was considered difficult to model, it was agreed not to define SBFD UE demodulation requirements.

Proposal 1: If SBFD BS interference from a realistic deployment scenario cannot be properly modeled, RAN4 shall not define BS demodulation requirements.

Proposal 2: RAN4 to keep the specification impact discussion as FFS.

- References

- R4-2512615, Way Forward for [116][328] NR_duplex_evo_demod, Samsung, 3GPP TSG RAN4#116, Bengaluru, India, August 23-29, 2025

- R4-2502310, Way Forward for [114][306] NR_duplex_evo_General, 3GPP TSG RAN4#114, Athens, GR, February 17 – 21, 2025

- 3GPP TR 38.858, Study on Evolution of NR Duplex Operation, V18.2.0, 12-2024

- 3GPP TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz, V18.0.0, 03-2024

- R4-2507626, On the Evolution of NR Duplex Operation - Sub-Band Full Duplex, Nokia, 3GPP TSG RAN4#115, Malta, May 19 – 23, 2025

- 3GPP TR 36.884, Performance requirements of MMSE-IRC receiver for LTE BS, V13.1.0, 09-2016

- 3GPP TS 38.101-1, User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone, V19.1.0, 03-2025

- RP-252905, New WID on NR base station (BS) RF requirement evolution for FR1 and testing phase 2, Huawei, 3GPP TSG RAN#109, Beijing, China, September 15-18, 2025

- RP-252691, Proposal for SBFD-SBFD adjacent channel co- existence scenario, China Unicom, et al., 3GPP TSG RAN#109, Beijing, China, September 15-18, 2025

- R4-2508721, Offline meeting minutes for [115][330] NR_duplex_evo_demod, Samsung, 3GPP TSG RAN4#115, Malta, May 19-23, 2025