# 3GPP TSG RAN WG4 Meeting #116	bis 														R4-2514300

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	6.17.2.1

Source: 	Ericsson

Title:	RRM performance requirements for event triggered L1 measurement reporting

Document for:	Discussion

# Introduction

In this contribution, we will address several aspects regarding the test case design for mobility RRM requirement.

# Discussion

## Event triggered L1 measurement reporting

Our understanding of the use case of the RAN2 introduced events is as follows:

- Event LTM2: As the name suggests this event is similar to event A2 for the event triggered L3 measurements. When the beam of the serving cell becomes worse than absolute threshold, NW may request an aperiodic report to schedule early TCI state activation or PDCCH order based RACH.

- Event LTM3: As the name suggests this event is similar to event A3 for the event triggered L3 measurements. When beam of candidate cell becomes amount of offset better than beam of serving cell, NW can trigger the cell switch based on the event triggered report from the UE.

- Event LTM4: As the name suggests this event is similar to event A4 for the event triggered L3 measurements. When beam of candidate cell becomes better than absolute threshold, based on the UE report, NW may activate the TCI states of the candidate cell or update the active TCI state list.

- Event LTM5: As the name suggests this event is similar to event A5 for the event triggered L3 measurements. When Beam of serving cell becomes worse than absolute threshold1 AND beam of candidate cell becomes better than another absolute threshold2, based on the UE report, NW may activate the TCI states of the candidate cell or update the active TCI state list or perform LTM cell switch.

These RAN2-agreed events are designed to request candidate cell Level 1 measurement reports to enable early DL/UL synchronization on candidate cells or trigger an LTM cell switch. We recommend testing at least one event intended for cell switching and one for early TCI state activation, specifically Event LTM3 and Event LTM4. To reduce the number of test cases, some are tested with Event LTM3 and some ae with Event LTM4.

- At least Event LTM3 and Event LTM4 be included in the RRM testcase. Some of the agreed test cases can be tested with Event LTM3 and some of the test cases with Event LTM4.

Test cases:

In last meeting following WF is agreed.

  - For SSB based L1 RSRP measurement reporting:

    - Approach 1: duplicate all R18 test cases and add event triggered reporting. UE which can pass R19 test cases can skip R18 test cases. (E///, Apple)

    - Approach 2: keep R18 test cases, develop necessary new R19 test to verify reporting behavior. (MTK, HW)

As discussed in the last meeting, if the UE supports both periodic, semi-persistent L1-RSRP for the LTM and event-triggered L1-RSRP for the LTM, we think it is sufficient to test the event-triggered reporting. This is because the UE needs to perform more frequent measurements for event-triggered reporting, and the associated requirements are more stringent for the UE to pass compared to Rel-18. Hence, to reduce the number of test cases burden on the UE, we suggest testing the Rel-19 LTM test cases and skipping Rel-18 for a UE that supports both rel-18 LTM L1-RSRP and Rel-19 event triggered L1-RSRP.

- If the UE supports both periodic, semi-persistent L1-RSRP for the LTM and event-triggered L1-RSRP for the LTM, it is sufficient to test the event-triggered reporting and skip the Rel-18 LTM L1-RSRP test cases.

List of test cases agreed in Rel-18:

| Intra-f L1-RSRP measurement for LTM | - Intra-f L1-RSRP measurement in FR1 - Single frequency layer - 1 serving cell, 2 neighbor cells |
| --- | --- |
|  | - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells, none of neighbor cells’ TCI state activated |
|  | - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells, one of neighbor cells’ TCI state activated |
| Inter-f L1-RSRP measurement with MG | - Inter-f L1-RSRP measurement with MG in FR1 - 2 neighbor cells |
|  | - Inter-f L1-RSRP measurement with MG in FR2 - 2 neighbor cells |
| Inter-f L1-RSRP measurement without gap for LTM | - Inter-f L1-RSRP measurement without gap in FR1 - Single frequency layer - 2 neighbor cells |
|  | - Inter-f L1-RSRP measurement without gap in FR2 - Single frequency layer - 2 neighbor cells, none of neighbor cells’ TCI state activated |

- Adopt the following list of test cases for Release 19 SSB-based event-triggered reporting.

| Intra-f L1-RSRP measurement for LTM | - Intra-f L1-RSRP measurement in FR1 - Single frequency layer - 1 serving cell, 2 neighbor cells |
| --- | --- |
|  | - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells, none of neighbor cells’ TCI state activated |
|  | - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells, one of neighbor cells’ TCI state activated |
| Inter-f L1-RSRP measurement with MG | - Inter-f L1-RSRP measurement with MG in FR1 - 2 neighbor cells |
|  | - Inter-f L1-RSRP measurement with MG in FR2 - 2 neighbor cells |
| Inter-f L1-RSRP measurement without gap for LTM | - Inter-f L1-RSRP measurement without gap in FR1 - Single frequency layer - 2 neighbor cells |
|  | - Inter-f L1-RSRP measurement without gap in FR2 - Single frequency layer - 2 neighbor cells, none of neighbor cells’ TCI state activated |

# Summary

The following have been proposed in the current contribution:

- At least Event LTM3 and Event LTM4 be included in the RRM testcase. Some of the agreed test cases can be tested with Event LTM3 and some of the test cases with Event LTM4.

- If the UE supports both periodic, semi-persistent L1-RSRP for the LTM and event-triggered L1-RSRP for the LTM, it is sufficient to test the event-triggered reporting and skip the Rel-18 LTM L1-RSRP test cases.

- Adopt the following list of test cases for Release 19 SSB-based event-triggered reporting.

| Intra-f L1-RSRP measurement for LTM | - Intra-f L1-RSRP measurement in FR1 - Single frequency layer - 1 serving cell, 2 neighbor cells |
| --- | --- |
|  | - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells, none of neighbor cells’ TCI state activated |
|  | - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells, one of neighbor cells’ TCI state activated |
| Inter-f L1-RSRP measurement with MG | - Inter-f L1-RSRP measurement with MG in FR1 - 2 neighbor cells |
|  | - Inter-f L1-RSRP measurement with MG in FR2 - 2 neighbor cells |
| Inter-f L1-RSRP measurement without gap for LTM | - Inter-f L1-RSRP measurement without gap in FR1 - Single frequency layer - 2 neighbor cells |
|  | - Inter-f L1-RSRP measurement without gap in FR2 - Single frequency layer - 2 neighbor cells, none of neighbor cells’ TCI state activated |

# References

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom, 3GPP TSG RAN #105, Melbourne, Australia, Sep 9-12, 2024.