3GPP TSG-RAN4 Meeting #116bis	R4-2514316

# Prague, Czech Republic, 13th Oct 2025 - 17th Oct 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR |  | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on HPUE MSD requirements simplification for TS38.101-3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm, Skyworks Solutions, Inc., LG Electronics, ZTE, Murata Manufacturing Co Ltd., Nokia, [T-Mobile USA, ZTE, Oppo, Samsung, MediaTek, [Vivo, Huawei, Apple] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | HPUE MSD requirements simplification was introduced into TS38.101-1 in CR R4-2511756. This CR adds similar simplification into TS38.101-3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following inter-band EN-DC HPUE MSD requirements are introduced in clause 7.3B.2.3.0: - New clause 7.3B.2.3.0.1 introduces the 1UL band 1UL CC MSD simplification look-up tables, - New clause 7.3B.2.3.0.2 introduces the 2UL band 2UL CCs with 1UL CC in each band MSD simplification look-up tables. Clause 7.3B.2.3.7. is updated accordingly |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inter-band EN-DC HPUE MSD simplification is not leveraged. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Beginning of the changes>

#### 7.3B.2.3	Inter-band EN-DC within FR1

##### 7.3B.2.3.0	General

Reference sensitivity exceptions are specified for the condition when there is uplink transmission only in the aggressor band.

#### 7.3B.2.3.0.1	PC2 and PC1.5 MSD requirements with look-up tables for two-band DL EN-DC with 1UL band single CC

The PC2 and the PC1.5 MSD requirements with look-up tables for two-band EN-DC reference sensitivity exceptions (MSD) due to 1UL band 1UL CC harmonic, harmonic mixing, and cross band isolation interference shall apply when the following criterias are met:

-		A PC3 reference sensitivity exception requirement is specified either in Table 7.3B.2.3.1-1, or in Table 7.3B.2.3.2-1 or in Table 7.3B.2.3.4-1, and

-		A PC2 reference sensitivity exception requirement is not specified respectively in Table 7.3B.2.3.2-1a, or in Table 7.3B.2.3.4-1a, and PC1.5 reference sensitivity exception requirement is not specified, and

-		PC2 is specified in Table 6.2B.1.3-1 and PC2 output power is specified as a valid per band power class in Table 6.2B.1.3-1a, denoted here “PC21Tx”, or PC2 is specified in Table 6.2H.1.3-1 and PC2 is specified as a valid per band power class in Table 6.2H.1.3-1a, denoted here “PC22Tx”, or PC1.5 is specified in Table 6.2H.1.3-1 and PC1.5 output power is specified as a valid per band power class in Table 6.2H.1.3-1a, denoted here “PC1.52Tx”, and,

-		The PCx aggressor UL band is the same aggressor UL band as in case of PC3 MSD, and,

-		The aggressor UL band is neither band n46, band B46, band n96 nor band n102.

For these cases, and where in the following PCx denotes either PC21Tx, PC22Tx or PC1.52Tx, the PCx MSD due to harmonic, harmonic mixing, and cross band isolation is specified as:

PCx MSD = PC3 MSD + MSD

where,

-		PCx MSD is the reference sensitivity exception specified for PC21Tx, PC22Tx, or PC1.52Tx,

-		PC3 MSD is the reference sensitivity exception specified for PC3 in Table 7.3B.2.3.1-1, or in Table 7.3B.2.3.2-1 or in Table 7.3B.2.3.4-1

-		MSD values are specified in Table 7.3B.2.3.0.1-1 output columns denoted “MSDmax 3, 6, 9”. These apply to the same uplink/downlink configurations as those specified for the minimum PC3 MSD requirements in Table 7.3B.2.3.1-1, or in Table 7.3B.2.3.2-1 or in Table 7.3B.2.3.4-1,

-		The correspondence between the MSDmax specified in Table 7.3B.2.3.0.1-1, the source of interference and PCx MSD is specified in Table 7.3B.2.3.0.1-2,

Table 7.3B.2.3.0.1-1: MSD per MSDmax look-up table for MSD due to harmonic, harmonic mixing, cross band isolation

| PC3 MSD (dB) | MSDmax / MSD(dB) |  |  |
| --- | --- | --- | --- |
|  | 3 | 6 | 9 |
| 0.1≤ PC3 MSD <1.0 | 0.7 | 1.1 | 2.5 |
| 1.0≤ PC3 MSD <3.0 | 1.6 | 2.7 | 5.1 |
| 3.0≤ PC3 MSD <5.0 | 2.0 | 3.8 | 6.5 |
| 5.0≤ PC3 MSD <7.0 | 2.3 | 4.5 | 7.3 |
| 7.0≤ PC3 MSD <9.0 | 2.5 | 5.0 | 7.9 |
| PC3 MSD ≥9.0 | 3 | 6 | 9 |

Table 7.3B.2.3.0.1-2: MSDmax correspondence look-up table for source of interference and PCx MSD

| Source of interference | MSDmax |  |  |
| --- | --- | --- | --- |
|  | PC21Tx MSD | PC22Tx MSD | PC1.52Tx MSD |
| UL harmonic | 3 | 6 | 9 |
| Harmonic mixing | 3 | 6 | 9 |
| Cross band isolation | 3 | 6 | 9 |

As an exception, for cases where:

-		The PC21Tx MSD is specified in Table 7.3B.2.3.2-1a or in Table 7.3B.2.3.4-1a, and

-		The PC3 MSD is not specified in Table 7.3B.2.3.2-1, or in Table 7.3B.2.3.4-1, and

-		The PC1.5 MSD is not specified, and

-		PC1.5 is specified in Table 6.2H.1.3-1 and PC1.5 output power is specified as a valid per band power class in Table 6.2H.1.3-1a, denoted here as “PC1.52Tx”, and,

-		The aggressor UL band is neither band n46, band B46, band n96 nor band n102,

then the PC1.52Tx MSD is specified as:

PC1.52Tx MSD = PC21Tx MSD + MSD,

where in the Table 7.3B.2.3.0.1-1,

-	MSD is specified output column denoted “MSDmax 6”,

-	The input column uses the specified “PC21Tx MSD” instead of “PC3 MSD”. These apply to the same uplink/downlink configurations as those specified for the minimum PC2 MSD requirements in Table 7.3B.2.3.2-1a or in Table 7.3B.2.3.4-1a.

#### 7.3B.2.3.0.2	PC2 and PC1.5 MSD requirements with look-up tables for two-band or three-band DL EN-DC with two-band UL EN-DC

The PC2 and the PC1.5 MSD requirements with look-up tables for two-band and three-band DL EN-DC reference sensitivity exceptions (MSD) due to 2UL EN-DC intermodulation interference shall apply when the following criterias are met:

-	A PC3 reference sensitivity exception requirement is specified either in Table 7.3B.2.3.5.1-1, or in Table 7.3B.2.3.5.2-1, and,

-	A PC2 reference sensitivity exception requirement is not specified respectively in Table 7.3B.2.3.5.1-1a or in Table 7.3B.2.3.5.2-1a and PC1.5 reference sensitivity exception requirement is not specified, and,

-	PC2 or PC1.5 two-band UL EN-DC for a total of 2Tx or 3Tx and 1CC in each UL band is specified as a valid two-band UL EN-DC configuration in Table 6.2B.1.3-1, or in Table 6.2H.1.3-1, and,

-	The PC2 or PC1.5 MSD is caused by the same uplink/downlink configurations as in case of PC3 MSD, and,

-	One of the constituent uplink band is neither band n46, Band B46, band n96 nor band n102.

For these cases, and where in the following PCx denotes either PC2 or PC1.5, the PCx MSD due to 2UL EN-DC intermodulation interference is specified as:

PCx MSD = PC3 MSD + MSD

where,

-	PCx MSD is the reference sensitivity exception specified for PC2 or PC1.5 with 2UL band EN-DC for a total of 2Tx or 3Tx and with 1UL CC in each UL band,

-	PC3 MSD is the reference sensitivity exception specified for PC3 in Table 7.3B.2.3.5.1-1, or in Table 7.3B.2.3.5.2-1,

-	MSD values are specified in Table 7.3B.2.3.0.2-1 output columns denoted “MSDmax 6, 9, 12, 15, 18, 24, 30”. These apply for the same uplink/downlink configurations as those specified for the minimum PC3 MSD requirements in Table 7.3B.2.3.5.1-1, or in Table 7.3B.2.3.5.2-1, and,

-	The correspondence between the MSDmax specified in Table 7.3B.2.3.0.2-1, the IMD order and PCx MSD is specified in Table 7.3B.2.3.0.2-2,

Table 7.3B.2.3.0.2-1: MSD per MSDmax look-up table for MSD due to 2UL EN-DC intermodulation interference

| PC3 MSD (dB) | MSDmax / MSD(dB) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 6 | 9 | 12 | 15 | 18 | 24 | 30 |
| 0.1≤ PC3 MSD <0.5 | 1.2 | 2.4 | 3.9 | 5.8 | 7.9 | 13.2 | 18.8 |
| 0.5≤ PC3 MSD <1.0 | 1.7 | 3.5 | 5.4 | 7.5 | 10.0 | 15.6 | 21.4 |
| 1.0≤ PC3 MSD <2.0 | 2.8 | 4.8 | 7.1 | 9.6 | 12.4 | 18.0 | 24.1 |
| 2.0≤ PC3 MSD <3.0 | 3.5 | 5.7 | 8.2 | 10.9 | 13.7 | 19.5 | 25.5 |
| 3.0≤ PC3 MSD <4.0 | 3.9 | 6.3 | 8.9 | 11.7 | 14.5 | 20.5 | 26.5 |
| 4.0≤ PC3 MSD <5.0 | 4.3 | 6.8 | 9.4 | 12.4 | 15.2 | 21.2 | 27.2 |
| 5.0≤ PC3 MSD <6.0 | 4.5 | 7.1 | 9.9 | 12.8 | 15.8 | 21.7 | 27.7 |
| 6.0≤ PC3 MSD <7.0 | 4.8 | 7.6 | 10.4 | 13.3 | 16.2 | 22.3 | 28.2 |
| 7.0≤ PC3 MSD <8.0 | 5.0 | 7.8 | 10.7 | 13.6 | 16.6 | 22.5 | 28.5 |
| 8.0≤ PC3 MSD <9.0 | 5.2 | 8.1 | 11.0 | 14.0 | 16.9 | 22.8 | 28.8 |
| PC3 MSD ≥9.0 | 6 | 9 | 12 | 15 | 18 | 24 | 30 |

Table 7.3B.2.3.0.2-2: MSDmax correspondence look-up table for IMD order and PCx MSD

| IMD order | MSDmax |  |
| --- | --- | --- |
|  | PC2 MSD | PC1.5 MSD |
| IMD2 | 6 | 12 |
| IMD3 | 9 | 18 |
| IMD4 | 12 | 24 |
| IMD5 | 15 | 30 |

As an exception, for cases where:

-	The PC2 MSD is specified in Table 7.3B.2.3.5.1-1a or in Table 7.3B.2.3.5.2-1a, and,

-	The PC3 MSD is not specified in Table 7.3B.2.3.5.1-1or in Table 7.3B.2.3.5.2-1, and,

-	The PC1.5 MSD is not specified, and,

-	PC1.5 two-band UL EN-DC for a total of 2Tx or 3Tx and 1CC in each UL band is specified as a valid two-band UL EN-DC configuration in Table 6.2B.1.3-1, or in Table 6.2H.1.3-1, and,

-	One of the constituent uplink band is neither band n46, band B46, band n96 nor band n102,

then the PC1.5 MSD is specified as:

PC1.5 MSD = PC2 MSD + MSD,

where,

-	In the Table 7.3B.2.3.0.2-1, MSD is specified with output columns denoted “MSDmax 6, 9, 12, 15” and where the input column uses the specified PC2 MSD specified in Table 7.3B.2.3.5.1-1a or in Table 7.3B.2.3.5.2-1a instead of the PC3 MSD, and

-	In the Table 7.3B.2.3.0.2-2, the correspondence between the MSDmax and the IMD order is specified using the column specified for “PC2 MSD”, and

-	These PC1.5 MSD requirements apply for the same uplink/downlink configurations as those specified in the PC2 MSD requirements of Table 7.3B.2.3.5.1-1a or in Table 7.3B.2.3.5.2-1a.

In all cases, the MSD requirements specified in Table 7.3B.2.3.0.2-1 and in Table 7.3B.2.3.0.2-2 do not apply to 2UL band EN-DC configurations with 3UL CCs, e.g. a combination of intra-band and inter-band dual connectivity.

## <Unchanged sections omitted>

#### 7.3B.2.3.7	Lower-MSD requirements for inter-band EN-DC within FR1

A UE can report better MSD performance than the minimum requirements as specified in clause 7.3B.2.3.1, 7.3B.2.3.2, 7.3B.2.3.4,7.3B.2.3.5, 7B.2.3.2.0.1, and 7B.2.3.2.0.2 by lowerMSD-r18 capability, except that the reporting for MSD caused by IMD with order higher than 5, IMD of UL intra-band CA or triple-beat is not supported in this release of the specification. The MSD performance after improvement is categorized into different lower-MSD capability classes, which are defined in Table 7.3A.7-1 of TS 38.101-1 [2].

The reported lower-MSD capability classes are subject to the same uplink/downlink configurations as defined for the minimum MSD requirements in clause 7.3B.2.3, 7B.2.3.2.0.1, and 7B.2.3.2.0.2. If a UE can support more than one test points for a given REFSENS exception case, the reported lower-MSD capability class is applicable for the test point having the largest specified MSD value. Otherwise, it’s only applicable for the test point which can be supported by the UE. If one or multiple power classes are requested by the network, the UE can, if supported, report lowerMSD-r18 capability for the requested power classes; otherwise, the UE shall report lowerMSD-r18 capability for the highest supported power class for the given DC configuration.

The UE shall meet one of the following conditions in order to report lowerMSD-r18 capability for a given REFSENS exception case:

- If the specified minimum requirement is tightly bounded by the range of a lower-MSD capability class (i.e, Thresholdi-1 < MSD ≤ Thresholdi, where i and (i-1) are two adjacent lower-MSD capability classes), the actual MSD shall be at least one-level lower (i.e., actual MSD ≤ Thresholdi-1); or

- If the specified minimum requirement is larger than the maximum threshold (corresponding to lower-MSD capability class VIII), the actual MSD shall be no more than the maximum threshold.

Otherwise, the UE shall not report lowerMSD-r18 capability for this REFSENS exception case.

If the special MSD type “ALL” is indicated in the lowerMSD-r18 capability, the reporting conditions as specified above shall be met for each MSD type that has been specified in this release for the given DC configuration.

NOTE 1: 	The lowerMSD-r18 capability is verified by reusing the MSD test point parameters and only replacing the minimum MSD requirement value by the threshold of the reported lower-MSD capability class. UE supporting lower MSD shall indicate the lower MSD capability for the requested power class if supported. If no power class is explicitly requested, the UE supporting lower MSD shall indicate the lower MSD capability for the highest supported power class of the band combination including victim band and aggressor band(s).. And, similar to the specified MSD minimum requirements, only the highest supported power class or the power class required by the certification/regulation body per UL configuration is verified.

NOTE 2: If the UE is equipped with four or eight Rx antenna ports for the victim band of the BC, the lowerMSD-r18 capability is verified with four or eight Rx antenna ports according to clause 7.1 under the condition mentioned above, but with the increased MSD values by the absolute value of ΔRIB,4R or ΔRIB,8R applied for the requirement based on the description in clause 7.3B.1.

## <End of the changes>