3GPP TSG RAN4 Meeting #116bis		R4-2514337
Prague, Czech Republic, 13th – 18th October, 2025

Source:	Nokia

Title:	Adopting the new channel model with satellite movements into RRM test cases

Agenda item:	6.6.3.2

Document for:	Discussion

Introduction

In RAN #101, RAN plenary has approved a simplification of the test framework for NGSO, amidst concerns of the time to complete the work item in RAN5 by the end of the Release 17. As a consequence, the specification work in RAN4 has concluded that the test cases for NGSO scenarios would be tested against a fixed (non-varying) propagation delay and fixed doppler delay.

As the speed of satellite in NGSO scenarios is a very relevant aspect of the NTN scenario that has not been covered by similar testing before and considering that NTN is a growing and relevant market, RAN has approved the WID RP-241985 [1] (NR_IoT_NTN_req_test_enh) to develop more meaningful and relevant test cases for this feature considering the motion trajectory of the satellite.

The following objectives were established for this WI:

NTN testing for NGSO - Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios - Inform RAN5 to assist specifying RF frequency error tests, if needed - Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements - Specify the applicability of the tests under the TE-emulated channel model

In the present meeting, RAN4 has to discuss the adaptation of the RRM specifications in TS 38.133 to encompass the new channel model to the test cases.

In this contribution we will discuss the open issues identified in the current WF [2].

Discussion

Applicability of the new channel model

The first point to be discussed in this document is related to the scope of changes of the test cases to adopt the new test cases. In the previous meeting, some companies presented their views relative to this matter and there are currently two main views. Some companies believe that the scope of the new channel model shall be limited to the new test cases. Other companies believe it shall be applied to all test cases.

In our view, implementing the new channel model to all test cases will not add significant workload, as the channel model has already being developed by RAN4 in previous meetings, it is just a matter of enabling the configuration in all test cases. Besides, there are the following advantages of doing so:

- It is economically wise for the UE implementation, as there will not be the need to create a new “test mode” in the UE chipset for enabling the NTN test cases

- RRM specs are already prepared for a non-static channel model.

- Minimum specification effort

- Consistency across all test cases and more meaningful tests that approximate the scenarios closer to the real market applications

Based on all the points above we propose:

Apply the channel model to all test cases in RRM.

Applicatility of the new TE-emulated channel model for pre-R19 UE

RAN4 has discussed whether the new channel model could be applicable to pre-Rel 19 UEs, and achieved the following WF:

WF: FFS on - Proposal 1: - For the pre-R19 UE, if the new TE-emulated channel model defined in Rel19 available for DUT, the test cases defined before Rel19 can be tested under these new channel model instead of the static channel. - Proposal 1a: - The satellite motion-based time varying channel model can be retroactively applied to pre-Rel-19 test cases subject to DUT declaration

We see the intent of the proposal as beneficial and also striving to achieve the economic benefits of not needing to implement a test mode in such UEs. Therefore, we support such proposals.

The satellite motion-based time varying channel model can be retroactively applied to pre-Rel-19 test cases subject to DUT declaration

Testing requirements

In what regards, the transmit timing test case, there are the following proposals to be evaluated in RAN4, according to the WF:

WF: FFS on: - Proposal 1 (Samsung): - For the timing advance test, FFS on whether addition margin should be added in the test requirements. - Proposal 2 (Samsung, Huawei): Clarify that how to calculated the requirements for NR NTN timing test: - Option 1a (Samsung): NTA,adjUE in the test requirements is calculated by UE itself. - Option 1b (Huawei): NTA,adjUE in test requirements is calculated based on the generated UL channel with time varying Doppler and delay shifts

In what regards proposal 1, we do not agree that a new margind should be added to the test case, there is already a test case margin provided in there. The discussion is whether this margin should be entirely removed. Besides, RAN4 has agreed that the tolerance of the test cases might be left for RAN5.

And in what regards proposal 2, it seems straightforward to us that the UE shall perform the timing pre-compensation.

NTA,adjUE in test requirements is calculated based on the generated UL channel with time varying Doppler and delay shifts

Do not add new additional margins in the test case. Test tolerance is left for RAN5.

Conclusion

In this paper, we provided our views on the performance test cases utilizing the new channel model that includes satellite movement. The following proposals were made:

Proposal 1: Apply the channel model to all test cases in RRM.

Proposal 2: The satellite motion-based time varying channel model can be retroactively applied to pre-Rel-19 test cases subject to DUT declaration

Proposal 3: NTA,adjUE in test requirements is calculated based on the generated UL channel with time varying Doppler and delay shifts

Proposal 4: Do not add new additional margins in the test case. Test tolerance is left for RAN5.

- References

- RP-241985, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, Samsung, RAN #105, Melbourne, Australia, September 9-12, 2024+

- R4-2512120, “WF on RRM requirements for NR_IoT_NTN_req_test_enh”, Moderator (Xiaomi), Bengaluru, India, August 25th – 29th, 2025