3GPP TSG-RAN WG4 Meeting #116bis	R4-2514352

Prague, Czech Republic, October 13th – October 17th, 2025

Source:	Nokia

Title:	RRM core requirements and testing framework for AI/ML positioning accuracy enhancement

Agenda item:	6.11.5

Document for:	Discussion

Introduction

The work item on artificial intelligence (AI)/machine learning (ML) for NR air interface (NR_AIML_Air) approved at RAN#102, was revised at RAN#108 [1] to refine the normative work of the general AI/ML framework for air interface and to enable the recommended use cases as depicted in the TR for the preceding study [2].

In this paper, open issues on RRM core and performance requirements for UE-based positioning (Case 1) and NG-RAN node assisted positioning (Cases 3a and 3b) from RAN4#115 are discussed and corresponding proposals are made.

Discussion

The open issues related to Cases 1, 3a and 3b are discussed in this section according to RAN4#115 WF [3] and Topic summary [4].

Requirements for Case1

This section discusses the requirements for the use cases with a UE-side model.

The following agreements were reached for Case 1 at RAN4#115 [3].

Issue 3-1: Requirements for case 1 Agreement: - Discuss how to define a measurement delay requirement (reporting delay) - Do not define any positioning accuracy requirement

The following agreements were reached for Case 1 at RAN4#116 [4].

Issue 3-1: Reporting Delay Requirements for case 1 Agreement: Take the framework of the existing reporting delay requirements - Reporting delay includes measurement delay, inference delay and the time needed until the UE send the report - Check until next meeting on the number of samples needed for the measurements

Hence RAN4 needs to investigate how to define the measurement delay requirement for the reported quantity in Case 1, i.e. the reporting delay for the reported UE location while no positioning accuracy requirements will be specified.

Measurement delay requirements

This paper proposes a framework for defining reporting delay requirements for AI/ML-based UE-assisted positioning methods in 3GPP NR by reusing the legacy (non-AI/ML) reporting delay structure defined in TS 38.133. The goal is to ensure consistency, interoperability, and feasibility in standardization while accommodating the characteristics of AI/ML-based inference mechanisms. We outline the components of reporting delay, compares legacy and AI/ML-based methods, and discusses implications for measurement sampling and system performance.

In Case 1, the reporting delay for AIML positioning is defined as the time from the measurement trigger to the transmission of the report. It includes:

- Measurement Delay(Tmeas): Time taken by the UE to perform the required measurements.

- Inference Delay (Tinf): Time taken to process or infer positioning-related data.

- Report sending/scheduling Delay (Treport,LPP): Time taken to prepare and transmit the report to the network/LMF.

In legacy UE-assisted positioning methods, the reporting delay consists solely of the time required for the UE to perform radio measurements and transmit the results to the network. This is because the UE does not perform significant computational processing or inference; instead, the network is responsible for interpreting the measurements and estimating the UE’s position. Consequently, the computational delay component is effectively absent in the legacy framework. This architectural separation simplifies the UE’s role and ensures that the reporting delay remains tightly bounded, typically within a predefined latency budget.

This legacy structure serves as a baseline for evaluating the feasibility of integrating AI/ML-based positioning methods, where inference functionality may shift to the UE side. According to the Measurement Reporting Requirements for UE-assisted positioning, the measurement reporting delay is defined as the time between the triggering of a periodic measurement report and the moment the UE starts to transmit the report over the air interface. Also, when UE is in the RRC_ IDLE state, prior to transmission, an additional reporting delay is considered for the UE to transition to the RRC_CONNECTED state.

Therefore, in Case 1 of AI/ML-based positioning, a similar definition can be applied to Positioning Reporting Requirements. Here, the positioning reporting delay is defined as the time between the triggering of a periodic measurement report and the moment the UE is ready to transmit the position report over the air interface. As in the legacy case, the UE must transition to the RRC_CONNECTED state before transmitting the report when the UE is in the RRC_IDLE state.

For Case 1, RAN4 should define the “Positioning Reporting Requirement” as follows:

- Change #1:

#### 9.9.X Positioning Reporting Requirements for UE-based direct AI/ML Positioning The positioning reporting delay is defined as the time between the moment when the periodic measurement report is triggered and the moment when the UE is ready to transmit the positioning report over the air interface. This requirement assumes that the positioning report is not delayed by other LPP signalling on the DCCH. This positioning reporting delay excludes a delay uncertainty resulted when inserting the positioning report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the positioning report is transmitted on the PUSCH with subframe or slot or subslot duration. This position reporting delay excludes any delay caused by no UL resources for UE to send the measurement report. When receiving a RequestLocationInformation message from the LMF via LPP [22, 34], the UE shall be able to send the positioning response, defined in [34], from configured PRS resources for configured TRPs on configured positioning frequency layers, within Treport, total=Tmeas+Tinf+Treport, LPP, where Tmeas is the time taken by the UE to perform the required measurements, Tinf is the time taken to process or infer positioning-related data, and Treport, LPP is the time taken to prepare and transmit the report to the network.

- Change #2:

#### 4.5.X Positioning Reporting Requirements for UE-based direct AI/ML Positioning The positioning reporting delay is defined as the time between the moment when the periodic measurement report is triggered and the moment when the UE is ready to transmit the measurement report over the air interface. The UE will transition to RRC_CONNECTED state prior to transmitting the positioning report. For positioning performed by a UE in RRC_IDLE state, the positioning reporting delay excludes all of the following: - additional delay caused other LPP signalling on the DCCH, - delay uncertainty introduced when inserting the measurement report in the TTI of the uplink DCCH, equal to 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration, - any delay caused by unavailability of UL resources to transmit the measurement report, - the time needed to transition to RRC_CONNECTED state to report the measurements. If a positioning measurement is performed in RRC_IDLE state, when receiving a RequestLocationInformation message from the LMF via LPP [34], the positioning reporting requirements defined in clause 9.9.X is re-used for the positioning reporting delay requirement.

Positioning latency should be lower than the positioning reporting requirement.

RAN4 has agreed not to define accuracy requirements for UE-assisted positioning. As a result, any test case would be considered passed by any UE, rendering the concept of a meaningful test case infeasible. Without accuracy benchmarks, the validity and utility of such tests are fundamentally compromised. Therefore, we recommend that for Case 1, RAN4 should refrain from defining any test cases related to the “Position Reporting Requirement,” as they would lack evaluative significance.

For Case 1, RAN4 should not define any test case for “Position Reporting Requirement”.

Conclusion

In this paper we share our views on potential RAN4 impacts from issues related to AI/ML based positioning. In the paper, the following observations and proposals were made:

Case 1

- For Case 1, RAN4 should define the “Positioning Reporting Requirement” as follows:

- Change #1:

#### 9.9.X Positioning Reporting Requirements for UE-based direct AI/ML Positioning The positioning reporting delay is defined as the time between the moment when the periodic measurement report is triggered and the moment when the UE is ready to transmit the positioning report over the air interface. This requirement assumes that the positioning report is not delayed by other LPP signalling on the DCCH. This positioning reporting delay excludes a delay uncertainty resulted when inserting the positioning report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the positioning report is transmitted on the PUSCH with subframe or slot or subslot duration. This position reporting delay excludes any delay caused by no UL resources for UE to send the measurement report. When receiving a RequestLocationInformation message from the LMF via LPP [22, 34], the UE shall be able to send the positioning response, defined in [34], from configured PRS resources for configured TRPs on configured positioning frequency layers, within Treport, total=Tmeas+Tinf+Treport, LPP, where Tmeas is the time taken by the UE to perform the required measurements, Tinf is the time taken to process or infer positioning-related data, and Treport, LPP is the time taken to prepare and transmit the report to the network.

- Change #2:

#### 4.5.X Positioning Reporting Requirements for UE-based direct AI/ML Positioning The positioning reporting delay is defined as the time between the moment when the periodic measurement report is triggered and the moment when the UE is ready to transmit the measurement report over the air interface. The UE will transition to RRC_CONNECTED state prior to transmitting the positioning report. For positioning performed by a UE in RRC_IDLE state, the positioning reporting delay excludes all of the following: - additional delay caused other LPP signalling on the DCCH, - delay uncertainty introduced when inserting the measurement report in the TTI of the uplink DCCH, equal to 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration, - any delay caused by unavailability of UL resources to transmit the measurement report, - the time needed to transition to RRC_CONNECTED state to report the measurements. If a positioning measurement is performed in RRC_IDLE state, when receiving a RequestLocationInformation message from the LMF via LPP [34], the positioning reporting requirements defined in clause 9.9.X is re-used for the positioning reporting delay requirement.

Positioning latency should be lower than the positioning reporting requirement.

For Case 1, RAN4 should not define any test case for “Position Reporting Requirement”.

- References

- RP-251186, Revised WID on AI/ML for NR air interface, RAN#108, Prague, Czech Republic, June 9-13, 2025

- 3GPP TR 38.843 V18.0.0, Technical Report (TR): Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- R4-2508080, WF on NR_AIML_air_part1, Qualcomm, RAN4#115

- R4-2511890, WF on AI/ML for air interface for [116][131], Qualcomm, RAN4#116