3GPP TSG-RAN4 Meeting #116bis		R4-2514457

# Prague, Czech Republic 13th – 17th October, 2025

Source: 	Qualcomm Incorporated

Title: 	RRM Core Requirements for AI/ML Beam Management

Agenda item:	6.11.4

Document for:	Discussion

# Introduction

This paper focuses on RRM core requirements for AI/ML based beam management.

# Discussion

## 2.1 Beam ID Prediction Accuracy Metric

RAN4 made the following agreements during the last couple of meetings.

RAN4 114 Agreement If RSRP and beam ID are reported: - RSRP accuracy will be one of the KPIs. - How option 1 is applied is FFS - Beam ID prediction accuracy will be one of the KPIs - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options If only beam ID is reported: - Beam ID prediction accuracy will be used as the KPI FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options RAN4 114bis Agreement - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

The beam ID prediction accuracy prediction related metric of RAN4 114bis kept multiple parameters and aspects FFS. They are:

- Whether the metric will evaluate the beam ID prediction accuracy, in terms of ground truth RSRP, of more than one beam (i.e., N > 1)

- X dB, the margin between the maximum ground truth RSRP of K predicted beams and the ground truth RSRP of the strongest genie-aided beam

- Value of K, number of beams that are predicted by UE and reported to network

Evaluating the beam ID prediction accuracy of more than one beam could lead to multiple different metrics.

- Let us assume, K = 4, N = 2, M = 5 and X = 0 dB. Then the metric would check if any two of the top 4 predicted beams are contained among the five strongest genie aided beams.

- Let us assume, K = 4, N = 2, M = 5 and X = 3 dB. Then the metric would check if the ground truth RSRP of any two of the top 4 predicted beams are within 3 dB of ground truth RSRP of any two of the five strongest genie aided beams.

- In another interpretation, let’s assume, K = 4, N = 2 and X = 3 dB. Then the metric could check if the ground truth RSRP of the strongest and 2nd strongest of the K predicted beams are within 3 dB of the ground truth RSRP of the genie aided strongest and 2nd strongest RSRP beams respectively.

  - Note: In above interpretation, strongest is defined based on ground truth RSRP of a beam.

The proponents of N > 1 primarily mention two arguments. First, they mention that the current version of the agreement only checks if one of top K predicted beams is within X dB margin, in terms of ground truth RSRP, of the genie aided strongest beam. Network does not know which of the top K predicted beams is within X dB margin of the genie aided strongest beam. Hence, network cannot automatically select a predicted beam for TCI state switching.

Second, N = 1 gives network idea about UE’s capability to predict one strong beam ID. If N = 1, network does not know if UE can predict more than one strong beam.

Observation 1: The FFS of N > 1 in beam ID accuracy prediction metric can be expanded to different metrics.

- Potential metric 1: The metric could check if N out of top K predicted beams are contained among the M strongest genie aided beams.

- Potential metric 2: The metric could also check if any N out of top K predicted beams are within X dB of any two of the M strongest genie aided beams.

- Potential metric 3: The metric could also check if the ground truth RSRP of top N predicted beams are within X dB of the ground truth RSRP of the two genie aided strongest beams.

Observation 2: The proponents of N > 1 claim that the current version of the agreement only checks if one of top K predicted beams is within X dB margin, in terms of ground truth RSRP, of the genie aided strongest beam. Network does not know which of the top K predicted beams is within X dB margin of the genie aided strongest beam.

Observation 3: The proponents N > 1 also claim that it gives network idea about UE’s capability to predict and report multiple strong beam IDs.

Observations 2 and 3, the arguments of N > 1, have some limitations. First, as long as K > 1 and K is not equal to N, network will have to transmit the top-K predicted beams in a 2nd round of beam sweep and collect UE’s report before changing active TCI state. Second, network and UE are not expected to see two many strong beams in FR2. Load balancing is not essential in existing FR2 deployments. Hence, RAN4 can just make K = N = 1 and play with the value of X during performance requirements based on UE or chip vendors’ inputs.

Observation 4: If K > 1, network must transmit the top-K predicted beams in a 2nd round of beam sweep and collect UE’s report regarding the strongest beam among top-K predicted beams before changing active TCI state.

Observation 5: Currently agreed procedures do not allow network to dynamically configure (e.g., via DCI or MAC-CE) the 2nd round of beam sweep based on UE’s inference results regarding top-K predicted beams.

Observation 6: The issue of 2nd round of beam sweep, mentioned in observation 4 and 5, can be avoided by making K = 1.

Observation 7: In FR2, networks typically cannot communicate with multiple UEs via the same beam. The knowledge of the 2nd strongest, defined via ground truth RSRP, predicted beam may not be essential for the network.

Proposal 1: Update the previous agreement with the following changes:

- “The successful rate for the correct prediction,

  - The correct prediction is considered as maximum ground-truth RSRP among of top-K1 predicted beams being larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB,

  - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation

    - FFS on N values

  - FFS on x values

- Note: if x=0 and N=1, it means Top-K1/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K1 predicted beams"

## 2.2. Definition of Relative Accuracy

During the last meeting, RAN4 decided to define requirements for relative RSRP accuracy through the following agreement.

Issue 2-9: Metrics/KPIs for beam ID prediction Agreement: - If RSRP prediction is reported, absolute/relative RSRP requirements applies to - reported beams where SNR side condition is met - The exact RSRP reporting including both absolute and relative will be defined in RAN1 - The related test can only apply to the top-K, where the value of K is FFS, beams. - The legacy beam management test and parameters can be taken as the reference to decide the exact value of K - The feasibility of the test will be further discussed and decided.

RAN4 defined relative RSRP accuracy in the following way in a previous meeting:

Previous Agreement: - RAN4 to use the follow as baseline for further discussions, if the relative RSRP accuracy requirement is defined - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]. - Reported L1-RSRP can be predicted, - FFS whether reported L1-RSRP can be measured RSRP - Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2

Above agreement contains two FFS points.

- First, it is open if reported L1-RSRP of beam index n can be measured L1-RSRP, too.

  - Note: Beam index n owns the largest reported L1-RSRP value.

- Second, it is open if the above definition applies for spatio-temporal beam prediction.

First, in this work item, UE measures properties of setB beams and reports properties of top beams of setA beams. SetB can be a subset of setA, e.g., in narrow-to-narrow beam prediction scenario, and the beam index n owning the largest reported L1-RSRP value can be an element of setB. In this scenario, the reported L1-RSRP of beam index n can come from measured L1-RSRP. However, UE still reports it as predicted L1-RSRP of beam index n. It does not matter whether UE calculates this predicted L1-RSRP of beam index n based on measured L1-RSRP of the same beam. Hence, the definition of relative L1-RSRP accuracy can simply focus on “predicted L1-RSRP” and remove the “measured RSRP” related phrase.

The above definition of relative L1-RSRP accuracy can apply to spatio-temporal beam prediction, in general. However, the key question is whether the reported L1-RSRP across two different time instances will be considered in the definition of spatio-temporal beam prediction.

In spatio-temporal beam prediction, UE predicts and reports properties (L1-RSRP and beam ID) of predicted beams for up to eight prediction instances. Channel may vary significantly across these eight prediction instances. Hence, it would not make sense to compare reported L1-RSRP across two different prediction instances to define requirement. So, the previous definition of relative L1-RSRP accuracy can be applied to case 2 where predicted and ground truth L1-RSRP of both beam index i and n are obtained at the same time instance.

Observation 8: Beam index n, owning the largest reported value, can be an element of setB. However, UE reports it as predicted L1-RSRP while reporting it as part of a top beam of setA. It does not matter whether the reported and predicted L1-RSRP of this beam is exactly equal to the measured L1-RSRP of this beam.

Observation 9: Channel may vary significantly across different prediction time instances. It is not meaningful to compare relative accuracy of reported L1-RSRP across two different prediction time instances.

Proposal 2: RAN4 uses the agreed baseline for relative L1-RSRP of case 1 with the following changes:

  - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i – reported predicted L1-RSRP of beam index n) -  (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value].

    - Reported L1-RSRP can be predicted,

    - FFS whether reported L1-RSRP can be measured RSRP

Proposal 3: Apply proposal 3 to case 2 where predicted and ground truth L1-RSRP of both beam index i and n correspond to the same time instance. In other words, define relative L1-RSRP accuracy in the following way for case 2:

  - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i at time instance t –  predicted L1-RSRP of beam index n at time instance t) -  (ground truth of L1-RSRP of beam index i  at time instance t- ground truth of L1-RSRP of beam index n at time instance t), where the beam index n owns the largest reported value at time instance t.

## 2.2 Evaluation Period of Spatio-Temporal Beam Prediction

RAN4 made the following agreement during the last meeting [21].

Issue 2-1: Measurement period for inference Agreement: observation period for prediction: (observation period is the amount of time during which UE samples the reference signals to make 1 prediction report) For case 1 reuse M,N,P from legacy measurement requirements For case 2, use T*M, N P (M,N,P same as legacy measurement requirement) FFS on T value, T can also be 1. T can also be based on capability This observation period is to be used for the prediction delay requirement

In above agreement, T refers to the minimum number of observation instances that UE requires to generate its report for case 2. RAN1 made the following agreement during the last meeting [22]

RAN1 #112 Agreement Agreement Support the following on the reporting condition of CSI reporting for inference - For BM-Case2, after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, UE transmit a CSI report for inference only if receiving at least K latest consecutive transmission occasion for each of the RS resources in Set B no later than the corresponding CSI reference resource, where K is indicated by a new UE capability.

Above RAN1 agreement allows UE, at least, K consecutive observation occasions before UE is required to transmit a CSI report for inference, where K would be indicated by a new UE capability. RAN4 should reuse this RAN1 defined capability as the required minimum number of observation instances in case 2 evaluation period.

Observation 10: RAN4 has kept the required minimum number of observation instances as FFS in the evaluation period definition of case 2.

Observation 11: RAN1 has recently agreed to allow UE, at least, K consecutive observation occasions before UE is required to transmit a CSI report for inference, where K would be indicated by a new UE capability.

Proposal 4: T in the measurement period for inference in case 2 of RAN4 agreement is equal to the RAN1 defined UE capability K, where K denotes the number of consecutive transmission occasions that UE needs to receive before transmitting a CSI report for inference in BM-Case 2.

## 2.2 TCI State Status of Predicted Tx Beam

RAN4 made the following agreement during the last meeting [21]:

Issue 2-3: TCI State Handling Agreement: - On Detectability and SNR conditions: - The UE has sent at least 1 L1-RSRP inference report for the target TCI state before the TCI state switch command - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3 dB - Time conditions: - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam predict reporting or measurement - Agreement on QCL relationship - If the predicted Tx beam in Set A is QCL Type-D to a known measured Tx beam, where TX beam can be both inside or outside set B, the corresponding Rx beam is known. - If the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam, known TCI state conditions shall be based on UE capability.

Figure 1: Wide-to-narrow prediction in BM case 1

Above agreement mentions that if the predicted TX beam in setA is QCL Type-D to a known measured Tx beam, the corresponding Rx beam is known. On the other hand, if the predicted Tx beam in set A is not QCL Type-D to a known Tx beam, known TCI state conditions will be based on UE capability.

However, the term “known measured Tx beam’ is not clear. In legacy RRM spec, generally speaking, a TX beam is known If the UE has measured it during the last 1.28 seconds and if UE has sent a report regarding that beam. In AI-ML BM, UE may measure a beam to generate its inference but may not report it. For example, in Figure 1, UE measures SSBs of setB and runs inference to generate properties of top beams of setA. Later, UE may send inference based reports of top beams of CSI-RS beams of setA without sending measurement reports of SSBs of setA. The CSI-RS beams of setA will be QCLed with the SSBs of setA. It needs to be clarified if the SSBs of setB can be classified as “known measured” TX beams in this scenario where network has not received measurement reports of setB.

Since UE has already sent an inference report of top beams of setA and since UE must have used the SSBs of setB to generate its inference report for setB, RAN4 should classify these SSBs as “known measured Tx beams” in this scenario.

Observation 12: RAN4 has agreed to the following regarding the QCL relationship of TCI state handling in AI-ML BM.

- If the predicted TX beam in setA is QCL Type-D to a known measured Tx beam, the corresponding Rx beam is known.

- If the predicted Tx beam in set A is not QCL Type-D to a known Tx beam, known TCI state conditions will be based on UE capability.

Observation 13:  In legacy RRM spec, a TX beam is classified as “known” if the UE has measured it during the last 1.28 seconds and if UE has sent a report regarding that beam.

Observation 14: UE measures setB beams to generate and send inference report for setA but UE may not send measurement reports of setB to the network.

Proposal 5: RAN4 clarifies that “known measured TX beams” include setB beams in the agreement on QCL relationship on TCI state handling.

## 2.3 Inference Timeline of AI-ML Beam Prediction

RAN4 made several agreements regarding delay timeline of AI-ML based beam prediction.

Agreement: - Prediction delay includes: - Measurement for prediction - FFS on whether the legacy measurement delay can be reused - Inference delay - Other components are FFS Agreement: - For measurement delay component of the prediction delay: - Use “legacy” L1-RSRP measurement as the baseline for Case 1 - FFS whether same values for M,N,P should be reused - FFS for case 2

RAN4 has not discussed and defined inference delay for AI-ML beam prediction. RAN1 has started discussing inference delay for AI-ML beam prediction. RAN1 has already agreed to extend the legacy L1-RSRP reporting timeline to accommodate inference timeline in AI-ML based predicted L1-RSRP reporting.

RAN1 captured the following in its most recent CR that focuses on Re-19 AI-ML air interface [20]:

Above text shows that Z3, the minimum time separation between DCI trigger and AP L1-RSRP report, and Z3’, the minimum time separation between RS and AP L1-RSRP report, will be extended by d and d’ respectively in AI-ML beam prediction. Besides, the value of d and d’ can differ between spatial only beam prediction and spatio-temporal beam prediction. RAN1 is expected to discuss the specific values of d and d’ during August 2025meeting. These d and d’ values are expected to capture the inference timeline of UE.

The inference timelines mentioned in RAN4 spec should be aligned with RAN1 spec. RAN4 is discussing inference timeline in the context of prediction delay, i.e., the time separation between resources for inference and the report containing predicted RSRP. This is exactly what RAN1 is trying to capture through the parameter d’. Hence, the inference timeline of RAN4 spec should just refer to d’.

Observation 15: RAN1 has agreed to increase the minimum time separation between reference signal and aperiodic L1-RSRP report by d’ in AI-ML beam prediction.

- Exact value of d’ is TBD in RAN1.

Observation 16: RAN4 has agreed to include inference delay in prediction delay of AI-ML BM. This is equivalent to the parameter d’ of RAN1 spec.

Proposal 6: The inference timelines mentioned in RAN4 spec should be aligned with d’ in RAN1 spec.

- Note 1: d’ is the extension in time separation between RS and AP L1-RSRP report in AI-ML based beam prediction.

- Note 2: d’ can vary between spatial only and spatio-temporal beam prediction. Exact values of d’ will be discussed on August 2025 meeting in RAN1.

# Conclusions

Observation 1: The FFS of N > 1 in beam ID accuracy prediction metric can be expanded to different metrics.

- Potential metric 1: The metric could check if N out of top K predicted beams are contained among the M strongest genie aided beams.

- Potential metric 2: The metric could also check if any N out of top K predicted beams are within X dB of any two of the M strongest genie aided beams.

- Potential metric 3: The metric could also check if the ground truth RSRP of top N predicted beams are within X dB of the ground truth RSRP of the two genie aided strongest beams.

Observation 2: The proponents of N > 1 claim that the current version of the agreement only checks if one of top K predicted beams is within X dB margin, in terms of ground truth RSRP, of the genie aided strongest beam. Network does not know which of the top K predicted beams is within X dB margin of the genie aided strongest beam.

Observation 3: The proponents N > 1 also claim that it gives network idea about UE’s capability to predict and report multiple strong beam IDs.

Observation 4: If K > 1, network must transmit the top-K predicted beams in a 2nd round of beam sweep and collect UE’s report regarding the strongest beam among top-K predicted beams before changing active TCI state.

Observation 5: Currently agreed procedures do not allow network to dynamically configure (e.g., via DCI or MAC-CE) the 2nd round of beam sweep based on UE’s inference results regarding top-K predicted beams.

Observation 6: The issue of 2nd round of beam sweep, mentioned in observation 4 and 5, can be avoided by making K = 1.

Observation 7: In FR2, networks typically cannot communicate with multiple UEs via the same beam. The knowledge of the 2nd strongest, defined via ground truth RSRP, predicted beam may not be essential for the network.

Observation 8: Beam index n, owning the largest reported value, can be an element of setB. However, UE reports it as predicted L1-RSRP while reporting it as part of a top beam of setA. It does not matter whether the reported and predicted L1-RSRP of this beam is exactly equal to the measured L1-RSRP of this beam.

Observation 9: Channel may vary significantly across different prediction time instances. It is not meaningful to compare relative accuracy of reported L1-RSRP across two different prediction time instances.

Observation 10: RAN4 has kept the required minimum number of observation instances as FFS in the evaluation period definition of case 2.

Observation 11: RAN1 has recently agreed to allow UE, at least, K consecutive observation occasions before UE is required to transmit a CSI report for inference, where K would be indicated by a new UE capability.

Observation 12: RAN4 has agreed to the following regarding the QCL relationship of TCI state handling in AI-ML BM.

- If the predicted TX beam in setA is QCL Type-D to a known measured Tx beam, the corresponding Rx beam is known.

- If the predicted Tx beam in set A is not QCL Type-D to a known Tx beam, known TCI state conditions will be based on UE capability.

Observation 13:  In legacy RRM spec, a TX beam is classified as “known” if the UE has measured it during the last 1.28 seconds and if UE has sent a report regarding that beam.

Observation 14: UE measures setB beams to generate and send inference report for setA but UE may not send measurement reports of setB to the network.

Observation 15: RAN1 has agreed to increase the minimum time separation between reference signal and aperiodic L1-RSRP report by d’ in AI-ML beam prediction.

- Exact value of d’ is TBD in RAN1.

Observation 16: RAN4 has agreed to include inference delay in prediction delay of AI-ML BM. This is equivalent to the parameter d’ of RAN1 spec.

Proposal 1: Update the previous agreement with the following changes:

- “The successful rate for the correct prediction,

  - The correct prediction is considered as maximum ground-truth RSRP among of top-K1 predicted beams being larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB,

  - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation

    - FFS on N values

  - FFS on x values

- Note: if x=0 and N=1, it means Top-K1/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K1 predicted beams"

Proposal 2: RAN4 uses the agreed baseline for relative L1-RSRP of case 1 with the following changes:

  - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i – reported predicted L1-RSRP of beam index n) -  (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value].

    - Reported L1-RSRP can be predicted,

    - FFS whether reported L1-RSRP can be measured RSRP

Proposal 3: Apply proposal 3 to case 2 where predicted and ground truth L1-RSRP of both beam index i and n correspond to the same time instance. In other words, define relative L1-RSRP accuracy in the following way for case 2:

  - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i at time instance t –  predicted L1-RSRP of beam index n at time instance t) -  (ground truth of L1-RSRP of beam index i  at time instance t- ground truth of L1-RSRP of beam index n at time instance t), where the beam index n owns the largest reported value at time instance t.

Proposal 4: T in the measurement period for inference in case 2 of RAN4 agreement is equal to the RAN1 defined UE capability K, where K denotes the number of consecutive transmission occasion that UE needs to receive before transmitting a CSI report for inference in BM-Case 2.

Proposal 5: RAN4 clarifies that “known measured TX beams” include setB beams in the agreement on QCL relationship on TCI state handling.

Proposal 6: The inference timelines mentioned in RAN4 spec should be aligned with d’ in RAN1 spec.

- Note 1: d’ is the extension in time separation between RS and AP L1-RSRP report in AI-ML based beam prediction.

- Note 2: d’ can vary between spatial only and spatio-temporal beam prediction. Exact values of d’ will be discussed on August 2025 meeting in RAN1.

# References

- R4-2414446, ‘AI/ML ad-hoc meeting minutes’, Moderator (Qualcomm)

- R4-2414323, ‘WF on AI/ML’, Qualcomm

- 3GPP TR 38.843 V18.0.0, Technical Report (TR): Study on artificial Intelligence (AI)/Machine Learning (ML) for NR air interface.

- RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, 3GPP TSG RAN #105.

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #117.

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #118.

- 38.133, Requirements for support of radio resource management, V 17.11.0.

- R4-2417212, WF on Requirements for AI/ML air interface, 3GPP TSG-RAN WG4 #112bis.

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 #118bis.

- R1-2308259, “Evaluations on AI/ML for Beam Management”, Qualcomm Incorporated, 3GPP TSG-RAN WG1 #114.

- R4-2417211, “Simulation assumptions for beam prediction”, 3GPP TSG-RAN WG4 #112bis

- R4-2420513, WF on AI/ML air interface part 1, RAN4 #113.

- R4-2420480, Updated Simulation Assumptions for Beam Prediction, RAN4 #113.

[14] R4-2502856, “WF on the requirements for AI/ML air interface”, 3GPP TSG-RAN WG4 Meeting#114

[15] R4-2505105, “WF on requirements for AI/ML air interface normative work”, 3GPP TSG-RAN WG4 Meeting#114bis

[16] R4-2505152, “Updated simulation assumptions for AI/ML BM”, 3GPP TSG-RAN WG4 Meeting#114bis

[17] 3GPP TSG WG RAN1 Chairman’s notes, RAN1 #120bis

[18] 38.133 V 18.7.0, Requirements for support of radio resource management.

[19] R4-2502856, WF on the requirements for AI/ML air interface, 3GPP RAN WG4 Meeting#114

[20] R1-2504992, “Introduction of AI/ML BM Prediction”, 3GPP TSG-RAN WG#121

[21] R4-2511890, “WF on AI-ML air interface”,  3GPP TSG RAN WG4 #116.

[22] 3GPP TSG WG RAN1 122 Chairman’s notes.