3GPP TSG-RAN WG4 Meeting #116bis	R4-2514815

Prague, Czech Republic, 13 - 17 October 2025

Agenda item:			6.17.1

Source:	Nokia

Title:		WF on NR_Mob_Ph4_RRM_Part1

Document for:	Information

# Remaining issues and CR split

Open Core part issues:

Issue 1-3-1: Whether to include unknown cell case on SSB triggered CLTM

Combined case list for Event Triggered reporting and CSI-RS measurements:

| Test case | Baseline test case | Test case | Company |
| --- | --- | --- | --- |
| 1 | LTM R18 A.6.6.26 | Intra-frequency SSB based L1-RSRP measurement in FR1 with event triggered reporting or periodic reporting | Ericsson |
| 2 | LTM R18 A.6.6.27 | Inter-frequency SSB based L1-RSRP measurement with measurement gap with event triggered reporting or periodic reporting | CATT |
| 3 | LTM R18 A.6.6.28 | Inter-frequency SSB based L1-RSRP measurement without measurement gap with event triggered reporting or periodic reporting | CATT |
| 4 | LTM R18 A.7.6.20 | Intra-frequency SSB based L1-RSRP measurement in FR2 with event triggered reporting or periodic reporting | ZTE |
| 5 | LTM R18 A.7.6.21 | Inter-frequency SSB-based L1-RSRP measurement with measurement gap for LTM in FR2 with event triggered reporting or periodic reporting | ZTE |
| 6 | LTM R18 A.7.6.22 | Inter-frequency SSB based L1-RSRP measurement without measurement gap in FR2 with event triggered reporting or periodic reporting | Nokia |
| 7 | New | CSI-RS based L1 RSRP measurement for neighbour cell in FR1 with event triggered reporting or periodic reporting | Nokia |
| 8 | New | CSI-RS based L1 RSRP measurement for neighbour cell in FR2 with SSB based L1-RSRP measurement with event triggered reporting or periodic reporting | Huawei |
| 9 | New | CSI-RS based L1 RSRP measurement for neighbour cell in FR2 without SSB based L1-RSRP measurement with event triggered reporting or periodic reporting | Ericsson |
| 10 | New | CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR2 | Apple |
| 11 | New | CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR1 | Apple |

Accuracy requirements CR split:

| Intra-frequency L1-RSRP accuracy requirements | 10.1.19D.2 CSI-RS based intra-frequency L1-RSRP accuracy requirements for FR1 10.1.19D.2.1 Absolute Accuracy 10.1.19D.2.2 Relative Accuracy | Nokia |
| --- | --- | --- |
|  | 10.1.20A.2 CSI-RS based intra-frequency L1-RSRP accuracy requirements for FR2 10.1.20A.2.1 Absolute Accuracy 10.1.20A.2.2 Relative Accuracy | Huawei |

# Topic #1 RRM Core requirements

## Event triggered L1 measurement reporting

Issue 1-1-1: Whether to introduce/update requirements for reporting criteria per measurement category

<Agreement>

- Not to introduce additional UE capability.

- Common understanding is that as long as the number of RS configured is within the UE’s measurement capability and the number of reporting configurations remains within the 48, there are no limitations on maximum number of events instances UE can run in parallel. No specification impact is expected.

Issue 1-1-2: Impact on DL synchronization

Agreement:

- No need to add new condition for Tfirst-RS and TRS-proc = 0 in cell switch delay, because the event triggered report within 160ms before cell switch command

## CSI-RS based L1 measurement

Issue 1-2-1: Capturing CSI-acquisition note in cell switch delay

Agreement:

- Common understanding is clear in RAN4 related to the timeline of CSI-acquisition. No need to add note to the specification and we can close the issue.

Issue 1-2-2: Clarification on CSI-acquisition frequency location

Agreement:

- No need to further discuss about the frequency location of the CSI-acquisition RS in this WI.

## Conditional Intra-CU LTM

Issue 1-3-1: Whether to include unknown cell case on SSB triggered CLTM

Technical background: RAN4 previously agreed that specifications only cover the known cell case for CSI-RS L1 triggered CLTM. But for SSB L1 triggered CLTM, no agreement had been reached.

Way forward in RAN4#115 For CSI-RS based L1-RSRP measurement: - For known cell, Tmeasure shall include the time period used to acquire the SFN information (TCSI-RS_SFN_intra) and CSI-RS based L1-RSRP measurement period. - Further discuss the conditions for TCSI-RS_SFN_intra = 0. - Further discuss whether to include unknown cell case. If included, add one more component of TPSS/SSS. Agreement in RAN4#116 Issue 1-1-4: Whether to include unknown cell case < Agreement > - Only cover the known cell case.

Moderator comment: The following is included in the CR:

1> If SSB based L1-RSRP measurement is used in the event, Tmeasure shall be no larger than the maximum of L1-RSRP measurement periods of the serving cell and target cell. 2> serving cell measurement period TL1-RSRP_Measurement_Period_SSB is defined in clause 9.5.4.1 assuming TReport = 0, and 2> if the target cell is an intra-frequency cell: 3> target cell measurement period is TPSS/SSS_sync_intra + TSSB_measurement_period_intra + TL1-RSRP_Measurement_Period_SSB_intra as defined in clause 9.2.5.1 and clause 9.14.5.1 assuming TReport = 0, if deriveSSB-IndexFromCell is enabled or UE has reported SSB index in L3 measurement report of the same cell, or

Proposals:

- Proposal 1 (CTC): Discuss whether to include unknown cell case on SSB triggered CLTM, i.e., whether TPSS/SSS_sync_intra + TSSB_measurement_period_intra are needed. If so, the conditions that TSSB_time_index_inter = 0 maybe need to be updated.

- Proposal 2 (Nokia): RAN4 to distinguish between known and unknown target cell cases in the CLTM measurement time requirements (section 6.3.2.2.1) when an SSB-based L1-RSRP measurement is used in the event.

Recommended WF:

Discuss the following:

- For SSB based L1-RSRP measurement used with CLTM. Discuss and confirm which scenario is supported:

  - Option 1: Support only known case (specification change needed)

  - Option 2: Support both unknown and known case (specification change needed)

  - Option 3: Support only unknown case (no specification change).

Way Forward:

- Further check this issue

Issue 1-3-2: The values and the conditions for Tfirst-RS = 0 and TRS-proc = 0

Tentative agreement:

- No need to add additional condition for Tfirst-RS = 0 and TRS-proc = 0 in R19 CLTM.

Issue 1-3-3: CLTM requirements for candidate cells without active TCI states in FR2

Agreement:

- Capture agreement, discuss further in CR.

# Topic #2 Event triggered L1 measurement reporting performance part

Issue 2-1-1: Testing approach for SSB-based L1 event triggered measurement reporting

<Agreement>:

  - Testing approach for SSB based L1 RSRP measurement reporting:

  - Duplicate all R18 LTM L1 SSB test cases and add event triggered reporting. UE which can pass R19 test cases can skip R18 test cases.

Issue 2-1-2: Testing approach for CSI-RS-based L1 event triggered measurement reporting

<Agreement>

Regarding the direction of defining test case for CSI-RS based measurements:

- Introduce test cases to verify 1) CSI-RS based L1-RSRP measurement and 2) CSI-RS based L1-RSRP measurement event triggered reporting.

  - For UE supporting event triggered reporting, UE only needs to pass 2)

  - For UE not supporting event triggered reporting, UE only needs to pass 1)

- Test case list for CSI-RS based L1-RSRP measurement event triggered reporting

  - CSI-RS based L1 RSRP measurement for neighbour cell in FR1

  - CSI-RS based L1 RSRP measurement for neighbour cell in FR2 with SSB based L1-RSRP measurement

  - CSI-RS based L1 RSRP measurement for neighbour cell in FR2 without SSB based L1-RSRP measurement

  - CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR1

  - CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR2

Issue 2-1-3: Supported events in test cases

- Agreement:

- Cover Event LTM3 and LTM4 in different test cases.

Issue 2-1-4: Test case list for event-triggered L1 reporting

Moderator’s comment: See the combined test case list.

# Topic #3 CSI-RS based L1 measurement Performance part

Issue 3-1-1: Whether to define test cases for FR1 and FR2:

Moderator’s comment: this issue was resolved in the test case list discussion.

Issue is closed.

Issue 3-1-2: Test configuration for CSI-RS based L1 measurement test cases

Agreement:

- The SINR of the CSI-RS L1-RSRP measurement on candidate cell is above -3dB. The accuracy of CSI-RS based L1-RSRP on serving cell can be used as a baseline. The conditions of CSI-RS L1-RSRP measurement on candidate cells are:

  - The bandwidth of CSI-RS is 48 PRBs and the density is 3.

  - The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements.

  - The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP.

Issue 3-1-3: Test case list for CSI-RS based L1 measurement

Agreement:

- For CSI-RS based L1 measurement, define the following test cases:

  - CSI-RS based L1 RSRP measurement for neighbour cell in FR1

  - CSI-RS based L1 RSRP measurement for neighbour cell in FR2 with SSB based L1-RSRP measurement

  - CSI-RS based L1 RSRP measurement for neighbour cell in FR2 without SSB based L1-RSRP measurement

  - CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR1

  - CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR2

- Periodic Reporting for all the test cases.

Issue 3-1-4: Scenarios to test with and without step 2 (Rel-18 SSB measurements):

Moderator’s comment: this issue was resolved in the test case list discussion.

Issue is closed.

Issue 3-1-5: SS/CSI-RSRP measurement L1 report mapping

Agreement:

Legacy SS/CSI-RSRP measurement L1 report mapping specified in clause 10.1.6 (including measured quantity reporting and differential reporting) can be reused for SS/CSI-RS based L1 measurement reporting on neighbor cell

Issue 3-1-6: Conditions when UE supports event triggered reporting and traditional reporting

Moderator’s comment: This issue is resolved in the Topic 2: Issue 2-1-2: Testing approach for CSI-RS-based L1 event triggered measurement reporting

The agreement made in the issue 2-1-2 discussion is:

<Agreement>

Regarding the direction of defining test case for CSI-RS based measurements:

- Introduce test cases to verify 1) CSI-RS based L1-RSRP measurement and 2) CSI-RS based L1-RSRP measurement event triggered reporting.

  - For UE supporting event triggered reporting, UE only needs to pass 2)

  - For UE not supporting event triggered reporting, UE only needs to pass 1)

Issue 3-1-7: Whether to test CSI-acquisition

Agreement: this issue is closed.

Issue 3-1-8: Accuracy requirements for CSI-RS based L1-RSRP candidate cell

Technical background from R4-2512141:

| #### Issue 4-2-1: CSI-RS based L1 measurement accuracy Agreement: - CSI-RS based L1-RSRP accuracy of the serving cell can be reused for neighbour cell when the following conditions are met: - Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled. - Conditions for L1-RSRP measurements are fulfilled according to annex B.2.4.2 for a corresponding Band for each relevant CSI-RS. - The bandwidth of CSI-RS is 48 PRBs and the density is 3. - The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP. - Legacy SS/CSI-RSRP measurement L1 report mapping specified in clause 10.1.6 (including measured quantity reporting and differential reporting) can be reused for SS/CSI-RS based L1 measurement reporting on neighbor cell. - New requirements are defined in the following sections |  |
| --- | --- |
| Accuracy requirements | Detail |
| Intra-frequency L1-RSRP accuracy requirements | 10.1.19D.2 CSI-RS based intra-frequency L1-RSRP accuracy requirements for FR1 10.1.19D.2.1 Absolute Accuracy 10.1.19D.2.2 Relative Accuracy |
|  | 10.1.20A.2 CSI-RS based intra-frequency L1-RSRP accuracy requirements for FR2 10.1.20A.2.1 Absolute Accuracy 10.1.20A.2.2 Relative Accuracy |