3GPP TSG RAN WG4 Meeting #116bis                  	                                    R4-2514823

Prague, CZ, 13th – 17th October 2025

Agenda item:			7.5.1

Source:	China Telecom

Title:	WF on NR_demod_Ph6_Ph5

Document for:	Approval

- Introduction

This Way Forward captures the agreements, open issues and candidate options for Rel-20 Demod performance requirements WI and Rel-19 Demod performance requirements maintenance.

- Topic #1: UE requirements for FR1 SU-MIMO with spatial channel model

- Potential updates to rCDL channel model

Angle scaling for rCDL

- Agreement:

  - Update the rCDL channel model based on the latest angular scaling procedure as in TR 38.901 v19.1.0 and further discuss the details.

- Agreement on the details:

  - Use the ‘s’ values for tabulated desired AS values rather than the desired AS values we used in 38.753 to save us the effort of obtaining ‘s’ and aligning among companies.

  - Obtain ‘s’ from the tabulated example AS desired values and use the following desired AS: ASD – 25; ASA – 60; ZSD – 5, ZSA – 15 as baseline to configure rCDL channel model

  - Additional rCDL Channel Models based on different AS are not precluded

Frequency generalization for rCDL channel modeling

- Agreement:

  - Define the requirements based on the same channel model for all different FR1 frequencies for this WI

- The maximum Doppler fD should be fixed as X Hz

    - As baseline for Rel-20 demod requirement, X =10

    - FFS if other fD can be considered

  - On the note to Maximum Doppler:

- The ratio between the UE speed v, i.e., magnitude of the velocity vector, and the wavelength is normalized to be X Hz (maximum Doppler).

UE antenna modelling

- <For INFORMATION> It is RAN4 common understanding that the following were added in Rel-19 version of TR 38.901:

  - #1: Candiadate location for UE antenna placements

  - #2: Directional antenna assumption for UE

- Further discuss the two options in the next meeting:

  - Option 1: Update the UE antenna placement and assumption of rCDL based on v19.0.0 of 38.901.

  - Option 2: Use CDL parameters listed in Table 6.1-2 of TR 38.753 as baseline

  - The moderator requests proponents of option 1 to provide detailed simulation assumptions in the next meeting.

Delay scaling after cluster truncation

- Candidate options:

  - Option 1: Consider omitting the post-truncation delay scaling step in rCDL model derivation. Methods other than linear scaling and/or metrics other than RMS delay spread could be investigated.

  - Option 2: Reuse the Rel-19 SI approach captured in TR38.753

Further rCDL-C1 channel alignment

- Candidate options:

  - Option 1: Measure and align channel statistics such as Spatial Domain Power Density (SDPD), Time Coherence (TC), and Frequency Coherence (FC) as described in TR 38.753 Section 6.4.

  - Option 2: Not to do further rCDL-C1 channel alignment

- Test scope and test parameters for SCM SU-MIMO requirements

- General

Antenna Array Virtualization (AAV) assumptions

- Candidate options:

  - Option 1: Only consider Ms=Ns=1

Parameters for CDL modelling

- Candidate options:

  - Option 1: Use CDL parameters listed in Table 6.1-2 of TR 38.753 as baseline

Duplex mode and SCS

- Agreement:

  - Cover TDD 30kHz SCS with 40MHz CHBW and FFS whether to also cover FDD 15kHz SCS with 10MHz CHBW

- PDSCH demodulation requirements with SCM

Test scope for PDSCH demodulation requirements with SCM

- Agreement:

  - Cover PDSCH mapping type A demodulation requirements with 8T8R 8 layer case and the 4T4R 4 layer case.

Precoding, MCS and test metric

- Agreement and candidate options:

  - For 4T4R 4L test, it is agreed to use the test metric: SNR @ 70% max TP.

  - For 8T8R 8L test, it is agreed to use test metric: Define single SNR and further discuss the SNR is based on:

    - Option a: Based on per CW thput with Different MCS per CW

    - Option b: Based on total thput with different MCS per CW

    - Option c: Based on total thput with same MCS per CW

    - Further discuss: How to define the SNR for option a. How to select the MCS for different CW.

Other test parameters for PDSCH demodulation requirements with SCM

- Candidate options:

  - Option 1: Reuse the test parameters for the Rel-19 SI, i.e., Table 6.1-1 and Table 6.1-2 in TR38.753.

- PMI reporting requirements with SCM

PMI bias

- Candidate options:

  - Option 1: RAN4 shall exploit approaches to address the PMI bias issue

- Option 1a: Apply the existing simple beam steering approach specified in TS 38.101-4 B.2.3.2.3

- Option 1b: time variant mean angles (AOA/AOD/ZOA/ZOD)

- Other options are not precluded

  - Option 2: No need to consider time-wise evolution for PMI reporting tests

  - Option 3: Defer a consideration of a comprehensive long-term solutions to the 6G Radio Study Item

Whether to define eType II PMI reporting requirements based on SCM

- Candidate options:

  - Option 1: Specify requirements for PMI performance with eType II precoding

- Option 1a: Select the lowest value among the ones companies provided.

- Option 1b: Outlier removal to be applied to form requirements, if outliers are not aligned in second simulation results delivery round.

  - Option 2: Further simulation result alignment is required

- Option 2a: Collect both TP gain and SNR @ 90% Max TP to evaluate alignment

- Option 2b: RAN4 can take 8Tx 4 layers Type-I PMI case for initial alignment if rCDL model is updated.

Test scope for eType II PMI reporting (if introduced)

- Candidate options:

  - Option 1: RAN4 needs to define requirements for both 8T4R and 4T4R cases as captured in the WID

Rank for eType II PMI reporting (if introduced)

- Candidate options:

  - Option 1: Only consider rank 2

  - Option 2: Consider rank 4

Other test parameters for eType II PMI reporting (if introduced)

- Candidate options:

  - Option 1: Reuse the test parameters for the Rel-19 SI, i.e., Table 6.2-1 and Table 6.2-2 in TR38.753.

Test metric for eType II PMI reporting (if introduced)

- Candidate options:

  - Proposal 1: Reuse the same test metric for the existing eType II requirement based on TDL, i.e., Thput of following PMI (eType II)/ Thput of Random PMI (Type I codebook) with SNR @ 90 % of the maximum throughput for following PMI.

  - Proposal 2: Check PMI simulation results on both normalized throughput and gamma values and do down selection.

- Test applicability for SCM SU-MIMO requirements

Test applicability for SCM SU-MIMO requirements

- Candidate options:

  - Proposal 1: Introduce new declaration for the SCM testing. RAN4 needs to discuss whether to define one single declaration on SCM requirements or define multiple declarations for each individual SCM test cases.

  - Proposal 2: Introduce new optional UE capability “SCM compatible MIMO receiver” and corresponding applicability rules

  - Proposal 3: If requirements with rCDL are introduced, their applicability should be optional and based on UE declaration to support these requirements.

Release independent for SCM SU-MIMO requirements

- Candidate options:

  - Proposal 1: Discuss whether to define release independent rules for the new SCM requirements.

- Topic #2: UE requirements for 6Rx UE with interference

- General

Coverage of UE types

- Observation:

  - It is clearly stated in the approved WID that the 6Rx requirements with interference will cover both FWA and handheld UEs.

- Proposals:

  - Proposal 1: FFS whether to define one set or two sets of requirements for handheld and FWA UEs until there is clear agreement in Rel-19.

  - Proposal 2: Define only a single requirement for both handheld and FWA devices.

  - Proposal 3: Define separate requirements for handheld and FWA devices

Test scope

- Agreements:

  - Focus PDSCH requirements on Mapping Type A only.

  - Do not cover PDSCH absolute physical layer throughput requirements

Antenna correlation

- Candidate options:

  - Proposal 1: Follow the decision in the Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’.

  - Proposal 2: Evaluate the performance with α = 0 and β = 0.07179 for handheld UE and low correlation for FWA

  - Proposal 3: For initial simulation alignment, cover both ULA Low and ULA “non-Low” as per outcome of NR_ENDC_RF_Ph4_Demod_6Rx.

Receiver assumption

- Agreement:

  - Only MMSE-IRC is considered

Duplex mode and SCS

- Agreement:

  - Align with the basic 6Rx requirement: Only to define requirements for FR1 TDD with 30kHz SCS/40MHz 7D1S2U with S: 6D+4G+4U

- PDSCH requirements with inter cell interference

Test scenarios and interference profile

- Agreement:

  - Cover both HomNet and HetNet scenarios

  - Interference modelling: Reuse the same interference modeling defined for Rel-17 2/4Rx and Rel-19 8Rx:

- INR based inter cell interference modeling with the following interference level: - INR1 = 7.77dB and INR2 = 2.29dB for HomNet (if covered) - INR1 = 7.58dB for HetNet (if covered) - Random rank with 70% and 30% probability for rank 1 and rank 2 for the interference cell(s) - Random 16QAM modulated symbols for the interference cell(s) - Frequency shift to the target cell: 300Hz for interference cell 1, -100Hz for interference cell 2 - SSB configuration: First SSB in Slot #0 for all interference cells (Same as serving cell)

Rank for the serving cell

- Agreement:

  - For initial evaluation purpose, cover rank 1,2 and 4 And make further selection based on simulation.

Antenna configuration

- Agreement:

  - For initial simulation purpose, 2T6R for rank 1/2 and 4T6R for rank 4

Propagation condition

- Candidate options:

  - Option 1: TDLA30-10

  - Option 2: Consider TDLC300-100 and TDLA30-10 for handheld UE. For FWA devices, consider TDLA30-10 only.

  - Option 3: TDLC300-100 for 2 interfering cells (HomNet) and TDLA30-10 for 1 interfering cell (HetNet)

MCS

- Candidate options:

  - Option 1: MCS 17 for HomNet and MCS 13 for HetNet

  - Option 2: MCS 17

  - Option 3: MCS 17 for rank 2 and MCS 13 for rank 4

Other parameters

- Agreement:

  - Reuse the same configuration for Rel-17 2/4Rx as the starting point.

- PDSCH requirements with intra cell inter user interference

MU-MIMO Beamforming modeling

- Agreement:

  - Reuse MU-MIMO Beamforming in TS 38.101-4 Annex B.4.2

Signal power assumptions and SNR assumptions

- Agreement:

  - Average target UE signal power is equal to RankTargetUE/RankTotal and average interference UE signal power is equal to RankInterfUE/RankTotal

  - SNR = (STargetUE+ SInterfUE)/N

Antenna configuration and rank for target + co-scheduled UE

- Agreement:

  - For initial evaluation purpose, cover Rank 1+1 (with 2T6R) and Rank 2+2 (with 4T6R).

  - Select rank 2+2 if simulation results show feasibility

Propagation condition

- Candidate options:

  - Option 1: TDLA30-10

  - Option 2: Consider TDLC300-100 and TDLA30-10 for handheld UE. For FWA devices, consider TDLA30-10 only.

  - Option 3: TDLC300-100 with rank 1+1 and TDLA30-10 with rank 2+2

Precoding method for the co-scheduled UE

- Candidate options:

  - Option 1: Random precoding with rank 1+1 and orthogonal precoding for rank 2+2 tests

  - Option 2: Orthogonal precoding for rank 2+2 tests

MCS for the target UE

- Candidate options:

  - Option 1: MCS 17 (Table 1) for 2+2 case, and MCS19 (Table 1) for 1+1 case

  - Option 2: MCS 17 (Table 1) for 2+2 case

  - Option 3: Cover MCS 13 and MCS 17 for evaluation

Other test parameters and simulation assumptions

- Agreement:

- 1 Co-scheduled UE number - Precoding for the target UE: Random Single Panel Type I precoding - 2PRBs for PRB bundling size and precoding granularity - PDCCH and PDSCH allocation in time domain - Use symbols #0 and #1 of each slot for PDCCH - PDSCH mapping type A, Start symbol 2, Duration 12 - PDSCH allocation in frequency domain - Full PRB

- CQI reporting requirements with inter-cell interference

Whether to introduce CQI reporting requirements with inter-cell interference

- Candidate options:

  - Option 1: No matter how RAN4 will design the CQI reporting tests for basic 6Rx, there is necessity to define CQI reporting requirements with inter-cell interference.

  - Option 2: Don’t define CQI reporting requirements with inter-cell interference for 6Rx

Interference modelling and test parameters for CQI reporting requirements with inter-cell interference (if introduced)

- Candidate options:

  - Option 1: RAN4 to reuse only Rel-17 interference models for 6Rx devices.

  - Option 2: Use the configuration provided in below table as starting point

| Parameter | Value |
| --- | --- |
| SCS/CBW | FDD: 15kHz/10MHz TDD: 30kHz/40MHz, 7D1S2U(S=6D+4G+4U) |
| Rank for serving cell | Rank 2 |
| Propagation conditions | TDLA30-5 for the serving cell AWGN for the interference cell |
| Antenna and MIMO correlation for serving cell | 2T6R with ULA Low 2T6R with ULA “non-Low” as per outcome of NR_ENDC_RF_Ph4_Demod_6Rx. |
| Antenna configuration for the interference cell: | 1T6R |
| Interference power level | INR = 10.04dB for the serving cell. |
| Other parameters | For other parameters re-use the same configuration as used in Rel-17 requirements. |

Test metric for CQI reporting requirements with inter-cell interference (if introduced)

- Candidate options:

  - Option 1: Reuse the same test metric for Rel-17 2Rx, 4Rx and 8Rx (Nokia)

a) the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥  b) when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to X%.

- Applicability and release independent

Test applicability for 6Rx requirements with interference

- Candidate options:

  - Option 1: 6Rx requirements with interference should be mandatory for all 6Rx UEs without additional test applicability rule

  - Option 2: Define a test applicability rule to skip 2/4Rx requirements if 6Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not

Release independent for 6Rx requirements with interference

- Candidate options:

  - Option 1: Follow the same conclusion in Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’

  - Option 2: All new 6Rx interference requirements introduced shall be release independent from Rel-17

- Topic #3: Rel-19 Demod performance requirements maintenance

Alignment on simulation results for BS MMSE-IRC requirements

- Way forward

  - Keep the way forward from RAN4#116 and continue the discussions in RAN4#117

  - Make decision on final SNR requirements in RAN4#117