3GPP TSG RAN4 Meeting #116bis		R4-2515132
Prague, Czech Republic, 13th – 18th October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR |  | rev |  | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR to TS 36.133 on conditional HARQ-ACK feedback for CB-Msg3 Requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IOT_NTN_PH3 |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the feature design, it is optional for the network to implement the HARQ-ACK resource in the MSG4 that responds to a CB-MSG3. Therefore, the UE ACK transmission shall be dependent on such configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fix the HARQ-ACK requirement in CB-MSG3 functionality |  |  |  |  |  |  |  |  |
| T |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification will be incosistent with TS 36.331 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4A, 6.6A4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of change 1>

### 6.2.4A	Random Access Requirements for Cat-M1 UEs with CB-Msg3 EDT for Satellite Access

The requirement in this clause applies for Cat-M1 CE Mode A UE for satellite access when CB-Msg3-EDT is configured by CB-Msg3-ConfigSIB-r19 and the UE supportsntn-MO-CB-Msg3-EDT-UP-r19. The UE in IDLE mode is allowed to initiate a contention-based PUSCH transmission.

The UE shall have the capability to calculate the transmission power for the initial CB-Msg3 transmission and the subsequent CB-Msg3 replica transmissions according to the formula defined in TS 36.213.

#### 6.2.4A.1	Correct behaviour when transmitting CB-Msg3

UE shall also execute the CB-Msg3-EDT procedure defined in clause 5.1x in TS 36.321 [17] using the PUSCH configuration contained in CB-Msg3-ConfigSIB-r19 in TS 36.331 [2]. The UE shall transmit CB-Msg3 replicas within the CB-Msg3 window, according to network configuration.

#### 6.2.4A.2	Correct behaviour when receiving CB-Msg4

If the CB-Msg4 contains a backoff indicator, the UE shall set the backoff time, according to the CB-Msg3-EDT backoff parameters as described in TS 36.321 [17].

If the CB-Msg4 contains a UE contention resolution identity, the UE shall send HARQ-ACK if the Contention Resolution is successful and if HARQ-ACK resource is provided.

Upon receiving a CB-Msg4 including a matching Contention Resolution Identity and a C-RNTI without a RRC message, UE shall continue monitoring PDCCH.

Upon receiving a CB-Msg4 including a matching Contention Resolution Identity without both a RRC message and a C-RNTI, UE shall terminate the CB-Msg3-EDT procedure and keep in RRC_IDLE.

#### 6.2.4A.3	Correct behaviour when not receiving CB-Msg4

If the Msg3ResponseTimer expires, the maximum attempt number of CB-Msg3 transmissions is not achieved, and the UE has not received a PDCCH message that includes a UE Contention Resolution Identity MAC control element that matches the CCCH SDU transmitted in the uplink message, the UE shall re-attempt a CB-Msg3 transmission when the backoff expires in the next CB-Msg3 window with the calculated PUSCH transmission power.

When the CB-Msg3ResponseTimer expires and the maximum number of retransmissions has been reached, RRC procedure is re-initiated, which procedure is initiated is up to UE implementation.

<End of Change1>

# <Start of change 2>

### 6.6A.4	Requirements for CB-Msg3-EDT procedure

The requirement in this clause applies for for UE category NB-IoT for satellite access when CB-Msg3-EDT is configured by CB-Msg3-ConfigSIB-NB-r19 and the UE supports ntn-MO-CB-Msg3-EDT-UP-r19. The UE in IDLE mode is allowed to initiate a contention-based NPUSCH transmission.

The UE shall have the capability to calculate the transmission power for the initial CB-Msg3 transmission and the subsequent CB-Msg3 replica transmissions according to the formula defined in TS 36.213.

#### 6.6A.4.1 Correct behaviour when transmitting CB-Msg3

UE shall also execute the CB-Msg3-EDT procedure defined in clause 5.1x in TS 36.321 [17] using the NPUSCH configuration contained in CB-Msg3-ConfigSIB-NB-r19 in TS 36.331 [2].The UE shall transmit CB-Msg3 replicas within the CB-Msg3 window, according to network configuration.

#### 6.6A.4.2	Correct behaviour when receiving a CB-Msg4 over CB-RNTI

If the CB-Msg4 contains a backoff indicator, the UE shall set the the backoff time, according to the CB-Msg3-EDT backoff parameters as described in TS 36.321 [17] .

If the CB-Msg4 contains a UE contention resolution identity, the  UE shall send HARQ-ACK if the Contention Resolution is successful and if the HARQ-ACK resource is provided.

Upon receiving a CB-Msg4 including a matching Contention Resolution Identity and a C-RNTI without a RRC message, UE shall continue monitoring PDCCH.

Upon receiving a CB-Msg4 including a matching Contention Resolution Identity without both a RRC message and a C-RNTI, UE shall terminate the CB-Msg3-EDT procedure and keep in RRC_IDLE.

#### 6.6A.4.3	Correct behaviour when detecting CB-Msg3-EDT failure

If the Msg3ResponseTimer expires, the maximum attempt number of CB-Msg3 transmissions is not achieved, and the UE has not received a PDCCH message that includes a UE Contention Resolution Identity MAC control element  that matches the CCCH SDU transmitted in the uplink message, the UE shall re-attempt a CB-Msg3 transmission when the backoff expires in the next CB-Msg3 window with the calculated PUSCH transmission power.

When the CB-Msg3ResponseTimer expires and the maximum number of retransmissions has been reached, RRC procedure is re-initiated, which procedure is initiated is up to UE implementation.

<End of Change2>