3GPP TSG-RAN4 Meeting #116-bis	R4-251xxxx

# Prague, Czech Republic, 13 – 17 October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | xxxx | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on RLM test cases for less than 5MHz for NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IoT_NTN_req_test_enh-Perf |  |  |  |  |  | Date: |  |  | 2025-09-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on work split in R4-2508261, RLM test cases should be introduced for less than 5MHz in NTN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce RLM test cases for less than 5MHz in NTN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Agreed test cases are missing for less than 5MHz in NTN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.14.4.1.X (new), A.14.4.1.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

#### A.14.4.1.X	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode

##### A.14.4.1.X.1	Test Purpose and Environment

The purpose of this test is to verify that the UE supporting support-3MHz-ChannelBW-r18 properly detects the out of sync for the purpose of monitoring downlink radio link quality of the PCell operating on a 3 MHz channel bandwidth. This test will partly verify the FR1 radio link monitoring requirements in clause 8.1C.

Supported test configurations are specified in table A.14.4.1.X.1-1. General test parameters as specified in table A.14.4.1.1.1-2 with config 1 apply except those specified in table A.14.4.1.X.1-2. Cell specific test parameters as specified in table A.14.4.1.1.1-3 apply except those specified in table A.14.4.1.X.1-3.

The test procedure specified in clause A.14.4.1.1.1 applies to this test.

Table A.14.4.1.X.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 3 MHz bandwidth |

Table A.14.4.1.X.1-2: General test parameters for FR1 OOS 15 PRB in non-DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| BWchannel | Config 1 | MHz | 3: NPRB,c = 15 |
| RMSI CORESET Reference Channel | Config 1 |  | CR.1.3 FDD |
| Dedicated CORESET Reference Channel | Config 1 |  | CCR.1.7 FDD |
| SSB Configuration | Config 1 |  | SSB.13 FR1 |
|  | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 3 |
|  | Aggregation level | CCE | 4 |
| In sync transmission parameters | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 0 |
|  | Number of Control OFDM symbols |  | 3 |
|  | Aggregation level | CCE | 8 |
| Out of sync transmission parameters | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 |
|  | REG bundle size |  | 6 |
|  | CP length |  | Normal |
|  | Mapping from REG to CCE |  | Non-Distributed |

Table A.14.4.1.X.1-3: Cell specific test parameters for FR1 PCell

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| SNR_SSB of set q0 | Config 1 | dB | 2 | -6 | -14 |

##### A.14.4.1.X.2	Test Requirements

Test requirements specified in clause A.14.4.1.1.2 apply to this test.

#### A.14.4.1.Y	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode for for less than 5 MHz BW

##### A.14.4.1.Y.1	Test Purpose and Environment

The purpose of this test is to verify that the UE supporting support-3MHz-ChannelBW-r18 properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell operating on a 3 MHz channel bandwidth. This test will partly verify the FR1 radio link monitoring requirements in clause 8.1C.

Supported test configurations are specified in table A.14.4.1.Y.1-1. General test parameters as specified in table A.14.4.1.4.1-2 with config 1 apply to this test, except those specified in table A.14.4.1.Y.1-2. Cell specific test parameters as specified in table A.14.4.1.4.1-3 apply except those specified in table A.14.4.1.Y.1-3.

The test procedure specified in clause A.14.4.1.4.1 applies to this test.

Table A.14.4.1.Y.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 3 MHz (15PRB) |
| NOTE: The UE is required to pass the test with configuration 1 for every supported 3 MHz band in FR1. |  |

Table A.14.4.1.Y.1-2: General test parameters for FR1 in-sync testing in DRX mode

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
| Parameter |  | Unit |  |  | Value |
|  |  |  |  |  | Test 1 |
| BWchannel | Config 1 | MHz |  |  | 3: NRB,c = 15 |
| RMSI CORESET Reference Channel | Config 1 |  |  |  | CR.1.3 FDD |
| Dedicated CORESET Reference Channel | Config 1 |  |  |  | CCR.1.7 FDD |
| SSB Configuration | Config 1 |  |  |  | SSB.13 FR1 |
| In sync transmission parameters | Number of Control OFDM symbols, Config 1 |  |  |  | 3 |
|  | Aggregation level, Config 1 | CCE |  |  | 4 |
|  | Mapping from REG to CCE, Config 1 | Config 1 |  |  | Non-Distributed |
| Out of sync transmission parameters | Number of Control OFDM symbols, Config 1 |  |  |  | 3 |
|  | Aggregation level, Config 1 | CCE |  |  | 8 |
|  | Mapping from REG to CCE, Config 1 | Config 1 |  |  | Non-Distributed |

Table A.14.4.1.Y.1-3: Cell specific test parameters for FR1 (Cell 1) for in-sync radio link monitoring tests in DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| SNR on RLM-RS | Config 1 | dB | 2 | -6 | -14 | -2.5 | 3 |

##### A.14.4.1.Y.2	Test Requirements

Test requirements specified in clause A.14.4.1.4.2 apply to this test.

<End of Change 1>