3GPP RAN WG4 Meeting #116bis		R4-2514628

Prague, Czech Republic, October 13th – 17th, 2025

Title:	Way forward on HPUE notes in 38.101-1 Clause 5 tables

Agenda item:	4.6.1.1

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Source:	Apple, Skyworks Solutions, Inc., Nokia, Samsung

Document for:	Approval

# 1	Background

- One of the main reasons the HPUE applicability notes in band combination configuration tables of the RAN4 UE RF specifications are necessitated is to ensure the corresponding MSD requirements are specified.

- On the other hand, despite many band combinations are not subject to MSD issue, to maintain the specifications consistency on indicating HPUE support for band combinations, the HPUE applicability notes still need to be added to those band combinations via formal CR process.

- As MSD analysis is a rather time-consuming process, enabling HPUE support for band combinations has been relatively slow and there remain many band combinations without the HPUE applicability notes which would prohibit UE from supporting HPUE for those band combinations despite it is hardware capable.

- To ease the HPUE band combination introducing process and simplify the RAN4 specifications, HPUE MSD look-up tables (LUTs) had been introduced in last RAN4 meeting [1].

- With HPUE MSD LUTs, the existing HPUE applicability notes in band combination configuration tables in Clause 5 of the RAN4 UE RF specifications in principle are no longer needed.

- However, due to the concern that the MSD from one-band UL with two CCs and two-band UL with 3 CCs are not covered by the existing HPUE MSD LUTs and there may be cases where PC3 MSD is too low to be specified while its PC2/PC1.5 MSD is substantial enough to be captured, RAN4 is not yet ready to remove the HPUE applicability notes and may require two more meeting cycles of studies and analysis to have a complete solution to replace the HPUE applicability notes [2].

- In this meeting, an interim solution is proposed to enable HPUE support for band combinations without HPUE applicability notes in Clause 5 [3] which has received substantial interest and support. However, due to that this solution requires coordinated changes in the HPUE MSD LUT CR agreed in last RAN4 meeting [1], it is postponed to next RAN4 meeting.

- This way forward intends to provide guidelines on how to handle the HPUE applicability notes in the coming meetings with respect to enabling HPUE support for band combinations whose MSD are covered by LUTs.

# Way forward

Proposal 1: Introduce an interim solution in RAN4 #117 meeting to enable HPUE support for band combinations without HPUE applicability notes in Clause 5 based on [3] and the coordinated changes in the HPUE MSD LUT core requirements of sub-clause 7.3A.2.3.1 and sub-clause 7.3A.2.3.2.

Proposal 2: Companies are encouraged to study the HPUE MSD LUT solution for MSD due to one-band UL with two CCs and two-band UL with 3 CCs and identify cases where PC3 MSD is too low to be specified while its PC2/PC1.5 MSD is substantial enough to be captured in the specifications.

Proposal 3: RAN4 to finalize the process in Proposal 2 and have a complete solution to replace the HPUE applicability notes in Clause 5 by end of RAN4 #118 meeting.

# References

R4-2511752 “Draft CR to 38101-1-j20 on introducing the HPUE MSD requirements simplification”, Skyworks Solutions Inc., et al, 3GPP RAN WG4 Meeting #116, Bengaluru, India, August 25th – 29th, 2025

R4-2514410 “Way Forward on HPUE notes in 38.101-1 Clause 5 tables”, Skyworks Solutions, Inc., 3GPP RAN WG4 Meeting #116bis, Prague, Czech Republic, October 13th – 17th, 2025

R4-2514599 “Draft CR for 38.101-1 on enabling HPUE support for band combinations without HPUE applicability notes in Clause 5”, Apple, 3GPP RAN WG4 Meeting #116bis, Prague, Czech Republic, October 13th – 17th, 2025