`3GPP TSG-RAN WG4 Meeting # 116bis	R4-2514792

Prague, Czech Republic, October 13th ‒ October 17th, 2025

Agenda item:			6.2.1

Source:	Ericsson

Title:	Ad-hoc minutes for NonCol_intraB_ENDC_NR_CA_Ph2_demod

Document for:	Approval

# Introduction

This topic summary summarizes the issues on Rel-19 WI: Support of intra-band non-collocated EN-DC/NR-CA deployment Phase2 (RP-240838).

This topic summary treats AI 6.2.2.

# Topic #1: Work plan

This topic discusses the work plan of WI Support of intra-band non-collocated EN-DC/NR-CA deployment Phase 2 UE demodulation performance part.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2513699 | KDDI, Ericsson | Work plan |

## Open issues summary

This issue discusses the work plan of UE demodulation performance part.

### Sub-topic 1-1	Work plan

Issue 1-1-1: Work plan

- Work plan proposal

  - RAN4#116bis, October 2025 (0.25 TU for Demod/RRM performance)

    - Discuss the scope of UE demodulation requirements.

    - Agree with the initial simulation assumption.

  - RAN4#117, November 2025 (0.25 TU for Demod/RRM performance)

    - Update the simulation assumption if necessary.

    - Collect the initial simulation results.

    - Decide the CR work split.

  - RAN4#118, February 2026 (0.25 TU for Demod/RRM performance)

    - Collect the simulation results.

    - Agree with the draft CRs for TS38.101-4.

    - Agree with the big CR for TS38.101-4 (email approval).

- Recommended WF

  - Confirm the work plan.

Tentative agreement

- Confirm the work plan.

# Topic #2: UE demodulation requirements

This topic discusses the UE demodulation requirements.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2513431 | Huawei, HiSilicon | Proposal 1: Only define CA requirements with Type 4a receiver. Proposal 2: Use following test setup as starting point: - CC1: 2x4, Rank2, QPSK, 0.30 - CC2: 2x4, Rank2, 256QAM, 0.69 - MRTD:33us, power imbalance: 25dB Proposal 3: Specify the following antenna connection: - “For Minimum requirements for non-collocated scenarios for intra-band non-contiguous NR-CA for type 4 UE in Clause xxxx, 4 out of 8Rx should be connected to one CC and the other 4 out of 8Rx should be connected to another CC, depending on UE’s declaration and AP configuration” |
| R4-2513976 | Ericsson | Observation 1: In Rel-18, RAN4 defined Type 2 UE PDSCH demodulation requirements for TDD+TDD NR-CA scenario. Observation 2: Rel-18 Type 2 UE demodulation requirement configures rank 1 with MCS4 in 64QAM table (QPSK 0.30) on for one carrier and rank 2 with MCS21 in 256QAM table (256QAM 0.69) for another carrier. Observation 3: Rel-18 Type 2 UE demodulation requirement configures the timing offset of the SCell from the PCell to 33us. Observation 4: In Rel-18 Type 2 UE demodulation requirement, RAN4 set MCS/rank so that the difference of SNRs of two CCs is less than 25dB. Proposal 1: Define Type 4a UE PDSCH demodulation requirement for 2CC NR-CA scenario, with the configuration FR1 TDD SCS=30kHz + FR1 TDD SCS=30kHz. Proposal 2: RAN4 defines Type 4 UE demodulation requirement, where at least one CC configures rank 3 or rank 4. Proposal 3: RAN4 defines Type 4 UE demodulation requirement where the timing offset of the SCell from the PCell is 33us. Proposal 4: RAN4 sets the MCS/rank of two CCs for Type 4 UE demodulation requirements so that the difference of SNRs of two CCs is less than 25dB, where the SNRs are average of companies’ impairment results. Proposal 5: Assume Tx EVM=3% for both QPSK and 256QAM as same assumption as Rel-18 Type 2 UE demodulation requirements. Proposal 6: Configure CBW/SCS=40MHz/30kHz and 4x4 static channel in TS 38.101-4 B.1.1. Proposal 7: RAN4 should evaluate the following MCS/rank combinations for Type 4 UE demodulation requirements: rank 2 with MCS4/MCS5 in 64QAM MCS table for CC#1 and rank 4 with MCS21/MCS22 in 256QAM MCS table for CC#2. |
| R4-2514287 | Nokia | Observation 1: The objective of the core part provides the following assumptions: up-to 4-layer DL MIMO per CC, MRTD>CP and up to 25 dB power imbalance. Observation 2: With power imbalance of up to 25 dB, each CC can have different MCS and different number of layers. Observation 3: For 2Rx case, TS 38.101-4 Clause 5.2A.2.6 uses the following parameters: SCS 30KHz, BW 40 MHz, QPSK with code rate 0.30 for lower power carrier and 256QAM with code rate 0.69 for the higher power carrier Observation 4: Type 4a FWA UE has 4 antennas for CC#1 and 2 antennas for CC#2, while Type 4b FWA UE has 4 antennas for both CCs. Observation 5: The 25dB power difference is hard on the TEs. While AWGN channels can be used to get enough power out of them, they may not be MIMO/multi-layer compatible. Proposal 1: RAN4 to specify PDSCH requirements for NR-CA with 2 CCs with different MCSs and number of layers for the 2 CCs to reflect the power imbalance, in which at least one of the CC shall take the maximum possible number of layers. Proposal 2: The following parameters can be used as a starting point: SCS 30KHz, BW 40 MHz, QPSK with code rate 0.30 for the lower power carrier and 256QAM with code rate 0.69 for the higher power carrier. Proposal 3: In the case of Type 4a FWA UE, the lower power carrier is assumed to use the lower number of Rx antennas, and, hence, the lower number of layers. Proposal 4: RAN4 to discuss how to enable up-to 4-layer if AWGN channels are used. |
| R4-2514290 | Qualcomm | Observation 1: Existing requirements for NR UE supporting non-colocated intra-band NR CA are defined with rank equal [2,1] for the strong and weak CC respectively, The SNR imbalance between the two CCs is 24.2dB. Proposal 1: RAN4 to introsduce NR-CA PDSCH demodulation simulation for Type 4 UEs supporting intra-band non-collocated. Proposal 2: RAN4 not to introduce EN-DC PDSCH demodulation simulation for Type 4 UEs supporting intra-band non-collocated. Proposal 3: If new PDSCH demodulation requirements for Type 4 UEs supporting intra-band non-collocated NR-CA are introduced, they should be applicable only if the UE indicates support for the intraBandNR-CA-non-collocated-r19 features and providing nonCollocatedTypeNR-CA-v19xy with value “type4”. Proposal 4: RAN4 should confirm with RAN5 that the existing test condition for intraBandNonColocatedCA-r18 which defines a minimum gap of 80MHz for the two CCs is applicable to new requirements in this WI, if introduced. Proposal 5: Simulation assumptions should align with the existing requirements in TS 38.101-4, Section 5.2A.2.6 where applicable. Proposal 6: The Rank to be considered for initial evaluation should be Rank 4 for the “stronger” CC, and Rank 2 for the “weaker” CC. Proposal 7: The final choice of MCS and Rank should consider a maximum SNR difference of 25dB. |

## Open issues summary

### Sub-topic 2-1 Test scope

Issue 2-1-1: Test scenario

- Proposals

  - Only define CA requirements with Type 4a receiver (Huawei, HiSilicon)

  - Define Type 4a UE PDSCH demodulation requirement for 2CC NR-CA scenario, with the configuration FR1 TDD SCS=30kHz + FR1 TDD SCS=30kHz. (Ericsson)

  - RAN4 to specify PDSCH requirements for NR-CA with 2 CCs with different MCSs and number of layers for the 2 CCs to reflect the power imbalance, in which at least one of the CC shall take the maximum possible number of layers. (Nokia)

  - RAN4 to introsduce NR-CA PDSCH demodulation simulation for Type 4 UEs supporting intra-band non-collocated. (Qualcomm)

  - RAN4 not to introduce EN-DC PDSCH demodulation simulation for Type 4 UEs supporting intra-band non-collocated. (Qualcomm)

- Observations

  - Type 4a FWA UE has 4 antennas for CC#1 and 2 antennas for CC#2, while Type 4b FWA UE has 4 antennas for both CCs. (Nokia)

- Recommended WF

  - Moderator: According to UE RF conclusion, NR-CA is only applicable for Type 4b (See Appendix).

  - Define PDSCH demodulation requirements for Type 4b UE with 2CC NR-CA scenario.

    - Specify FR1 TDD + FR1 TDD only.

Tentative agreement

Define PDSCH demodulation requirements for Type 4b UE with 2CC NR-CA scenario.

- Specify FR1 TDD + FR1 TDD only.

Issue 2-1-2: Received time difference between two carriers

- Observation (Ericsson):

  - Rel-18 Type 2 UE demodulation requirement configures the timing offset of the SCell from the PCell to 33us.

- Proposals (Huawei, HiSilicon, Ericsson):

  - Timing offset of the SCell from the PCell is 33us.

- Recommended WF

  - Configure the timing offset of the SCell from the PCell is 33 µs.

Tentative agreement

Configure the timing offset of the SCell from the PCell is 33 µs.

### Sub-topic 2-2 Simulation assumption

Issue 2-2-1: Channel bandwidth and SCS

- Proposals (Ericsson, Nokia, Qualcomm):

  - CBW=40MHz and SCS=30kHz

- Recommended WF

  - Configure CBW=40MHz and SCS=30kHz for both CCs.

Tentative agreement

Configure CBW=40MHz and SCS=30kHz for both CCs.

Issue 2-2-2: MCS/rank

- Proposals

  - Option 1 (Huawei, HiSilicon):

    - CC#1: Rank 2, QPSK 0.30

    - CC#2: Rank 2, 256QAM 0.69

  - Option 2 (Ericsson):

    - CC#1: Rank 2, MCS4/MCS5 in 64QAM MCS table

    - CC#2: Rank 4, MCS21/MCS22 in 256QAM MCS table

  - Option 3 (Nokia):

    - QPSK with code rate 0.30 for the lower power carrier and 256QAM with code rate 0.69 for the higher power carrier.

  - Option 4 (Qualcomm):

    - The Rank to be considered for initial evaluation should be Rank 4 for the “stronger” CC, and Rank 2 for the “weaker” CC.

- Recommended WF

Evaluate the following rank/MCS options.

  - For weaker CC:

    - Option 1: Rank 2 + QPSK 0.3 (MCS4 in MCS index table 1)

    - Option 2: Rank 2 + QPSK 0.37 (MCS5 in MCS index table 1)

  - For stronger CC:

    - Option 1: Rank 2 + 256QAM 0.69 (MCS21 in MCS index table 2)

    - Option 2: Rank 4 + 256QAM 0.69 (MCS21 in MCS index table 2)

    - Option 3: Rank 4 + 256QAM 0.74 (MCS22 in MCS index table 2)

Discussion

Apple: For stronger CC, we should focus on rank 4.

Huawei: Type 4a UE can be configured for rank 2.

Qualcomm: Support rank 4 only for stronger CC.

Tentative agreement

Evaluate the following rank/MCS options.

- For weaker CC:

  - Option 1: Rank 2 + QPSK 0.3 (MCS4 in MCS index table 1)

- For stronger CC:

  - Option 1: Rank 2 + 256QAM 0.69 (MCS21 in MCS index table 2)

  - Option 2: Rank 4 + 256QAM 0.69 (MCS21 in MCS index table 2)

  - Option 3: Rank 4 + 256QAM 0.74 (MCS22 in MCS index table 2)

Issue 2-2-3: Channel model and antenna configuration

According to the RAN4 RF assumption, Type 4b UE has 8 Rx antennas, and it can use 4 of 8Rx antennas for one CC and other 4 of 8Rx antennas for another CC. In Rel-18, Type 2 UE demodulation requirements set 2x2 static channel model.

- Proposals

  - Use 2x4 (Huawei)

  - Specify the following antenna connection: “For Minimum requirements for non-collocated scenarios for intra-band non-contiguous NR-CA for type 4 UE in Clause xxxx, 4 out of 8Rx should be connected to one CC and the other 4 out of 8Rx should be connected to another CC, depending on UE’s declaration and AP configuration” (Huawei, HiSilicon)

  - Use 4x4 static channel in TS 38.101-4 B.1.1 (Ericsson)

  - In the case of Type 4a FWA UE, the lower power carrier is assumed to use the lower number of Rx antennas, and, hence, the lower number of layers (Nokia)

  - RAN4 to discuss how to enable up-to 4-layer if AWGN channels are used. (Nokia)

- Recommended WF

  - Use the static channel model specified in TS 38.101-4 B.1.1.

    - Option 1: 2x4 for each CC

    - Option 2: 2x4 for weaker CC, 4x4 for stronger CC

    - Option 3: 4x4 for each CC

  - Specify the following antenna connection in TS 38.101-4:

    - “For Minimum requirements for non-collocated scenarios for intra-band non-contiguous NR-CA for type 4 UE in Clause xxxx, 4 out of 8Rx should be connected to one CC and the other 4 out of 8Rx should be connected to another CC, depending on UE’s declaration and AP configuration”

Discussion

Qualcomm: Support Option 3.

Apple: Support Option 3.

Huawei: Use 2x4 for rank 2, use 4x4 for rank 4.

Tentative agreement

Use the static channel model specified in TS 38.101-4 B.1.1.

- Option 1: 2x4 for CC with rank 2, 4x4 for CC with rank 4.

- Option 2: 4x4 for each CC

Specify the following antenna connection in TS 38.101-4:

- “For Minimum requirements for non-collocated scenarios for intra-band non-contiguous NR-CA for type 4 UE in Clause xxxx, 4 out of 8Rx should be connected to one CC and the other 4 out of 8Rx should be connected to another CC, depending on UE’s declaration and AP configuration”

Issue 2-2-4: How to choose MCS/rank

- Observation (Ericsson):

  - In Rel-18 Type 2 UE demodulation requirement, RAN4 set MCS/rank so that the difference of SNRs of two CCs is less than 25dB.

- Proposals

  - Power imbalance: 25dB (Huawei, HiSilicon)

  - RAN4 sets the MCS/rank of two CCs for Type 4 UE demodulation requirements so that the difference of SNRs of two CCs is less than 25dB, where the SNRs are average of companies’ impairment results (Ericsson)

  - The final choice of MCS and Rank should consider a maximum SNR difference of 25dB (Qualcomm)

- Recommended WF

  - The final choice of MCS and Rank should consider the maximum SNR difference of 25dB, where the SNRs are average of companies’ impairment results.

Tentative agreement

- The final choice of MCS and Rank should consider the maximum SNR difference of 25dB, where the SNRs are average of companies’ impairment results, plus additional margin (0.5dB for QPSK, 0.8dB for 256QAM).

Issue 2-2-5: Tx EVM assumption

- Proposals (Ericsson)

  - Assume Tx EVM=3% for both QPSK and 256QAM as the same assumption as Rel-18 Type 2 UE demodulation requirements.

- Recommended WF

  - Assume Tx EVM=3% for both CCs.

Tentative agreement

- Assume Tx EVM=3% for both CCs.

### Sub-topic 2-3 Applicability

Issue 2-3-1: Separation of two CCs

- Proposals

  - RAN4 should confirm with RAN5 that the existing test condition for intraBandNonColocatedCA-r18 which defines a minimum gap of 80MHz for the two CCs is applicable to new requirements in this WI, if introduced (Qualcomm).

- Recommended WF

  - Discuss whether to send a LS to RAN5 on the separation of CC as same as Rel-18.

  - Moderator thinks it is reasonable to send a similar LS to RAN5 as we have sent in Rel-18, because of the fact that:

    - In Rel-18, RAN4 sent LS R4-2316951 to RAN5.

RF conformance tests should introduce the following test condition in RAN5: - Center of BWanother relative to edge of BWwanted is assumed to be at least 80MHz+BWanother/2 away from the edge of the wanted CC in R18. The same agreement should be applied to NR-CA PDSCH demodulation requirement.

    - In RAN4#116, RF session agreed to send LS R4-2511768 to RAN5.

RAN4 agreed that the following test condition should be introduced in the corresponding conformance test case for both type of UEs. - Center of BWanother relative to edge of BWwanted is assumed to be at least 80MHz+BWanother/2 away from the edge of the wanted CC in R19

Tentative agreement

Send LS to RAN5 as same as Rel-18 NonCol_intraB_ENDC_NR_CA UE demodulation performance part in RAN4#117.

Qualcomm volunteers to send LS to RAN5.

Issue 2-3-2: Test applicability

- Proposals

  - If new PDSCH demodulation requirements for Type 4 UEs supporting intra-band non-collocated NR-CA are introduced, they should be applicable only if the UE indicates support for the intraBandNR-CA-non-collocated-r19 features and providing nonCollocatedTypeNR-CA-v19xy with value “type4”. (Qualcomm)

- Recommended WF

  - If new PDSCH demodulation requirements for Type 4 UEs supporting intra-band non-collocated NR-CA are introduced, it is applicable for UEs with the capability of:

    - Supporting ‘intraBandNR-CA-non-collocated-r19’,

    - Providing ‘nonCollocatedTypeNR-CA-v1900’ with value ‘type4’.

Tentative agreement

Type 4b UE demodulation requirement is applicable for UEs with the capability of:

- Supporting ‘intraBandNR-CA-non-collocated-r19’,

- Providing ‘nonCollocatedTypeNR-CA-v1900’ with value ‘type4’.

# Appendix (WID: RP-242083, WF: R4-2217734)

| UE Type | CC# | antenna / LNA | Mixer | Analog BB | #Rx | NRCA / ENDC | power imbalance | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 (NR) | 4 shared | 4 shared | 4 shared | 4Rx | NR-CA EN-DC | 6dB full range | Baseline architecture (i.e. legacy architecture) |
|  | 2 (LTE/NR) |  |  |  | 4Rx |  |  |  |
| 2 | 1 (NR) | 2 | 2 | 2 | 2Rx | NR-CA EN-DC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 (LTE/NR) | 2 | 2 | 2 | 2Rx |  |  |  |
| 4a | 1 (NR) | 4 | 6 total | 4 | 4Rx | EN-DC | 25dB full range | Requires 6 antennas and FWA |
|  | 2 (LTE/NR) | 2 |  | 2 | 2Rx |  |  |  |
| 4b | 1 (NR) | 4 | 8 total | 4 | 4Rx | NR-CA EN-DC | 25dB full range | Requires 8 antennas and LNA |
|  | 2 (LTE/NR) | 4 |  | 4 | 4Rx |  |  |  |