3GPP TSG-RAN4 Meeting #116-bis	R4-2515047

# Prague, Czech Republic, 13th Oct. – 17th Oct. 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.102 | CR | - | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to introduce a new annex to 36.102 for NGSO time varying doppler shift and propagation delay modelling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IoT_NTN_req_test_enh-Perf |  |  |  |  |  | Date: |  |  | 2025-10-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | It is necessary to capture the agreements on the time varying Doppler shift and propagation delay modelling for NGSO as normative conditions. Note the initial conditions in annex E.4.1 are based on the WF (R4-2512525). However, since there was one typo with the agreed coordinate in the WF, positionX-r17 is described based on the correct value taken from GMAT coordinate for LEO-600. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Created a new Annex E and captured following agreements: - Calculation procedures and formulae based on Newton method - Calculation procedures and formulae based on 4th order Runge-Kutta method - Calculation of Doppler shift, propagation delay and elevation angle. - Initial conditions of NTN Ephemeris information - Descriptions on the influence of the receiver movement on propagation delay - Accuracy requirements for TE to obtain delay and Doppler shift |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Normative conditions remain undefined regarding the methods to calculate time varying Doppler shift and propagation delay for NGSO. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (New) Annex E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2513920. Some wording improvements are made. |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

<<Start of change>>

Annex D (normative): 
Propagation conditions

# D.1	Multi-path fading propagation conditions

The multipath propagation conditions consist of several parts:

-	A delay profile in the form of a "tapped delay-line", characterized by a number of taps at fixed positions on a sampling grid. The profile can be further characterized by the r.m.s. delay spread and the maximum delay spanned by the taps.

-	A combination of channel model parameters that include the Delay profile and the Doppler spectrum that is characterized by a classical spectrum shape and a maximum Doppler frequency.

## D.1.1	Delay profiles

The delay profiles are derived from the TR 38.811 [13] NTN-TDL models for the desired delay spread and tap resolution. After scaling the normalized delay spread values for each tap by the desired RMS delay spread, the tap delays are quantized to a delay resolution of 5ns by rounding to the nearest multiple of the delay resolution.

Table D.1.1-1: Delay profiles for IoT NTN channel models

| Type | Model | Delay spread (r.m.s.) | Delay resolution |
| --- | --- | --- | --- |
| NLOS | NTN-TDLA100 | 100 ns | 5 ns |
| LOS | NTN-TDLC5 | 5 ns | 5 ns |

Table D.1.1-2: NTN-TDLA100 (DS = 100 ns)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | 0 | Rayleigh |
| 2 | 110 | -4.7 | Rayleigh |
| 3 | 285 | -6.5 | Rayleigh |

Table D.1.1-3 NTN-TDLC5 (DS = 5 ns)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.6 | LOS path |
|  | 0 | -8.9 | Rayleigh |
| 2 | 60 | -21.5 | Rayleigh |
| Note 1: Tap #1 follows a Rician distribution. |  |  |  |

## D.1.2	Combinations of channel model parameters

The propagation conditions used for the performance measurements in multi-path fading environment are indicated as a combination of a channel model name and a maximum Doppler frequency, i.e., NTN-TDLA<DS>-<Doppler>, or NTN-TDLC<DS>-<Doppler> where '<DS>' indicates the desired delay spread and '<Doppler>' indicates the maximum Doppler frequency (Hz).

Table D.1.2-1 show the propagation conditions that are used for the performance measurements in multi-path fading environment for NLOS and LOS propagation conditions.

Table D.1.2-1: Channel model parameters for NTN

| Combination name | Model | Maximum Doppler frequency |
| --- | --- | --- |
| NTN-TDLA100-10 | NTN-TDLA100 | 10 Hz |
| NTN-TDLA100-200 | NTN-TDLA100 | 200 Hz |
| NTN-TDLC5-30 | NTN-TDLC5 | 30 Hz |
| NTN-TDLC5-200 | NTN-TDLC5 | 200 Hz |

Annex E (Normative):  
Modelling of time varying Doppler shift and propagation delay for NGSO

E.1	General

This annex specifies the methodologies for time varying Doppler shift and propagation delay modelling for NGSO.

E.2	Satellite position/velocity estimation

As a first step, it is necessary to perform the satellite’s orbital calculations. There are two methodologies applicable to model the time varying Doppler shift and propagation delay by a test equipment, i.e. Newton-Raphson method to solve Keplerian model and fourth-order Runge-Kutta method to solve equation of motion. It is up to the implementation for test equipment on which method to apply to.

Table E.2-1: Input values for calculation of satellite orbit

| Parameters | Unit | Description |
| --- | --- | --- |
| r0ECEF=r0,xECEFr0,yECEFr0,zECEF | km | Initial satellite position state vector at time 0 in Earth-centred earth-fixed frame (ECEF) |
| v0ECEF=v0,xECEFv0,yECEFv0,zECEF | km/s | Initial satellite velocity state vector at time 0 in Earth-centred earth-fixed frame (ECEF) |
| t | second | Time to derive the satellite position and velocity |

Table E.2-2: Output values

| Parameters | Unit | Description |
| --- | --- | --- |
| rtECEF=rt,xECEFrt,yECEFrt,zECEF | km | Satellite position state vector at time t in Earth-centred earth-fixed frame (ECEF) |
| vtECEF=vt,xECEFvt,yECEFvt,zECEF | km/s | Satellite velocity state vector at time t in Earth-centred earth-fixed frame (ECEF) |

Table E.2-3: Constant parameters

| Parameters | Description | Values | Unit |
| --- | --- | --- | --- |
| μ | Gravitational parameter for Earth | 3.986004418 x 105 | km2/s2 |
| ωE | Earth angular speed | 7.2921151467 x 10-5 | rad/s |

E.2.1	Newton-Raphson method

E.2.1.1 Keplerian model based estimation

The method specified in this sub clause is applicable to cases that Eccentricity (e) in Step 1-4 is more than zero. In case e=0, use the method specified in E.2.2.

Step 1	Derive six orbital elements (a, e, i, Ω, ω, M0)

Step 1-0	Convert the initial position/velocity state vectors from ECEF format to ECI format

Note we assume the x-axis of ECI and ECEF the same for simplicity.

r0=r0ECEF=r0,xECEFr0,yECEFr0,zECEF

v0ECI=v0,xECIv0,yECIv0,zECI=v0ECEF+00ωE×r0ECEF

=v0,xECEF-ωEr0,yECEFv0,yECEF+ωEr0,xECEFv0,zECEF

Step 1-1	Position magnitude (r), velocity magnitude (v), and orbital angular momentum (h)

r=r0ECI=r0,xECI2+r0,yECI2+r0,zECI2

v=v0ECI=v0,xECI2+v0,yECI2+v0,zECI2

vr=v0ECI⋅r0ECIr=r0,xECIv0,xECI+r0,yECIv0,yECI+r0,zECIv0,zECIr

h=hxhyhz=r×v=r0,yECIv0,zECI-r0,zECIv0,yECIr0,zECIv0,xECI-r0,xECIv0,zECIr0,xECIv0,yECI-r0,yECIv0,xECI

h=h=hx2+hy2+hz2

Step 1-2	Inclination (INC, i)

K=[001]

i=cos-1h⋅Kh=cos-1hzh

Note the range of INC is between 0 and π (radian).

Step 1-3	Right Ascension of the Ascending Node (RAN, Ω)

n=nxnynz=K×h=-hyhx0

n=n=nx2+ny2+nz2

Ω=cos-1nxn,ny≥02π-cos-1nxnny<0

Note the range of RAN is between 0 and 2π (radian).

Note Ω is called as ‘Longitude of ascending node’ in TS 36.331 [6] ephemerisInfo-r17.

Step 1-4	Eccentricity (ECC, e), Semi-major axis (SMA, a), Period (P)

e=exeyezT=1μv2-μrr0ECI-r0ECI⋅v0ECIv0ECI=1μv2-μrr0,xECIr0,yECIr0,zECIT-r⋅vrv0,xECIv0,yECIv0,zECIT

e=e=ex2+ey2+ez2

a=h2μ(1-e2)

Period of the satellite around earth, P (sec), is given by:

P=2πa3μ

Step 1-5	Argument of Periapsis (AP, ω)

ω=cos-1e⋅n e⋅n=cos-1exnx+eyny+eznzenx2+ny2+nz2,ez≥02π-cos-1exnx+eyny+eznzenx2+ny2+nz2,ez<0

Note the range of AP is between 0 and 2π (radian).

Step 1-6	Mean Anomaly at time 0 (MA, M0)

True Anomaly at time 0 (ν0):

ν0=cos-1e⋅r0ECIe⋅r=cos-1exr0,xECI+eyr0,yECI+ezr0,zECIe⋅rvr≥02π-cos-1exr0,xECI+eyr0,yECI+ezr0,zECIe⋅rvr<0

Eccentric Anomaly at time 0 (E0):

E0=2tan-11-e1+etanv02

Mean Anomaly at time 0 (M0):

M0=E0-esinE0

Note the range of M0 is between 0 and 2π (radian).

Step 2	Determine the satellite position and velocity at time t (sec)

Step 2-1	Mean Anomaly at time t (Mt):

Mt=M0+2πtP=M0+tμa3

Step 2-2	Derive Eccentric Anomaly at time t (Et) by solving Kepler’s equation with Newton-Raphson method

Build the Kepler’s equation between Mt and Et:

Mt=Et-esinEt

Step 2-2-1: Set Et(0)=Mt

Step 2-2-2: Calculate fEt(n)=Et(n)-esinEtn-Mt

Step 2-2-3: Calculate f'Et(n)=dfEt(n)dEt(n)=1-ecosEt(n)

Step 2-2-4: Update Et(n+1) from Et(n), fEt(n), and f'Et(n) , as follows:

Et(n+1)=Et(n)-fEt(n)f'Et(n)=Et(n)-Et(n)-esinEt(n)-Mt1-ecosEt(n)

Step 2-2-5: If n≥4, then set Et=Et(n+1) and go to Step 2-3. Otherwise, go to Step 2-2-2 by setting n := n+1.

Step 2-3 Derive True Anomaly at time t (νt)

νt=2tan-11+e1-etanEt2

Note the range of νt is between 0 and 2π (radian).

Step 2-4	Convert the orbital elements to the state vector in ECI frame

Convert to the state vector in perifocal frame,

rt,xpqwrt,ypqwrt,zpqw=rtcosνtsinνt 0=h2μ11+ecosνtcosνtsinνt0=a1-e21+ecosνtcosνtsinνt0

vt,xpqwvt,ypqwvt,zpqw=μh-sinνt e+cosνt0=μa1-e2-sinνt(e+cosνt)0

Convert the state vector from perifocal frame to ECI.

rt,xECIrt,yECIrt,zECI=cosΩ-sinΩ0sinΩcos(Ω)00011000cosi-sini0sin(i)cos(i)cosω-sinω0sinωcosω0001rt,xpqwrt,ypqwrt,zpqw

vt,xECIvt,yECIvt,zECI=cosΩ-sinΩ0sinΩcos(Ω)00011000cosi-sini0sin(i)cos(i)cosω-sinω0sinωcosω0001vt,xpqwvt,ypqwvt,zpqw

Convert the state vector from ECI to ECEF.

rt,xECEFrt,yECEFrt,zECEF=cos-ωEt-sin-ωEt0sin-ωEtcos(-ωEt)0001rt,xECIrt,yECIrt,zECI

vt,xECEFvt,yECEFvt,zECEF=cos-ωEt-sin-ωEt0sin-ωEtcos-ωEt0001vt,xECIvt,yECIvt,zECI-0-ωE0ωE00000rt,xECEFrt,yECEFrt,zECEF

E.2.2	Fourth-order Runge-Kutta method

E.2.2.1 Equations of motion based estimation

Step 1: Initialization

n:=0

rnΔt=rnΔt,xrnΔt,yrnΔt,z=r0,xECEFr0,yECEFr0,zECEF=r0ECEF

vnΔt=vnΔt,xvnΔt,yvnΔt,v=v0,xECEFv0,yECEFv0,zECEF=v0ECEF

Set time step size of position/velocity updates. Set Δt=1 (sec), for example, if updating the satellite position/velocity every one second.

Step 2: Solve the equation of motion with the fourth-order Runge-Kutta method.

k1,r=k1,r,xk1,r,yk1,r,z=vnΔtΔt

k1,v=k1,v,xk1,v,yk1,v,z=frnΔt,vnΔtΔt

k2,r=k2,r,xk2,r,yk2,r,z=vnΔt+12k1,vΔt

k2,v=k2,v,xk2,v,yk2,v,z=frnΔt+12k1,r,vnΔt+12k1,vΔt

k3,r=k3,r,xk3,r,yk3,r,z=vnΔt+12k2,vΔt

k3,v=k3,v,xk3,v,yk3,v,z=frnΔt+12k2,r,vnΔt+12k2,vΔt

k4,r=k4,r,xk4,r,yk4,r,z=vnΔt+k3,vΔt

k4,v=k4,v,xk4,v,yk4,v,z=frnΔt+k3,r,vnΔt+k3,vΔt

rn+1Δt=rn+1Δt,xrn+1Δt,yrn+1Δt,z=rnΔt+16k1,r+2k2,r+2k3,r+k4,r

vn+1Δt=vn+1Δt,xvn+1Δt,yvn+1Δt,z=vnΔt+16k1,v+2k2,v+2k3,v+k4,v

where

frt,vt=frt,x,rt,y,rt,z,vt,x,vt,y,vt,z=-μrt3rt,x+2ωEvt,y+ωE2rt,x-μrt3rt,y-2ωEvt,x+ωE2rt,y-μrt3rt,zT

rt=|rt|=rt,x2+rt,y2+rt,z2

Step 3: Determine the satellite position/velocity at time t.

If t=n+1Δt:

rtECEI=rn+1Δt=rn+1Δt, xrn+1Δt, yrn+1Δt, z

vtECEI=vn+1Δt=vn+1Δt, xvn+1Δt, yvn+1Δt, z

Otherwise, set n≔n+1, and go to Step 1.

E.3	Varying Doppler shift and propagation delay generation

E.3.1 Determine UE position

This step calculates the UE position according to the earth angular speed. This clause assumes UE location is given by a format of latitude, longitude, and altitude. This clause specifies how to convert values to ECEF format to align with the satellite position and velocity state vectors.

Table E.3.1-1: Input values for calculation of UE position

| Parameters | Unit | Description |
| --- | --- | --- |
| UElatitude | degree | Initial UE latitude |
| UElontitude | degree | Initial UE longitude |
| UEaltitude | km | Initial UE altitude |

Table E.3.1-2: Output values

| Parameters | Unit | Description |
| --- | --- | --- |
| UEt,xECEFUEt,yECEFUEt,zECEF | km | UE position state vector at time t in Earth-centred earth-fixed frame (ECEF) |

Table E.3.1-3: Constant parameters

| Parameters | Description | Values | Unit |
| --- | --- | --- | --- |
| aE | Earth radius | 6378.137 | km |
| eE2 | Square of Earth eccentricity | 6.6943799014 x 10-3 |  |

ϕ=π180UElatitude

λ=π180UElongitude

N=aE1-eE2sin2ϕ

Since the UE is stationary during the tests, UE position does not change regardless of time t.

UEt,xECEF=N+UEaltitudecosϕcosλ UEt,yECEF=N+UEaltitudecosϕsinλUEt,zECEF=N1-eE2+UEaltitudesinϕ

E.3.2 Doppler shift and propagation delay

This step calculates the Doppler shift and propagation delay based on the satellite position/velocity and UE position state vectors calculated in the earlier steps.

Table E.3.2-1: Input values for calculation of Doppler shift and propagation delay

| Parameters | Unit | Description |
| --- | --- | --- |
| UEt,xECEFUEt,yECEFUEt,zECEF | km | UE position state vector at time t in Earth-centred earth-fixed frame (ECEF) |
| rtECEF=rt,xECEFrt,yECEFrt,zECEF | km | Satellite position state vector at time t in Earth-centred earth-fixed frame (ECEF) |
| vtECEF=vt,xECEFvt,yECEFvt,zECEF | km/s | Satellite velocity state vector at time t in Earth-centred earth-fixed frame (ECEF) |
| fc | Hz | Carrier frequency (e.g., 2.0 x 109 for band n256) |

Table E.3.2-2: Output values

| Parameters | Unit | Description |
| --- | --- | --- |
| Doppler | Hz | Doppler shift at time t |
| DelayDL | sec | Downlink propagation delay at time t |
| DelayUL | sec | Uplink propagation delay at time t |

Table E.3.2-3: Constant parameters

| Parameters | Description | Example of values | Unit |
| --- | --- | --- | --- |
| c | Speed of light | 299792.458 | km/s |
| ωE | Earth angular speed | 7.2921151467 x 10-5 | rad/s |

Distance between satellite and UE is derived as follows:

ρtECEF=ρt,xECEFρt,yECEFρt,zECEF=rt,xECEF-UEt,xECEFrt,yECEF-UEt,yECEFrt,zECEF-UEt,zECEF

ρtECEF=ρt,xECEF2+ρt,yECEF2+ρt,zECEF2

Using the distance above and the following range rate, Doppler shift is also derived as follows:

RangeRate(t)=ρtECEF⋅vtECEFρtECEF=ρt,xECEF⋅vt,xECEF+ρt,yECEF⋅vt,yECEF+ρt,zECEF⋅vt,zECEFρtECEF

Doppler(t)=-RangeRate(t)×fcc

Next, total propagation delay for downlink and uplink case are derived as follows. Note an influence of the receiver movement (i.e. either UE movement due to the Earth’s rotation or Satellite movement) is considered in the following equations.

vtECI=vt,xECIvt,yECIvt,zECI=vtECEF+00ωE×rtECEF

=v0,xECEF-ωEr0,yECEFv0,yECEF+ωEr0,xECEFv0,zECEF

DelayDLtR=ρtRECEF/c1+1cvtRECI⋅ρtRECEFρtRECEF

Note tR is the reception time at UE.

DelayULtT=ρtTECEF/c1-1cvtTECI⋅ρtTECEFρtTECEF

Note tT is the transmission time at UE. It is not mandated that UE follows this formula for UL propagation delay estimation.

Doppler shift and propagation delay for LEO-600, calculated by the Newton-Raphson and fourth-order Runge-Kutta methods, are shown in Figure E.3.2-1 and E.3.2-2. Refer to Annex E.4 for initial conditions.

Figure E.3.2-1: Doppler shift of LEO-600 calculated by Newton-Raphson method and fourth-order Runge-Kutta method

Figure E.3.2-2: Propagation delay of LEO-600 calculated by Newton-Raphson method and fourth-order Runge-Kutta method

E.3.3 Elevation angle and Azimuth angle (for information)

An elevation angle, El (degrees), of the satellite position from the UE viewpoint at time t can be derived as follows:

e=-sinλcosλ0, n=-sinϕcos λ-sinϕsin λcosϕ, u=cosϕcosλcosϕsinλsinϕ

Note e, n, and u are the unit vectors for east, north, and zenith direction at the UE position at time t.

lt,e=ρtECEI⋅e=ρt,xECEI⋅ex+ρt,yECEI⋅ey+ρt,zECEI⋅ez

lt,n=ρtECEI⋅n=ρt,xECEI⋅nx+ρt,yECEI⋅ny+ρt,zECEI⋅nz

lt,u=ρtECEI⋅u=ρt,xECEI⋅ux+ρt,yECEI⋅uy+ρt,zECEI⋅uz

lt,h=lt,e2+lt,n2

Elt=180πtan-1lt,ult,h

Azimuth angle, Az (degrees), is also derived as follows.

Azt=180πtan-1lt,elt,n

For example, elevation angle of LEO-600 with maximum positive Doppler scenario is shown in Figure E.3.3-1. Starting the elevation angle from 30 degrees, available connection time between UE and one satellite is approximately 250 seconds.

Figure E.3.3-1: Elevation angle for LEO-600 with maximum positive Doppler scenario

E.4 Initial conditions for numerical solution

E.4.1 Initial ephemeris information for satellites

For LEO-600 scenario, the initial position and velocity of a satellite is specified in Table E.4.1-1. The initial position of a UE is specified in E.4.2 (TS 36.508 [14]). Note these values are described in ECEF frame.

Table E.4.1-1: Initial ephemeris information for NGSO satellites (LEO-600 with Max Doppler)

| Information Element | Value/remark |
| --- | --- |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |
| ephemerisInfo-r17 CHOICE { |  |
| stateVectors SEQUENCE { |  |
| positionX-r17 | -2654549 |
| positionY-r17 | 4386991 |
| positionZ-r17 | 1594205 |
| velocityVX-r17 | 14581 |
| velocityVY-r17 | -34487 |
| velocityVZ-r17 | 120182 |
| } |  |
| NOTE 1: Satellite-UE elevation angle equal to 30.11 degrees, one-way delay equal to 3.60 ms and Doppler equal to 19.83 ppm. |  |

The position and velocity of the satellite described in ECEF Frame need to be converted based on the definitions in TS 36.331 [6] as EphemerisInfo field descriptions.

EphemerisInfo field descriptions

positionX, positionY, positionZ X, Y, Z coordinate of satellite position state vector in ECEF. Unit is meter. Step of 1.3 m. Actual value = field value * 1.3.

velocityVX, velocityVY, velocityVZ X, Y, Z coordinate of satellite velocity state vector in ECEF. Unit is meter/second. Step of 0.06 m/s. Actual value = field value * 0.06.

The converted satellite coordinate and velocity in ECEF Frame are as follows.

Satellite coordinate: ( -3450913.7 [m], 5703088.3 [m], 2072466.5 [m])

Satellite velocity: (874.86 [m/s], -2069.22 [m/s], 7210.92 [m/s])

E.4.2 Initial location information for UE

UE location can be referred from TS 36.508 [14].

5.6.1 UE location UE shall determine its location during the test using any of the following means. 1. UE location for the test shall be provided to the UE via pre-configured means. During the test the UE location is not expected to change unless explicitly stated as a requirement for the test. 2. Other options such as providing UE location via AT command are not precluded. UE location provided to the UE before the start of the test for RF Tx/Rx and DEMOD test cases for both GSO and NGSO config is as follows: Longitude: 121.56076999 Latitude: 25.08439333 (NGSO satellites), 55.0 (GSO satellites) Altitude: 0

Based on the equations described in the annex E.3.1, converted UE location in ECEF Frame is as follows.

UE location: (-3025296.935779 [m], 4925102.789830 [m], 2687544.210048 [m])

E.5	Accuracy requirements for maximum delay errors to obtain the propagation delay and Doppler shift trajectory in TE side

The mean of absolute error for Doppler shift should be less than 1 Hz and the mean of absolute error for propagation delay should be less than 0.05µs based on the following error calculation method.

ErrDopplert=11000n=11000Dopplerbaselinet+n1000-Dopplerinterpt+n1000, t=0,1,2,…(s)

ErrDelayt=11000n=11000Delayrbaselinet+n1000-Delayrinterpt+n1000, t=0,1,2,…(s)

where Dopplerbaselinen, Dopplerinterp(n), Delayrbaselinen and Delayinter(n) are calculated once per 1ms, and the average is taken over every 1 second.

Annex F (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-08 | RAN4#104-e | R4-2215118 |  |  |  | Initial Skeleton | 0.1.0 |
| 2022-10 | RAN4#105 | R4-2218376 |  |  |  | Added approved TPs in RAN4#104-bis-e including: R4-2217750, R4-2217752, R4-2217753, R4-2217755, R4-2217807, R4-2217810 | 0.2.0 |
| 2022-11 | RAN4#105 | R4-2218377 |  |  |  | Added approved TPs in RAN4#105 including: R4-2218767, R4-2220803, R4-2220804, R4-2220805, R4-2220806, R4-2220812, R4-2220828, R4-2220835, R4-2220836 | 0.3.0 |
| 2022-12 | RAN#98-e | RP-223233 |  |  |  | 1-step Approval of version 1.0.0 | 1.0.0 |

<<End of change >>