3GPP TSG-RAN WG4 Meeting #116bis										                  R4-2515165

Prague, Czech Republic, 13 – 17 October, 2025

Agenda item:			6.23

Source:	Moderator (Qualcomm)

Title:	WF for [116bis][309] Rel-19 RRM performance_Part1

Document for:	Approval

# Introduction

This WF captures all the agreements, open issues and work-split related to performance requirements for R19 WI NR_duplex_evo, based on the documents submitted under agenda 6.13.3.1 and 6.13.3.2.

# Agreements

# Topic #1: UE-to-UE CLI handling

### Issue 1-1-1: Time offset for L1-SRS-RSRP measurements

- Agreement

  - Specify the time offset conditions for L1-SRS-RSRP measurement accuracy requirements with cell phase error for 30kHz SCS and without cell phase error for other SCS

### Issue 1-1-2: SRS BW for L1-SRS-RSRP measurements

- Agreement

  - SRS BW of 24 RBs shall be used for performance requirements of L1-SRS-RSRP measurements

### Issue 1-1-3: SRS Es/Iot for L1-SRS-RSRP measurements

- Agreement

  - SRS Es/Iot side-condition of 1db shall be used for performance requirements of L1-SRS-RSRP measurements

### Issue 1-1-4: L1-SRS-RSRP measurement accuracy requirements

- Agreement

  - L1-SRS-RSRP measurements accuracy requirements are defined based on simulation results for the agreed side conditions

### Issue 1-2-1: L1-CLI-RSSI measurement accuracy requirements

- Agreement

  - Reuse the R16 CLI-RSSI measurement accuracy requirements for L1-CLI-RSSI measurements

### Issue 1-3-1: Absolute report mapping

- Agreement

  - Re-use Table 10.1.22.1.2-1 and Table 10.1.22.2.2-1 for report mapping of absolute L1-SRS-RSRP and L1-CLI-RSSI, respectively.

### Issue 1-3-2: Differential report mapping

- Agreement

  - Use Table 1 and Table 2 (based on Table 10.1.6.1-2) for reporting mapping of differential L1-SRS-RSRP and L1-CLI-RSSI, respectively.

Table 1: Reporting mapping of differential L1-SRS-RSRP

| Reported value | Measured quantity value (difference in measured SRS-RSRP from strongest SRS-RSRP) | Unit |
| --- | --- | --- |
| DIFF_SRS-RSRP_0 | 0≥ΔSRS-RSRP>-2 | dB |
| DIFF_SRS-RSRP_1 | -2≥ΔSRS-RSRP>-4 | dB |
| DIFF_SRS-RSRP_2 | -4≥ΔSRS-RSRP>-6 | dB |
| DIFF_SRS-RSRP_3 | -6≥ΔSRS-RSRP>-8 | dB |
| DIFF_SRS-RSRP_4 | -8≥ΔSRS-RSRP>-10 | dB |
| DIFF_SRS-RSRP_5 | -10≥ΔSRS-RSRP>-12 | dB |
| DIFF_SRS-RSRP_6 | -12≥ΔSRS-RSRP>-14 | dB |
| DIFF_SRS-RSRP_7 | -14≥ΔSRS-RSRP>-16 | dB |
| DIFF_SRS-RSRP_8 | -16≥ΔSRS-RSRP>-18 | dB |
| DIFF_SRS-RSRP_9 | -18≥ΔSRS-RSRP>-20 | dB |
| DIFF_SRS-RSRP_10 | -20≥ΔSRS-RSRP>-22 | dB |
| DIFF_SRS-RSRP_11 | -22≥ΔSRS-RSRP>-24 | dB |
| DIFF_SRS-RSRP_12 | -24≥ΔSRS-RSRP>-26 | dB |
| DIFF_SRS-RSRP_13 | -26≥ΔSRS-RSRP>-28 | dB |
| DIFF_SRS-RSRP_14 | -28≥ΔSRS-RSRP>-30 | dB |
| DIFF_SRS-RSRP_15 | -30≥ΔSRS-RSRP | dB |

Table 2: Reporting mapping of differential L1-CLI-RSSI

| Reported value | Measured quantity value (difference in measured CLI-RSSI from strongest CLI-RSSI) | Unit |
| --- | --- | --- |
| DIFF_CLI-RSSI_0 | 0≥ΔCLI-RSSI>-2 | dB |
| DIFF_CLI-RSSI_1 | -2≥ΔCLI-RSSI>-4 | dB |
| DIFF_CLI-RSSI_2 | -4≥ΔCLI-RSSI>-6 | dB |
| DIFF_CLI-RSSI_3 | -6≥ΔCLI-RSSI>-8 | dB |
| DIFF_CLI-RSSI_4 | -8≥ΔCLI-RSSI>-10 | dB |
| DIFF_CLI-RSSI_5 | -10≥ΔCLI-RSSI>-12 | dB |
| DIFF_CLI-RSSI_6 | -12≥ΔCLI-RSSI>-14 | dB |
| DIFF_CLI-RSSI_7 | -14≥ΔCLI-RSSI>-16 | dB |
| DIFF_CLI-RSSI_8 | -16≥ΔCLI-RSSI>-18 | dB |
| DIFF_CLI-RSSI_9 | -18≥ΔCLI-RSSI>-20 | dB |
| DIFF_CLI-RSSI_10 | -20≥ΔCLI-RSSI>-22 | dB |
| DIFF_CLI-RSSI_11 | -22≥ΔCLI-RSSI>-24 | dB |
| DIFF_CLI-RSSI_12 | -24≥ΔCLI-RSSI>-26 | dB |
| DIFF_CLI-RSSI_13 | -26≥ΔCLI-RSSI>-28 | dB |
| DIFF_CLI-RSSI_14 | -28≥ΔCLI-RSSI>-30 | dB |
| DIFF_CLI-RSSI_15 | -30≥ΔCLI-RSSI | dB |

# Topic #2: SBFD operation impacts

### Issue 2-1-1: Measurement accuracy for CSI-RS based L1 measurements

- Background

  - Different cases for DUD

    - Case 1: X1 >= X3 and X2 >= X3

    - Case 2: X1 >= X3 and X2 < X3

    - Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3

    - where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD).

  - Agreement from RAN4#114bis

    - Note: The following are for UE that can support simultaneous PDSCH reception on the two DL subbands for DUD

    - Existing delay (number of samples) requirements and accuracy requirements apply for Case 1 and Case 2

    - For Case 3

      - For L1-RSRP and L1-SINR, existing delay requirements apply and accuracy requirements are relaxed provided that X1 > X4 or X2 > X4, where the accuracy requirements depend on the X4 = 24 PRBs

      - In the test case, not model the interfering UE.

    - If UE indicates that CSI processing time is scaled by 2, further discuss whether and how to update the delay requirements

      - Option 1: Add a lower bound of 8ms on top of existing delay requirements.

      - Option 2: other

- Way forward

  - Define the relaxed accuracy for L1-RSRP and L1-SINR based on 24 RB by adding [TBD] relaxation on top of existing accuracy (based on 48 RB)

  - For case 3 (X1 < X3 and X2 < X3, (X1 + X2) >= X3; X3 = 48 RB),

    - for a UE indicating CSI processing time scaled by 2

      - Option 1: existing accuracy requirements based on 48 RB apply.

      - Option 2: accuracy requirements based on 24 RB apply.

    - for UE not indicating CSI processing time scaled by 2

      - Option 1: accuracy requirements based on 24 RB apply

# Topic #3: Test configurations and applicability

### Issue 3-1-1: Test case scenarios

- Agreements

  - Define test cases for both FR1 and FR2

### Issue 3-1-2: Test applicability

- Agreements

  - TDD test cases are also applicable to SBFD aware UEs. The new test cases introduced for SBFD aware UEs are additional to the test cases defined for TDD UEs.

### Issue 3-1-2: Test configurations

- Agreements

  - FR1: NR 30 kHz SCS, 100 MHz bandwidth, TDD duplex

  - FR2:

    - Option 1: NR 120 kHz SCS, 100 MHz bandwidth, TDD duplex mode

    - Option 2: NR 120 kHz SCS, 400 MHz bandwidth, TDD duplex mode

  - TDD UL/DL configuration : DDDSU

  - Time domain configuration of SBFD slots/symbols: DXXXU

  - Frequency domain configuration of SBFD subbands for FR1

    - DU: 80 MHz – 20 MHz

    - DUD: 40 MHz – 20 MHz – 40 MHz

  - FFS: Frequency domain configuration of SBFD subbands for FR2

# Topic #4: Test-cases and Work-split

### Issue 4-1-1: Test case scenarios

- Agreements

  - Agreed test-cases (details in the work-split table)

    - L1-SRS-RSRP measurement delay and scheduling restrictions

    - L1-CLI-RSSI measurement delay and scheduling restrictions

    - L1-SRS-RSRP measurement accuracy

    - L1-CLI-RSSI measurement accuracy

    - FFS: Radio Link Monitoring Out-of-sync Test for PCell configured with CSI-RS-based RLM in SBFD

    - Radio Link Monitoring In-sync Test for PCell configured with CSI-RS-based RLM in SBFD

    - Beam Failure Detection and Link Recovery Test for PCell configured with CSI-RS-based BFD and LR in SBFD

    - CSI-RS based L1-RSRP measurement in SBFD

    - CSI-RS based L1-RSRP measurement accuracy in SBFD

  - Further discuss whether to define the following test-cases:

    - CSI-RS L1-SINR measurement (delay and accuracy)

### Issue 4-1-1: Work-split

| No. | Title | Clause | Test purpose | Test setup and procedure | Company |
| --- | --- | --- | --- | --- | --- |
| 1-2 | L1-SRS-RSRP measurement in FR1/FR2 | A.6.6.x.1 A.7.6.x.1 | Delay and scheduling restriction for L1-SRS-RSRP | DU P resource on SBFD symbols; AP report 2 time periods, RSRP changed at beginning of T2. time offset: 17.67 for FR1, 7.67 for FR2 Power level refer to existing L3 CLI test cases UE is continuously scheduled except for the restricted symbols | Qualcomm |
| 3-4 | L1-CLI-RSSI measurement in FR1/FR2 | A.6.6.x.2 A.7.6.x.2 | Delay and scheduling restriction for L1-CLI-RSSI | DUD RSSI resource in 2 DL subbands, i.e. method#1 resource type#2 P resource on SBFD symbols; AP report 2 time periods, RSSI/Noc changed at beginning of T2. Power level refer to existing L3 CLI test cases UE is continuously scheduled except for the restricted symbols | Samsung |
| 5-6 | L1-SRS-RSRP measurement accuracy in FR1/FR2 | A.6.7.x.1 A.7.7.x.1 | Accuracy for L1-SRS-RSRP | DU P resource on SBFD symbols; AP report time offset: 17.67 for FR1, 7.67 for FR2 Power level refer to existing L3 CLI test cases | CATT |
| 7-8 | L1-CLI-RSSI measurement accuracy in FR1/FR2 | A.6.7.x.2 A.7.7.x.2 | Accuracy for L1-CLI-RSSI | DUD RSSI resource in UL subband, i.e. method#3 P resource on SBFD symbols; AP report Power level refer to existing L3 CLI test cases | Huawei |
| 9-10 | FFS:Radio Link Monitoring Out-of-sync Test for FR1/FR2 PCell configured with CSI-RS-based RLM in SBFD | A.6.5.1.x A.7.5.1.x | CSI-RS based RLM, collision with UL | FFS: DU/DUD Non-DRX All CSI-RS on SBFD symbols UL scheduling hit some CSI-RS occasions | Mediatek |
| 11-12 | Radio Link Monitoring In-sync Test for FR1/FR2 PCell configured with CSI-RS-based RLM in SBFD | A.6.5.1.y A.7.5.1.y | CSI-RS based RLM, collision with UL | DU DRX All CSI-RS on SBFD symbols UL scheduling hit some CSI-RS occasions | CMCC |
| 13-14 | Beam Failure Detection and Link Recovery Test for FR1/FR2 PCell configured with CSI-RS-based BFD and LR in SBFD | A.6.5.5.x A.7.5.5.x | CSI-RS based BFR, puncturing in frequency domain | DUD Non-DRX All CSI-RS on SBFD symbols, CSI-RS BW is same as CBW | LGE |
| 15-16 | CSI-RS based L1-RSRP measurement in SBFD FR1/FR2 | A.6.6.4.x A.7.6.3.x | CSI-RS based L1-RSRP delay, distribution in two symbol types | DU Non-DRX CSI-RS has occasions on both SBFD and non-SBFD symbols with differnet RSRP level | Ericsson |
| 17-18 | CSI-RS based L1-RSRP measurement accuracy in SBFD FR1/FR2 | A.6.7.4.x A.7.7.4.x | CSI-RS based L1-RSRP accuracy, puncturing in frequency domain (Case 3) | DUD Non-DRX CSI-RS has occasions on SBFD symbol, CSI-RS BW is 24 on each DL subband plus UL subband | vivo |
| Acc1 | Performance requirements for L1 CLI measurement | 10.1.X | Accuracy and report mapping for L1-SRS-RSRP and L1-CLI-RSSI | N/A | Nokia |
| Acc2 | Performance requirements for CSI-RS based L1-RSRP and L1-SINR measurement | 10.1.19.2 10.1.20.2 10.1.27 10.1.28 | Relaxed accuracy based on 24RB for L1-RSRP and L1-SINR | N/A | Oppo |
| TestConf | Test configuration for SBFD | A.3.x A.3.10 A.3.1.4 | Time and frequency domain configuration for SBFD SSB configuration TDD UL/DL configuration | N/A | ZTE |