3GPP TSG-RAN WG4 Meeting #117	R4-2520019

Dallas, USA, Nov. 17-21, 2025

Agenda item:	6.1.2

Source:	MediaTek Inc.

Title:	Discussion on demodulation and CSI requirements for 6Rx UE

Document for:	Discussion

# Introduction

Based on the WF of demodulation and CSI requirements for 6Rx UE [1], we provide our perspectives on the unresolved issues in this contribution.

- Discussion

## 2.1 General

Issue 1-2: Test applicability rule for 6Rx 6 layers performance requirements - Agreement: - Agree to define the applicability rules that 6 MIMO layers performance requirements are applicable for UEs supporting maxNumberMIMO-LayersPDSCH-v1900 = 6. - Proposals: - Discuss whether to explicitly capture the capability rules that “6 layers requirements are only applicable for FWA UE” in 38.101-4

RAN#109 officially approved RAN2 CR [2], which introduces maxNumberMIMO-LayersPDSCH-v1900 exclusively for FWA. This capability clearly indicates the 6 MIMO layers performance is only included for FWA. Also, RAN4 agreed to define the applicability rules that 6 MIMO layers performance requirements are applicable for UEs supporting maxNumberMIMO-LayersPDSCH-v1900 = 6.

maxNumberMIMO-LayersPDSCH, maxNumberMIMO-LayersPDSCH-v1900 Defines the maximum number of spatial multiplexing layer(s) supported by the UE for DL reception. For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2 (except for FR2-NTN). If supportOf2RxXR is indicated, for single CC standalone NR, it is mandatory with capability signalling to support 2 MIMO layers in the bands specified in Table 7.3.2-2b in TS 38.101-1 [2]. If absent, the UE does not support MIMO on this carrier. The maxNumberMIMO-LayersPDSCH-v1900 is only included for FWA UE as defined in TS 38.101-1 [2]. The UE supporting maxNumberMIMO-LayersPDSCH-v1900 shall also indicate support of maxNumberMIMO-LayersPDSCH for the corresponding component carrier. When maxNumberMIMO-LayersPDSCH and maxNumberMIMO-LayersPDSCH-v1900 are reported together for a component carrier, the network which is able to decode maxNumberMIMO-LayersPDSCH-v1900 ignores maxNumberMIMO-LayersPDSCH. For the bands where pdsch-1024QAM-2MIMO-FR1-r17 is indicated, MIMO layers for 1024 QAM is the smaller value between 2 and maxNumberMIMO-LayersPDSCH.; FSPC; CY; N/A; N/A.

Observation 1: RAN2 agreed that the UE capability maxNumberMIMO-LayersPDSCH-v1900, which supports a maximum of 6 MIMO layers, only be included for FWA as specified in TS 38.306.

Observation 2: RAN4 agreed to establish the applicability rules stating that the performance requirements for 6 MIMO layers are applicable to UEs supporting maxNumberMIMO-LayersPDSCH-v1900 = 6.

Based on Observation 1 and 2, the 6 MIMO layers requirements defined in RAN4 are only applicable to FWA. We think it is correct to add a note in the applicably rules for 6 MIMO layers requirements.

Proposal 1: Capture the capability rules that “6 layers requirements are only applicable for FWA UE” in 38.101-4.

Issue 1-5: Specification structure - WF - Add a new clause for 6Rx after the current 8Rx performance requirements - Section 5.2.5 6Rx requirement

As for specification structure, modifying the current clause numbering for 4Rx and 8Rx would significantly increase the complexity of CR maintenance. Such changes could lead to unnecessary confusion and inefficiencies in managing updates and revisions. To ensure clarity and streamline the process, we propose adding new clauses specifically for 6Rx. This approach not only preserves the integrity of the existing structure but also provides a dedicated space to address the unique requirements and parameters associated with 6Rx, facilitating better organization and future scalability.

Observation 3: Modifying the current clause numbering for 4Rx and 8Rx would significantly increase the complexity of CR maintenance.

Proposal 2: Add new clauses for 6Rx after the current 8Rx performance requirements, 5.2.5 for PDSCH requirements and 6.2.5 for CQI requirements.

## 2.2 SDR requirements

Issue 3-1-2: MCS value for 256QAM/ 6 layers with scaling factor 1 - Proposals - Option 1: 27 - Option 2: 24 - Option 3: 25 Issue 3-1-3: MCS value for 64QAM/ 6 layers with scaling factor 1 - Proposals - Option 1: 28 - Option 2: 26 Issue 3-1-4: MCS value for 1024QAM/ 2 layers with scaling factor 1 - Proposals - Option 1: 23 - Option 2: 24

RAN4 agreed to define SDR requirements for QPSK/16QAM/64QAM/256QAM with 6 MIMO layers and 1024QAM with 2 MIMO layers. Based on the simulation assumptions in [1], we provide evaluations in Table 1.

Table 1. Summary for the required SNR achieving 85% maximum throughput

| MCS | SNR(dB)@85% of maxTP |  |  |
| --- | --- | --- | --- |
|  | MCS Table 1 8T6R6L Fixed precoder with i1,1=i1,2=i2=0 | MCS Table 2 8T6R6L Fixed precoder with i1,1=i1,2=i2=0 | MCS Table 4 2T6R2L Random precoder |
| 28 | 21.2 |  |  |
| 27 | 19.2 | 26.2 |  |
| 26 | 18.1 | 25.1 |  |
| 25 | 17.1 | 24.1 | 31.6 |
| 24 | 16.1 | 23.1 | 27.1 |
| 23 | 15.0 | 22.0 | 24.8 |
| 22 |  | 20.3 |  |

Table 2. EVM assumptions and corresponding SNR

| Modulation scheme for PDSCH | EVM assumptions | Corresponding SNR |
| --- | --- | --- |
| QPSK/16QAM/64QAM | 6 % | 24.44 dB |
| 256QAM | 3 % | 30.46 dB |
| 1024QAM | 2.5 % | 32.04 dB |

Based on the EVM assumptions in table 2, we have the following proposals for MCS in SDR requirements.

Proposal 3: Use MCS 27 for 256QAM/ 6 layers with scaling factor 1 in SDR test

Proposal 4: Use MCS 28 for 64QAM/ 6 layers with scaling factor 1 in SDR test

Proposal 5: Use MCS 24 for 1024QAM/ 2 layers with scaling factor 1 in SDR test

## 2.3 CSI requirements

Issue 4-1-1: CQI test for 6Rx - Agreements: - Define CQI requirements with rank 4 under static channel - Propagation channel: AWGN - Antenna: 4x6 with static channel specified in Annex B.1 - RAN4 to only define one CQI reporting requirement applying to both handheld UEs and FWA UEs without additional applicability rules.

RAN4 has agreed to define CQI requirements for rank 4 under static channel conditions for 6Rx UEs [1]. Typically, two criteria are used to verify whether a UE meets the CQI requirements under static channel conditions

a)	The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90 % of the time.

b)	If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 0.1.

Based on our simulation results in [3] and above two criteria, we propose using the following SNR in 6Rx CQI requirements.

Proposal 6: Use lower SNR [5,6] dB and higher SNR [11,12] dB for 6Rx static channel CQI testing.

# Conclusion

In this contribution, we provide our views about performance requirements for 6Rx UEs. Observations and proposals are summarized:

Observation 1: RAN2 agreed that the UE capability maxNumberMIMO-LayersPDSCH-v1900, which supports a maximum of 6 MIMO layers, only be included for FWA as specified in TS 38.306.

Observation 2: RAN4 agreed to establish the applicability rules stating that the performance requirements for 6 MIMO layers are applicable to UEs supporting maxNumberMIMO-LayersPDSCH-v1900 = 6.

Proposal 1: Capture the capability rules that “6 layers requirements are only applicable for FWA UE” in 38.101-4.

Observation 3: Modifying the current clause numbering for 4Rx and 8Rx would significantly increase the complexity of CR maintenance.

Proposal 2: Add new clauses for 6Rx after the current 8Rx performance requirements, 5.2.5 for PDSCH requirements and 6.2.5 for CQI requirements.

Proposal 3: Use MCS 27 for 256QAM/ 6 layers with scaling factor 1 in SDR test

Proposal 4: Use MCS 28 for 64QAM/ 6 layers with scaling factor 1 in SDR test

Proposal 5: Use MCS 24 for 1024QAM/ 2 layers with scaling factor 1 in SDR test

Proposal 6: Use lower SNR [5,6] dB for 16QAM and higher SNR [11,12] dB for 64QAM for 6Rx static channel CQI testing.

# Reference

- R4-2514788, Way Forward for [116bis][215] NR_ENDC_RF_Ph4_Demod_6Rx, Huawei, HiSilicon

- R2-2506467, Introduction of UE capability on 6 DL MIMO layers, Huawei, HiSilicon, Samsung, MediaTek Inc., Apple

- R4-2520020, Simulation results on 6Rx UE demodulation performance requirements, MediaTek