3GPP TSG-RAN WG4 Meeting #117		   				 		  								 	   R4-2520037

Dallas, USA, 17th – 21th Nov., 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | draft | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of test case for CB-Msg3 procedure for NB-IoT with Satellite Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_Ph3-Perf |  |  |  |  |  | Date: |  |  | 2025-11-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Introdcue a test case for CB-Msg3-EDT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a new test case for CB-Msg3-EDT. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - No test case for CB-Msg3-EDT. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) A.13.3.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | NA |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES 1>>>

#### A.13.3.2.4	Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in normal coverage for CB-Msg3-EDT procedure

##### A.13.3.2.4.1	Test Purpose and Environment

The purpose of this test is to verify whether the behavior of the random access procedure of a category NB1 UE in Normal Coverage is according to the requirements when connected to a NTN NB-IoT cell, whether the CB-Msg3 power settings and timing are within specified limits, and whether the UE determines properly the enhanced coverage level based on the NRSRP measurement and the configured criterion in NRSRP-ThresholdsPrach [2]. This test will verify the requirements in Clause 6.6A.4, 7.20A in an AWGN model.

For this test a single NB-IoT cell is used. The test parameters are given in tables A.13.3.2.4.1-1, A.13.3.2.4.1-2 and A.13.3.2.4.1-3. The UE shall perform timing pre-compensation before the initial CB-Msg3 transmission using AT command-based test approach.

Table A.13.3.2.4.1-1: nCell specific test parameters for HD-FDD contention based random access test for UE category NB1 Standalone mode in Normal Coverage

| Parameter | Unit | Value | Comments |
| --- | --- | --- | --- |
| NB-IOT operational mode |  | Standalone |  |
| BWchannel | kHz | 200 |  |
| NPDSCH parameters Note 2 |  | R.18 HD-FDD | As defined in A.3.1.5.3 |
| NPDCCH parameters Note 2 |  | R.30 HD-FDD | As defined in A.3.1.6.3 |
| NPBCH_RA | dB | 0 |  |
| NPBCH_RB | dB | 0 |  |
| NPSS_RA | dB |  |  |
| NSSS_RA | dB |  |  |
| NPDCCH_RA | dB |  |  |
| NPDCCH_RB | dB |  |  |
| NPDSCH_RA | dB |  |  |
| NPDSCH_RB | dB |  |  |
| NOCNG_RA Note 1 | dB |  |  |
| NOCNG_RB Note 1 | dB |  |  |
| DRX |  | OFF |  |
|  | dBm/15 kHz | -98 |  |
|  | dB | 3 |  |
| Note 3 | dB | 3 |  |
| NRSRP Note 3 | dBm/15 kHz | -95 |  |
| Io Note 3 | dBm/180 KHz | -82.45 |  |
| Propagation Condition | - | AWGN |  |
| Antenna Configuration |  | 1x1 |  |
| Note 1: NOCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: The NPDSCH and NPDCCH reference measurement channels are used in the test only when a downlink transmission dedicated to the UE under test is required. Note 3: Es/Iot, NRSRP and Io level has been derived from other parameters for information purpose. They are not settable parameters themselves. |  |  |  |

Table A.13.3.2.4.1-2: NTN specific test parameters for HD-FDD contention based random access test for UE category NB1 Standalone mode in Normal Coverage

| Parameter | Value | Comment |
| --- | --- | --- |
| Configuration 1 | SSC.1 | GSO Test Configuration |
| Configuration 2 | SSC.2 | NGSO Test Configuration |

Table A.13.3.2.4.1-3: NPUSCH Configuration parameters for CB-Msg3 for HD-FDD contention based random access test for UE category NB1 Standalone mode in Normal Coverage

| Field | Value |  | Comment |  |
| --- | --- | --- | --- | --- |
| Parameters not per NPRACH resource |  |  |  |  |
| cb-Msg3-MinRSRP-Threshold-NB | {39, 24} |  | Corresponding to {-101, -116} dBm as defined in Table 9.1.4-1 |  |
| Configured UE transmitted power () | 23 dBm for power class 3, 20 dBm for power class 5 |  | As defined in clause 6.2B.4 in TS 36.102 |  |
| powerRampingStep-NB-r19 | dB2 |  |  |  |
| cb-Msg3-InitialReceivedTargetPower-NB-r19 | dBm-112 |  |  |  |
| cb-Msg3-MaxAttemptNum-NB-19 | n6 |  |  |  |
| cb-Msg3-TBS-NB-r19 | b144 |  |  |  |
| cb-Msg3-NumOfReplicas-NB-r19 | 2 |  |  |  |
| Parameters per NPUSCH Resource |  |  |  |  |
| NPUSCH Resource | Level 0 |  |  |
| npusch-Periodicity-r19 | ms40 |  |  |
| npusch-NumRepetitionsIndex-r19 | n2 |  |  |
| npusch-NumRepetitionsIndex-r19 | r4 |  |  |
| npdcch-StartSF-CSS-r19 | v1dot5 |  |  |
| npdcch-Offset-CSS-r19 | zero |  |  |
| cb-Msg3-ResponseWindow-NB-r19 | pp2 |  |  |
| mac-ContentionResolutionTimer | pp8 |  |  |
| Note 1: See Clause 6.7.3 in TS 36.331 for further information on the parameters in this table. |  |  |  |  |

##### A.13.3.2.4.2	Test Requirements

A.13.3.2.4.2.1	Transmitting CB-Msg3

To test the UE behavior specified in subclause 6.6A.4.1, the UE shall calculate the transmission power for the initial CB-Msg3 transmission and [the subsequent CB-Msg3 replica transmissions] according to the formula defined in TS 36.213.

The transmit timing of all CB-Msg3 transmissions shall be within the accuracy specified in Subclause 7.20A.

A.13.3.2.4.2.2	receiving a CB-Msg4 over CB-RNTI

To test the UE behavior specified in Subclause 6.6A.4.2.

The System Simulator shall transmit CB-Msg4 that include the backoff indicator field.

The UE shall set the backoff time, according to the CB-Msg3-EDT backoff parameters as described in TS 36.321.

A.13.3.2.4.2.3	Reception of an Incorrect CB-Msg4 over CB-RNTI

To test the UE behavior specified in Subclause 6.6A.4.3.

The System Simulator shall send a message addressed to the CB-RNTI with a UE Contention Resolution Identity MAC control element that not matches the CCCH SDU transmitted in the uplink message.

The UE shall re-attempt a CB-Msg3 transmission when the backoff expires in the next CB-Msg3 window with the calculated PUSCH transmission power, until the maximum number of re-transmissions defined by cb-Msg3-MaxAttemptNum-NB-19 in the table A.13.3.2.4.1-3 is reached.

A.13.3.2.4.2.4	Reception of a Correct CB-Msg4 over CB-RNTI

To test the UE behavior specified in Subclause 6.6A.4.2.

The System Simulator shall send a message addressed to the CB-RNTI with a UE Contention Resolution Identity MAC control element that matches the CCCH SDU transmitted in the uplink message.

The System Simulator shall allocate and indicate HARQ-ACK resources to the UE.

The UE shall send HARQ-ACK if the Contention Resolution is successful.

<<< END OF CHANGES 1>>>