3GPP TSG-RAN WG4 #117	R4-2520045

Dallas, US, November 17-21, 2025

Agenda item:	8.12.2.2		RRM spec

Source:	Qualcomm Incorporated

Title:	Further comments on RAN4 operational efficiency for RRM

Document for:	Discussion

# Introduction

The 6G study item was approved in RAN #108 [1]. One important topic from a RAN4 point of view is to improve the operational efficiency in RAN4. The discussion was initiated in RAN4 #116bis and a way forward was agreed in [2]

In this contribution, we continue to discuss topics related to RAN4 operation efficiency for RRM.

# Views on the Operation Efficiency for RRM

## Interworking of RAN4 and RAN5

We think it is also important to address the interworking of RAN4 and RAN5 in the 6GR study item. We believe RAN4 does not need to define all details of the parameters and configuration of the RRM performance tests.  Typically, RRM features become relevant for the field several years after they have been specified by RAN4. Hence, RAN4 is not in a good position to decide on all parameters of the performance test cases. Instead, RAN5 is better suited to define the detailed parameter settings of the test cases to address the importance for field deployments. We think that RAN4 should define only the scope and the framework for the RRM performance tests, but details of the parameter configuration could be addressed by RAN5.

Observation 1: RRM features are usually deployed in the field several years later after they are specified by RAN4. Hence, RAN4 is not in a good position to decide on important field-relevant parameter settings of the RRM performance test cases.

Proposal 1: We propose to discuss between RAN4 and RAN5 whether in 6GR RAN4 could focus on the scope and framework for defining RRM performance tests and RAN5 could specify the detailed parameter configurations of the RRM performance tests.

## Structure and Drafting Principles for TS 38.133

Specification improvement of TS 38.133 has already been discussed in Rel-19 for 5G NR and good progress has been made. We repeat here the agreements that have been made in RAN4 #113 and propose that they should be applied fully for the 6G RRM spec

Proposal 2: The 6G RRM spec should follow the agreements for RRM specification improvement made in R4-2420107.

Proposal 3: The 6G RRM specification should adopt RAN2 pseudo-code approach in all sections.

Proposal 4: The following drafting principles have been agreed in Rel-19 RRM specification improvement and should be applied for the 6G RRM specification.

- Put conditions before requirements

- For requirements with multiple applicable conditions, strive to start the paragraph with UE capability, if applicable, and provide all remaining conditions in a list

- When both new and legacy requirement are to be put in the same hierarchy level, separate new and legacy requirement with If and Else if

  - If (condition of legacy requirement)

    - Legacy requirements

  - Else if (condition of new requirement)

    - New requirement

Proposal 5: RAN4 should study whether further drafting principles should be defined to enhance the readability of the spec and should document the drafting principles clearly.

RAN4 should also investigate in the 6G study item whether the set of RRM procedures can be reduced, and requirements can be defined only for key procedures. Corner cases or less common scenarios may not require defining core and performance requirements. That would also help to reduce the complexity and the readability of the RRM specification significantly. For example, for event-triggered reports requirements may only be defined if a clear network action follows within a well-defined interval. One of the goals of the 6G study item should be to define a slim RRM specification that focuses on the main procedures and does not define requirements for corner cases. Similarly, RAN4 should also aim to avoid defining too many different options for the same feature and focus on a very few core procedures. An example is SCell activation that got enhanced in 5G NR since Rel-15 but only the Rel-15 procedures are commercialized in our understanding.

Proposal 6: RAN4 should study in the 6G study item whether the amount of RRM procedures can be reduced. Requirements should only be defined for key RRM procedures.

It is important that these drafting principles are very closely followed to avoid the risk that the RRM spec ends up in a messy status again. Since the drafting principles are new it may not be easy for everybody to always follow them strictly. Therefore, we believe that the editor of the RRM spec has to spend some effort to maintain the good quality of the spec, i.e., by making sure that CRs are only accepted if they clearly follow the drafting principles.

Proposal 7: CRs for the 6GR RRM spec should only be accepted if they clearly follow the drafting principles.

In our view it is important that all drafting rules for writing the specification and CRs are collected in a single document that is published for everybody’s reference.

Proposal 8: RAN4 should collect all drafting rules for writing the specification and CRs in a single document and publish it as a tdoc.

## Additional Comments to Issue 3-2-1 and Issue 3-2-4

Issue 3-2-1 in the way forward [2] lists Proposal 7 and Proposal 11 under general principles and targets:

- Proposal 7:

  - Proposal 7a: Avoid corner cases and focus on most typical and practical use cases.

  - Proposal 7b: Focus on enhancing the testing of RRM procedures to ensure that functionality and performance is tested under conditions that reflect field-relevant scenarios.

  - Proposal 7c: Study how to ensure that real UE implementations are tested as much as possible.

  - Proposal 7d: Discuss whether to define two threads requirements, one focus on the baseline and another focus on the strict performance with real field request.

  - Proposal 7e: Study how to define RRM requirements that allow UE implementation based on minimum requirements but also allow UEs that can outperform the minimum requirements the benefits from such better performance

- Proposal 11:

  - Introduce a more intuitive and simpler way to define RRM requirements, and new forms of representation if necessary.

In our view proposals 7a – 7c are general comments that are only partially actionable in the 6G study item under the operational efficiency track. For example, the operation efficiency track could try to develop a general definition of corner cases and practical use cases. In our view proposals, 7a – 7c are rather a guideline for the 6G normative phase when concrete core and performance requirements are defined.

However, proposals 7d, 7e and partially proposal 11 do not belong to operational efficiency of the RRM specification in our opinion. Proposals 7d and 7e suggest introducing two set of requirements, namely the minimal set of requirements and enhanced requirements, called strict performance in proposal 7d and outperforming the minimum requirements in proposal 7d. These two proposals are closely related to the proposal on RRM-specific categories in [3]. All three proposals have the same underlying idea to make the network aware that some UEs may outperform the minimal requirements of the RRM specification. In our view, such a discussion is not related at all to operational efficiency but should be discussed in the main RRM track. According to the way forward in 6G RRM, the proposal of RRM-specific categories is covered under Issue 12: Other PHY signal/channel/procedure related RRM [4]. We suggest that proposals 7d and 7e are combined with the proposal of RRM-specific categories and discussed in the RRM session.

Observation 2: Proposals 7d, 7e and 11 in Issue 3-2-1 do not belong to operational efficiency of the RRM specification. These proposals suggest introducing two set of requirements, namely the minimal set of requirements and enhanced requirements

Proposal 9: We suggest that proposals 7d, 7e and 11 of Issue 3-2-1 are combined with the proposal of RRM-specific categories in R4-2514644 and discussed in the main RRM session.

Issue 3-2-4 in the way forward [2] lists Proposal 1 under readability and simplification:

- Proposal 1: simplification on core requirements and tests by considering real demands, typical scenarios and realistic UE implementation

  - Proposal 1b: Re-evaluate existing RRM core and performance requirements whether they still reflect state-of-the art UE implementations.

  - Proposal 1d: Investigate how the network can be enabled to follow true UE performance in its RRM procedures instead of assuming that all UEs just support minimal requirements.

  - Proposal 1g: Defining standardized and meaningful measurement metrics for 6G from day one, ensuring that these metrics are clearly specified and consistently implemented across vendors.

  - Proposal 1h: Define 6G feature requirements based on realistic UE architecture assumptions, rather than relying solely on basic UE profiles that may not support the intended feature.

In our view, these proposals do not belong proposals do not belong to RRM operation efficiency. Proposal 1d is closely related to the comments for Issue 3-2-1 above. Proposals 1b and 1h indicate that implementations in 5G have changed over time and new architectures and state-of-the-art implementations should be considered in the definition of RRM requirements for 6G. Such an analysis should be handled in the main RRM session and does not belong to RRM operational efficiency.

Observation 3: Proposals 1b, 1d, 1g and 1h in Issue 3-2-4 do not belong to operational efficiency of the RRM specification.

Proposal 10: We suggest that proposals 1b, 1d, 1g and 1h in Issue 3-2-4 are discussed in the main RRM session.

# Conclusions

In this paper we provided our views related to the operation efficiency in RAN4. Our observations and proposals are summarized as follows:

Observation 1: RRM features are usually deployed in the field several years later after they are specified by RAN4. Hence, RAN4 is not in a good position to decide on important field-relevant parameter settings of the RRM performance test cases.

Proposal 1: We propose to discuss between RAN4 and RAN5 whether in 6GR RAN4 could focus on the scope and framework for defining RRM performance tests and RAN5 could specify the detailed parameter configurations of the RRM performance tests.

Proposal 2: The 6G RRM spec should follow the agreements for RRM specification improvement made in R4-2420107.

Proposal 3: The 6G RRM specification should adopt RAN2 pseudo-code approach in all sections.

Proposal 4: The following drafting principles have been agreed in Rel-19 RRM specification improvement and should be applied for the 6G RRM specification.

- Put conditions before requirements

- For requirements with multiple applicable conditions, strive to start the paragraph with UE capability, if applicable, and provide all remaining conditions in a list

- When both new and legacy requirement are to be put in the same hierarchy level, separate new and legacy requirement with If and Else if

  - If (condition of legacy requirement)

    - Legacy requirements

  - Else if (condition of new requirement)

    - New requirement

Proposal 5: RAN4 should study whether further drafting principles should be defined to enhance the readability of the spec and should document the drafting principles clearly.

Proposal 6: RAN4 should study in the 6G study item whether the amount of RRM procedures can be reduced. Requirements should only be defined for key RRM procedures.

Proposal 7: CRs for the 6GR RRM spec should only be accepted if they clearly follow the drafting principles.

Proposal 8: RAN4 should collect all drafting rules for writing the specification and CRs in a single document and publish it as a tdoc.

Observation 2: Proposals 7d, 7e and 11 in Issue 3-2-1 do not belong to operational efficiency of the RRM specification. These proposals suggest introducing two set of requirements, namely the minimal set of requirements and enhanced requirements

Proposal 9: We suggest that proposals 7d, 7e and 11 of Issue 3-2-1 are combined with the proposal of RRM-specific categories in R4-2514644 and discussed in the main RRM session.

Observation 3: Proposals 1b, 1d, 1g and 1h in Issue 3-2-4 do not belong to operational efficiency of the RRM specification.

Proposal 10: We suggest that proposals 1b, 1d, 1g and 1h in Issue 3-2-4 are discussed in the main RRM session.

# References

- RP-251881, New SID: Study on 6G Radio, RAN #108, June 2025

- R4-2514648, WF on 6G operation efficiency, RAN4 #116bis, October 2025

- R4-2513040, On radio resource management aspects of 6GR, Qualcomm, RAN4 #116bis, October 2025

- R4-2514644, WF on [116bis][105] 6G RRM, RAN4 #116bis, October 2025