3GPP TSG-RAN WG4 Meeting #117                                                                                                           R4-2520056

Dallas, USA, 17th –21st November, 2025

Source: 	CATT

Title: 	Discussion on note for FR1-NTN frequency range for NR Ku band

Agenda item:	4.2.7.1

Document for:	Approval

- Introduction

In this contribution, we provide our views on note for FR1-NTN frequency range for Ku-band for NR NTN.

- Discussion

The frequency range for TN defined in Table 5.1-1 of TS 38.104 is copied as below:

Table 5.1-1: Definition of frequency ranges

| Frequency range designation |  | Corresponding frequency range |
| --- | --- | --- |
| FR1 |  | 410 MHz – 7125 MHz |
| FR2 | FR2-1 | 24250 MHz – 52600 MHz |
|  | FR2-2 | 52600 MHz – 71000 MHz |

The frequency range for NTN defined in Table 5.1-1 of TS 38.108 after introducing Ku-band in endorsed draft CR R4-2515135 [2] is copied as below:

Table 5.1-1: Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range (MHz) |
| --- | --- |
| FR1-NTN (NOTE 1) | 410 – 14500 |
| FR2-NTN (NOTE 2) | 10700 – 30000 |
| NOTE 1: NTN bands within this frequency range with band numbering less than or equal to 256 are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range with band numbering from 257 to 512 are regarded as a FR2-1 band when references from other specifications unless otherwise stated. |  |

As above NOTE 1 in Table 5.1-1, the NOTE 1 for FR1-NTN in TS 38.108 indicates that FR1-NTN is regarded as FR1 when referenced from other specification.

Comparing the corresponding frequency range for FR1 for TN in TS 38.104 and that for FR1-NTN for NTN in TS 38.108, we can find that the FR1-NTN and FR1 isn’t identical due to introduction of Ku-band, so there may be ambiguous understanding for NOTE 1 for FR1-NTN frequency range when referenced from other specification.

The wording “unless otherwise stated” in Note 2 for FR2-NTN can avoid ambiguous understanding when referenced from other specification. So, it is reasonable to add wording “unless otherwise stated” in Note 1 for FR1-NTN to avoid ambiguous understanding as below:

Table 5.1-1: Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range (MHz) |
| --- | --- |
| FR1-NTN (NOTE 1) | 410 – 14500 |
| FR2-NTN (NOTE 2) | 10700 – 30000 |
| NOTE 1: NTN bands within this frequency range with band numbering less than or equal to 256 are regarded as a FR1 band when references from other specifications unless otherwise stated. NOTE 2: NTN bands within this frequency range with band numbering from 257 to 512 are regarded as a FR2-1 band when references from other specifications unless otherwise stated. |  |

So, we propose,

Proposal 1: Add “unless otherwise stated” to NOTE 1 for FR1-NTN to follow the treatment in Note 2 for FR2-NTN.

Table 5.1-1: Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range (MHz) |
| --- | --- |
| FR1-NTN (NOTE 1) | 410 – 14500 |
| FR2-NTN (NOTE 2) | 10700 – 30000 |
| NOTE 1: NTN bands within this frequency range with band numbering less than or equal to 256 are regarded as a FR1 band when references from other specifications unless otherwise stated. NOTE 2: NTN bands within this frequency range with band numbering from 257 to 512 are regarded as a FR2-1 band when references from other specifications unless otherwise stated. |  |

Based on above proposals, two draft CR [2,3] for TS 38.101-5 and TS 38.108  for FR1-NTN frequency range for NR NTN Ku band are provided for approval.

- Conclusion

This contribution provides analysis on system parameters for Ku-band for NR NTN, and the following proposal is concluded as follows:

Proposal 1: Add “unless otherwise stated” to NOTE 1 for FR1-NTN to follow the treatment in Note 2 for FR2-NTN.

Table 5.1-1: Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range (MHz) |
| --- | --- |
| FR1-NTN (NOTE 1) | 410 – 14500 |
| FR2-NTN (NOTE 2) | 10700 – 30000 |
| NOTE 1: NTN bands within this frequency range with band numbering less than or equal to 256 are regarded as a FR1 band when references from other specifications unless otherwise stated. NOTE 2: NTN bands within this frequency range with band numbering from 257 to 512 are regarded as a FR2-1 band when references from other specifications unless otherwise stated. |  |

Based on above proposals, two draft CR [2,3] for TS 38.101-5 and TS 38.108  for FR1-NTN frequency range for NR NTN Ku band are provided for approval.

- References

- R4-2515135, draftCR on NTN frequency range defintion (38.108), Xiaomi, RAN4#116bis

- R4-2520057, (NR_NTN_Ku_bands-Core)CR for 38.101-5, On FR1-NTN frequency range for NR NTN Ku band, CATT, RAN4#117

- R4-2520058, (NR_NTN_Ku_bands-Core)CR for 38.108, On FR1-NTN frequency range for NR NTN Ku band, CATT, RAN4#117