3GPP TSG-RAN WG4 Meeting #117                                                                                                      R4-2520062

Dallas, USA, 17th –21st November, 2025

Source: 	CATT

Title: 	Further discussion on SAN conformance tests for NR NTN Ku-band

Agenda item:	5.10.4

Document for:	Approval

- Introduction

In RAN4#116bis meeting, A [116bis][308] WF on Rel-19_BS_SAN_conformance_Part2 was agreed [1]. It included the open issues and agreements for Ku-band SAN conformance requirement. There are some issues in WF such as MU for 10-15GHz for Ku-band SAN conformance requirement needs to be discussed.

In this contribution, we provide our views on SAN conformance requirements for Ku-band for NR NTN.

- Discussion

Measurement uncertainties

As per the WF [1], the remaining issue concerning MU for Ku-band is shown as below:

### Issue 1-1-1: MU is an Open issue that needs further discussion For SAN Type 1-O above 10 GHz, it is not a straightforward choice what MUs to use, since the range of 10-15 GHz was not studied before for TN and cannot be reused for NTN. Agreement: Inputs from TE vendors would be appreciated, with respect to Measurement Uncertainties (MUs) for: - 13.75GHz-14.5GHz for FR1-NTN Ku-band and FR2-NTN Ku-band OTA receiver tests - 10.7GHz-12.75GHz for FR1-NTN Ku-band and FR2-NTN Ku-band OTA transmitter tests. Companies to further discuss if current FR2-NTN MUs could be reused.

Current FR2-NTN MUs are defined for Ka-band SAN type 2-O. in RAN4#110 meeting, the agreement on SAN MUs for Ka-band in agreed WF R4-2402854 is copied as below:

From above the agreement on SAN MUs for Ka-band, we can find that TX and RX MUs for Ka-band are reused from TN MUs for 24.25 – 29.5 GHz in TR 37.941 with [].

Observation 1: Current FR2-NTN MUs for Ka-band are reused from TN MUs for 24.25 – 29.5 GHz in TR 37.941 with [].

From MUs in TR 37.941, the TX and RX MUs for 4.2<f≤6 GHz are defined in Table 17-1 and Table 17-3, respectively. And the TX and RX MUs for 24.25 – 29.5 GHz are defined in Table 17-2 and Table 17-4, respectively.

Referring to MUs in TR 37.941, the TX MUs differences between TX MUs for 4.2<f≤6 GHz and TX MUs for  24.25 – 29.5 GHz are captured in the following Table 1, and the RX MUs differences between RX MUs for 4.2<f≤6 GHz and RX MUs for  24.25 – 29.5 GHz are captured in the following Table 2.

Table 1: TX MUs differences between TX MUs for 4.2<f≤6 GHz and TX MUs for  24.25 – 29.5 GHz

| Clause | Maximum OTA Test System uncertainty | Maximum OTA Test System uncertainty |  |
| --- | --- | --- | --- |
|  | (24.25 – 29.5 GHz) | 4.2<f≤6 GHz | Difference |
| 9.2 Radiated transmit power | Normal condition: ±1.7 dB | Normal condition: ±1.3 dB | 0.4 |
| 9.3 OTA base station output power | ±2.1 dB | ±1.5 dB | 0.6 |
| 9.4.3 OTA total power dynamic range | ±0.4 dB | ±0.4 dB | 0 |
| 9.6.2 OTA frequency error | ±12 Hz | ±12 Hz | 0 |
| 9.6.3 OTA modulation quality | 1% | ±1 % | 0 |
| 9.7.2 OTA occupied bandwidth | 600 kHz | ±100 kHz, BWChannel 5 MHz, 10 MHz ±300 kHz, BWChannel from 15 MHzup to 50 MHz ±600 kHz, BWChannel from 60 MHz up to 100 MHz | need to consider BWChannel |
| 9.7.3 OTA ACLR | ±2.3 dB (Relative ACLR) | ±1.2 dB(Relative ACLR) | 1.1 |
| 9.7.4 OTA out-of-band emissions | ±2.7 dB | ±2 dB | 0.7 |
| 9.7.5.2 OTA transmitter spurious emissions, mandatory requirements | ±2.3 dB, 30 MHz ≤ f ≤ 6 GHz | ±2.3 dB, 30 MHz < f ≤ 6 GHz | MU for 24.25 – 29.5 GHz is more reasonable |
|  | ±2.7 dB, 6 GHz < f ≤ 40 GHz | ±4.2 dB, 6 GHz < f ≤ 26 GHz |  |
|  | ±5.0 dB, 40 GHz < f ≤ 60 GHz |  |  |
| NOTE: Test system uncertainty values are applicable for normal condition unless otherwise stated. |  |  |  |

Table 2: RX MUs differences between TX MUs for 4.2<f≤6 GHz and TX MUs for  24.25 – 29.5 GHz

| Clause | Maximum OTA Test System uncertainty |  |  |
| --- | --- | --- | --- |
|  | (24.25 – 29.5 GHz) | 4.2<f≤6 GHz | Difference |
| 10.3 OTA reference sensitivity level | ±2.4 dB | ±1.6 dB | 0.8 |
| 10.5.1 OTA adjacent channel selectivity | ±3.4 dB | ±2.4 dB | 1 |
| 10.6 OTA out-of-band blocking | ±3.6 dB | ±3.6 dB, finterferer ≤ 3 GHz ±3.7 dB, 3 GHz < finterferer ≤ 4.2 GHz ±3.8 dB, 4.2 GHz < finterferer ≤ 6 GHz | about 1 |
| 10.9 OTA in-channel selectivity | ±3.4 dB | ±2.4 dB | 1 |
| NOTE: Test system uncertainty values are applicable for normal condition unless otherwise stated. |  |  |  |

From above Table 1, the maximum difference between TX MUs for 4.2<f≤6 GHz and TX MUs for  24.25 – 29.5 GHz is about 1.1dB, From above Table 2, the maximum difference between RX MUs for 4.2<f≤6 GHz and RX MUs for  24.25 – 29.5 GHz is about 1.

Since the frequency range for Ku-band is 10-15GHz, which is between  4.2<f≤6 GHz and 24.25 – 29.5 GHz. So the TX and RX MUs for 10-15GHz for Ku-band should be between TX and RX MUs for 4.2<f≤6 GHz and TX and RX MUs for 24.25 – 29.5 GHz.

So if reusing MUs for 24.25 – 29.5 GHz in TR37.941 for MUs for 10-15GHz for Ku-band, the maximum MU relaxation is less than 1dB.

Observation 2: If reusing MUs for 24.25 – 29.5 GHz in TR37.941 for MUs for 10-15GHz for Ku-band, the maximum MU relaxation is less than 1dB.

From our understanding, the less than 1dB MU relaxation is acceptable. Since the current FR2-NTN MUs is reused from MUs for 24.25 – 29.5 GHz in TR 37.941, so the current FR2-NTN MUs can be reused for FR1-NTN Ku-band and FR2-NTN Ku-band, and additional inputs from TE vendors for more reasonable MUs for Ku-band are encouraged, the Values can be kept in [].

So, we propose,

Proposal 1: For MUs for 10.7GHz-12.75GHz for FR1-NTN Ku-band and FR2-NTN Ku-band OTA transmitter tests:

Consider current MUs for SAN type 2-O transmitter tests in 17.3-20.2 GHz for FR1-NTN Ku-band and FR2-NTN Ku-band OTA transmitter tests in 10.7GHz-12.75GHz. Values are kept in [], encouraging additional inputs from TE vendors.

Proposal 2: For MUs for 13.75GHz-14.5GHz for FR1-NTN Ku-band and FR2-NTN Ku-band OTA receiver tests:

Consider current MUs for SAN type 2-O receiver tests in 27.5-30.0 GHz for FR1-NTN Ku-band and FR2-NTN Ku-band OTA receiver tests in 13.75GHz-14.5GHz. Values are kept in [], encouraging additional inputs from TE vendors.

- Conclusion

This contribution provides analysis on SAN conformance requirement for NR NTN Ku-band, and the following proposal is concluded as follows:

Observation 1: Current FR2-NTN MUs for Ka-band are reused from TN MUs for 24.25 – 29.5 GHz in TR 37.941 with [].

Observation 2: If reusing MUs for 24.25 – 29.5 GHz in TR37.941 for MUs for 10-15GHz for Ku-band, the maximum MU relaxation is less than 1dB.

Proposal 1: For MUs for 10.7GHz-12.75GHz for FR1-NTN Ku-band and FR2-NTN Ku-band OTA transmitter tests:

Consider current MUs for SAN type 2-O transmitter tests in 17.3-20.2 GHz for FR1-NTN Ku-band and FR2-NTN Ku-band OTA transmitter tests in 10.7GHz-12.75GHz. Values are kept in [], encouraging additional inputs from TE vendors.

Proposal 2: For MUs for 13.75GHz-14.5GHz for FR1-NTN Ku-band and FR2-NTN Ku-band OTA receiver tests:

Consider current MUs for SAN type 2-O receiver tests in 27.5-30.0 GHz for FR1-NTN Ku-band and FR2-NTN Ku-band OTA receiver tests in 13.75GHz-14.5GHz. Values are kept in [], encouraging additional inputs from TE vendors.

- References

- R4-2515065, [116bis][308] WF on Rel-19_BS_SAN_conformance_Part2, Moderator (Iridium), RAN4#116bis

- R4-2402854, Way forward on [110][306] NR_NTN_enh_Part2, Ericsson, RAN4#110