3GPP TSG RAN WG4 Meeting #117		R4-2520093

Dallas, Texas, United States, Nov 17th – 21st , 2025

Title: 	Discussion on General RF and UE RF for 6G study

Agenda Item:	8.3

Source: 	CATT

Document for:	Approval

1 Introduction

In the RAN4#116bis meeting, a WF [6] on General RF and UE RF for 6G study was approved. It can be observed that some topics will be prioritized for discussion, for example:

Topic #3: Single carrier output power requirements - Agreements - Study 6G output power requirements to enable more flexibility to different UE implementations to reflect the RF front-end capability for single band - Encourage inputs on how to allow UE to deliver more output power and how power class framework includes e.g. Tx diversity and UL-MIMO - Study improvements on configured maximum output power requirements and associated parameters - Strive to improve clarity on UE power ability under different UE condition (e.g. configured, activated, scheduled) in all output power requirements - Study the default power class for FDD and TDD bands for 6G - Study how to handle SAR/MPE for 6GR Topic #4: Power class framework for CA - Agreements - Strive to improve clarity on UE power ability under different UE condition (e.g. configured, activated, scheduled) in all output power requirements - Study how to maximize the UL transmission power capability for CA - Prioritize discussion for single carrier in RAN4#117 and RAN4#118

Topic #5: Tx requirements - Agreements - PA modelling is discussed in system parameter thread - In initial 6GR evaluations, use 5G NR unwanted emission requirements as starting point until requirement framework for 6GR is clear - Spurious emissions requirements, SEM and ACLR will be defined for 6GR - FFS for - OOB boundary - potential accommodations for different frequency sub-ranges - Encourage inputs on regulatory survey being conducted in spectrum agenda - Study the UE Tx impairment requirements/assumptions (e.g. carrier leakage, image rejection, CIM3 and CIM5) - In RAN4#117, prioritize discussion for UE Tx impairments and unwanted emission framework Topic #8: Spectrum aggregation - Agreements - Below spectrum aggregation topics can be discussed as starting point. - RF switch-time performance - Need for intra-band support of non-contiguous UL CA and RB allocations - Study benefits-drawbacks of using CA vs. using wider single carrier BW - Study how to account for simultaneous TX/RX between the bands properly from the beginning - Discuss band combination simplification in spectrum agenda Topic #9: Joint UE and BS RF - Agreements - Encourage companies to contribute whether there are concrete study points for low power receiver. - Coexistence study including need for coex study will be discussed in agenda for BS RF and co-existence. - Prioritize work for the baseline single carrier output power requirement framework before detailed studies on ACLR and SEM relaxation. - Discuss PAPR reduction topics in system parameter agenda - For Tx EVM relaxation - Discuss non-AI based UL demod in UE RF thread - The impact on BS receiver could be considered. - Do not discuss AI based solution(s) in UE RF thread, except the potential common part. - The potential common part between AI and non-AI solutions will be discussed in this thread. AI solutions, which are discussed under AI agenda, should leverage the agreements in this thread as much as possible. - Strive to study TX non-linearity sources, including PA, in this thread - It is FFS whether and how, if agreed, to take into consideration of the existing 5G requirements and RF impairment assumptions for both 5GA and 6G. - Strive to avoid overlapping work between different threads - In RAN4#117, prioritize discussion on study scope and necessary details aligned with RAN guidance, e.g. - Which aspects need to be considered to conclude on feasibility - Need for network control

Based on the inputs from 6G RAN WG SI and RAN4 Chair’s guideline, the following aspects can be summarized from RAN4 UE RF perspective.

1) Frequency Range:

| Frequency Range | FR1 (410 or FFS ~ 7125MHz) | between FR1 and FR2-1 (TBD & including around ~7GHz) | FR2-1 (24.25GHz ~ 52.6GHz) |
| --- | --- | --- | --- |
| TR 38.921(R17) | 6.425 - 7.025 GHz, 7.025 - 7.125 GHz | 10.0 - 10.5 GHz | NA |
| TR 38.922(R19) | 4400 - 4800 MHz | 7125 - 8400 MHz and 14800 - 15350 MHz | NA |

2) Single technology framework based on a stand-alone architecture (General principle):

b)	Energy efficiency and energy saving: both for network and device.

d)	Enhanced overall coverage, focus on cell-edge performance and UL coverage.

f)	Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

h)	Improved spectrum utilization and operations taking into account diverse spectrum allocations.

3) General RF and UE RF scope in Q4'25 and Q1'26

a) non-spectrum and non-AI UE RF

b) spectrum aggregation framework [General principle h)]

c) UE RF related coverage [General principle d) and f)]

d) UE RF related energy efficiency [General principle b)]

e) other joint UE-BS RF issues

Based on the observations and summaries above, we’d like to prioritize the following topics for discussion in this paper.

- Single carrier output power requirements

- UE Tx impairments and unwanted emission framework

- Spectrum aggregation framework

- Joint UE and BS RF

And some new areas:

5) UE RF related coverage [General principle d) and f) in the 6G SI]

6) UE RF related energy efficiency [General principle b) in the 6G SI]

2 Discussion

2.1 Single carrier output power requirements

The following table shows the applicable frequency range for conducted requirements and OTA requirements during the LTE stage and NR stage.

Table 2 the applicable frequency range for conducted requirements and OTA requirements

| 3GPP RAT | LTE UE RF | NR UE RF (SA) |
| --- | --- | --- |
| FR1 (410MHz ~7125MHz) | Conducted requirements | Conducted requirements |
| FR2-1 (24.25GHz ~ 52.6GHz) | NA | OTA requirements |

There is a classical statement or assumption to clarify the consideration of UE antenna’s capability for conductive requirements.

TS 36.101/TS 38.101-1 For UE(s) with an integral antenna only, a reference antenna(s) with a gain of 0 dBi is assumed for each antenna port(s).

Firstly, “For UE(s) with an integral antenna only” is outdated since most of UE especially for smartphone are implemented with multiple integral antennas without any external antennas. But even for a few kinds of UEs, e.g. FWA, even if external antenna(s) are implemented, a reference antenna(s) with a gain of 0dBi is also assumed from conduced requirements’ perspective.

However, in the real field, the antenna efficiency of integral antennas for most of UEs are less than 100%. And some special antenna designs may consider the impacts from hand phantom and head phantom to improve the antenna efficiency. In other words, the antenna efficiency of UE can vary for different scenarios, different phantoms, different frequency and different UE form factor.

Thus, one view holds that the performance of UE’s antennas is considered as part of large scale path loss because of the varied UE antenna efficiency when RAN4 only specify conducted requirements from UE RF perspective. The following figure show the logic of this view.

Figure 1 The illustration for the relationship among BS/UE antenna performance, Transmission Channel, conducted requirements and OTA requirements.

From testability and deterministic perspective, the UE RF requirements at antenna connectors are under control, stable and easily verified.

Based on the views above, the conducted requirements were specified as UE RF core requirements for FR1. However, as the number of antennas is increasing, it becomes more and more important to verify some OTA requirements for FR1 UE, e.g, MIMO related performance and reference signal related performance.

Although the absolute OTA requirements for FR1 UE are not stable with large variance considering different scenarios, different frequency and different UE form factor, alternatively the relative OTA requirements for FR1 UE can be considered by comparing the same UE between different configurations.

In the last meeting, RAN4 reached the consensus on studying the 6G output power requirements in order to reflect the RF front-end capability, improving the configured maximum output power requirements and associated parameters and studying the default power class.

- Agreements - Study 6G output power requirements to enable more flexibility to different UE implementations to reflect the RF front-end capability for single band - Encourage inputs on how to allow UE to deliver more output power and how power class framework includes e.g. Tx diversity and UL-MIMO - Study improvements on configured maximum output power requirements and associated parameters - Strive to improve clarity on UE power ability under different UE condition (e.g. configured, activated, scheduled) in all output power requirements - Study the default power class for FDD and TDD bands for 6G - Study how to handle SAR/MPE for 6GR

In order to confirm a baseline, RAN4 need to discuss whether the following first paragraph is outdated and need to be updated.

6 Transmitter characteristics 6.1 General Unless otherwise stated, the transmitter characteristics are specified at the antenna connector of the UE with a single or multiple transmit antenna(s). For UE with integral antenna only, a reference antenna with a gain of 0 dBi is assumed.

Maybe it’s too early to decide whether the whole Transmitter characteristics are specified at the antenna connector. For 410MHz ~ 7125MHz, RAN4 can decide that the output power requirements are specified at the antenna connector of the UE with a single or multiple transmit antenna(s). Unless otherwise stated, a reference antenna with a gain of 0 dBi is assumed for UE.

Proposal 1: For 410MHz ~ 7125MHz, RAN4 can decide that the output power requirements are specified at the antenna connector(s) of the UE with a single or multiple transmit antenna(s). FFS the following sentence is still valid or not for output power requirements as general UE antenna efficiency is less than 100%.

Unless otherwise stated, a reference antenna with a gain of 0 dBi is assumed for UE.

For the improvements on configured maximum output power requirements and associated parameters, RAN4 can initially discuss whether the following three parameters are still valid or not. i.e. ΔTIB,c, ∆TC,c , ∆TRxSRS. To my understanding, there is no need to consider ∆TC,c based on the state-of-the-art technology. For ΔTIB,c, based on the state-of-the-art RFIC technology, there is no need to discuss it per band-combination specific and FFS whether ΔTIB,c still need to be kept or not. For ∆TRxSRS, RAN4 need to explore how to reflect it in the RF requirements. Alternatively, RAN4 can consider to specify a separate Pcmax,SRS.

Proposal 2: For the improvements on configured maximum output power requirements and associated parameters, the following three points are proposed.

P2-1: There is no need to consider ∆TC,c based on the state-of-the-art technology.

P2-2: For ΔTIB,c, based on the state-of-the-art RFIC technology, there is no need to discuss it per band-combination specific and FFS whether ΔTIB,c still need to be kept or not.

P2-3: For ∆TRxSRS, RAN4 need to explore how to reflect it in the RF requirements. Alternatively, RAN4 can consider to specify a separate Pcmax,SRS.

For the study of the default power class, RAN4 could consider it from FDD and TDD’s perspective. Alternatively, RAN4 can consider the default power class from sub-frequency group’s perspective as current main path LPAMiDs are designed based on sub1GHz, 1.4GHz~3GHz, 3GHz~5GHz and FFS 6GHz~8.4GHz. When RAN4 discuss the UE default power class, apart from the UE implementation’s perspective, RAN4 still need to study the link budget considering different frequency range.

The UL MCL (Maximum Coupling Loss) can be calculated based on the following formula:

UL MCL (Maximum Coupling Loss) = UE Total transmit power – BS Receiver sensitivity + BS antenna gain

| Frequency Range | Reference Frequency | UE Total transmit power | UL Bandwidth | BS Receiver sensitivity (5dB NF) -168+10log(BW) | BS antenna gain TR 38.922 | UL MCL (Maximum Coupling Loss) |
| --- | --- | --- | --- | --- | --- | --- |
| Sub1GHz | 600MHz 900MHz | 23dBm, 26dBm | 20MHz 40MHz | -95dBm -92dBm | 18dB for 1-C | 139dB (26dBm, 20MHz) 136dB (26dBm, 40MHz) 136dB (23dBm, 20MHz) 133dB (23dBm, 40MHz) |
| 1.4GHz~3GHz | 1.4GHz 2GHz | 23dBm, 26dBm | 50MHz 100MHz | -91dBm -88dBm | 18dB for 1-C | 135dB (26dBm, 50MHz) 132dB(26dBm, 100MHz) 132dB (23dBm, 50MHz) 129dB(23dBm, 100MHz) |
|  |  |  |  |  | 29.2 for AAS | 146.2dB (26dBm, 50MHz) 143.2dB(26dBm, 100MHz) 143.2dB (23dBm, 50MHz) 140.2dB(23dBm, 100MHz) |
|  | 2.6GHz | 26dBm, 29dBm | 100MHz 200MHz | -88 dBm -85 dBm | 29.2 for AAS | 146.2dB(29dBm, 100MHz) 143.2dB(29dBm, 200MHz) 143.2dB(26dBm, 100MHz) 140.2dB(26dBm, 200MHz) |
| 3GHz~5GHz | 3.5GHz 4.9GHz | 26dBm, 29dBm | 100MHz 200MHz 400MHz | -88 dBm -85 dBm -82 dBm | 29.2 for AAS | 146.2dB(29dBm, 100MHz) 143.2dB(29dBm, 200MHz) 140.2dB(29dBm, 400MHz) 143.2dB(26dBm, 100MHz) 140.2dB(26dBm, 200MHz) 137.2dB(26dBm, 400MHz) |
| 6GHz~8.4GHz | 7GHz 8.4GHz | 26dBm, 29dBm | 100MHz 200MHz 400MHz | -88 dBm -85 dBm -82 dBm | 35.2 for AAS | 152.2dB(29dBm, 100MHz) 149.2dB(29dBm, 200MHz) 146.2dB(29dBm, 400MHz) 149.2dB(26dBm, 100MHz) 146.2dB(26dBm, 200MHz) 143.2dB(26dBm, 400MHz) |
| Around 15GHz | 15GHz | FFS | FFS |  |  |  |

Based on the UL MCL calculation above, we can get the margin gap between UE UL MCL budget and Rural Macro Path Loss. Max Distance (3D) is assumed as 1155.06m.

| BS Type | MCL max | MCL min | Reference Frequency (GHz) | LOS/NLOS (38.901) | Shadow fading (dB) | Penetration loss (dB) | Total Pathloss (dB) | Margin Max (dB) | Margin Min (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-C | 139 | 133 | 0.6 | PL_RMa_LOS | 6 | 0 | 108.17 | 30.83 | 24.83 |
| 1-C | 139 | 133 | 0.9 | PL_RMa_LOS | 6 | 0 | 111.69 | 27.31 | 21.31 |
| 1-C | 135 | 129 | 1.4 | PL_RMa_LOS | 6 | 0 | 115.53 | 19.47 | 13.47 |
| 1-C | 135 | 129 | 2 | PL_RMa_LOS | 6 | 0 | 118.63 | 16.37 | 10.37 |
| AAS | 146.2 | 140.2 | 1.4 | PL_RMa_LOS | 6 | 0 | 115.53 | 30.67 | 24.67 |
| AAS | 146.2 | 140.2 | 2 | PL_RMa_LOS | 6 | 0 | 118.63 | 27.57 | 21.57 |
| AAS | 146.2 | 140.2 | 2.6 | PL_RMa_LOS | 6 | 0 | 120.90 | 25.30 | 19.30 |
| AAS | 146.2 | 137.2 | 3.5 | PL_RMa_LOS | 6 | 0 | 123.49 | 22.71 | 13.71 |
| AAS | 146.2 | 137.2 | 4.9 | PL_RMa_LOS | 6 | 0 | 126.41 | 19.79 | 10.79 |
| AAS | 152.2 | 143.2 | 7 | PL_RMa_LOS | 6 | 0 | 129.51 | 22.69 | 13.69 |
| AAS | 152.2 | 143.2 | 8.4 | PL_RMa_LOS | 6 | 0 | 131.09 | 21.11 | 12.11 |

In the following meetings, we will further calculate the path loss and margin gaps considering UMa, NLOS, Penetration loss scenarios and so on.

Proposal 3: For the study of the default power class, RAN4 should not only consider UE implementation, but also need to study the link budget considering different frequency range, e.g. comparison between UE UL MCL and Maximum Path Loss.

2.2 UE Tx impairments and unwanted emission framework

For the Tx impairments from Quadrature Modulation in the transceiver, carrier leakage, image rejection, CIM3 and CIM5 will be further studied. These factors will affect the UL EVM. In current specification, the UL EVM requirements, carrier leakage and image leakage requirements are shown below.

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| Pi/2-BPSK | % | 30 |
| QPSK | % | 17.5 |
| 16 QAM (Mandatory in LTE) | % | 12.5 |
| 64 QAM (Mandatory in NR) | % | 8 |
| 256 QAM (Mandatory in 6GR?) | % | 3.5 |

| IQ Image | dB | -28 | Image frequencies when output power > 10 dBm | Image frequencies (NOTES 2, 3) |
| --- | --- | --- | --- | --- |
|  |  | -25 | Image frequencies when output power ≤ 10 dBm |  |
| Carrier leakage | dBc | -28 | Output power > 10 dBm | Carrier leakage frequency (NOTES 4, 5) |
|  |  | -25 | 0 dBm ≤ Output power ≤ 10 dBm |  |
|  |  | -20 | -30 dBm ≤ Output power < 0 dBm |  |
|  |  | -10 | -40 dBm ≤ Output power < -30 dBm |  |

In NR phase, 64QAM was approved as UE mandatory capability. In the 6GR phase, if it’s agreed to mandatorily support 256QAM, the carrier leakage and image leakage requirements above 10dBm output power should be adjusted to meet total 3.5% EVM budget of UE Tx chain. Current -28dBc IQ image and carrier leakage above 10dBm output power are not enough to meet total 3.5% EVM budget of UE Tx chain.

Generally, RAN4 assume 60dB CIM3 and 70dB CIM5 for MPR simulation. As new frequency range should be studied, e.g. around 7GHz and 15GHz, RAN4 need to discuss whether 60dB CIM3 and 70dB CIM5 for MPR simulation are still valid or necessary for MPR simulation in around 7GHz and 15GHz.

Proposal 4: For UE Tx impairments, the following parts are proposed.

P4-1: In the 6GR phase, if it’s agreed to mandatorily support 256QAM, the carrier leakage and image leakage requirements above 10dBm output power should be adjusted to meet total 3.5% EVM budget of Tx chain.

P4-2: RAN4 need to discuss whether 60dB CIM3 and 70dB CIM5 for MPR simulation are still valid or necessary for MPR simulation in around 7GHz and 15GHz.

2.3 Spectrum aggregation framework

If companies check the first release of LTE UE RF spec (TS 36.101) and NR UE RF spec (TS 38.101-1), the framework of contents were created based on the First Principle (single carrier operation in one band) referring to the Annex. Although inter band UL CA, SUL and intra-band contiguous CA and inter-band CA for DL were introduced together with single carrier operation in the first release during 5G phase, these features were still considered as enhanced feature instead of the basic feature from First Principle.

Table 1 Supported feature(s) in the first release of LTE and NR for UE RF spec

| 3GPP RAT | LTE UE RF (Rel-8) | NR UE RF (SA) (Rel-15) |
| --- | --- | --- |
| Supported feature in the first release of one generation for UE RF spec | 1. Single carrier operation for both DL and UL. | 1. Single carrier operation for both DL and UL. 2. inter-band CA for UL 3. SUL with both UL and DL RF requirements 4. UL MIMO 5. intra-band contiguous CA and inter-band CA for DL 6. DL MIMO, but without UE RF impacts… |

Based on the RAN 6G SI and SID of RAN WGs 6G, the spectrum aggregation framework (i.e. multi carriers operation) become more and more important at 6G stage.

Observation 1: Based on the RAN 6G SI and SID of RAN WGs 6G, the spectrum aggregation framework (i.e. multi carriers operation) become more and more important at 6G stage.

Referring to the contribution [3], it’s observed that most of the operators own at least one LEGACY FDD band below 1GHz and one LEGACY FDD band between 1.4GHz ~2.7GHz. Thus, at the 6G stage, operator could re-farm one LEGACY FDD at least for 6G usage and leave one LEGACY FDD band for the legacy RAT (LTE or NR) & legacy UE.

For large bandwidth TDD bands for 6G usage, operators may have spectrum resources in around 3 GHz to 6 GHz range (such as 3.5 GHz, 4.9 GHz, etc.). Furthermore, candidate new spectrum resources have been discussed, ranging from around 7 to 24 GHz, such as 6.425 – 7.125 GHz (identified in WRC-23 meeting [4]), 7.125 – 8.400 GHz, and 14.800 – 15.350 GHz (recommended for WRC-27), and FR2-1 (24.25GHz ~ 52.6GHz, legacy 5G NR mmWave).

As the center frequency of TDD band with large channel bandwidth become higher and higher, it’s very important for operators to choose one LEGACY FDD band below 2.7GHz to guarantee the UL capability and UL coverage especially for deep coverage for 6G usage.

Observation 2: at the 6G stage, most of the operators have the adequate spectrum resources to initiate one UL band below 2.7GHz and one “band” with large channel bandwidth above 3GHz for 6G usage. This strategy can guarantee a good UL capability and UL coverage especially for the demands of deep coverage at the 6G stage.

On the other hand, current main path LPAMiD modules and diversity path LFEMiD modules can be split in different frequency range, e.g. Sub1GHz, 1.4GHz~2.7GHz, 3.3~5GHz, and so on. It’s UE common capability to support concurrent operation between one UL band below 2.2GHz and one DL band above 3.3GHz, and concurrent operation between one DL band below 2.2GHz and one DL band above 3.3GHz.

Proposal 5: the following common UE capabilities should be mandatorily supported in 6GR from RF’s perspective:

P5-1: To support concurrent operation between one UL band below 2.2GHz and one DL band above 3.3GHz.

P5-2: To support concurrent operation between one DL band below 2.2GHz and one DL band above 3.3GHz.

For RF switch-time performance between UL band, and (UL band or DL band), the following values were summaried in LTE/NR stage.

| Scenarios | Values | The RF components which may affect the RF switch-time performance |
| --- | --- | --- |
| UL and DL switching in the same LTE TDD carrier | 20us | - The switch component in main path between PA/LNA and bands filter. - PA on-off time. |
| UL and DL switching in the same NR TDD carrier | 10us | - The switch component in main path between PA/LNA and bands filter. - PA on-off time. |
| UL and UL switching between different NR bands/carriers. | 0us | Two Tx chains were considered as mandatory UE capability. One Tx chain can be switched to the other Tx chain which have been prepared. |
|  | 35us | One Tx chain is shared between two different UL bands/carriers. Power supply for PA need to be switched without much LO retuning time due to close carrier frequency between two carriers. |
|  | 140us | One Tx chain is shared between two different UL bands/carriers. Longer enough time to retune the LO between two different UL bands/carriers. |
|  | 210us | One Tx chain is shared between two different UL bands/carriers. Some BB parameters should be reconfigured between two different UL bands/carriers. |
| Sub1GHz switching between FDD and SDL. | 35us | ASM switching + Antenna retuning?? The LO is not shared. |
|  | 70us | There is no technical discussion on this value. |
|  | 140us | ASM switching + Antenna retuning + LO retuning Rx chain below LNA including LO is shared between FDD DL band and SDL band. |

It’s unclear whether the PA on-off time can be merged into the UL Tx switching time between different bands/carriers. Otherwise, if 35us is reported by UE, two symbols will be blanked in the real deployment.

Proposal 6: For UL Tx switching scenarios between different bands/carriers, it’s proposed to merge PA on-off time into the UL Tx switching time. Otherwise, more symbols will be blanked in the real deployment which is unexpected.

2.4 Joint UE and BS RF

According to the WF [6], for Tx EVM relaxation, it’s agreed to prioritize discussion on study scope and necessary details aligned with RAN guidance. In addition, there is FFS on whether and how, if agreed, to take into consideration of the existing 5G requirements and RF impairment assumptions for both 5GA and 6G. Thus, we’d like to share our views on these aspects.

- Agreements - Encourage companies to contribute whether there are concrete study points for low power receiver. - Coexistence study including need for coex study will be discussed in agenda for BS RF and co-existence. - Prioritize work for the baseline single carrier output power requirement framework before detailed studies on ACLR and SEM relaxation. - Discuss PAPR reduction topics in system parameter agenda - For Tx EVM relaxation - Discuss non-AI based UL demod in UE RF thread - The impact on BS receiver could be considered. - Do not discuss AI based solution(s) in UE RF thread, except the potential common part. - The potential common part between AI and non-AI solutions will be discussed in this thread. AI solutions, which are discussed under AI agenda, should leverage the agreements in this thread as much as possible. - Strive to study TX non-linearity sources, including PA, in this thread - It is FFS whether and how, if agreed, to take into consideration of the existing 5G requirements and RF impairment assumptions for both 5GA and 6G. - Strive to avoid overlapping work between different threads - In RAN4#117, prioritize discussion on study scope and necessary details aligned with RAN guidance, e.g. - Which aspects need to be considered to conclude on feasibility - Need for network control

Observation 3:

For 5GA Tx EVM relaxation, it’s the truth that the existing 5G requirements and RF impairment assumptions should be took into consideration.

For 6G Tx EVM relaxation, RAN4 has to wait for the discussion on 6G RF requirements and RF impairment assumptions.

Proposal 7: For the SI scope on Tx EVM relaxation, the following points should be captured.

- Study the impacts on BS receiver from both RF and demod perspectives.

- Study the system gain for Tx EVM relaxation.

- Study the potential impacts on the other WGs if network control is needed. Study NBC issues of BS receiver for 5GA if network control is not needed.

3 Discussion on New Area

3.1 UE RF related coverage

As the targeted frequency is increasing, the UL features, e.g. UL coverage enhancement, UL capability enhancement, UL throughput enhancement, SRS enhancement etc., become more and more important. However, current framework of RAN4 spec can’t support the verification of UE UL features from system/UL performance perspective referring the following figure 3. Based on the clause 6 transmitter characteristics of TS 36.101/TS 38.101-1, only some discrete UE requirements, e.g. output power, transmit signal quality etc., are specified or used to verify whether the UL enhancements meet the minimum requirements or not, but RAN4 missed the UE UL verification from system perspective. In addition, current UL enhancements including UL coverage become more and more complicated, it’s impossible to achieve the standardization purpose from system perspective by designing some discrete UE RF requirements.

Although RAN4 specified the EVM requirements as the core requirements for transmit signal quality, EVM requirements just indirectly indicate the quality of transmit signal. Here, we need a new framework to verify UE UL signal performance from system perspective or link level perspective.

F.2 Basic Error Vector Magnitude measurement The EVM is the difference between the ideal waveform and the measured waveform for the allocated RB(s) , where is a set of modulation symbols with the considered modulation scheme being active within the measurement period, are the samples of the signal evaluated for the EVM, is the ideal signal reconstructed by the measurement equipment, and is the average power of the ideal signal. For normalized modulation symbols is equal to 1. The basic EVM measurement interval is defined over one slot in the time domain for PUCCH and PUSCH and over one preamble sequence for the PRACH.

Furthermore, current UL RMC definition for Type A PUSCH can’t be applicable to the complicated UL features. And some key RMCs for SRS feature and HPUE (PC2 above) are missing in current specification. From completion perspective, the new framework for UL features including UE UL coverage should considering the testability of these UL features including appropriated UL RMC definition.

Proposal 8: RAN4 study new UE framework for the missing UE UL performance and the verification of UL features including UE RF related coverage from system perspective or verification of the whole UE RF&BB Tx chains.

Figure 3 the illustration on the missing UE UL Performance

3.2 UE RF related energy efficiency

As we are approaching the 6G Era, to achieving good energy efficiency is very important for both UE vendors and consumers. This is a new area for RAN4 since RAN4 never specify the relative requirements for UE energy efficiency. One model is proposed to illustrate the relationship between energy consumption (costs) and communication service/Throughput (gain) as the figure below.

Figure 4 Model between energy consumption (costs) and communication service/Throughput (gain)

Although the UE energy efficiency is brand new for RAN4, the relative metrics for energy efficiency are not totally brand new for the whole industry. The first metric for energy efficiency (EE) is the bits/Joule (bit/J), which directly reflect the energy consumed by a communication terminal to transmit one unit of bit information.

Energy Efficiency (EE, unit bit/J) = Throughput (bps) / Power (Watt) or = d Throughput(Power)d (Power).

If we go through the open research and discussion, the following metrics can also be found.

Bits Transmitted per Watt (bit/W): commonly used in power-constrained mobile communication terminals, it reflects how many bits of information a terminal can transmit at a fixed Power level, demonstrating the terminal’s data transmission capability under specific power conditions.

Bits Transmitted per Watt (bit/W) = Energy Efficiency (EE, unit bit/J) × time (s) or = EEtdt

Energy Efficiency Ratio (EER): Represents the ratio between the electrical energy consumed by a communication terminal and the effective communication energy it generates per unit time. The effective communication energy refers to the energy used by the terminal for transmitting and processing information in its normal working state.

Tx Energy Efficiency Ratio (EER, unit %) = Tx Output Power (PA/antenna connector/OTA, unit Watt) / Total Input Power for Tx chain (unit Watt)

Standby Power Consumption: Refers to the power consumed by a communication terminal when it is in standby mode, with the unit being Watt (W) or milliwatt (mW). Lower standby power consumption means the terminal consumes less energy when not performing actual communication operations, which may also extend the battery life.

Operating Power Consumption: Refers to the power consumed by a communication terminal when conducting communication services (such as calling, data transmission, video playback, etc., representing different throughputs). It can be measured separately according to different service types, such as call power consumption and internet access power consumption, and reflects the energy consumption of the terminal during actual operation.

Standby Power Consumption (Background) VS Operating Power Consumption

The whole industry aim to achieve the zero vision, i.e. zero bit zero watt, but it seems very difficult. One possible way is to specify the Standby Power Consumption, i.e. how much is Power Consumption when there is no UL/DL throughput transmission. RAN4 may need to answer How many Watt with zero bit.

Proposal 9: RAN4 study the following metrics at least for UE related energy efficiency and model for UE related energy efficiency between energy consumption and Throughput.

1) Energy Efficiency (EE, unit bit/J) = Throughput (bps) / Power (Watt) or = d Throughput(Power)d (Power)

2) Bits Transmitted per Watt (bit/W) = Energy Efficiency (EE, unit bit/J) × time (s) or = EEtdt

3) Energy Efficiency Ratio: e.g. Tx Energy Efficiency Ratio (EER, unit %) = Tx Output Power (PA/antenna connector/OTA, unit Watt) / Total Input Power for Tx chain (unit Watt)

4) Standby Power Consumption VS Operating Power Consumption, but it’s targeted to achieve zero vision, i.e. Zero Bit Zero Watt.

4 Summary

2.1 Single carrier output power requirements

Proposal 1: For 410MHz ~ 7125MHz, RAN4 can decide that the output power requirements are specified at the antenna connector(s) of the UE with a single or multiple transmit antenna(s). FFS the following sentence is still valid or not for output power requirements as general UE antenna efficiency is less than 100%.

Unless otherwise stated, a reference antenna with a gain of 0 dBi is assumed for UE.

Proposal 2: For the improvements on configured maximum output power requirements and associated parameters, the following three points are proposed.

P2-1: There is no need to consider ∆TC,c based on the state-of-the-art technology.

P2-2: For ΔTIB,c, based on the state-of-the-art RFIC technology, there is no need to discuss it per band-combination specific and FFS whether ΔTIB,c still need to be kept or not.

P2-3: For ∆TRxSRS, RAN4 need to explore how to reflect it in the RF requirements. Alternatively, RAN4 can consider to specify a separate Pcmax,SRS.

Proposal 3: For the study of the default power class, RAN4 should not only consider UE implementation, but also need to study the link budget considering different frequency range, e.g. comparison between UE UL MCL and Maximum Path Loss.

2.2 UE Tx impairments and unwanted emission framework

Proposal 4: For UE Tx impairments, the following parts are proposed.

P4-1: In the 6GR phase, if it’s agreed to mandatorily support 256QAM, the carrier leakage and image leakage requirements above 10dBm output power should be adjusted to meet total 3.5% EVM budget of Tx chain.

P4-2: RAN4 need to discuss whether 60dB CIM3 and 70dB CIM5 for MPR simulation are still valid or necessary for MPR simulation in around 7GHz and 15GHz.

2.3 Spectrum aggregation framework

Observation 1: Based on the RAN 6G SI and SID of RAN WGs 6G, the spectrum aggregation framework (i.e. multi carriers operation) become more and more important at 6G stage.

Observation 2: at the 6G stage, most of the operators have the adequate spectrum resources to initiate one UL band below 2.7GHz and one “band” with large channel bandwidth above 3GHz for 6G usage. This strategy can guarantee a good UL capability and UL coverage especially for the demands of deep coverage at the 6G stage.

Proposal 5: the following common UE capabilities should be mandatorily supported in 6GR from RF’s perspective:

P5-1: To support concurrent operation between one UL band below 2.2GHz and one DL band above 3.3GHz.

P5-2: To support concurrent operation between one DL band below 2.2GHz and one DL band above 3.3GHz.

Proposal 6: For UL Tx switching scenarios between different bands/carriers, it’s proposed to merge PA on-off time into the UL Tx switching time. Otherwise, more symbols will be blanked in the real deployment which is unexpected.

2.4 Joint UE and BS RF

Observation 3:

For 5GA Tx EVM relaxation, it’s the truth that the existing 5G requirements and RF impairment assumptions should be took into consideration.

For 6G Tx EVM relaxation, RAN4 has to wait for the discussion on 6G RF requirements and RF impairment assumptions.

Proposal 7: For the SI scope on Tx EVM relaxation, the following points should be captured.

- Study the impacts on BS receiver from both RF and demod perspectives.

- Study the system gain for Tx EVM relaxation.

- Study the potential impacts on the other WGs if network control is needed. Study NBC issues of BS receiver for 5GA if network control is not needed.

3.1 UE RF related coverage

Proposal 8: RAN4 study new UE framework for the missing UE UL performance and the verification of UL features including UE RF related coverage from system perspective or verification of the whole UE RF&BB Tx chains.

3.2 UE RF related energy efficiency

Proposal 9: RAN4 study the following metrics at least for UE related energy efficiency and model for UE related energy efficiency between energy consumption and Throughput.

1) Energy Efficiency (EE, unit bit/J) = Throughput (bps) / Power (Watt) or = d Throughput(Power)d (Power)

2) Bits Transmitted per Watt (bit/W) = Energy Efficiency (EE, unit bit/J) × time (s) or = EEtdt

3) Energy Efficiency Ratio: e.g. Tx Energy Efficiency Ratio (EER, unit %) = Tx Output Power (PA/antenna connector/OTA, unit Watt) / Total Input Power for Tx chain (unit Watt)

4) Standby Power Consumption VS Operating Power Consumption, but it’s targeted to achieve zero vision, i.e. Zero Bit Zero Watt.

Reference

[1] RP-252912, “Revised SID: Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone

[2] R4-2511652, “Plan on RAN4 6G discussion and meeting arrangement in Rel-20”, RAN4 Chair (Apple)

[3] RP-252137, “Consideration on cell management and multi-carrier operation”, Huawei, HiSilicon

[4] https://www.gsma.com/connectivity-for-good/spectrum/wrc-series/

[5] RP-220681, revised WID: RF requirements enhancement for NR frequency range 1 (FR1), Huawei, HiSilicon

[6] R4-2514642, WF for [116bis][102] 6G general RF and UE RF, Feature lead (Qualcomm)

Annex

In the RAN#109 meeting, a 6G RAN WG SI [1] was updated with the following objectives related to UE RF parts.

The study aims to develop one non-backward compatible radio access technology (hereafter “6GR”) to meet a broad range of use cases, and requirements as defined during the RAN requirements study [TR38.914]. The study should strive at dimensioning an appropriate set of functionalities, minimizing the adoption of multiple options for the same functionality, focusing on practical user experience. The study should identify principles to ensure extensibility and deliver superior performance. NOTE: Section 4.1.1 shows interim milestones to reach certain agreements, including early consensus items. From a technology perspective, the study will address frequency ranges up to 52.6GHz, including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around ~7GHz), and FR2-1 (24.25 GHz – 52.6GHz). NOTE: TR 38.921, TR 38.922 will be taken into account. NOTE: Frequency ranges beyond 52.6 GHz are not in scope of the work. The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion. - Physical Layer structure for 6GR, - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4] - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1] - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1] - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4] - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2] - MIMO operation [RAN1, RAN4] - Duplexing [RAN1, RAN4] - Initial access [RAN1, RAN2, RAN4] - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4] - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4] - 6GR spectrum utilization and aggregation. [RAN1, RAN2, RAN4] - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4] - Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study [RAN1]. - RAN4 can be involved, if necessary, based on the LS from RAN1 …… - 6GR core and performance requirements - General RF aspects [RAN4] - Intra-3GPP co-existence studies - Study RF related system parameters and requirements - BS RF requirement aspects including band [RAN4] - BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation - Study how to improve 6G BS core, conformance specifications, including structure and drafting principles - UE RF requirement aspects including band and band combination [RAN4] - UE RF requirement framework aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G UE RF specification(s), including structure, drafting principles, and database for band combination - Study UE RF capabilities considering different device types and implementations - RRM aspects for 6GR [RAN4, RAN1, RAN2] - RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G requirement specification, including structure and drafting principles - Demodulation and performance aspects [RAN4] - Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS - Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS - Aspects related to the testability [RAN4] - Testability methodology framework and key assumptions, aiming to ensure that requirements can be properly tested considering the applicability and feasibility of conductive and/or OTA testing with reasonable complexity - Other aspects - Handling irregular channel bandwidths including the definition [RAN4, RAN1] - Definition of ‘frequency range(s)’[RAN4]

Based on the RAN4 Chair’s guideline [2] on the RAN4 6G discussion and meeting arrangement in Rel-20 and the draft of RAN4#117 meeting agenda, the general scope for UE RF requirements include the following parts.

| 6G Topics | Scope | Scope in Q4'25 and Q1'26 |
| --- | --- | --- |
| General RF and UE RF | non-spectrum UE RF, MIMO operation, duplexing, spectrum aggregation, initial access, multi-TRP, UE RF related coverage and energy efficiency and other joint UE-BS RF issues | non-spectrum and non-AI UE RF, spectrum aggregation, UE RF related coverage and energy efficiency |

The draft of RAN4#117 meeting agenda: # 8.3 General RF and UE RF [FS_6G_Radio] # NOTE: The scope includes non-spectrum and non-AI UE RF, spectrum aggregation framework, UE RF related coverage and energy efficiency, and other joint UE-BS RF issues