3GPP TSG-RAN WG4 Meeting #116bis                                                           R4-2520100

Dallas, US, 17th –21th Nov, 2025

Source: 	CATT

Title: 	BS RF requirements for 4.9GHz new band

Agenda item:	5.11.2

Document for:	Approval

- Introduction

In RAN#108 meeting, a new WI was established for 4.9GHz new band [1] and revised in RAN#109 in [2]. The objective is as below. In this contribution, we focus on system parameters on this new band.

A one quarter study phase to identify and document coexistence requirements for the new band: - Study coexistence issues with adjacent spectrum and other users of the 4940-4990 MHz band. - Identify adjacent spectrum protection requirements - Identify any potential emission limits more stringent than the normal BS and UE emission masks - Identify other technologies operating in the 4940-4990 MHz band - Identify potential coexistence mechanism(s) - Frequency division sharing - Geographic separation - TDD synchronization - Identify any potential emission limits more stringent than the normal BS and UE emission masks Following the completion of the study phase, the core objectives of this WI are: - Specify a new NR TDD operating band for the frequency range, 4940-4990 MHz to support the following SCS and channel bandwidths: SCS (kHz); UE Channel bandwidth (MHz). 3; 5; 10; 15; 20; 25; 30; 35; 40; 45; 50; 60; 70; 80; 90; 100. 15; 10; 15; 20; 25; 30; 40; 50. 30; 10; 15; 20; 25; 30; 40; 50. 60; 10; 15; 20; 25; 30; 40; 50. - Specify band numbering, system parameters and RF characteristics for the channel bandwidths defined for the new band. - Address potential BS and UE co-existence issues, if any. - Specify band specific BS, repeater and IAB emissions requirements if needed - Specify UE additional emission limits and A-MPR if needed - Specify the BS, repeater and IAB RF requirements. - Specify the UE RF requirements for the new band. Reuse NR band n79 UE hardware assumption if possible. - Update the related technical specifications to include support for the new band - CRs shall not be approved until ongoing litigation is resolved to avoid specification rework and potential NBC changes Note: The new NR band will be introduced in a REL-independent way starting from REL-15.

- Discussion

Operating band

4.9GHz new band is overlapping with n79 and should be defined in operating bands in FR1 as in Table 1.

Table 1  NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n114 | 4940 MHz – 4990 MHz | 4940 MHz – 4990 MHz | TDD |

Channel bandwidth

According to the objectives in WI, we can require the channel bandwidth for 4.9GHz band in Table 2.

Table 2  Channel bandwidths for each NR band

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n114 | 15 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |

Channel raster

The applicable NR-ARFCN for new 4.9GHz operating band is listed in Table 3.

Table 3  Applicable NR-ARFCN per operating band

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n114 | 15 | 729334 – <1> – 732666 | 729334 – <1> – 732666 |
|  | 30 | 729334 – <2> – 732666 | 729334 – <2> – 732666 |

Synchronization raster

The applicable SS raster for new 4.9GHz operating band is listed in Table 4.

Table 4  Applicable SS raster entries per operating band for above 3 MHz channel bandwidth

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n114 | 30 kHz | Case C | 8850 – <1> – 8877 |

Proposal: Approve Table 1 to Table 4 on system parameters for new 4.9GHz NR operating band.

- Conclusion

This contribution provides system parameters for 4.9GHz new band and the proposal is as below:

Proposal: Approve Table 1 to Table 4 on system parameters for new 4.9GHz NR operating band.

- References

- RP-251887 New WID on Introduction of NR TDD 4.9GHz Band

- RP-252929 Revised WID on Introduction of NR TDD 4.9GHz band for US operation