3GPP TSG-RAN WG4 Meeting #116bis                                                           R4-2520102

Dallas, US, 17th –21th Nov, 2025

Source: 	CATT

Title: 	Further Considerations on coexistence for UAV

Agenda item:	7.7.3

Document for:	Approval

- Introduction

In RAN4#116bis meeting, some agreements on co-existence were reached in [1], but there are still some issues need to be further discussed, in this contribution, we give our analysis on the open issues.

- Discussion

2.1 Interference cases

According to [1], coexistence scenarios are listed in below table.

Table 1: coexistence scenarios in the WF[1]

|  | Aggressor |  | Victim |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No. | deployment scenario UL/DL | System BW duplex mode | deployment scenario UL/DL | System BW duplex mode | Simulation frequency | ACLR/ACS requirements |
| 1 | TN UL | 100MHz TDD | UAV UL | 100MHz TDD | 4 GHz | UAV BS ACS |
| 2 | UAV DL | 100MHz TDD | TN DL | 100MHz TDD | 4 GHz | UAV BS ACLR |
| 3 | TN DL | 100MHz TDD | UAV DL | 100MHz TDD | 4 GHz | UAV UE ACS |
| 4 | UAV UL | 100MHz TDD | TN UL | 100MHz TDD | 4 GHz | UAV UE ACLR |
| 5 | TN UL | [20MHz] FDD | UAV UL | [20MHz] FDD | 2 GHz | UAV BS ACS |
| 6 | UAV DL | [20MHz] FDD | TN DL | [20MHz] FDD | 2 GHz | UAV BS ACLR |
| 7 | TN DL | [20MHz] FDD | UAV DL | [20MHz] FDD | 2 GHz | UAV UE ACS |
| 8 | UAV UL | [20MHz] FDD | TN UL | [20MHz] FDD | 2 GHz | UAV UE ACLR |

The interference are analysed as below:

1	TN UL to	UAV UL, TN UE TX interfere UAV BS RX, if TN UE amount is numerous, the aggregated interferences at UAV BS RX antenna connector may be high, but as the UE is on the ground, the pathloss between TN UE and  UAV BS may also be large.

2	UAV DL	to TN DL, UAV BS TX interfere TN UE RX, as UAV BS has similar output power as TN BS and uptilt, it is expected the interference is not severe.

3	TN DL to	UAV DL, TN BS TX interfere UAV UE RX, if TN BS ISD is much smaller than UAV BS ISD, UAV UE will be interfered by more TN BS, the pathloss between UAV UE and TN BS close to freespace, so the interference need to be concerned.

4	UAV UL to TN UL, as UAV UE assume higher power, so this case needs to be mainly concerned. If TN BS ISD is much smaller than UAV BS ISD, UAV UE will be easily close to TN BS. But if TN BS ISD is equivalent to UAV BS ISD, more UAV UE will interfere TN BS.

As analysis above, we can achieve below observations:

Observation 1: If TN BS ISD is much smaller than UAV BS ISD, the interference case 1 and 3 may be more sensible.

Observation 2: If TN BS ISD is equivalent to UAV BS ISD, more UAV UE will interfere TN BS, the case 4 may suffer more interference.

2.2 Deployment scenario

Deployment scenario related agreements in [1] are listed as below.

Issue 2-2-1: Deployment scenario - Agreements: - RAN4 needs to further discuss deployment scenarios for coexistence analysis - Option 1: UMa - Option 2: RMa - Option 3: UMa and RMa Issue 2-2-2: Network layout - Agreements: - RAN4 needs to further discuss network layout used for coexistence analysis - Option 1: co-located - Option 2: co-located and non-collocated Figure 2.1.1-1: Illustration of co-located network layout Figure 2.1.1-2: Illustration of non-collocated network layout - Other network layouts such as only covering UAV routes are not precluded. - ISDUAV =X*ISDNR, FFS on X - ISD for UAV: 2-4km for UAV ISD

As analysis and observations in section 2.1, we can use two cases for UAV and TN BS layout, one is the UAV BS ISD is much bigger, the other is they have comparable ISD and coverage.

Table 2: Deployment layout of UAV and TN (extreme case)

| Deployment layout | ISD for UAV | ISD for TN | ISDUAV =X*ISDNR |
| --- | --- | --- | --- |
| 1 | 4km | 500m (UMa) | X=8 |
| 2 | 2km | 1732m (RMa) | X=1.15 |

However, in the real deployment, UAV may have similar network planning as TN, the urban of TN may also be the urban of UAV, the rural of TN may also be the rural of UAV. Then the deployment layout may become:

Table 3: Deployment layout of UAV and TN (realistic case)

| Deployment layout | ISD for UAV | ISD for TN | ISDUAV =X*ISDNR |
| --- | --- | --- | --- |
| 1 | 4km (Rural) | 1732m (RMa) | X=2.3 |
| 2 | 2km(Urban) | 500m (UMa) | X=4 |

Proposal 1: The ISD of UAV and TN BS in the same geographical area should be discussed and decided first. In the co-existence study, both maximum and minimum ISD difference of UAV and TN BS should be studied.

Figure 1 NR BS and UAV BS non-co-located with ISDUAV =2*ISDNR

Figure 2 NR BS and UAV BS non-co-located with ISDUAV =4*ISDNR

In order to take into the account all the necessary interference, at least one site with 3 sectors should be set for UAV BS, and the TN BS should fully cover the UAV 3 sectors’ coverage.

Proposal 2: At least one site with 3 sectors should be set for UAV BS, and the TN BS should fully cover the UAV 3 sectors’ coverage.

- Conclusion

This contribution provides proposals as below:

Observation 1: If TN BS ISD is much smaller than UAV BS ISD, the interference case 1 and 3 may be more sensible.

Observation 2: If TN BS ISD is equivalent to UAV BS ISD, more UAV UE will interfere TN BS, the case 4 may suffer more interference.

Proposal 1: The ISD of UAV and TN BS in the same geographical area should be discussed and decided first. In the co-existence study, both maximum and minimum ISD difference of UAV and TN BS should be studied.

Proposal 2: At least one site with 3 sectors should be set for UAV BS, and the TN BS should fully cover the UAV 3 sectors’ coverage.

- References

- R4-2515159 Way Forward for [116bis][319] NR_UAV