3GPP TSG-RAN WG4 Meeting #116bis                                                           R4-2520103

Dallas, US, 17th –21th Nov, 2025

Source: 	CATT

Title: 	Further Considerations on UE RF for UAV

Agenda item:	7.7.4

Document for:	Approval

- Introduction

In RAN4#116bis meeting, some agreements on UE RF were reached in [1], but there are still some issues need to be further discussed, in this contribution, we give our analysis on the open issues.

- Discussion

Maximum output power

Below is the agreement for maximum output power in last meeting in [1].

Issue 3-2-1: Maximum output power - Agreements: - RAN4 to specify maximum output power for UAV higher power UE considering the following two options: - Option 1: Introduce the new capability for high power UAV to indicate the rated maximum output power (Similar as ATG) - Option 2: Introduce a new power class for UAV UE for up to 33dBm max total conductive output power for the example bands (Similar as legacy TN UE power class definition) - Option 3: Other options are not precluded.

For ATG UE, no power class was introduced, UE can report its rated maximum output power at maximum modulation order and full PRB configurations with granularity as 1dB from 23~40dBm. That is because there seems to be no limitation of ATG UE in terms of size, power consumption, and so on, so ATG UE should have a more linear PA mounted in airplane.

For UAV, as it is targeted to the UAV in the height of 600~1000m in WID, this kind of UAV may have enough size and power to mount a more linear PA similar as ATG. If it is the case, option 1 should be chose.

MPR & A-MPR

If option 1 is chose for maximum output power, no MPR & A-MPR requirement is needed.

Proposal 1: If there is no size and power limitation for PA mounted on UAV, option 1 for maximum output power is proposed and no MPR & A-MPR requirement is needed.

Minimum output power & Maximum input level

Minimum output power & Maximum input power consider the UAV BS and UAV UE are nearest from each other or with the minimum coupling loss between them, these two requirements can based on link level calculation to obtain. In [1], the minimum height for UAV UE is [50m-150m], the minimum horizontal distance [20m], if assuming UAV BS height is 25~30m, the minimum 3D distance between UAV BS and UAV UE is about 30m.

Table 1 Minimum output power from link level calculation

| Parameters | Omni-directional antenna | phased array antenna |
| --- | --- | --- |
| Center frequency GHz | 2.00 | 4.00 |
| Distance meter | 30.00 | 30.00 |
| Pathloss dB | -68.00 | -74.03 |
| BS antenna gain dB | 15.00 | 15.00 |
| UE antenna gain dB | 0.00 | 11.00 |
| Thermal noise power density dBm/MHz | -114.00 | -114.00 |
| BS Noise figure dB | 5.00 | 5.00 |
| Minimum output power dBm/MHz | -56.00 | -60.97 |
| Minimum output power dBm/20MHz | -42.98 | -47.96 |

It should be noted that if we assume the minimum height and distance as 50m and 20m respectively, the minimum output power is stricter than legacy TN UE requirement, we need be careful about the minimum height UAV UE activated.

In the same way, we calculated maximum input level as below table, the maximum input level is much stricter than legacy TN UE requirement.

Table 1 Maximum input level from link level calculation

| Parameters | Omni-directional antenna | phased array antenna |
| --- | --- | --- |
| Center frequency GHz | 2.00 | 4.00 |
| Distance meter | 30.00 | 30.00 |
| Pathloss dB | -68.00 | -74.03 |
| BS output power dBm | 46.00 | 53.00 |
| BS antenna gain | 15.00 | 15.00 |
| UE element gain | 0.00 | 5.00 |
| maximum input level dBm | -7.00 | -1.03 |

Proposal 2: it should be careful about the minimum height UAV UE activated to take into consideration of minimum output power in power control and maximum input level of UAV UE.

ACLR & ACS

ACLR/ACS requirements depend on the coexistence study or analysis. The adjacent channel simulation includes two cases:

- NR and UAV in adjacent channel from same or different operator;

- UAV to UAV in adjacent channel from another operator.

In-band blocking

In-band blocking level should come from Monte-Carlo simulation by interference statistics. The simulation can reuse the coexistence simulation with interference level statistics, the interference comes from below two cases:

- NR BS in adjacent channel from same or different operator;

- UAV BS in adjacent channel from another operator.

Proposal 3: ACLR/ACS requirements and In-band blocking level should base on coexistence study or analysis.

- Conclusion

This contribution provides proposals as below:

Proposal 1: If there is no size and power limitation for PA mounted on UAV, option 1 for maximum output power is proposed and no MPR & A-MPR requirement is needed.

Proposal 2: it should be careful about the minimum height UAV UE activated to take into consideration of minimum output power in power control and maximum input level of UAV UE.

Proposal 3: ACLR/ACS requirements and In-band blocking level should base on coexistence study or analysis.

- References

- R4-2515159 Way Forward for [116bis][319] NR_UAV