3GPP TSG-RAN WG4 Meeting #117	R4-2520104

# Dallas, US, 17th – 21st Nov, 2025

Title: 	Discussion on RRM core maintenance for Rel-19 LTM

Source: 	CATT

Agenda item:	4.23.1

Document for:	Discussion

- Introduction

In the last meeting, the WF on NR_Mob_Ph4_RRM_Part1 was approved [1], but there still some issues left. This document will further discuss the remaining RRM core issues about Conditional LTM, and give our understanding and proposals.

- Discussion

- Conditional Intra-CU LTM

Issue 1-3-1: Whether to include unknown cell case on SSB triggered CLTM

RAN4 previously agreed that specifications only cover the known cell case for CSI-RS L1 triggered CLTM. But for SSB L1 triggered CLTM, no agreement had been reached.  The related conclusions are copied as below:

RAN4#115 For CSI-RS based L1-RSRP measurement: - For known cell, Tmeasure shall include the time period used to acquire the SFN information (TCSI-RS_SFN_intra) and CSI-RS based L1-RSRP measurement period. - Further discuss the conditions for TCSI-RS_SFN_intra = 0. - Further discuss whether to include unknown cell case. If included, add one more component of TPSS/SSS. RAN4#116 Issue 1-1-4: Whether to include unknown cell case < Agreement > Only cover the known cell case.

In the last meeting, RAN4 discussed the related issues and the candidate options are copied as follows [1]:

Discuss the following: - For SSB based L1-RSRP measurement used with CLTM. Discuss and confirm which scenario is supported: - Option 1: Support only known case (specification change needed) - Option 2: Support both unknown and known case (specification change needed) - Option 3: Support only unknown case (no specification change).

In existing spec, the related description is copied as below [2]:

If SSB based L1-RSRP measurement is used in the event, Tmeasure shall be no larger than the maximum of L1-RSRP measurement periods of the serving cell and target cell. 2> serving cell measurement period TL1-RSRP_Measurement_Period_SSB is defined in clause 9.5.4.1 assuming TReport = 0, and 2> if the target cell is an intra-frequency cell: 3> target cell measurement period is TPSS/SSS_sync_intra + TSSB_measurement_period_intra + TL1-RSRP_Measurement_Period_SSB_intra as defined in clause 9.2.5.1 and clause 9.14.5.1 assuming TReport = 0, if deriveSSB-IndexFromCell is enabled or UE has reported SSB index in L3 measurement report of the same cell, or 3> target cell measurement period is TPSS/SSS_sync_intra + TSSB_measurement_period_intra + TL1-RSRP_Measurement_Period_SSB_intra + TSSB_time_index_intra as defined in clause 9.2.5.1 and clause 9.14.5.1 assuming TReport = 0, otherwise. 2> if the target cell is an inter-frequency cell: 3> target cell measurement period is TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TL1-RSRP_Measurement_Period_SSB_inter as defined in clause 9.3.4 and clause 9.15.5 or 9.15.6 assuming TReport = 0, if deriveSSB-IndexFromCellInter-r17 is enabled or UE has reported SSB index in L3 measurement report of the same cell, or 3> target cell measurement period is TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TL1-RSRP_Measurement_Period_SSB_inter + TSSB_time_index_inter as defined clause 9.3.4 and in clause 9.15.5 or 9.15.6 assuming TReport = 0, otherwise.

Some companies believe that although the existing spec seem to define only the requirements for the unknown case, in fact, both known and unknown cases are considered. Moreover, here it is just about following the logic of the legacy CHO to define the minimum requirements, as the legacy CHO also only considered the unknown case.

In our view, CLTM is different with legacy CHO, as there is no concept of known case in legacy CHO requirements, whereas there was definition of known neighbor cell in LTM. The related requirement for legacy CHO and CLTM is copied as below [2]:

CHO: For conditional intra-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify intra with index or Tidentify_intra_without_index defined in clause 9.2.5.1 or clause 9.2.6.2. For Tidentify intra with index: Tidentify_intra_without_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra) ms Tidentify_intra_with_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra + TSSB_time_index_intra) ms LTM: If a neighbor cell is known according to clause 9.14.2, the UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_intra if deriveSSB-IndexFromCell is enabled or UE has reported SSB index in L3 measurement report of the same cell. Otherwise, UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_intra+TSSB_time_index_intra, where TSSB_time_index_intra is the time period used to acquire the index of the SSB being measured given in table 9.2.5.1-3. The cell is considered as known if the following conditions are met in this requirement: - The UE has performed L3 measurement on the target cell during the last 5 seconds, and - The SSB from the target cell configured for L1 measurement remains detectable according to the cell identification requirements specified in clause 9.2. Otherwise, the cell is unknown.

Therefore, in our view, it is better to distinguish between known and unknown target cell cases in the CLTM measurement time requirements (section 6.3.2.2.1) when an SSB-based L1-RSRP measurement is used in the event, and option 2 is ok for us.

Observation 1: CLTM is different with legacy CHO, as there is no concept of known case in legacy CHO requirements, whereas there was definition of known neighbor cell in LTM.

Proposal 1: It is better to distinguish between known and unknown target cell cases in the CLTM measurement time requirements when an SSB-based L1-RSRP measurement is used in the event, and specification change is needed.

- Conclusion

This document discussed the issues for conditional Intra-CU LTM and presented following observation and proposal:

Observation 1: CLTM is different with legacy CHO, as there is no concept of known case in legacy CHO requirements, whereas there was definition of known neighbor cell in LTM.

Proposal 1: It is better to distinguish between known and unknown target cell cases in the CLTM measurement time requirements (section 6.3.2.2.1) when an SSB-based L1-RSRP measurement is used in the event.

- Reference

[1]     R4-2514815, WF on NR_Mob_Ph4_RRM_Part1, Nokia

[2]     3GPP TS 38.133, NR Requirements for support of radio resource management