3GPP TSG-RAN WG4 Meeting #117	R4-2520111

# Dallas, US, 17th – 21st Nov, 2025

Title: 	Discussion on RRM performance requirements for on-demand SSB SCell operation

Source: 	CATT

Agenda item:	6.15.2.1

Document for:	Discussion

- Introduction

In last RAN4 meeting, WF on Netw_Energy_NR_enh_RRM_Part1 was approved [1], and there was still some issues for FFS. In this contribution, we continue to discuss and provide our views on the performance requirements for on-demand SSB SCell operation.

- Discussion

  - Test scope

General – Different OD-SSB and AO-SSB relation in Case 2

In the last meeting, RAN4 discussed test cases for different OD-SSB and AO-SSB relation in Case 2 and two options were proposed. The agreement is copied as below [1]:

Agreement: RAN4 to define TCs for Alt Time-C1. Meanwhile, - Option 1: Not define TC for Alt Time-C2 - Option 2: Select one test for Alt Time-C2. For UE supporting Alt Time-C2, the corresponding test case for Alt Time-C1 can be skipped. Further discuss in one test case or separate test cases.

In core part, RAN4 agreed to define the related requirements for Alt Time-C1 and Alt Time-C2 for case 2, and the definitions of Alt Time-C1 and Alt Time-C2 are copied as follows:

- Case #2: Always-on SSB is periodically transmitted on the cell - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification) when the center frequency locations of always-on SSB and on-demand SSB are same. - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported when the center frequency locations of always-on SSB and on-demand SSB are same.

Some companies think that RAN4 define the requirement for Alt Time-C2 only based on the OD-SSB periodicity, which is the same with case 1, however, we still think that at least one test case for Alt Time-C2 should be introduced to verify the UE behaviour when configuring both OD-SSB and AO-SSB, and Option 2 is ok for us.

In addition, we also believe that Alt Time-C2 is more typical and common case since the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern, which means the restriction in time domain is less compared with Alt Time-C1 from the configuration perspective.

Observation 1: Alt Time-C2 is more typical and common case since the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern, which means the restriction in time domain is less compared with Alt Time-C1 from the configuration perspective.

Proposal 1: RAN4 to select one test for Alt Time-C2. For UE supporting Alt Time-C2, the corresponding test case for Alt Time-C1 can be skipped.

- Conclusion

This document discussed the performance requirements for on-demand SSB SCell operation and presented following observation and proposal:

Observation 1: Alt Time-C2 is more typical and common case since the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern, which means the restriction in time domain is less compared with Alt Time-C1 from the configuration perspective.

Proposal 1: RAN4 to select one test for Alt Time-C2. For UE supporting Alt Time-C2, the corresponding test case for Alt Time-C1 can be skipped.

- Reference

[1] 	R4-2514808, WF on Netw_Energy_NR_enh_RRM_Part1, Ericsson, RAN4#116-bis