3GPP TSG-RAN WG4 Meeting #117	R4-2520118

# Dallas, US, 17th – 21st Nov, 2025

Title: 	Discussion on RRM requirements for (e)RedCap UE enhancement

Source: 	CATT

Agenda item:	7.4.3.2

Document for:	Discussion

- Introduction

A new RAN4 leading work item for NR RRM Phase 6 was approved in RAN#109. At the first stage, (e)RedCap UE enhancement including relaxed RLM/BFD RRM requirements and support of less than 5 MHz channel bandwidth will be discussed. The objectives of core part for NR RRM Phase 6 are as following [1]:

- Specify the following requirements for (e)RedCap UEs [RAN4] - Relaxed RLM/BFD RRM requirements - Use R17 RLM/BFD relaxation for legacy UE as a baseline - Support of less than 5 MHz channel bandwidth (15 PRB transmission bandwidth configuration and 12 PRB SSB) - RF requirements, considering example bands: n100, n106, n26, n28 and n85 - RRM requirements including relaxed RLM/BFD RRM requirements - Note 1: Use R18 3 MHz requirements for non-RedCap UE as a baseline - Note 2: Consider both 1Rx and 2Rx (e)RedCap UEs and HD-FDD - For intra-band co-located contiguous CA, specify necessary RRM requirements to support data scheduling on SCell prior to UE transmitting valid CSI report (to be started in Q4 2026) [RAN4] - Note 3: No new PDSCH demodulation requirement shall be defined.

According to the schedule of this WI, this document will discuss the RRM issues about (e)RedCap UE enhancement and present our proposals.

- Discussion

Topic 1: RRM requirements for less than 5 MHz channel bandwidth

- Issue 1-1: LT5 for HD-FDD (e)RedCap UE

In the last meeting, RAN4 agreed that the existing LT5 requirements for non-RedCap UE can be directly reused for 2Rx (e)RedCap UE with full-duplex FDD, TDD modes. However, the requirements for HD-FDD RedCap UE still need more discussion. The agreement is copied as follows [2]:

Agreement: - The existing LT5 requirements for non-RedCap UE can be directly reused for 2Rx (e)RedCap UE with full-duplex FDD, TDD modes. - Handover - RLM/BFD - Intra-frequency/Inter-frequency measurement - Further check for HD-FDD

In our view, all of HD-FDD related requirements for RedCap UE are requirements applicability rules due to collision handling approach defined in RAN1, which included the following aspects:

- Cell reselection, including paging reception

- RRC Connection Release with Redirection

- Handover

- Random access

- RLM/BFD, including scheduling availability and L1 indication

- intra/inter-frequency measurements, including scheduling availability

- L1-RSRP measurement, including scheduling availability

Take the HD-FDD requirements for maximum interruption in paging reception for RedCap as an example, the following requirement applicability rule is added specifically [3]:

The 1 Rx RedCap in HD-FDD shall meet all applicable requirements specified in clause 4.2.2.6 under the following conditions - at least 1 SSB is available at the UE in the serving cell during the last 160 ms duration.

According to the above, we believe that the related requirements applicability rules for HD-FDD are unrelated to the reduction of transmission bandwidth. So the existing applicability rules defined for HD-FDD can be reused for (e)RedCap UE supporting LT5.

Observation 1: The related requirements applicability rules for HD-FDD are unrelated to the reduction of transmission bandwidth.

Proposal 1: The existing applicability rules defined for HD-FDD can be reused for (e)RedCap UE supporting LT5.

- Issue 1-2: Specification structure for (e)RedCap UE with LT5

For the specification structure of less than 5MHz requirements for (e)RedCap UEs in TS 38.133[2],  since legacy less than 5MHz requirements were defined under clauses defined for normal UE directly, so less than 5MHz requirements for (e)RedCap UEs will be captured under clauses defined for (e)RedCap UEs.

Proposal 2: RAN4 to define less than 5MHz related requirements on the top of those RedCap requirements in Table 2, which will be captured under clauses defined for (e)RedCap UEs.

Topic 2: Relaxed RLM/BFD RRM requirements

Power saving is important for (e)RedCap UEs due to their lower cost and complexity and smaller battery size in devices such as wearables. Therefore, the further enhancement on power saving for (e)RedCap UE is needed. Given that the existing Rel-17 RLM/BFD relaxation for power saving WI has been specified only for non-RedCap UEs in FR1 and FR2, it is natural to extend it to (e)RedCap UEs [1].

- Issue 2-1: Requirement impact for relaxed RLM/BFD

According to Rel-17 RLM/BFD relaxation, the following requirements were defined for non-RedCap UEs for power saving [3]:

Table 1 RRM Requirements defined for RLM/BFD relaxation in Rel-17

| RLM | - Minimum requirement of SSB based radio link monitoring for UE fulfilling relaxed measurement criteria - Minimum requirement of CSI-RS based radio link monitoring for UE fulfilling relaxed measurement criteria |
| --- | --- |
| LR | - Minimum requirement of SSB based beam failure detection for UE fulfilling relaxed measurement criteria - Minimum requirement of CSI-RS based beam failure detection for UE fulfilling relaxed measurement criteria |

Therefore, minimum requirement of SSB/CSI-RS based RLM and BFD for RedCap UE fulfilling relaxed measurement criteria need to be defined for 1Rx and 2Rx (e)RedCap UEs. In detail, in legacy RedCap related requirements, since there were no requirements defined for SSB/CSI-RS based RLM/BFD measurement relaxation 2Rx RedCap UE in FR2, so the related scenarios also excluded for RedCap UE with relaxed RLM/BFD, and the following requirements need to be defined for RedCap UE with relaxed RLM/BFD:

- SSB based RLM/BFD measurement relaxation for 2Rx RedCap UE in FR1

- SSB based RLM/BFD measurement relaxation for 1 Rx RedCap UE in FR1

- CSI-RS based RLM/BFD measurement relaxation 2Rx RedCap UE in FR1

- CSI-RS based RLM/BFD measurement relaxation 1 Rx RedCap UE in FR1

- SSB based RLM/BFD measurement relaxation 2Rx RedCap UE in FR2

- CSI-RS based RLM/BFD measurement relaxation 2Rx RedCap UE in FR2

In addition, since LT5 also need to consider the SSB based relaxed RLM/BFD impact, to follow the principle of only defining the requirements in NR FR1 for (e)RedCap UE supporting LT5 agreed in the last meeting [2], only the following relaxed RLM/BFD requirements need to be defined for RedCap UE supporting LT5:

- SSB based RLM/BFD measurement relaxation for 2Rx RedCap UE in FR1

- SSB based RLM/BFD measurement relaxation for 1 Rx RedCap UE in FR1

Proposal 3: RAN4 to define the following minimum requirements for RedCap UE with relaxed RLM/BFD in R20:

- SSB based RLM/BFD measurement relaxation for 2Rx RedCap UE in FR1

- SSB based RLM/BFD measurement relaxation for 1 Rx RedCap UE in FR1

- CSI-RS based RLM/BFD measurement relaxation 2Rx RedCap UE in FR1

- CSI-RS based RLM/BFD measurement relaxation 1 Rx RedCap UE in FR1

- SSB based RLM/BFD measurement relaxation 2Rx RedCap UE in FR2

- CSI-RS based RLM/BFD measurement relaxation 2Rx RedCap UE in FR2

Proposal 4: RAN4 to define the following minimum relaxed RLM/BFD requirements for RedCap UE supporting LT5 in R20:

- SSB based RLM/BFD measurement relaxation for 2Rx RedCap UE in FR1

- SSB based RLM/BFD measurement relaxation for 1 Rx RedCap UE in FR1

- Issue 2-2: Relaxed RLM/BFD for 2Rx (e)RedCap UE

In the last meeting, RAN4 agreed to reuse the existing R17 RLM/BFD relaxation requirements for both FR1 and FR2, and for full-duplex FDD and TDD. However, it is still FFS for HD-FDD related requirements. The agreement reached in last meeting is copied as below [2]:

Agreement: - Reuse the existing R17 RLM/BFD relaxation requirements for both FR1 and FR2, and for full-duplex FDD and TDD.

In our view, all of HD-FDD related requirements for RedCap UE are requirements applicability rules due to collision handling approach defined in RAN1, which are unrelated to the relaxation of RLM/BFD. So the existing applicability rules defined for HD-FDD can be reused for (e)RedCap UE with RLM/BFD relaxation.

Observation 2: The related requirements applicability rules for HD-FDD are unrelated to the relaxation of RLM/BFD.

Proposal 5: The existing applicability rules defined for HD-FDD can be reused for (e)RedCap UE with RLM/BFD relaxation.

- Issue 2-3: Relaxed RLM/BFD for 1Rx (e)RedCap UE

In R17 requirements of SSB/CSI-RS based RLM/BFD for RedCap UE fulfilling relaxed measurement criteria, only the relaxations for TEvaluate_out_SSB/CSI-RS and TEvaluate_BFD_SSB/CSI-RS were considered, and relaxation factor K1 and the relaxation factor for the lower bound K3 were introduced for non-RedCap UE. In R20, how to relax TEvaluate_out_SSB/CSI-RS and TEvaluate_BFD_SSB/CSI-RS also need to be further discussed for 1Rx (e)RedCap UEs.

It is worth emphasizing that when discussing the specific relaxation for 1Rx (e)RedCap UE with relaxed RLM/BFD (relaxation 3), the legacy relaxation due to reduction in the number of UE Rx branches (relaxation 1) need to be taken into consideration together with RLM/BFD relaxation (relaxation 2) to avoid relaxing too much.

Proposal 6: RAN4 to discuss how to relax TEvaluate_out_SSB/CSI-RS and TEvaluate_BFD_SSB/CSI-RS for 1Rx (e)RedCap UEs.

Proposal 7: When discussing the specific relaxation for 1Rx (e)RedCap UE with relaxed RLM/BFD (relaxation 3), the legacy relaxation due to reduction in the number of UE Rx branches (relaxation 1) need to be taken into consideration together with RLM/BFD relaxation (relaxation 2) to avoid relaxing too much.

In detail, the legacy evaluation period for 1Rx RedCap UE and normal UE with relaxation in FR1 in TS 38.133 are copied as following [3]:

Table 8.1B.2.2-3: Evaluation period TEvaluate_out_SSB,RedCap and TEvaluate_in_SSB,RedCap for FR1 for 1Rx RedCap UE (relaxation 1)

| Configuration | TEvaluate_out_SSB,RedCap (ms) | TEvaluate_in_SSB,RedCap (ms) |
| --- | --- | --- |
| no DRX | Max(400, Ceil(20 P) TSSB) | Max(100, Ceil(5 P) TSSB) |
| DRX cycle≤320 ms | Max(400, Ceil(30 P) Max(TDRX,TSSB)) | Max(100, Ceil(7.5 P) Max(TDRX,TSSB)) |
| DRX cycle>320 ms | Ceil(20 P) TDRX | Ceil(5 P) TDRX |
| NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. |  |  |

Table 8.1.2.4-1: Evaluation period TEvaluate_out_SSB_Relax for FR1 (relaxation 2)

| Configuration | TEvaluate_out_SSB_Relax (ms) |
| --- | --- |
| Max(TDRX,TSSB) ≤80 ms | Max(200 K3 NOTE3, Ceil(15 K1NOTE2 P) Max(TDRX,TSSB)) |
| 80 ms < Max(TDRX,TSSB) ≤160 ms | Ceil(15 P) Max(TDRX,TSSB) |
| NOTE 1: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length and no longer than 80 ms. NOTE 2: K1 = 4 for Max(TDRX,TSSB) ≤40 ms and K1 = 2 for 40 ms<Max(TDRX,TSSB) ≤80 ms. NOTE 3: K3 = K1, if K1 ≤ 2; otherwise K3 = 1. |  |

Take SSB based RLM requirements as an example,

- Case 1: Max(TDRX,TSSB) ≤40 ms and P = 1,

- For relaxation 1: TEvaluate_out_SSB,RedCap = Max(400, 30  Max(TDRX,TSSB))

- For relaxation 2:  K1 = 4 and K3 =1, TEvaluate_out_SSB_Relax = Max(200, 60 Max(TDRX,TSSB))

In Case 1, since TSSB is always larger than or equal to 5ms, so TEvaluate_out_SSB_Relax = 60 Max(TDRX,TSSB) ≤ 2400ms and TEvaluate_out_SSB,RedCap = 400 or 30  Max(TDRX,TSSB) in the above cases. It is obvious that TEvaluate_out_SSB_Relax is always larger than TEvaluate_out_SSB,RedCap, that is, relaxation 2 is more relax than relaxation 1.

- Case 2: 40 ms<Max(TDRX,TSSB) ≤80 ms and P = 1,

- For relaxation 1: TEvaluate_out_SSB,RedCap = Max(400, 30  Max(TDRX,TSSB))

- For relaxation 2:  K1 = 2 and K3 =2, TEvaluate_out_SSB_Relax = Max(400, 30 Max(TDRX,TSSB))

In Case 2, TEvaluate_out_SSB_Relax is always equal to TEvaluate_out_SSB,RedCap, and relaxation 2 is equal to relaxation 1.

- Case 3: 80 ms < Max(TDRX,TSSB) ≤160 ms and P = 1,

- For relaxation 1: TEvaluate_out_SSB,RedCap = Max(400, 30  Max(TDRX,TSSB))

- For relaxation 2:  TEvaluate_out_SSB_Relax = 15 Max(TDRX,TSSB))

In Case 2, TEvaluate_out_SSB_Relax is always larger than TEvaluate_out_SSB,RedCap, that is, relaxation 2 is more relax than relaxation 1.

In summary, TEvaluate_out_SSB_Relax is always larger than or equal to TEvaluate_out_SSB,RedCap in above two cases, and the same conclusion will apply for CSI/RS based RLM requirements and SSB/CSI/RS based BFD requirements.

Observation 3: TEvaluate_out_SSB_Relax for 1Rx RedCap UE is always larger than or equal to TEvaluate_out_SSB,RedCap regardless of the value of Max(TDRX,TSSB), and the same conclusion will apply for CSI/RS based RLM requirements and SSB/CSI/RS based BFD requirements.

Therefore, in our view, for specific relaxation for 1Rx (e)RedCap UE with relaxed RLM/BFD (relaxation 3), the principle of TEvaluate_out_SSB/CSI-RS_Relax,RedCap not exceeding TEvaluate_out_SSB/CSI-RS_Relax defined in Rel-17 RLM can used for 1 Rx RedCap UE, and the principle of TEvaluate_BFD_SSB/CSI-RS_Relax,RedCap not exceeding TEvaluate_BFD_SSB/CSI-RS_Relax defined in Rel-17 BFD can used for 1 Rx RedCap UE.

Proposal 8: For specific relaxation for 1Rx (e)RedCap UE with relaxed RLM/BFD (relaxation 3),

- The principle of TEvaluate_out_SSB/CSI-RS_Relax,RedCap not exceeding TEvaluate_out_SSB/CSI-RS_Relax defined in Rel-17 RLM can used for 1 Rx RedCap UE.

- The principle of TEvaluate_BFD_SSB/CSI-RS_Relax,RedCap not exceeding TEvaluate_BFD_SSB/CSI-RS_Relax defined in Rel-17 BFD can used for 1 Rx RedCap UE.

- Issue 2-4: Entry and exit criteria, singling indication, applicability rules

In the last meeting, RAN4 discussed entry and exit criteria for 2Rx (e)RedCap UE, and agreed to reuse R17 RLM/BFD Entry and exit criteria from legacy UE as a baseline, and the agreement is copied as follows [2]:

Agreement: Reuse R17 RLM/BFD Entry and exit criteria from legacy UE as a baseline.

Another issue is about singling indication. Since RAN2 signalling does not differentiate non-RedCap and RedCap UE, and entry and exit criteria can be reused as baseline, so we prefer to reuse network signalling goodServingCellEvaluationRLM and goodServingCellEvaluationBFD for R20 (e)RedCap UEs.

Observation 4: RAN2 signalling does not differentiate non-RedCap and RedCap UE.

Proposal 9: Prefer to reuse network signalling goodServingCellEvaluationRLM and goodServingCellEvaluationBFD for R20 (e)RedCap UEs.

For whether to reuse UE feature rlm-Relaxation-r17, RAN4 need to firstly discuss whether to support early implementation or release independence from R17. If release independence from R17 can be supported, then we can support to reuse UE feature rlm-Relaxation-r17.

Proposal 10: For whether to reuse UE feature rlm-Relaxation-r17, RAN4 need to firstly discuss whether to support early implementation or release independence from R17.

As we know, the same issue was discussed in R19 (e)RedCap UE with NTN, considering that there were some new things introduced in R19 and the legacy RedCap UE may not support them, so a new UE capability was introduced for R19 (e)RedCap UE with NTN. In R20 for this WI, (e)RedCap UE will support both LT5 and relaxed RLM/BFD. However, LT5 was introduced in R18 and relaxed RLM/BFD was introduced in R17, but there may be no new things will be defined for R20 RedCap UEs. However, at least at this early stage, the related issue can be postponed until most of issues are resolved.

Proposal 11: At least at this early stage, singling indication can be postponed until most of issues are resolved.

- Issue 2-5: Specification structure for relaxed RLM/BFD for (e)RedCap

The legacy requirements for relaxed RLM/BFD for normal UE were defined in separate clauses in 8.1.2.4, 8.1.3.4 for RLM and 8.5.2.4, 8.5.3.4 for BFD. To follow the same principle, the new separate clauses are also needed for (e)RedCap with relaxed RLM/BFD under those clauses defined for (e)RedCap, such as: clauses 8.1B.2.4, 8.1B.3.4 for RLM and clauses 8.5B.2.4, 8.5B.3.4 for BFD.

Proposal 12: RAN4 to introduce new separate clauses for (e)RedCap with relaxed RLM/BFD under those clauses defined for (e)RedCap, such as: clauses 8.1B.2.4, 8.1B.3.4 for RLM and clauses 8.5B.2.4, 8.5B.3.4 for BFD.

- Conclusion

This document discussed RRM requirements for (e)RedCap UE enhancement and presented following observations and proposals:

Observation 1: The related requirements applicability rules for HD-FDD are unrelated to the reduction of transmission bandwidth.

Proposal 1: The existing applicability rules defined for HD-FDD can be reused for (e)RedCap UE supporting LT5.

Proposal 2: RAN4 to define less than 5MHz related requirements on the top of those RedCap requirements in Table 2, which will be captured under clauses defined for (e)RedCap UEs.

Proposal 3: RAN4 to define the following minimum requirements for RedCap UE with relaxed RLM/BFD in R20:

- SSB based RLM/BFD measurement relaxation for 2Rx RedCap UE in FR1

- SSB based RLM/BFD measurement relaxation for 1 Rx RedCap UE in FR1

- CSI-RS based RLM/BFD measurement relaxation 2Rx RedCap UE in FR1

- CSI-RS based RLM/BFD measurement relaxation 1 Rx RedCap UE in FR1

- SSB based RLM/BFD measurement relaxation 2Rx RedCap UE in FR2

- CSI-RS based RLM/BFD measurement relaxation 2Rx RedCap UE in FR2

Proposal 4: RAN4 to define the following minimum relaxed RLM/BFD requirements for RedCap UE supporting LT5 in R20:

- SSB based RLM/BFD measurement relaxation for 2Rx RedCap UE in FR1

- SSB based RLM/BFD measurement relaxation for 1 Rx RedCap UE in FR1

Observation 2: The related requirements applicability rules for HD-FDD are unrelated to the relaxation of RLM/BFD.

Proposal 5: The existing applicability rules defined for HD-FDD can be reused for (e)RedCap UE with RLM/BFD relaxation.

Proposal 6: RAN4 to discuss how to relax TEvaluate_out_SSB/CSI-RS and TEvaluate_BFD_SSB/CSI-RS for 1Rx (e)RedCap UEs.

Proposal 7: When discussing the specific relaxation for 1Rx (e)RedCap UE with relaxed RLM/BFD (relaxation 3), the legacy relaxation due to reduction in the number of UE Rx branches (relaxation 1) need to be taken into consideration together with RLM/BFD relaxation (relaxation 2) to avoid relaxing too much.

Observation 3: TEvaluate_out_SSB_Relax for 1Rx RedCap UE is always larger than or equal to TEvaluate_out_SSB,RedCap regardless of the value of Max(TDRX,TSSB), and the same conclusion will apply for CSI/RS based RLM requirements and SSB/CSI/RS based BFD requirements.

Proposal 8: For specific relaxation for 1Rx (e)RedCap UE with relaxed RLM/BFD (relaxation 3),

- The principle of TEvaluate_out_SSB/CSI-RS_Relax,RedCap not exceeding TEvaluate_out_SSB/CSI-RS_Relax defined in Rel-17 RLM can used for 1 Rx RedCap UE.

- The principle of TEvaluate_BFD_SSB/CSI-RS_Relax,RedCap not exceeding TEvaluate_BFD_SSB/CSI-RS_Relax defined in Rel-17 BFD can used for 1 Rx RedCap UE.

Observation 4: RAN2 signalling does not differentiate non-RedCap and RedCap UE.

Proposal 9: Prefer to reuse network signalling goodServingCellEvaluationRLM and goodServingCellEvaluationBFD for R20 (e)RedCap UEs.

Proposal 10: For whether to reuse UE feature rlm-Relaxation-r17, RAN4 need to firstly discuss whether to support early implementation or release independence from R17.

Proposal 11: At least at this early stage, singling indication can be postponed until most of issues are resolved.

Proposal 12: RAN4 to introduce new separate clauses for (e)RedCap with relaxed RLM/BFD under those clauses defined for (e)RedCap, such as: clauses 8.1B.2.4, 8.1B.3.4 for RLM and clauses 8.5B.2.4, 8.5B.3.4 for BFD.

- Reference

[1]	  RP-252955, WID: NR Radio Resource Management (RRM) Phase 6, Apple

[2]      R4-2514821, WF on NR_RRM_Ph6, Ericsson

[3]      3GPP TS 38.133, NR Requirements for support of radio resource management