3GPP TSG-RAN WG4 Meeting #117	R4-2520120

# Dallas, US, 17th – 21st Nov, 2025

Title: 	Discussion on RRM aspects for 6G

Source: 	CATT

Agenda item:	8.6

Document for:	Discussion

- Introduction

The study for 6G Radio Access Technology aims to develop one non-backward compatible radio access technology, and the motivation is deeply rooted in the growing demands and technological evolution expected in the coming decade. This paper initially shares some views on 6G study.

- Discussion

As 5G systems mature and become widely deployed, it is becoming increasingly clear that merely evolving existing technologies will not suffice to meet the full range of future use cases, particularly those requiring extreme performance, flexibility, and scalability [1].

The study of 6G will focus on a transformative leap in capability, efficiency, and simplicity. Given that new physical layer design and network architecture may be introduced, and RRM study will heavily rely on this, so at least the following RRM aspects according to the WF on 6G RRM approved in the last meeting could be considered at this stage [1].

- Measurement gap (MG) and interruption

Measurement gaps are critical for some types of measurements and it is also is closely related to the overall system throughput. In the last meeting, most of companies are interested in measurement gap and interruption related issues and believed that it is a RAN4 driven topic and the discussion can be started from RAN4.

According to the WF on 6G RRM approved in the last meeting, the following agreements for measurement gap and interruption were reached as follows [2]：

Agreement in main session (Tue): Measurement gap, including gap-less measurement, is agreed as part of RAN4 RRM 6G study. The detailed scope will be further decided. - Interruption enhancement due to gap-less measurement will be part of the MG discussion.

In existing design of Measurement gap, there were too many Gap patterns with different Gap lengths and Gap Repetition Periods for different usages/features and scenarios, such as positioning MG, MUSIM MG, so the design for Gap patterns is so complicated [3]. In addition, considering the complexity and cost of UE implementation, only a few types of Gap patterns are commonly used in practice. In order to simplifying design for Measurement gap in the upcoming 6GR, MG pattern reduction from 5G can be considered.

Observation 1: There were too many Gap patterns with different Gap lengths and Gap Repetition Periods for different usages/features and scenarios, such as positioning MG, MUSIM MG, so the design for Gap patterns is so complicated.

Observation 2: Considering the complexity and cost of UE implementation, only a few types of Gap patterns are commonly used in practice.

Proposal 1: RAN4 to consider MG pattern reduction from 5G.

In addition, we believe that unified MG will become an important trend in the future and RAN2 also discussed this direction in the last meeting. 5G NR has introduced a variety of gap types including per-UE/per-FR gaps, pre-configured MGs, NCSG, autonomous gaps, and concurrent gaps, etc., and some gap mechanism can be applied at the same time, which results in an increase in complexity on both the UE and network sides. To follow the principle of 6G design, we propose that a flexible and simple unified MG framework need to be studied. However, since gap design is closely related to the measurement framework, so the unified MG framework needs to be discussed together with new measurement framework.

Observation 3: 5G NR has introduced a variety of gap types including per-UE/per-FR gaps, pre-configured MGs, NCSG, autonomous gaps, and concurrent gaps, etc., and some gap mechanism can be applied at the same time, which results in an increase in complexity on both the UE and network sides.

Proposal 2: RAN4 to study a flexible and simple unified MG framework in 6G.

Proposal 3: The unified MG framework needs to be studied together with new measurement framework in 6G.

Moreover, measurement Gap is entirely configured by the network itself in 5G, whereas the UE has the best understanding of its own measurements and mobility. Therefore, we believe that some information or request from UE side will help the network to decide whether configured MG is actually used and what kind of gap pattern will be configured, which can be called ‘UE assisted MG configuration’.

Observation 4: Some information or request from UE side will help the network to decide whether configured MG is actually used and what kind of gap pattern will be configured.

Proposal 4: RAN4 to study UE assisted MG configuration in 6G.

In legacy measurements with gap in 5G NR, only one CC measurement was assumed in each MG occasion. For reference signal configured per frequency, it is reasonable that one CC measurement is related to each MG occasion. However, the reference signal for measurements is not necessarily always configured per frequency (such as the configuration per resource set in LTM), especially for 6G. In one resource set, there may be multiple frequency layers measurement, therefore, how to perform Multi-CC measurements in one MG occasion should be discussed to cope with various approach of configurations for future reference signals in 6G.

Observation 5: In legacy measurements with gap in 5G NR, only one CC measurement was assumed in each MG occasion.

Observation 6: The reference signal for measurements is not necessarily always configured per frequency (such as the configuration per resource set in LTM), especially for 6G. In one resource set, there may be multiple frequency layers measurement.

Proposal 5: RAN4 to study Multi-CC measurements in one MG occasion in 6G.

- RRM framework

In last meeting, RAN4 discussed RRM framework involving measurement capability, delay, overhead, accuracy, quantities, and unified measurement. The agreement is copied as below [2]:

Agreement: RRM framework: measurement capability/delay/overhead/accuracy/quantities/unified measurement, is agreed as part of RAN4 RRM 6G study. The detailed scope will be further decided. Note: the title of the main topic can be revised after selection of the sub-topics.

We believe that united/integrated measurement is an important trend in the future and RAN2 also discussed this direction in the last meeting.

5G NR has introduced a variety of measurements from different dimension, including L1/L3 measurements, serving cell/neighbour cell measurements, etc., which are also defined in different clauses and introduced for different usages/motivations. If a united/integrated cross-layers and cross- functions measurement framework can be introduced, all types of measurements for different usages/motivations will be able to use a more integrated design, making it easier for RAN4 to define measurement requirements, and the overall framework will become more organized and clearer. In addition, considering backward compatibility, if new types of measurements are introduced in the future, the measurement framework will also be highly scalable.

Observation 7: 5G NR has introduced a variety of measurements from different dimension, including L1/L3 measurements, serving cell/neighbour cell measurements, etc., which are also defined in different clauses and introduced for different usages/motivations.

Proposal 6: RAN4 to study united/integrated cross-layers (L1/L3) and cross- functions (MIMO/LTM) measurement framework in 6G.

- NTN related RRM

In 6G, harmonized NTN and TN is a mainstream direction for study. Since delay for handover is more important for NTN, so the existing enhancement for HO delay reduction in 5G TN can be considered for NTN as baseline, such as LTM. In addition, there were additional designs for handover and measurement in NTN, so RAN4 need to study how to integrate the requirements of TN and NTN in 6G day-1, at least the requirements will be defined in the same clause as a whole.

Proposal 7: The existing enhancement for HO delay reduction in 5G TN can be considered for NTN as baseline, such as LTM.

Proposal 8: RAN4 to study how to integrate the requirements of TN and NTN in 6G day-1, at least the requirements will be defined in the same clause as a whole.

- RRM related energy efficiency

In 6G, networks and UEs will deeply collect a variety of information, so the energy efficiency issue has become particularly important for networks and UEs. Therefore, in 6G, energy savings is no longer just a direction for optimization, but rather an important core KPI that stands alongside spectral efficiency, latency, and other metrics. In many scenarios, it will even become the primary target, for example, high energy efficiency in low buffer is more valuable for operator. It is necessary to study more extreme energy saving solutions and processes than those in 5G.

Observation 8: In 6G, energy savings is no longer just a direction for optimization, but rather an important core KPI that stands alongside spectral efficiency, latency, and other metrics. In many scenarios, it will even become the primary target, for example, high energy efficiency in low buffer is more valuable for operator.

Proposal 9: RAN4 to study enhanced energy saving solutions and processes than those in 5G for 6G.

Regarding radio resource management, RAN4 could consider energy based RRM. For example, when UE is in a low battery state, the RRM strategy (such as cell selection, handover, etc.) can prioritize networks and frequency bands that are more energy-efficient. For the network, it is also necessary to weigh the energy consumption and the benefit under a certain decision (such as scheduling, awakening, etc.).

Proposal 10: In order to further save energy, RAN4 to consider energy based RRM strategy for 6G.

- Other PHY signal/channel/procedure related RRM

User-centric operation

6G study represents an opportunity to revisit the foundations of radio system design, free from the constraints of backward compatibility, so it is a good chance to consider new architecture for 6G.

In legacy, the characteristic of a centralized network architecture is that its decision-making, control, and main processing functions are highly concentrated in a few or a single central node, with disadvantages including high transmission delay, single point of failure and low data transmission efficiency, etc.

In our view, with the evolution of network architecture, UE centric network could be considered. It is a comprehensive design, which involves many aspects across different layers. The main difference between traditional cellular architecture and UE centric architecture is that there is no concept of cells and it is user-centric, that is, multiple distributed Access Point (APs) will serve a single UE simultaneously. For such a new architecture, RAN4 need to study new framework to support radio resource management.

Proposal 11: RAN4 to study the RRM impact for user-centric operation.

Multi-functional RAN

The existing RRM measurement in NR mainly involves the measurement of link quality, such as RSRP/RSRQ/SINR, and the measurement is mainly used for handover and beam management, etc. In the context of integrated communication and sensing, 6G networks themselves possess high-precision sensing capabilities, where the signals transmitted by BS and UE will be better utilized for environmental sensing.

Observation 9: The existing RRM measurement in NR mainly involves the measurement of link quality, such as RSRP/RSRQ/SINR, and the measurement is mainly used for handover and beam management, etc.

Based on the above, RAN4 can consider to further enriching RRM measurement content and introducing sensing information based mobility management. In addition to link quality measurement such as RSRP/RSRQ/SINR [2], the measurement content and reporting can also include speed, distance, angle, positioning, imaging, or activity detection, etc.

In addition, we believe that RAN4 could consider multi-functional RAN, where communication and sensing functionalities are jointly supported and the resources may be used to both communication and sensing. How to dynamically allocate resources for communication and sensing to achieve optimal overall performance and the related RRM impacts need to be studied.

Proposal 12: RAN4 to consider enriching RRM measurement content and introducing sensing information based mobility management for 6G.

- In addition to link quality measurement such as RSRP/RSRQ/SINR, the measurement content can also include speed, distance, angle, positioning, imaging, or activity detection, etc.

Proposal 13: RAN4 to consider RRM impact for multi-functional RAN, where communication and sensing functionalities are jointly supported for 6G.

- Conclusion

This document discussed the RRM aspects in 6G and presented following proposals and observations:

Observation 1: There were too many Gap patterns with different Gap lengths and Gap Repetition Periods for different usages/features and scenarios, such as positioning MG, MUSIM MG, so the design for Gap patterns is so complicated.

Observation 2: Considering the complexity and cost of UE implementation, only a few types of Gap patterns are commonly used in practice.

Proposal 1: RAN4 to consider MG pattern reduction from 5G.

Observation 3: 5G NR has introduced a variety of gap types including per-UE/per-FR gaps, pre-configured MGs, NCSG, autonomous gaps, and concurrent gaps, etc., and some gap mechanism can be applied at the same time, which results in an increase in complexity on both the UE and network sides.

Proposal 2: RAN4 to study a flexible and simple unified MG framework in 6G.

Proposal 3: The unified MG framework needs to be studied together with new measurement framework in 6G.

Observation 4: Some information or request from UE side will help the network to decide whether configured MG is actually used and what kind of gap pattern will be configured.

Proposal 4: RAN4 to study UE assisted MG configuration in 6G.

Observation 5: In legacy measurements with gap in 5G NR, only one CC measurement was assumed in each MG occasion.

Observation 6: The reference signal for measurements is not necessarily always configured per frequency (such as the configuration per resource set in LTM), especially for 6G. In one resource set, there may be multiple frequency layers measurement.

Proposal 5: RAN4 to study Multi-CC measurements in one MG occasion in 6G.

Observation 7: 5G NR has introduced a variety of measurements from different dimension, including L1/L3 measurements, serving cell/neighbour cell measurements, etc., which are also defined in different clauses and introduced for different usages/motivations.

Proposal 6: RAN4 to study united/integrated cross-layers (L1/L3) and cross- functions (MIMO/LTM) measurement framework in 6G.

Proposal 7: The existing enhancement for HO delay reduction in 5G TN can be considered for NTN as baseline, such as LTM.

Proposal 8: RAN4 to study how to integrate the requirements of TN and NTN in 6G day-1, at least the requirements will be defined in the same clause as a whole.

Observation 8: In 6G, energy savings is no longer just a direction for optimization, but rather an important core KPI that stands alongside spectral efficiency, latency, and other metrics. In many scenarios, it will even become the primary target, for example, high energy efficiency in low buffer is more valuable for operator.

Proposal 9: RAN4 to study enhanced energy saving solutions and processes than those in 5G for 6G.

Proposal 10: In order to further save energy, RAN4 to consider energy based RRM strategy for 6G.

Proposal 11: RAN4 to study the RRM impact for user-centric operation.

Observation 9: The existing RRM measurement in NR mainly involves the measurement of link quality, such as RSRP/RSRQ/SINR, and the measurement is mainly used for handover and beam management, etc.

Proposal 12: RAN4 to consider enriching RRM measurement content and introducing sensing information based mobility management for 6G.

- In addition to link quality measurement such as RSRP/RSRQ/SINR, the measurement content can also include speed, distance, angle, positioning, imaging, or activity detection, etc.

Proposal 13: RAN4 to consider RRM impact for multi-functional RAN, where communication and sensing functionalities are jointly supported for 6G.

- Reference

[1]     RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator), RAN#108

[2]     R4-2514644, WF on 6G RRM, Feature lead (Apple)

[3]     3GPP TS 38.133, NR Requirements for support of radio resource management