3GPP TSG-RAN WG4 Meeting # 117                                                              R4-2520126

Dallas, USA, Nov. 17-21, 2025

Title: 	Discussion on BS conformance requirements for existing Tx requirements for SBFD operation in FR1 and FR2-1

Source: 	CATT

Agenda item:	6.13.2.2

Document for:	Discussion

- Background

The WF of RAN4#116bis [1] concluded that it is necessary to study RF conformance testing of Tx requirements for SBFD operation. In this contribution, we provide our analysis of them.

- Conformance testing for Tx requirements

  - Additional manufacture declarations and updates

In TS 38.104, the Tx requirements for SBFD capable BS and NR BS are specified in separate clauses. In both TS 38.141-1 and TS 38.141-2, references to TS 38.104 Tx requirements shall be updated to include the relevant clauses for SBFD capable BS.

Proposal 1: In both TS 38.141-1 and TS 38.141-2, references to TS 38.104 Tx requirements shall be updated to include the relevant clauses for SBFD capable BS.

The conducted and OTA output power requirements for SBFD capable BS are defined differently from those for NR BS. Therefore, it is necessary to add the definitions of output power requirements for SBFD capable BS in TS 38.141-1 and TS 38.141-2.

Proposal 2: It is necessary to add the definitions of output power requirements for SBFD capable BS to TS 38.141-1 and TS 38.141-2.

The definitions of the conducted and OTA total power dynamic range of SBFD capable BS and NR BS are different. Therefore, it is necessary to add the definition of the total power dynamic range requirement of SBFD capable BS in TS 38.141-1 and TS 38.141-2.
Proposal 3: It is necessary to add the definition of total power dynamic range requirement for SBFD capable BS to TS 38.141-1 and TS 38.141-2.

The definitions of EVM and averaged EVM for SBFD capable BS are different from those of NR BS, and need to be added in TS 38.141-1 and TS 38.141-2.

Proposal 4: The definitions of EVM and averaged EVM for SBFD capable BS need to be added to TS 38.141-1 and TS 38.141-2.

According to existing TS 38.141-2, OTA test requirements specific and OTA measurement chamber specific calibration (and measurement) procedures were captured in TR 37.941 for the following requirements sets:

-	TX and Rx directional requirements

-	In-band and out-of-band TRP requirements

-	Co-location requirements

-	In-band and out-of-band blocking requirements

For SBFD Capable BS, although the transmitter and receiver operate simultaneously, Tx performance remains unaffected by receiver operation; therefore, the OTA measurement chamber setup, calibration, and measurement procedures defined for NR BS can serve as the baseline for testing Tx requirements of SBFD capable BS. However, it must be noted that the useful signal is configured only in the DL subband, certain requirement definitions and calculations differ from conventional NR BS tests, and these differences must be added to TS 38.141-1 and TS 38.141.

Proposal 5: The OTA measurement chamber setup, calibration, and measurement procedures defined for NR BS can serve as the baseline for testing Tx requirements of SBFD capable BS.

Proposal 6: The frequency location of useful signal, certain requirement definitions and calculations differ from conventional NR BS tests, and these differences must be added to TS 38.141-1 and TS 38.141-2.

  - Beam configurations

For SBFD Capable BS, although the transmitter and receiver operate simultaneously, Tx performance remains unaffected by receiver operation, so when test Tx requirements, receivers operation is not required. In the test, the Tx antennas beam is test output beam.

Proposal 7: 	When test Tx requirements of SBFD Capable BS, receivers can be turned off.

Proposal 8: 	In the test of Tx requirements of SBFD Capable BS, the Tx antennas beam is test output beam.

For OTA Tx IMD requirement testing, in addition to considering the beam configuration of the OTA Tx requirements, it is also necessary to consider the isolation requirement that needs to be satisfied between the beam of the co-location interference signal input antenna and the Tx antenna beam.

Proposal 9: For OTA Tx IMD requirement testing, it is necessary to consider the isolation requirement between the beam of the co-location interference signal input antenna and the Tx antenna beam.

  - Additional test models for SBFD-capable BS

For the Tx requirements testing of SBFD-capable BS, since the desired Tx signal is allocated only to the DL subband, so the test signal can reuse those specified for NR BS, but the allocated channel differences needed to be added to each test model specified for NR BS.

Proposal 10: In TS 38.141-1 and TS 38.141-2, the test models specified for NR BS can be reused for the existing Tx requirements testing of SBFD capable BS, but with the different allocated channel in tests.

  - Test signal used to build Test Configurations

For output power exceeding the high/low power threshold, all declared channel bandwidths and SBFD modes (DUD or DU/UD) must be tested, so configuration-limited is needed. Conversely, for output power below the threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested, so configuration-flexible is also needed.

For the subcarrier spacing of the test signals used to build test configurations, based on common values in design implementation, we recommend 30 kHz for FR1 and 120 kHz for FR2-1.

Proposal 11: In TS 38.141-1 and TS 38.141-2, for the subcarrier spacing of the test signals used to build test configurations, we recommend 30 kHz for FR1 and 120 kHz for FR2-1.

- Summary

In this contribution, we have the following proposals.

Proposal 1: In both TS 38.141-1 and TS 38.141-2, references to TS 38.104 Tx requirements shall be updated to include the relevant clauses for SBFD capable BS.

Proposal 2: It is necessary to add the definitions of output power requirements for SBFD capable BS to TS 38.141-1 and TS 38.141-2.

Proposal 3: It is necessary to add the definition of total power dynamic range requirement for SBFD capable BS to TS 38.141-1 and TS 38.141-2.

Proposal 4: The definitions of EVM and averaged EVM for SBFD capable BS need to be added to TS 38.141-1 and TS 38.141-2.

Proposal 5: The OTA measurement chamber setup, calibration, and measurement procedures defined for NR BS can serve as the baseline for testing Tx requirements of SBFD capable BS.

Proposal 6: The frequency location of useful signal, certain requirement definitions and calculations differ from conventional NR BS tests, and these differences must be added to TS 38.141-1 and TS 38.141-2.

Proposal 7: 	When test Tx requirements of SBFD Capable BS, receivers can be turned off.

Proposal 8: 	In the test of Tx requirements of SBFD Capable BS, the Tx antennas beam is test output beam.

Proposal 9: For OTA Tx IMD requirement testing, it is necessary to consider the isolation requirement between the beam of the co-location interference signal input antenna and the Tx antenna beam.

Proposal 10: In TS 38.141-1 and TS 38.141-2, the test models specified for NR BS can be reused for the existing Tx requirements testing of SBFD capable BS, but with the different allocated channel in tests.

Proposal 11: In TS 38.141-1 and TS 38.141-2, for the subcarrier spacing of the test signals used to build test configurations, we recommend 30 kHz for FR1 and 120 kHz for FR2-1.

Reference

[1] R4-, “Way Forward for [116][307] NR_duplex_evo_BSRF”, Huawei, RAN4#116bis