3GPP TSG-RAN WG4 Meeting # 117                                                              R4-2520127

Dallas, USA, Nov. 17-21, 2025

Title: 	Discussion on BS conformance requirements for existing Rx requirements for SBFD operation in FR1 and FR2-1

Source: 	CATT

Agenda item:	6.13.2.3

Document for:	Discussion

- Background

The WF of RAN4#116bis [1] concluded that it is necessary to study RF conformance testing of Rx requirements for SBFD operation. In this contribution, we provide our analysis of them.

- Conformance testing for Rx requirements

  - Additional manufacture declarations and updates

In TS 38.104, the Rx requirements for SBFD capable BS and NR BS are specified in separate clauses. In both TS 38.141-1 and TS 38.141-2, references to TS 38.104 Rx requirements shall be updated to include the relevant clauses for SBFD capable BS.

Proposal 1: In both TS 38.141-1 and TS 38.141-2, references to TS 38.104 Rx requirements shall be updated to include the relevant clauses for SBFD capable BS.

According to existing TS 38.141-2, OTA test requirements specific and OTA measurement chamber specific calibration (and measurement) procedures were captured in TR 37.941 for the following requirements sets:

-	TX and Rx directional requirements

-	In-band and out-of-band TRP requirements

-	Co-location requirements

-	In-band and out-of-band blocking requirements

The definition of OTA sensitivity for SBFD capable BS takes self-interference into account. Therefore, when testing Rx requirements, all the Tx transmitters must be turned on. Thus, the OTA measurement chamber setup and calibration (and measurement) procedures for SBFD capable BS are different from those of NR systems. Therefore, the differences must be added to TS 38.141-2.

Proposal 2: The OTA measurement chamber setup and calibration (and measurement) procedures of Rx requirements testing for SBFD capable BS are different from those of NR systems, and these differences must be added to TS 38.141-2.

  - Beam configurations

The definition of OTA sensitivity accounts for self-interference, thus the beam configurations of Tx and Rx antennas need to consider the isolation requirement between them. The SBFD capable BS implements separated Rx and Tx antenna planes, when testing Rx OTA requirements, the antenna beam configurations of both the interference signal input antenna and the test output antenna must also be considered.

Proposal 3: For OTA Rx requirements testing of SBFD capable BS, the beam configurations of Tx and Rx antennas need to consider the isolation requirement between them.

For OTA Rx requirements, in addition to the beam configurations of the Tx and Rx antennas, it is also necessary to consider the directional correspondence that needs to be satisfied between the test input antenna beam and the Rx antenna beam.

Proposal 4: For OTA Rx requirements testing of SBFD capable BS, it is necessary to consider the directional correspondence between the test input antenna beam and the Rx antenna beam.

For OTA co-location blocking requirements, in addition to the beam configurations of the Tx and Rx antennas, it is also necessary to consider the isolation requirement that needs to be satisfied between the beam of the co-location interference signal input antenna and the Rx antenna beam.

Proposal 5: For OTA co-location blocking requirement testing of SBFD capable BS, it is necessary to consider the isolation requirement between the beam of the co-location interference signal input antenna and the Rx antenna beam.

  - Additional test models for SBFD-capable BS

Since the definition of OTA sensitivity takes self-interference into account, both the transmitter and the receiver operate simultaneously, and a new test model which transmitter and the receiver operate simultaneously must be set up for Rx requirement testing of SBFD capable BS.

Proposal 6: A new test model which transmitter and the receiver operate simultaneously must be set up for Rx requirement testing of SBFD capable BS.

In existing TS 38.141-1 and TS 38.141-2, there are test requirements for each Rx requirement of NR BS. These test requirements can serve as the baseline for SBFD capable BS, but the NR channel bandwidth needs to be updated to the UL subband bandwidth of the SBFD capable BS. Additionally, any test requirements for SBFD capable BS that differ from NR BS test requirements must be specified for SBFD capable BS in TS 38.141-1 and TS 38.141-2.

Proposal 7: Test requirements for each Rx requirement of NR BS in TS 38.141-1 and TS 38.141-2 can serve as the baseline for SBFD capable BS.

Proposal 8: In test requirements, NR channel bandwidth needs to be updated to the UL subband bandwidth of the SBFD capable BS.

Proposal 9: Any test requirements for SBFD capable BS that differ from NR BS test requirements must be specified for SBFD capable BS in TS 38.141-1 and TS 38.141-2.

- Summary

In this contribution, we have the following proposals and observations.

Proposal 1: In both TS 38.141-1 and TS 38.141-2, references to TS 38.104 Rx requirements shall be updated to include the relevant clauses for SBFD capable BS.

Proposal 2: The OTA measurement chamber setup and calibration (and measurement) procedures of Rx requirements testing for SBFD capable BS are different from those of NR systems, and these differences must be added to TS 38.141-2.

Proposal 3: For OTA Rx requirements testing of SBFD capable BS, the beam configurations of Tx and Rx antennas need to consider the isolation requirement between them.

Proposal 4: For OTA Rx requirements testing of SBFD capable BS, it is necessary to consider the directional correspondence between the test input antenna beam and the Rx antenna beam.

Proposal 5: For OTA co-location blocking requirement testing of SBFD capable BS, it is necessary to consider the isolation requirement between the beam of the co-location interference signal input antenna and the Rx antenna beam.

Proposal 6: A new test model which transmitter and the receiver operate simultaneously must be set up for Rx requirement testing of SBFD capable BS.

Proposal 7: Test requirements for each Rx requirement of NR BS in TS 38.141-1 and TS 38.141-2 can serve as the baseline for SBFD capable BS.

Proposal 8: In test requirements, NR channel bandwidth needs to be updated to the UL subband bandwidth of the SBFD capable BS.

Proposal 9: Any test requirements for SBFD capable BS that differ from NR BS test requirements must be specified for SBFD capable BS in TS 38.141-1 and TS 38.141-2.

- Reference

[1] R4-, “Way Forward for [116][307] NR_duplex_evo_BSRF”, Huawei, RAN4#116bis