3GPP TSG-RAN WG4 Meeting # 117                                                             R4-2520130

Dallas, USA, Nov. 17-21, 2025

Title: 	Discussion on RF conformance testing of A-IoT

Source: 	CATT

Agenda item:	6.14.2

Document for:	Discussion

- Background

The WF of RAN4#116bis [1] concluded that it is necessary to study RF conformance testing of A-IoT BS and CW node. In this contribution, we provide our analysis of them.

- Discussion

2.1 Measurement of CW node

Table 1: Maximum Test System uncertainty for CW node tests

| Clause | Maximum Test System Uncertainty |
| --- | --- |
| CW output power | ±0.7 dB, f ≤ 3 GHz |
| Frequency error | ± 12 Hz |
| Operating band unwanted emissions | ±1.5 dB, f ≤ 3 GHz |
| Transmitter spurious emissions, Mandatory Requirements | 9 kHz < f ≤ 4 GHz: ±2.0 dB |

We recommend adding the maximum test system uncertainty for CW node testing to TS 38.195, with reference to the related content shown in Table 1.

Proposal 1: We recommend adding the maximum test system uncertainty for CW node testing to TS 38.195, with reference to the related content shown in Table 1.

2.2	Manufacturer declarations

Table 2 Manufacturer declarations for CW node conducted test requirements

| Declaration identifier | Declaration | Description | Applicability |
| --- | --- | --- | --- |
|  |  |  | CW node |
|  | CW node requirements set | Declaration of one of the CW node requirement's set as defined for CW node. | x |
|  | Operating bands and frequency ranges | List of CW operating band(s) supported by the antenna connector of the CW node and if applicable, frequency range(s) within the operating band(s) that the CW node operate in. Declarations shall be at the antenna connector for CW node. | x |
|  | Maximum CW RF Bandwidth | Maximum CW RF Bandwidth in the operating band for single-tone operation. Declared antenna connector for CW node. |  |
|  | Single or multiple tone | Capable of operating with a single -tone (only) or multiple -tone. Declared at the antenna connector for CW node. | x |
|  | Rated total output power | Mean power of the CW signal. Declared at the antenna connector for CW node. | x |
|  | Spurious emission category | Declare the CW node spurious emission category as category B with respect to the limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [5]. | x |

The Rated total output power (Prated) of the CW node is the mean power of the single-tone signal that the manufacturer has declared to be available at the antenna connector during the transmitter ON period and it needs to be added in the manufacturer declarations for A-IoT BS test requirements. The CW node requirements set, operating bands and frequency ranges, the maximum CW RF bandwidth, single or multiple-tone and Tx spurious emissions of CW node are also needed to be added in the manufacturer declarations.

Maximum CW RF Bandwidth is not specified in TS 38.194, but in conformance testing, the test configuration and RF channel configuration need this parameter, this parameter can be set as 15kHz.

Proposal 2: Maximum CW RF Bandwidth is not specified in TS 38.194, but this parameter is needed in conformance testing, this issue needs to be discussed in the meeting.

We recommend adding the manufacturer declarations for CW node conducted test requirements to TS 38.195, with reference to the related content shown in Table 2.

Proposal 3: We recommend adding the manufacturer declarations for CW node conducted test requirements to TS 38.195, with reference to the related content shown in Table 2.

2.3 A-IoT test configuration for CW node

CW node supports only one-tone signal. Test configuration can be as follows:

The purpose of test configuration TC * is to test all CW node requirements.

TC* generation

TC* is constructed using the following method:

-	The CW RF Bandwidth shall be the declared maximum CW RF Bandwidth.

-	Place a single-tone CW signal within the CW RF Bandwidth.

-	Set the power of CW signal to the rated output power Prated, according to the manufacturer’s declaration.

Proposal 4: Define single-tone CW signal test configuration for CW node.

2.4 Requirement set applicability

Table 3: Requirement set applicability

| Requirement | A-IoT BS | CW node |
| --- | --- | --- |
| BS output power | x |  |
| Output power dynamics | NA |  |
| Transmit ON/OFF power | x |  |
| Transmitted signal quality | x |  |
| Occupied bandwidth | x |  |
| ACLR | x |  |
| Operating band unwanted emissions | x |  |
| Transmitter spurious emissions | x |  |
| Transmitter intermodulation | NA |  |
| Reference sensitivity level | x |  |
| Dynamic range | NA |  |
| In-band selectivity and blocking | x |  |
| Out-of-band blocking | x |  |
| Receiver spurious emissions | x |  |
| Receiver intermodulation | x |  |
| In-channel selectivity | NA |  |
| Performance requirements | x |  |
| CW Output power |  | x |
| Frequency error |  | x |
| Phase noise |  | x |
| Operating band unwanted emissions |  | x |
| Transmitter spurious emissions |  | x |

The requirement set applicability listed in table 3 can be used for A-IoT BS and CW node.

Proposal 5: The requirement set applicability listed in table 3 can be used for A-IoT BS and CW node.

2.5 RF channels

For the single-tone signal testing, many tests can perform with appropriate frequencies in the bottom, middle and top frequency point of the supported frequency range of the CW node. These are denoted as RF frequency point B (bottom), M (middle) and T (top).

Unless otherwise stated, the test shall be performed with a single tone at each of the RF frequency point B, M and T.

Many tests can perform with the maximum CW RF Bandwidth located at the bottom, middle and top of the supported frequency range in the operating band. These can be denoted as BRFBW_CW (bottom), MRFBW_CW (middle) and TRFBW_CW (top).

Unless otherwise stated, the test shall be performed at BRFBW_CW, MRFBW_CW and TRFBW_CW defined as following:

-	BRFBW_CW: maximum CW node RF Bandwidth located at the bottom of the supported frequency range in the operating band.

-	MRFBW_CW: maximum CW node RF Bandwidth located in the middle of the supported frequency range in the operating band.

-	TRFBW_CW: maximum CW node RF Bandwidth located at the top of the supported frequency range in the operating band.

When a test is performed by a test laboratory, the position of B, M and T for single tone, BRFBW_CW, MRFBW_CW and TRFBW_CW shall be specified by the laboratory. The laboratory may consult with operators, the manufacturer or other bodies.

Proposal 6: We recommend adding the RF channels for CW node to TS 38.195.

Table 4: RF channels to be tested for CW node

| Requirement | RF channels to be tested |
| --- | --- |
| CW Output power | B, M, T |
| Frequency error | M |
| Phase noise | M |
| Operating band unwanted emissions | B, M, T |
| Transmitter spurious emissions | B, T |

The advised RF channel to be tested for each requirement of CW node is shown in table 4.

Proposal 7: The advised RF channel to be tested for each requirement of CW node is shown in table 4.

2.6 A-IoT test model for CW node

CW node only transmit single-tone CW signal, the CW signal can be used for tests on:

-	  CW output power

-	  Frequency error

-	  Unwanted emissions

-	Phase noise

-	Operating band unwanted emissions

-    Transmitter spurious emissions

Proposal 8: The single-tone CW signal can be used for tests of all requirements of CW node.

2.7 Environmental requirements for the BS equipment

The Annex B.2, B.3 and B.5 in TS 38.141-1 can be reused for A-IoT BS test and CW node test.

Proposal 9: The Annex B.2, B.3 and B.5 in TS 38.141-1 can be reused for A-IoT BS test and CW node test.

2.8	 Measurement system set-up

Figure 1: Measurement system set-up of CW node

The measurement system set-up of CW node needs to be added in annex D of TS 38.195, as shown in figure 1.

Proposal 10: The measurement system set-up of CW node needs is shown in figure 1, and it needs to be added to TS 38.195.

- Summary

In this contribution, we have the following proposals.

Proposal 1: We recommend adding the maximum test system uncertainty for CW node testing to TS 38.195, with reference to the related content shown in Table 1.

Proposal 2: Maximum CW RF Bandwidth is not specified in TS 38.194, but this parameter is needed in conformance testing, this issue needs to be discussed in the meeting.

Proposal 3: We recommend adding the manufacturer declarations for CW node conducted test requirements to TS 38.195, with reference to the related content shown in Table 2.

Proposal 4: Define single-tone CW signal test configuration for CW node.

Proposal 5: The requirement set applicability listed in table 3 can be used for A-IoT BS and CW node.

Proposal 6: We recommend adding the RF channels for CW node to TS 38.195.

Proposal 7: The advised RF channel to be tested for each requirement of CW node is shown in table 4.

Proposal 8: The single-tone CW signal can be used for tests of all requirements of CW node.

Proposal 9: The Annex B.2, B.3 and B.5 in TS 38.141-1 can be reused for A-IoT BS test and CW node test.

Proposal 10: The measurement system set-up of CW node needs is shown in figure 1, and it needs to be added to TS 38.195.

Reference

[1] R4-2515058, “WF for [116bis][314] A-IoT_BSCW”, Huawei, RAN4#116bis