3GPP TSG-RAN WG4 Meeting #117				                                                R4-2520138

Dallas, USA, November 17th – November 21st, 2025

Title:	Discussion on the maintenance of RRM core requirements of NR RRM Ph5

Source:	CATT

Agenda Item:	4.15.1

Document for:	Discussion

1 Introduction

In last meeting, RAN4 discussed the remaining issues on enhancement for NR RRM Ph5 including FR2-1 SSB based measurement delay reduction by optimizing Rx BSF and FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC. The agreements were captured in the approved WF [1].

In this contribution, we would like to discuss some maintenance issues on the core part and present our views.

2 Discussion

Issue 1: Condition of re-entering FSB after exiting

RAN4 discussed the re-evaluation mechanism of FBS in last meeting and the following way forward was provided:

RAN4#116bis: Issue 1-1-1: Condition of re-entering FSB after exiting <Way Forward>: - Further discuss in next meeting considering the below options: - Option1: Define T2 for re-evaluation after the UE exits FBS mode. - Option2: Not to define T2 for re-evaluation after the UE exits FBS mode. - Option 2.1: Need to update core requirements. - Option 2.2: Not need to update core requirements.

Regarding the re-evaluation mechanism after UE falls back to normal measurements, companies still have different views. In order to make progress in this WI, one compromise is to introduce time duration T2, and UE shall re-enter FBS mode no later than T2 expires when the triggering conditions remain satisfied during T1 and T2.

Proposal 1: RAN4 to introduce time duration T2 and UE shall re-enter FBS mode no later than T2 expires when the triggering conditions remain satisfied during T1 and T2.

Issue 2: Applicability of FBS when the target and serving carrier frequency are on different bands

The way forward is reproduced below:

<Way Forward>: - Further discuss the options: - Option 1: When the target and serving carrier frequency are different, FBS applies to target carrier only. - Option 2: When the target and serving carrier frequency are different, FBS applies to the serving carrier and target carrier. - Option 3: Introduce UE capability indicating UE can support FBS on both target and serving carrier frequency.

Firstly we do not see the necessity of introducing a UE capability for this clarification issue and the Option 3 can be ruled out. Between Option 1 and Option 2, the only difference is whether to apply the FBS to serving cell as well. From our perspective, the motivation of introducing FBS is to quickly complete the measurements on FR2-1 neighbouring cells, which is beneficial for potential handover. But for serving cell, it depends on UE implementation whether to use FBS to perform the measurement.

Proposal 2: When the target and serving carrier frequency are different, FBS at least applies to the target carrier.

3 Conclusion

In this paper, we discussed some maintenance issues on the core part of NR RRM Ph5 WI and the following proposals are provided:

Proposal 1: RAN4 to introduce time duration T2 and UE shall re-enter FBS mode no later than T2 expires when the triggering conditions remain satisfied during T1 and T2.

Proposal 2: When the target and serving carrier frequency are different, FBS at least applies to the target carrier.

4 References

- R4-2514800, WF on RRM requirements for NR_RRM_Ph5_Part1, Moderator (Ericsson), RAN4#116bis.