3GPP RAN WG4 Meeting #117	R4-2520144

Dallas, USA, November 17th – November 21st, 2025

Title: 	Discussion on RRM core requirements and testing framework for BM

Source: 	CATT

Agenda item:	6.11.4

Document for:	Discussion

Introduction

In RAN4#116bis meeting, RAN4 continued to discuss AI/ML-based beam management in Rel-19 AI/ML for NR air interface and the agreements were captured in [1]. In this contribution, we will further discuss some open issues summarized in [2].

Discussion

Metrics/KPIs for beam ID prediction

In previous RAN4 meetings, this issue was continuously discussed and RAN4 reached the following agreement:

Issue 2-1: Metrics/KPIs for beam prediction RAN4#114bis Agreement: - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" RAN4#115 Proposals: - Option 1: adopt N - N=2 - Further study the actual value - N>2 - N= K2+1 - Option 2: Do not use N, remove the FFS on top-N - Option 3: x<3dB - Option 4: further study x based on simulations - Option 5: Others (e.g. different distributions, etc.) RAN4#116 Agreement: - If RSRP prediction is reported, absolute/relative RSRP requirements applies to - reported beams where SNR side condition is met - The exact RSRP reporting including both absolute and relative will be defined in RAN1 - The related test can only apply to the top-K, where the value of K is FFS, beams. - The legacy beam management test and parameters can be taken as the reference to decide the exact value of K - The feasibility of the test will be further discussed and decided. RAN4#116bis Agreement: - Regarding the metric for beam ID only prediction, where top-K predicted beam(s) are reported, RAN4 only specifies the requirements for the following scenario(s): - K=1 - In this case, the ground truth RSRP of the predicted beam is larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB

In previous meetings, RAN4 have agreed to use the successful rate as a metric/KPI for the correct prediction when only beam ID is reported. For K=1, RAN4 agreed that the concept of successful rate applies. But for K>1, different views still exist. A main discussion point is whether there will be measurement and report configuration for the reported K beams. Based on our understanding on RAN1 discussion and design, the main motivation is to reduce the number of reference signals transmitted and measured by UE. Therefore it is okay to not validate more than 1 reported beam, i.e., N can be removed from the concept of successful rate.

Proposal 1: Remove N in the definition of successful rate.

Conclusions

This paper discussed of RRM core requirement issues related to AI/ML for beam management, and following proposals are provided:

Proposal 1: Remove N in the definition of successful rate.

References

[1] R4-2514631, WF on [116bis][112] R19 AI for air interface, Qualcomm Incorporated, RAN4#116bis.

[2] R4-2514519, Topic summary for [116bis][112] R19 AI for air interface, Qualcomm Incorporated, RAN4#116bis.