3GPP RAN WG4 Meeting #117	R4-2520146

Dallas, USA, November 17th – November 21st, 2025

Title: 	Discussion on RRM performance requirements for SBFD

Source: 	CATT

Agenda item:	6.13.3

Document for:	Discussion

Introduction

In RAN4#116bis meeting, RAN4 continued to discuss the SBFD RRM performance requirements. In this contribution, we will further discuss the remaining issues for performance requirements for both UE to UE CLI measurements and SBFD operation based on the RAN4#116bis WF [1] and open issue summary [2].

Discussion

Time offset for L1-SRS-RSRP measurements

In last meeting, RAN4 discussed the time offset for L1-SRS-RSRP measurements and achieved the agreement shown below:

- Agreement - Specify the time offset conditions for L1-SRS-RSRP measurement accuracy requirements with cell phase error for [15kHz SCS and] 30kHz SCS and without cell phase error for other SCS

For 15 kHz SCS, the square bracket was added and the argument is that there will no test cases defined for 15 kHz SCS, with which we also agree. But from the perspective of integrity of performance requirements, requirements with 15 kHz SCS will be defined and this agreement also applies for the requirements with 15 kHz SCS. Therefore we propose to remove the square bracket and keep 15 kHz SCS in the agreement.

Proposal 1: RAN4 to remove the square bracket and keep 15 kHz SCS in the agreement.

Measurement accuracy for CSI-RS based L1 measurements

RAN4#116bis meeting further discussed the measurement accuracy requirements for CSI-RS based L1 measurement without agreements:

RAN4#116bis: - Background - Different cases for DUD - Case 1: X1 >= X3 and X2 >= X3 - Case 2: X1 >= X3 and X2 < X3 - Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3 - where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD). - Agreement from RAN4#114bis - Note: The following are for UE that can support simultaneous PDSCH reception on the two DL subbands for DUD - Existing delay (number of samples) requirements and accuracy requirements apply for Case 1 and Case 2 - For Case 3 - For L1-RSRP and L1-SINR, existing delay requirements apply and accuracy requirements are relaxed provided that X1 > X4 or X2 > X4, where the accuracy requirements depend on the X4 = 24 PRBs - In the test case, not model the interfering UE. - If UE indicates that CSI processing time is scaled by 2, further discuss whether and how to update the delay requirements - Option 1: Add a lower bound of 8ms on top of existing delay requirements. - Option 2: other - Way forward - Define the relaxed accuracy for L1-RSRP and L1-SINR based on 24 RB by adding [TBD] relaxation on top of existing accuracy (based on 48 RB) - For case 3 (X1 < X3 and X2 < X3, (X1 + X2) >= X3; X3 = 48 RB), - for a UE indicating CSI processing time scaled by 2 - Option 1: existing accuracy requirements based on 48 RB apply. - Option 2: accuracy requirements based on 24 RB apply. - for UE not indicating CSI processing time scaled by 2 - Option 1: accuracy requirements based on 24 RB apply

In RAN4#115 meeting, RAN4 agreed to define relaxed accuracy requirements based on 24 PRBs for Case 3:

RAN4#115: - Agreement - Define relaxed accuracy requirements based on 24 RB for Case 3 following RAN4#114-bis agreement, based on the simulation assumption in Annex. - Further discuss the upper bound of the measurement period due to processing time and its applicability.

For UE supporting simultaneous DL reception in 2 DL subbands, UE still could indicate whether the CSI processing time needs to be scaled by 2. For UE not indicating CSI processing time scaled by 2, considering that 2 DL subbands are not contiguous and there might be performance degradation when UE receives both DL subbands simultaneously. Therefore, it is okay to apply the accuracy requirements based on 24 PRBs for Case 3. For UE indicating CSI processing time scaled by 2, in our understanding, UE can meet the accuracy requirements based on 48 PRBs since more time is used for processing CSI obtained in 2 DL subbands.

Proposal 2: For UE simultaneously receiving 2 DL subbands for CSI-RS L1 measurement:

- For UE indicating CSI processing time scaled by 2, existing accuracy requirements based on 48 RB for Case 3 apply.

- For UE not indicating CSI processing time scaled by 2, accuracy requirements based on 24 RB for Case 3 apply.

Test configurations

In RAN4#116bis meeting, test configurations for SBFD were discussed and the following agreements were achieved:

RAN4#116bis: - Agreements - FR1: NR 30 kHz SCS, 100 MHz bandwidth, TDD duplex - FR2: - Option 1: NR 120 kHz SCS, 100 MHz bandwidth, TDD duplex mode - Option 2: NR 120 kHz SCS, 400 MHz bandwidth, TDD duplex mode - TDD UL/DL configuration : DDDSU - Time domain configuration of SBFD slots/symbols: DXXXU - Frequency domain configuration of SBFD subbands for FR1 - DU: 80 MHz – 20 MHz - DUD: 40 MHz – 20 MHz – 40 MHz - FFS: Frequency domain configuration of SBFD subbands for FR2

For the bandwidth configuration in FR2 where 120 kHz SCS will be used, there are two options, i.e., 100 MHz and 400 MHz. The value of RB for each bandwidth is specified in clause 5.3.2 in 38.104:

- 5.3.2 Transmission bandwidth configuration … Table 5.3.2-2: Transmission bandwidth configuration NRB for FR2-1 SCS (kHz); 50 MHz; 100 MHz; 200 MHz; 400 MHz. NRB; NRB; NRB; NRB. 60; 66; 132; 264; N/A. 120; 32; 66; 132; 264. Note: The BS channel bandwidth for SBFD operation shall be no less than 100MHz.. Table 5.3.2-3: Transmission bandwidth configuration NRB for FR2-2 SCS (kHz); 100 MHz; 400 MHz; 800 MHz; 1600 MHz; 2000 MHz. NRB; NRB; NRB; NRB; NRB. 120; 66; 264; N/A; N/A; N/A. 480; N/A; 66; 124; 248; N/A. 960; N/A; 33; 62; 124; 148. NOTE: All Tx and Rx requirements are defined based on transmission bandwidth configuration specified in table 5.3.2-1 for FR1 and table 5.3.2-2 and table 5.3.2-3 for FR2. The transmission bandwidth configuration for NB-IoT is specified in TS 36.104 [13] clause 5.6. For SBFD operation, the transmission bandwidth configuration for SBFD UL subband NRB,SBFD,UL shall be configured according to the values provided for BS transmission bandwidth configuration NRB corresponding to bandwidth no less than 10MHz in Table 5.3.2-1 for FR1, and NRB in Table 5.3.2-2 for FR2-1. Transmission bandwidth configuration for SBFD DL subband in SBFD (i.e., NRB,SBFD,DL for SBFD DU or UD operation, and NRB,SBFD,DL,1 and NRB,SBFD,DL,2 for SBFD DUD operation) and guardband between SBFD UL subband and DL subband shall be configured by SBFD-capable BS. For SBFD DUD operation, NRB,SBFD,DL,1 and NRB,SBFD,DL,2 shall have the same size or different sizes by only 1RB.

In order to ensure that enough PRBs are configured in UL subband and DL subband(s) to verify measurement accuracy requirements, there are two ways to configure bandwidth in the test cases for both DU and DUD. One way is to adopt large bandwidth, e.g., 400 MHz. However, as companies mentioned in the discussion, one concern is that the number of UE supporting such large bandwidth may not be high. The other way is to dynamically change the ratio of DL/UL RBs for different test cases with 100 MHz used. For example, when accuracy requirements for L1 CSI-RS measurement are verified, more RBs can be allocated to DL subband. When accuracy requirements for L1-SRS-RSRP measurement are verified, more RBs can be allocated to UL subband. From our perspective, the 2nd way is more practical. Otherwise, we do not see any other feasible ways to define test cases for SBFD in FR2.

Proposal 3: For SBFD test in FR2 for both DU and DUD, dynamically change the ratio of DL/UL RBs for different test cases with 100 MHz used.

Conclusions

This contribution discussed the RRM performance requirements for SBFD and some proposals are provided:

Proposal 1: RAN4 to remove the square bracket and keep 15 kHz SCS in the agreement.

Proposal 2: For UE simultaneously receiving 2 DL subbands for CSI-RS L1 measurement:

- For UE indicating CSI processing time scaled by 2, existing accuracy requirements based on 48 RB for Case 3 apply.

- For UE not indicating CSI processing time scaled by 2, accuracy requirements based on 24 RB for Case 3 apply.

Proposal 3: For SBFD test in FR2 for both DU and DUD, dynamically change the ratio of DL/UL RBs for different test cases with 100 MHz used.

References

[1] R4-2515166, WF for [116bis][309] Rel-19 RRM performance_Part1, Moderator (Qualcomm), RAN4#116bis.

[2] R4-2514432, Topic summary for [116bis][309] NR_duplex_evo, Moderator (Qualcomm), RAN4#116bis.