3GPP TSG-RAN WG4 Meeting #117				                                                R4-2520150

Dallas, USA, November 17th – November 21st, 2025

Title:	Discussion on the RRM core requirements of NR MIMO ph6

Source:	CATT

Agenda Item:	7.10.3

Document for:	Discussion

1 Introduction

In RAN#109 meeting, RAN approved a new WI on NR MIMO Phase 6 [1] with the following objectives:

The detailed objectives are as follows: RAN1: - On enhancing UL capacity and coverage, specify the following enhancements: - For SRS, reusing the legacy port numbering for SRS resource, comb design, SRS sequence, SRS power control, symbol-level dropping rules (when SRS collides with another UL signal), TCI/UL-spatial-relation frameworks, and SRS bandwidth configuration for SRS frequency hopping, with the maximum number of SRS resources per set following the legacy specification: - Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding - Note: On phase continuity, the applicable conditions and requirements from the legacy RAN4 spec for DMRS bundling should be retained as much as possible. - Cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set - When used for one SRS with repetition, cross-slot SRS symbol mapping is limited to within one SRS resource, with a common timing advance (TA), a common UL spatial filter, and common transmit power for the SRS resource across the two consecutive slots - On enhancing DL CSI acquisition, targeting FR1, specify the following enhancements: - Early SRS/CSI/CSI-RS triggering for UE transitioning from IDLE/INACTIVE to CONNECTED mode via MSG4 of 4-step RACH, as well as for SCell activation and switching out of SCell dormancy in CONNECTED mode via the legacy signaling mechanisms (based on legacy SCell activation MAC CE, and legacy switching DCI out of SCell dormancy, respectively) - Reusing the legacy design, limit the scope only to SRS for antenna switching, aperiodic wideband CSI with Rel-15 Type-I Single-Panel codebook (targeting FDD) and PMI-free report (targeting TDD) via the legacy UCI, as well as CSI-RS for CSI and (only for IDLE/INACTIVE to CONNECTED mode) CSI-RS for tracking - For IDLE mode, UE capability for early triggering and resource/reporting configuration are signaled via MSG3 and System Information (SI), respectively - For 48, 64, and 128 CSI-RS ports aggregated over multiple CSI-RS resources per legacy specification: CSI-RS density of 1/3, 1/4, 1/6, and 1/8 RE/RB/port while fully reusing legacy CSI-RS RE mapping per RB for CDM group generation (including OCC length, CDM group type, number of CDM groups), without impacting the legacy specification for rate matching on CSI-RS REs - Density of 1/3 and 1/6 RE/RB/port are intended only for 48 ports RAN2: Specify higher layer support of the enhancements listed above. RAN4: Specify necessary core requirements for the enhancements listed above.

In RAN4#116bis meeting, RAN4 started the discussion on this WI and agreements are captured in the WF [2]. In this contribution, we will discuss the potential impacts of this WI on RAN4 requirements and present our views.

2 Discussion

  - Cross-slot SRS

In last meeting, one issue on cross-slot SRS is whether to introduce additional RRM requirements of SRS antenna port switching and no agreement achieved yet. The proposals are copied below for reference:

RAN4#116bis: < Way forward > FFS the following options: - Option 1: No RRM impact. - Option 2: Wait for further progress in RAN1. - MTK: wait for RAN1 conclusions and check whether the legacy interruption requirements are still applicable. - LG: Regarding cross-slot SRS, RAN4 to postpone the discussion until further progress in RAN1 - Option 3: - RAN4 to define SRS AS requirements for the case for cross-slot SRS resource. - RAN4 start from Scenario 1 as agreed in RAN1, and whether to consider other scenarios depends on further conclusion in RAN1. - Option 4: - FFS on whether to extend the applicability of SRS antenna port switching requirements to the case of SRS resource(s) not only in last 6 symbols of a slot.

In Option 4, it was mentioned that the applicability of SRS antenna port switching requirements needs to be extended for cross-slot SRS. In current spec [3], SRS antenna port switching requirements are defined as below:

8.2.2.2.16 Interruptions at NR SRS antenna port switching The requirements in this clause are applicable to SRS antenna port switching on FR1 and SRS resource(s) is only configured within the last 6 symbols of a slot. For interruption caused by SRS antenna port switching, the victim cell is based on the entry number of the band indicated by txSwitchImpactToRx and/or txSwitchWithAnotherBand regardless of per-FR MG capability. An UL interruption is allowed on any of the serving cells as indicated in txSwitchWithAnotherBand, and a DL interruption is allowed on any of the serving cells as indicated in txSwitchImpactToRx. …

According to the RAN1 design, SRS resource can span one U slot and one adjacent S slot, which is always in last 6 symbols of a slot. And with following principle in the WID, we think the legacy SRS antenna port switching requirements can reused with updating the applicability requirements.

    - Cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set

      - When used for one SRS with repetition, cross-slot SRS symbol mapping is limited to within one SRS resource, with a common timing advance (TA), a common UL spatial filter, and common transmit power for the SRS resource across the two consecutive slots

Two options can be considered to update the applicability requirements of RS antenna port switching requirements:

Alt 1: Delete “and SRS resource(s) is only configured within the last 6 symbols of a slot”.

Alt 2: Add “or configured in one uplink slot and one adjacent S slot” after the “…the last 6 symbols of a slot”.

On SRS carrier based switching requirements, we do not identify any impacts. RAN4 can follow RAN1 discussions and see if there will be any impacts in a later phase.

Proposal 1: RAN4 to reuse the legacy SRS antenna port switching requirements with updating the applicability requirements:

Alt 1: Delete “and SRS resource(s) is only configured within the last 6 symbols of a slot”.

Alt 2: Add “or configured in one uplink slot and one adjacent S slot” after the “…the last 6 symbols of a slot”.

Proposal 2: Cross-slot SRS has no impacts on SRS carrier based switching requirements.

  - Early SRS/CSI/CSI-RS triggering

According to the 2nd objective in the WID, the early SRS/CSI/CSI-RS triggering mechanism will be applied to the three procedures in FR1:

1. UE transitioning from IDLE/INACTIVE to CONNECTED mode;

2. SCell activation;

3. Switching out of SCell dormancy in CONNECTED mode.

UE transitioning from IDLE/INACTIVE to CONNECTED mode

In RAN1#122bis meeting, RAN1 discussed UE transition from IDLE/INACTIVE to CONNECTED mode scenario and reached some agreements [4]:

RAN1#122bis: Agreement For UE transition from IDLE/INACTIVE to CONNECTED mode, using MAC-CE in the PDSCH for MSG4 to trigger early aperiodic SRS-AS/CSI-RS/CSI - Note: How to determine the PUSCH for aperiodic CSI reporting is separately discussed. Agreement For early aperiodic SRS-AS/CSI-RS/CSI triggering (i.e., early triggering of aperiodic SRS-AS transmission, aperiodic CSI-RS reception, aperiodic CSI reporting) when UE transition from INACTIVE to CONNECTED mode, reuse the procedure for UE transition from IDLE to CONNECTED mode. Agreement: For early triggering of aperiodic CSI reporting and the associated CSI-RS for CSI when UE transition from IDLE/INACTIVE to CONNECTED mode, support Option-2 as follows: - In Step-1, SIBx provides one or multiple CSI report configurations based on one or multiple UE capability assumptions - Each CSI reporting configuration is associated with a CSI resource configuration for channel measurement. - The CSI-RS resource set(s) in the CSI resource configuration for channel measurement is provided with a same number of CSI-RS ports - Each CSI reporting configuration is associated with a CSI resource configuration for CSI-IM based interference measurement. - FFS: Support of NZP CSI-RS based interference measurement - FFS: Information to be provided in a CSI report/resource configuration - In Step-2, UE reports through MSG3 which/whether the CSI report configuration(s) provided in SIBx is/are supported. - FFS: How to report which/whether the CSI report configuration(s) provided in SIBx is/are supported. Agreement: On PUSCH allocation for MSG4-triggered aperiodic CSI reporting associated with aperiodic CSI-RS for CSI, when UE transition from IDLE/INACTIVE to CONNECTED mode, down select one from the following alternatives to determine PUSCH for carrying the aperiodic CSI report by RAN1#123: - Alt-1: The PUSCH is scheduled by MAC CE in MSG4 along with the aperiodic CSI report triggering. - Alt-2: The PUSCH is scheduled by a legacy DCI after MSG4 - Alt-3: The PUSCH is configured by SIBx providing resource/reporting configuration for early triggering of aperiodic CSI reporting. Note: The timeline of PUSCH for aperiodic CSI reporting is separately discussed.

Based on RAN1 design, UE will obtain CSI reporting configurations associated with aperiodic CSI-RS for CSI via SIBx and report supported CSI report configuration(s) through Msg3. The PUSCH for CSI report is not decided yet which will be determined in RAN1#123. However, in current RRM spec, there is almost no delay requirement for random access-related procedures and CSI reporting. Following this principle, RAN4 do not need to define core requirements for early SRS/CSI/CSI-RS triggering for UE transition from IDLE/INACTIVE to CONNECTED mode.

Observation 1: There is not delay requirement for random access-related procedures and CSI reporting in RRM spec.

Proposal 3: RAN4 not to define core requirements for early SRS/CSI/CSI-RS triggering for UE transition from IDLE/INACTIVE to CONNECTED mode.

Early SRS/CSI/CSI-RS triggering in SCell activation procedure

RAN1 agreements achieved in RAN1#122bis for early SRS/CSI/CSI-RS triggering in SCell activation procedure are reproduced below:

RAN1#122bis: Agreement For SCell transition from deactivation to activation, support the followings for aperiodic CSI reporting triggered based on legacy SCell activation command: - For PMI-based reporting, support the report quantity ‘CRI-RI-PMI-CQI’ - FFS: ‘CRI-RI-LI-PMI-CQI’ - For PMI-free reporting, support the report quantity ‘CRI-RI-CQI’ Agreement: On triggering mechanism for early aperiodic SRS-AS transmission on a SCell and early aperiodic CSI reporting for a SCell, based on the legacy SCell activation activating the SCell, down-select one from the followings in RAN1#123 meeting: - Alt-1 (Implicit mechanism): - For early aperiodic SRS-AS transmission, the SRS resource set(s) triggered for the SCell is determined according to RRC configuration. - For early aperiodic CSI reporting, the CSI report configuration(s) triggered for the SCell is determined according to RRC configuration - Alt-2 (Explicit mechanism): - For early aperiodic SRS-AS transmission, the SRS resource set(s) triggered for the SCell is determined according to an indication in SCell activation command. - For early aperiodic CSI reporting, the CSI report configuration(s) triggered for the SCell is determined according to an indication in SCell activation command.

According to the above RAN1 agreements, the mechanism of triggering SRS-AS transmission and early aperiodic CSI reporting for a SCell needs to be decided by RAN1 in RAN1#123 meeting. Therefore we suggest waiting for more progress before RAN4 start to discuss the detailed impacts on SCell activation delay requirements.

Proposal 4: RAN4 to wait for more progress made by RAN1 before RAN4 start to discuss the detailed impacts on SCell activation delay requirements.

Switching out of SCell dormancy in CONNECTED mode

In RAN1#122bis meeting, RAN1 discussed the switching out of SCell dormancy in CONNECTED mode scenario and reached some agreements:

RAN1#122bis: Agreement For early triggering of SRS-AS when SCell transition from a dormant BWP to a non-dormant BWP, support aperiodic SRS-AS transmission on the non-dormant BWP, triggered via a DCI indicating switching out of SCell dormancy. - Legacy DCI format(s) for switching out of SCell dormancy is reused without introducing new DCI field or resizing the existing DCI field - FFS: Which DCI format(s) from DCI format 0_1/0_3/1_1/1_3/2_6 is supported - FFS: Triggering mechanism(s) - FFS: Timeline of the aperiodic SRS-AS transmission (requirement is up to RAN4) Agreement: For early triggering of CSI/CSI-RS when SCell transition from a dormant BWP to a non-dormant BWP, support aperiodic CSI reporting for the non-dormant BWP, triggered via a DCI indicating switching out of SCell dormancy. - The aperiodic CSI reporting is associated with aperiodic CSI-RS for CSI on the non-dormant BWP - Legacy DCI format(s) for switching out of SCell dormancy is reused without introducing new DCI field or resizing the existing DCI field - FFS: Which DCI format(s) from DCI format 0_1/0_3/1_1/1_3/2_6 is supported - FFS: Triggering mechanism(s) - FFS: Timeline of the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI (requirement is up to RAN4)

Based on above RAN1 agreement, the legacy DCI triggering BWP switching from dormant BWP to non-dormant BWP will be reused to triggering aperiodic SRS-AS transmission on non-dormant BWP. In our understanding, the current RRM requirements related to dormant BWP are not impacted, such as BWP switch delay requirements. However, from RAN4 perspective, UE is only able to transmit SRS with AS after completing the BWP switching to non-dormant BWP, which may impact the timeline of early triggering of SRS-AS during SCell transition from a dormant BWP to a non-dormant BWP. Similar issues may exist for early triggering of CSI reporting as well. RAN4 should pay attention to RAN1 timeline design and identify any issues that may misalign with RAN4 requirements. If so, RAN4 can send LS to RAN1 for clarification.

Proposal 5: Based on current RAN1 design, RRM requirements related to dormant BWP are not impacted.

Proposal 6: UE is only able to transmit SRS with AS after completing the BWP switching to non-dormant BWP, which may impact RAN1 timeline design of early SRS triggering.

Proposal 7: UE is only able to measure CSI-RS and report CSI after completing the BWP switching to non-dormant BWP, which may impact RAN1 timeline design of early CSI triggering.

3 Conclusion

In this paper, we discussed potential impacts of NR MIMO Ph6 WI on RAN4 RRM requirements and the following observations and proposals are provided:

Proposal 1: RAN4 to reuse the legacy SRS antenna port switching requirements with updating the applicability requirements:

Alt 1: Delete “and SRS resource(s) is only configured within the last 6 symbols of a slot”.

Alt 2: Add “or configured in one uplink slot and one adjacent S slot” after the “…the last 6 symbols of a slot”.

Proposal 2: Cross-slot SRS has no impacts on SRS carrier based switching requirements.

Observation 1: There is not delay requirement for random access-related procedures and CSI reporting in RRM spec.

Proposal 3: RAN4 not to define core requirements for early SRS/CSI/CSI-RS triggering for UE transition from IDLE/INACTIVE to CONNECTED mode.

Proposal 4: RAN4 to wait for more progress made by RAN1 before RAN4 start to discuss the detailed impacts on SCell activation delay requirements.

Proposal 5: Based on current RAN1 design, RRM requirements related to dormant BWP are not impacted.

Proposal 6: UE is only able to transmit SRS with AS after completing the BWP switching to non-dormant BWP, which may impact RAN1 timeline design of early SRS triggering.

Proposal 7: UE is only able to measure CSI-RS and report CSI after completing the BWP switching to non-dormant BWP, which may impact RAN1 timeline design of early CSI triggering.

4 References

- RP-252936, Revised WID: NR MIMO Phase 6, MediaTek, RAN#109.

- R4-2514825, WF on NR_MIMO_Ph6_RRM, MediaTek Inc., RAN4#116bis.

- 3GPP TS 38.133, NR; Requirements for support of radio resource management, V19.2.0.

- RAN1 chair notes, RAN1#122bis.

- R4-2513531, Topic summary for [116bis] [225] NR_MIMO_Ph6_RRM, Moderator (MediaTek Inc.), RAN4#116bis.