3GPP TSG RAN WG4 Meeting #117		R4-2520177

Dallas, Texas, United States, Nov 17th – 21st , 2025

Title: 	Further discussion on modulation for 6GR

Agenda Item:	8.2.2

Source: 	CATT

Document for:	Discussion

# 1 Introduction

At RAN4#116-bis, the first meeting where RAN4 initiated its 6GR studies, extensive discussions were held on system parameters, including modulation schemes for 6GR, leading to the following agreements:

- Agreement - RAN4 evaluation work focus on the feasibility study and RF requirements impact - To establish the foundational evaluation framework in RAN4 firstly when no solid progress and inputs from RAN1 - Identify the main affected requirements for modulation evaluations - The existing 5G NR requirements will serve as the baseline, which are subject to future updates based on RAN4’s 6G UE RF discussions. - Align on the evaluation assumptions. - Study on how to align on the transmitter chain model, including PA model, for consistent evaluations on the modulation. - RF evaluation could be done firstly for 5G supported modulations with new assumptions for 6G study, such as assumed new spectrum, CBW, new PA models, etc. Co-ordination with 6G UE RF study is needed. - Both link-level and system-level simulations should be performed as usual for high-order modulations study done by RAN4 in prior releases, pending on the progress of RAN1. - Model and evaluate the performance and the implementation complexity of higher-order modulations, e.g. 1024QAM on the UL and/or new constellations - For high order QAM with uniform constellation, e.g. 1024QAM on the UL, RAN4 can work concurrently with RAN1 studies. - For new non-uniform constellation, the evaluation in RAN4 should depend on RAN1 progress and request.

In this contribution, we present our views on modulation studies from RAN4 perspective.

# 2 Discussion

Since modulation for 6GR is also one of the main focus in RAN1 at the current stage, the following agreements have been made in RAN1#122 in August and RAN1#122bis in October with regards to modulation [2] [3]:

Agreement - For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for study for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for CP-OFDM for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for DFT-s-OFDM for data channel - FFS: Enhancements and other modulation schemes

Agreement For 6GR constellation shaping evaluation for CP-OFDM, and improved MCS table, the proposed scheme will be compared with non-shaping with NR MCS table. The evaluation and comparison should consider at least the following: - BLER performance under AWGN channel (at least for performance calibration) - 1st transmission (baseline) and with HARQ re-transmission - BLER performance under fading channel with fixed MCS - 1st transmission (baseline) and with HARQ re-transmission - Throughput performance with link adaptation (adaptive MCS and rank) under fading channel - Needs to provide assumptions on rate adaptation (e.g., target BLER for 1st transmission, maximum # of retransmissions) - Transmitter and receiver complexity (e.g., shaping/deshaping, demapper), latency, parallelism implementation, and storage requirements, - Other KPI not excluded, such as PAPR, EVM, MPR/A-MPR - Expected spec impact - FFS detailed assumption of constellation shaping and improved MCS table - System level evaluation can be done after link level evaluation. Agreement For 6GR constellation shaping study, proponent is encouraged to provide details for the PS/GS schemes considered for evaluation and comparison, including at least the following - Probabilistic shaping for CP-OFDM and DFT-s-OFDM - Use the list of spectrum efficiencies in NR MCS table as starting point, and provide constellation (including normalization), coding rate and target probabilistic distribution for each SE - If multiple coding rate and target probabilistic distribution pairs are provided for each SE, how to switch between them - Relationship between shaping and FEC, coded bits to modulation symbol mapping, and other modules (such as scrambling, interleaving), in transmit and receive chains. How to handle HARQ retransmission - PS algorithm details (for example, source coding based, channel coding based, etc) and parameters (such as block length, rate loss) - Geometric shaping for CP-OFDM and DFT-s-OFDM - Use the list of spectrum efficiencies in NR MCS table as starting point, and provide target constellation shape (including normalization) (1D-NUC, 2D-NUC, QAM-CS, etc) for each SE - If multiple constellation shapes are provided for each SE, how to switch between them - GS mapping details, such as bit to constellation point mapping and shape - Relationship with other blocks (such as scrambling, interleaving). How to handle HARQ retransmission Agreement For link level simulation for modulation evaluation, companies are encouraged to evaluate with the following assumptions and should report the exact scheme evaluated. - channel configuration, including Channel profiles,Tx/RX antenna settings - For MIMO scenario: SU-MIMO or MU-MIMO, follow agenda item 11.2 for MIMO when available. - Precoder assumption - Close loop MIMO (reciprocal beamforming (e.g., SVD, SLR/RZF, etc.), codebook based) - Realistic CSI/SRS/AP-SRS periodicity and delay, and SRS chanEst assumptions, - or genie beamforming - Open loop MIMO - Receiver assumption (for MIMO): LMMSE (baseline) for UL, rML or LMMSE for DL - LLR demapper: Max-log (baseline) or Log-MAP - Channel estimation: Realistic (baseline) or ideal - Other assumptions: Channel coding NR-LDPC (baseline), PxSCH bandwidth, SCS, FD interleaver used or not, 5GNR BICM interleaver usage - Note: For MIMO, SIMO, MISO and SISO are included when possible Agreement For 6GR constellation shaping evaluation for DFT-s-OFDM, and improved MCS table, the proposed scheme will be compared with non-shaping with NR MCS table. In addition to what has been agreed for CP-OFDM in earlier agreement, the evaluation and comparison should further consider at least the following: - PAPR/CM of the resulting waveform - EVM, MPR/A-MPR Agreement For the study of uniform 4096QAM for DL and uniform 1024QAM for UL, need to study performance (assuming realistic channel estimation, time/freq synchronization assumption, phase noise assumption, etc), complexity/power consumption, requirements, benefit/necessity under applicable scenarios, associated restrictions, and challenges (such as EVM requirement, PAPR increase, MPR or A-MPR increase under realistic PA model). - FFS: How to involve RAN4 early - FFS: Shaping of higher order modulation System level evaluation can be done after link level evaluation.

It can be observed that RAN1 is currently studying advanced constellation shaping techniques beyond uniform constellations, including Geometric constellation Shaping (GS) and Probabilistic Shaping (PS). These techniques aim to improve link efficiency by optimizing symbol placement or occurrence probability, thereby maximizing overall performance gain when coding and modulation are jointly considered.

GS modifies the geometrical placement of constellation points in the I/Q plane to maximize mutual information or minimize symbol error rate under a given SNR. Unlike uniform constellations (e.g. square QAM with equally spaced points), GCS optimizes the distance and position of constellation points — often resulting in non-uniform spacing or irregular shapes (e.g. circular, elliptical, or probabilistically optimized clusters). The goal is to achieve better power efficiency and improved robustness against nonlinear distortions and channel noise, while maintaining the same nominal modulation order (e.g. 64QAM).

PS maintains a fixed constellation geometry (e.g. standard square QAM) but changes the probability distribution of symbol occurrences — giving higher probability to lower-energy symbols near the origin and lower probability to outer symbols. This reduces the average symbol energy, thereby improving the achievable information rate closer to Shannon capacity. Typical realizations include Maxwell–Boltzmann distributions or entropy-coded shaping methods. PS primarily targets coding and rate adaptation gains rather than geometrical optimization, and it is often implemented jointly with FEC encoding and decoding.

However, from the RAN4 point of view, the primary interest is in assessing how different constellation shaping techniques affect the RF characteristics after power amplification. Specifically, RAN4 would focus on analyzing emission behavior, including out-of-band emissions and ACLR, when the shaped constellations are transmitted through a realistic PA model (e.g., the composite GMP + Rapp + Saleh model). RAN4’s analysis and evaluation of different constellation shaping techniques can proceed in parallel once the realistic PA model has been agreed upon.

Proposal 1: RAN4 to analyse and evaluate independently how GS or PS constellation shaping techniques affect the RF characteristics after power amplifier.

In particular, after the selection or establishment of a representative PA model, RAN4 can begin quantitative assessments of how various shaping methods—such as geometric constellation shaping (GS) and probabilistic shaping (PS)—affect emission performance. This includes evaluating differences in out-of-band emissions, ACLR, and EVM characteristics when the shaped constellations are passed through the agreed PA model. Conducting these evaluations in parallel with the ongoing RAN1 studies would enable timely feedback on potential RF implications and facilitate coordinated progress between the two groups.

Proposal 2: RAN4 to begin quantitative assessments on constellation shaping techniques after establishment of a realistic PA model, including evaluating differences in out-of-band emissions, ACLR and EVM characteristics.

Proposal 3: RAN4 to provide timely feedback to RAN1 on potential RF implications and facilitate coordinated progress between two groups.

# 3 Conclusion

In this contribution, we have the following proposals on modulation study for 6GR in RAN4:

Proposal 1: RAN4 to analyse and evaluate independently how GS or PS constellation shaping techniques affect the RF characteristics after power amplifier.

Proposal 2: RAN4 to begin quantitative assessments on constellation shaping techniques after establishment of a realistic PA model, including evaluating differences in out-of-band emissions, ACLR and EVM characteristics.

Proposal 3: RAN4 to provide timely feedback to RAN1 on potential RF implications and facilitate coordinated progress between two groups.

# Reference

[1] R4-2514641, “WF for 6GR system parameter”, Huawei, HiSilicon

[2] R1-25xxxxx, “Minutes_report_RAN1#122bis_v104”, RAN1

[3] R1-2506755, “On Modulation for 6G Radio Air Interface”, Nokia