# 3GPP TSG-RAN WG4 Meeting #117	R4-2520187

# Dallas, United States, November 17th – 21st, 2025

Agenda item:			8.1

Source:	Feature lead (CATT)

Title:	Running summary of 6G operation efficiency after RAN4#116bis

Document for:	Information

# Introduction

As part of the scope defined in the working group–level 6G study item (FS_6G_Radio), RAN4 operation efficiency focuses on improving the quality and effectiveness of RAN4 6G specifications. The detailed objectives related to enhancing RAN4 operational efficiency are outlined below, in accordance with the latest SID RP-252912:

- 6GR core and performance requirements - General RF aspects [RAN4] - Intra-3GPP co-existence studies - Study RF related system parameters and requirements - BS RF requirement aspects including band [RAN4] - BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation - Study how to improve 6G BS core, conformance specifications, including structure and drafting principles - UE RF requirement aspects including band and band combination [RAN4] - UE RF requirement framework aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G UE RF specification(s), including structure, drafting principles, and database for band combination - Study UE RF capabilities considering different device types and implementations - RRM aspects for 6GR [RAN4, RAN1, RAN2] - RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G requirement specification, including structure and drafting principles - Demodulation and performance aspects [RAN4] - Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS - Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS - Aspects related to the testability [RAN4] - Testability methodology framework and key assumptions, aiming to ensure that requirements can be properly tested considering the applicability and feasibility of conductive and/or OTA testing with reasonable complexity - Other aspects - Handling irregular channel bandwidths including the definition [RAN4, RAN1] - Definition of ‘frequency range(s)’[RAN4]

Moreover, as instructed by the RAN4 leadership regarding 6G plans and meeting arrangements (see R4-2511652), a dedicated topic entitled “RAN4 operation efficiency”—i.e., this thread—has been established to coordinate discussions and activities related to the above-mentioned objectives. In particular, discussions during Q4’25 and Q1’26 will focus on the following aspects:

- CR, spec improvement and modernization in general, new tools for band/band combinations, other aspects

This running summary aims to provide key information and updates on the related discussions and activities, including:

- Status update providing a concise overview of the current progress, key agreements, regarding RAN4 operation efficiency discussions

- Key RAN4 Tdocs and major agreements achieved in this thread up to the current meeting in a reverse chronological order (i.e., latest first).

- Essential agreements related to 6G specification modernization under the 6G RAN/SA study item – 6G specification modernization (6GSM).

This running summary is prepared and maintained by the feature lead of this thread, with the following contact details:

| Feature Lead | Company | Email address & Telephone |
| --- | --- | --- |
| Aijun CAO | CATT | ajcao@catt.cn, +46 72 252 66 88 |

# Status update

The following status updates reflect changes since the previous summary, based on discussions and progress in the latest meeting cycle:

- The discussion on RAN4 6G operation efficiency was initiated.

- Initial views and proposals were collected, leading to some agreements.

- A sub-agenda structure was agreed to support and streamline upcoming discussions.

# RAN4 – Agreements on 6G operation efficiency discussion

## RAN4 #116bis meeting (Oct-2025)

### Key tdocs

| R4-2514518 | Topic summary for [116bis][111] 6G operation efficiency |
| --- | --- |
| R4-2514648 | Way Forward for [116bis][111] 6G operation efficiency |

### Key agreements

Guidelines for the November meeting

Agreement: - By taking the identified issues in the feature lead’s summary into consideration, in RAN4#117, a dedicated singe sub-agenda item will be introduced for the spec improvement. Interested companies are encouraged to structure their contributions such that the spec specific proposals and the general proposals for all RAN4 spec can be separately presented. - Interested companies are encouraged to continue populating the list of identified issues to be addressed in 6G as necessary. - A dedicated sub-agenda will be introduced on CR handling improvement from RAN4#117

On the conclusions of 6GSM SID

Conclusions: - Postponed until March 2026 - Companies encouraged to be actively involved into SID discussion.

# RAN/SA – Related agreements summary on 6G Specification Modernization (6GSM)

## Shortcomings, pain-points and potential benefits of current tools

### Conference call #2 (Sept-2025)

According to the agreed pCR 6GSM-250121, the following table summarizes the identified Shortcomings, pain-points and potential benefits of current tools:

| # | Shortcoming / pain-point / potential benefit | Possible improvement approaches with current tools | Summary of feasibility of addressing the shortcoming / pain-point / potential benefit with current tools | Applicable WGs and users of the specification |
| --- | --- | --- | --- | --- |
| 1 | Delays in specification availability Specifications are not available until after the TSG plenaries, sometimes very close to submission deadlines for the next WG meeting. Thus, writing new CRs is delayed or CRs are written based on an old version. There is also limited time for review of the merged spec. | Solution 1 – Increase workforce for CR merging Solution 2 – Scripting to automatically merge CRs | Partly feasible – Solution 1 for decreasing the number of TSes handled by a single MCC officer with the caveat that even a single large specification with many CRs still takes time to handle, while there are drawbacks about redistributing the load from MCC to delegates who would take on some of the work. Solution 2 (maybe feasible) is being developed for ETSI, but its feasibility is unknown. | WGs All groups Users Consumer, Contributor, Editor |
| 2 | CR cover page errors Errors can be present in the cover page details such as specification number and version, date, release, CR revision, affected clauses, and changes are not reflected in database | Solution 3 – [New tool] CR conformance checking Solution 4 – [New tool] CR auto-generation Solution 5 – CR cover auto-generation | Maybe feasible – Solution 5 is available and can be useful if the cover page is generated and attached to the CR just before submission. Solution 3 is being developed for ETSI (new CR parser that will perform a number of verifications on CR cover fields and also verify that the clauses affected listed on cover correspond to the actual changes. The parser will be available on-demand and run upon CR upload). Solution 4 would solve the problem, but the tools do not exist. | WGs All Groups Users Contributor, Editor |
| 3 | CR content errors - Incorrect specification base text, e.g., from the incorrect version - Errors introduced by copying and pasting content such as additional sections - Errors introduced by copying a CR for a later release to mirror CRs for prior releases - Unused references, e.g., references that are used but not defined, or defined which are not used, and references to non-existent clauses - Incorrect use of styles, including manually applying text formatting to mimic a style - Use of different settings in the docx editor such as language can lead to inconsistent formatting and styles which diverge from the official template - Omission of trademarks, commercial references, etc. | Solution 3 – [New tool] CR conformance checking Solution 4 – [New tool] CR auto-generation Solution 6 – Restrict editing Solution 7 – Training Solution 8 – Light version of Microsoft Word Solution 9 – EditHelp Consulting from ETSI Solution 10 – Exhaustive analysis of docx files against 21.801 | Not feasible - Solutions 4 and 8 require new tools, which need to be written. Today they are infeasible since they do not exist. Not feasible – Solutions 7, 9, and 10 resolve parts of the issues. Solution 7, 9, and 10 are already done today and we still have the problems. Maybe feasible – Solution 6 could be feasible to resolve the style errors as long as everyone uses Microsoft Word. Solution 3 is under development by ETSI in a new CR parser as indicated in solution 3 of shortcoming #2. The new CR parser will carry out a number of verifications, including use of latest reference version, no changes done without revision marks, or styles. (NOTE 3) | WGs All Groups Users Contributor, Editor |
| 4 | CR Merging - Multiple changes to the same section sometimes remove specification text due to human errors. - It is not possible to easily check the effect of the implementation of more than one CR or pCR simultaneously to check for side-effects - It isn’t trivial to identify conflicting CRs such that the relevant experts, e.g., CR editors or WI rapporteurs, could discuss the appropriate resolution. - Implementation of a CR is a manual process | Solution 2 – Scripting to automatically merge CRs | Maybe feasible – if the tool would be available for all specification contributors and editors, then it would be possible to check the effect of CRs. A pilot tool by ETSI that detects CR clashes is currently in trial in some of the 3GPP WGs (e.g., CT3), This assists rapporteurs to facilitate resolution of clashes during the meeting rather than during CR implementation. Further development of the tool is in progress based on the feedback of the pilot Furthermore, an automated CR implementation tool is being developed by ETSI. | WGs All groups Users Contributor, Editor |
| 5 | CR Tracking - It is difficult to associate a spec change with a CR, e.g., to determine where a change came from between two non-contiguous versions of a specification. For example, something changed between 18.1 and 18.7, but the exact version is unknown. - CRs which are part of mega CRs or aggregate CRs are not visible individually in the draft spec. - Changes to CRs to resolve merging conflicts, including those present in tables, ASN.1 definitions, and other text conflicts are not identifiable. - Checking implementation of CRs is a manual process - Changes in a new version of a specification are hard to track for implementors. It is particularly difficult to track an individual feature. - Missing background information on how certain test steps or approaches (pertaining to test parameters or implementation) were adopted for in conf test procedure |  |  | WGs All groups Users Consumer, Contributor, Editor |
| 6 | Data Structure Styling, Syntax and Representation - APIs and data structures, e.g., ASN.1, can contain formatting errors such as indentation for readability and colorization of keywords. - Code-like and data model or table representations coexist and can be inconsistent, e.g., ASN.1, OpenAPI and XML in tabular form can be inconsistent and don’t match the code representations. - OpenAPI: Different WGs (e.g. CT4 and SA5) often use the same datatypes but they are defined slightly differently. - In some groups, when modifications are made during a meeting, syntax errors can be introduced due to lack of verification. - In some groups, the contributions to the meeting can include syntax errors. - It isn’t easy to extract code | Solution 11 – Externalization of APIs and data structures Solution 12 – Single representation of APIs and data structures Solution 13 – Automatic syntax checking | Feasible – Solution 11 is already used by some WGs to store certain data structures and APIs separately from the specification, e.g., RAN4, CT4, and SA5. Solution 12 is already used in RAN2 which moved away from tabular ASN.1. Maybe Feasible – Solution 11 could be adopted. Groups such as RAN2 and RAN3 include field definitions and descriptions of conditionals in field tables. A solution for storing the field descriptions, which contain normative text, needs to be devised. Additionally, many find it convenient to have the procedures and protocol definitions in the same document. Solution 13 would require new scripting to extract text from the specification and run the syntax checker. | WGs All groups Users Consumer, Contributor, Editor |
| 7 | Cross-specification reference and navigation - References to other specification documents are made using numbered references in square brackets and are sometimes accompanied by a section or clause number. - Navigating across a work item over multiple specifications and working groups can be time consuming. For example, a RAN2 procedure could be triggered by RAN3 procedure, triggered by an SA2 procedure. - Conformance Test specifications use heavy cross-referencing between the test spec (e.g. 38.521-1) and associated details in test point analysis (TR 38.905), MU/TT (TR 38.903), UE/UE connection diagrams (TS 38.508-1) | Solution 14 - Include the specification number and clause number in references Solution 15 – Use hyperlinks in references Solution 16 - Script to create, validate, and automate verification of cross-references between specifications | Feasible – Solution 14 at least solves the problem of accurately referencing another specification or the same specification. Partly feasible – Solution 15 – while feasible, links would be to zip files hosted on the 3GPP FTP server. These would have to be unzipped, which is inconvenient, and the reference could not be to a specific clause. Infeasible – Solution 16 doesn’t exist. | WGs All groups Users Consumer, Contributor, Editor |
| 8 | Figures can become impossible to edit - Occasionally, CRs convert Visio figures to a format which cannot be edited. Sometimes Visio figures are drawn in an older format. - MSC-Generator diagrams can become corrupt | Solution 17 – Store figure source files | Maybe feasible – Solution 17 is feasible in the current tools because figure source files could be attached in the zip files. However, specifications can contain dozens to hundreds of figures, which could lead to error in omission of files and could be unscalable. Additionally, each attached file would have to be individually opened. | WGs All groups Users Contributor, Editor |
| 9 | Quality and size of figures - Some specifications and TRs include many images or a large body of text which can contribute significantly to the size of a docx file, leading to high upload and download times. - Figures are resized in dimension or compressed, thereby being distorted. - Microsoft has disallowed usage of equation editor 3.0 due to security issues with equation editor. When opening equations that were originally written using equation editor are stored as rasterized pixel images. Such images have image distortion issues when resizing the scale of the view. | Solution 18 – Store images as vector graphics | Feasible – Solution 18 is feasible for non-equation graphics. | WGs All groups Users Contributor, Editor |
| 10 | Inconsistent use of tools and formats for figures - e.g., Word, Visio, MSC-Generator, PlantUML, MS paint, PowerPoint, Equation editor, Latex, etc. - Use of non-cross-platform formats (Visio, objects embedded in documents using Windows OLE are not available on macOS). | Solution 19 - Support a limited number of supported formats which work across WGs and platforms | Feasible – Solution 19 would consider tools supported by the overall specification format, so they would be compatible. (NOTE 1) | WGs All groups Users Contributor, Editor |
| 11 | Large tables are not handled well - MS word does not handle large tables well and large documents well - can cause MS Word to crash or operate slowly. | Solution 11 – Externalization of APIs and data structures Solution 25 – Split large specifications into smaller parts | Maybe feasible – Solution 11, depending on the nature of the table’s content, it could be externalized, e.g., as in RAN4 Feasible – Solution 25 is used today. | WGs All groups Users Consumer, Contributor, Editor |
| 12 | Collaboration on CRs - Limitations of change tracking include: specific changes to figures are impossible to see; change marks not shown for columns deleted from tables; change marks are not shown when merging or unmerging cells; and change marks are shown in different colors, which poses an accessibility issue. - Collaborative editing relies on downloading a document, providing input, and uploading it. There is a race condition when multiple delegates are working at the same time. - Commenting bubbles do not scale well and having too many which affect the same clause makes it impossible to read. - It is difficult to update documents when used to collect input since manual locking does not work and progress becomes slow and error prone | Solution 20 - Provide comments on CRs in a separate file instead of using bubble comments Solution 21 – Use NWM to collect comments Solution 22 - Extract text under review and use Git to manage reviews Solution 23 - Use FTP to download latest inputs and upload new inputs Solution 24 - Split the CR for review into multiple files | Infeasible – Solution 20 and 21 could become unscalable when commenting on a large CR with a large number of comments and responses. Solution 22 would cause the loss of content most likely and would result in an inaccurate review. Maybe feasible – Solution 23 is used today, but many companies may be restricting access to FTP and SFTP. Feasible – Solution 24 could also work, but it has problems similar to Solution 23 and could increase confusion. | WGs All groups Users Contributor, Editor |
| 13 | Specification opening and navigation delay - Opening a TR or TS of 100s or 1000s of pages can take many minutes or even be impossible due to crashing. - Searching with keywords can be slow. - The specifications are stored as ZIP files, which add another step to opening the specification. - There seems to be some issue in the .docx specification template itself, regarding the header that appears on top of each page which checking for every single page how the header should be filled even if there is only one outcome. | Solution 6 – Restrict editing Solution 7 - Training Solution 8 – Light version of Microsoft Word Solution 11 – Externalization of APIs and data structures Solution 25 – split large specifications into smaller parts Solution 26 - Open specification and change to draft mode Solution 27 - Produce 3GPP PDF version of the specification after each plenary Solution 28 - Make all specs available in HTML | Maybe feasible – Solution 6, Solution 7, Solution 8 could help solve the problem, but so far these have not resulted in faster loading specs. Solution 25 works but has downsides like lack of navigability. Solution 26 works once the document is open, but crashing can occur prior to being able to switch to draft mode. Solution 28 may be infeasible because the conversion of Word to HTML could be lossy. Solution 11 is feasible but has more impact. Feasible – Solution 27 is already done by ETSI and other SDOs, but 3GPP could possibly release a version more quickly. (NOTE 2) | WGs All groups Users Contributor, Editor |
| 14 | Numbering of PRs and CPRs - Potential Requirements (PR), Consolidated Potential Requirements (CPR), requirements are numbered manually and inconsistently within TR/TS, making it error prone for tracking or later reference. - In some groups, requirements are not numbered which makes reference to requirements very difficult - it must be done by copying the text of the requirement. This can become misaligned, if the text is corrected in the specification where it is a provision. | Solution 29 – Mandate the numbering of requirements (PR and CPR) | Feasible – In Solution 29, numbering just needs to be enforced. |  |
| 15 | Automatic processing of specifications - Access from automated text processing tools, e.g., Automata, to CRs and TSs/TRs is very cumbersome, requiring a lot of preprocessing and manual intervention |  | The docx format is not easily processed. The file format is a mix of text and binary (to store images and objects). Conversions, e.g., docx to markdown, do not produce perfect representations of the original docx. | WGs All Groups Users Contributor, Editor |
| NOTE 1: This could also solve pain-point 8 depending on what is chosen. NOTE 2: No solution has been provided for how to deal with ZIP files. NOTE 3: It has been noted that some of the errors are due to not following the process. It should be considered which of these errors could be prevented by following rules and whether it is feasible to enforce the following of such rules. |  |  |  |  |

Furthermore, in the same pCR, there are also several approaches which can potential improve current tools as illustrated in the following table:

| # | Possible improvement approaches with current tools | Pros of possible improvement approaches | Cons of possible improvement approaches | Implementation Feasibility Analysis |
| --- | --- | --- | --- | --- |
| 1 | Increase workforce for CR merging – In WGs where all CR merging is done by a single MCC officer today, there is the possibility to: - Increase the MCC staff for CR merging. - Offload this task to each spec rapporteur. 3GPP could provide regular training courses for delegates that are candidates to become TS rapporteurs. | Drastic decrease in the workload for MCC officers, reduces the need to look for automated tools for merging CRs into specs. | Increase in workload for delegates. Need to ensure that all TS rapporteurs complete the task timely and respect all 3GPP drafting rules with proper and regular training. |  |
| 2 | Scripting to automatically merge CRs – ETSI is having a tool developed to automatically merge CRs into the specification. | Reduces human error and decreases the time to produce a merged specification. | It is unknown if such a script could be released to delegates and if it would comply with company security policies. For example, if macros are used, many would not be able to run them. |  |
| 3 | [New tool] CR conformance checking – write a new tool to check and report the location and nature of errors in a CR. These errors could include style errors, non-contiguous clause numbering, non-conforming figure and table numbering, and mismatches between the CR database and the CR cover page. | By using the tool pre-submission, delegates would not experience delays in submission and less time would be spent during meetings discussing styles and formatting. | It is difficult to write and maintain consistent tools based on docx due to high variability in docx file structures. Ensuring that styles didn’t change between versions and ensuring that the correct approved styles are used could be challenging and encounter corner cases. |  |
| 4 | [New tool] CR auto-generation - Write a tool, e.g., an application-native script, Python script or other programming language-based tool to automatically create a CR from a modified version of a specification, including the following: - Auto-filled cover page including the specification number, specification version, and affected clauses - Automatic inclusion of affected clauses in the body of the CR, including change marks - Additionally, the macro would need to be available for local use such that delegates could check their CRs prior to submission. | - Many error-prone fields of the CR cover page would be guaranteed to be correct. - Style errors would be prevented - The correct specification base text would be used | - Modifying additional clauses or removing affected clauses requires a regeneration of the CR - Difficult to write and maintain consistent tools based on docx due to high variability in docx file structures - Some companies do not allow the use of macros. - This solution only works for the initial version of the CR. |  |
| 5 | CR cover auto-generation | The tool exists online at the time of reserving a TDoc for a CR. | The tool is only available for online use. |  |
| 6 | Restrict editing | The feature is available in current tool used for writing CRs and the specification. | This feature is not available in all docx editors. |  |
| 7 | Training – (1) correct use of styles | Easy to implement through reminder by Chairs or MCC with detailed guidance. It is possible for MCC to organize training sessions and to provide tutorial documents. | Some tools other than used for processing docx files may not allow delegates full control of the styles. |  |
| 8 | Light version of Microsoft Word - light version would include only features used in 3GPP without any additional feature, e.g. removing the possibility of adding new styles, and many other functions. | Removing most of Microsoft Word features should help reduce file sizes, ensure consistent use of styles and formatting, and may make automatic processing of compressed docs files easier. | Microsoft is not likely to create a custom version of Word for 3GPP. |  |
| 9 | EditHelp Consulting from ETSI - This is done before entering change control. After change control secretary review is possible but does not scale well. | Current rules require this [In principle all authors must use official templates and settings. In practice, no one enforces this] | Forcing adherence to rules leads to slower progress. Those who did so were called CR police - respected, not loved. This solution relies on accountability by contributors for repeated errors, but this is disrespectful in a professional setting. So this solution is not feasible. [In principle all authors must use official templates and settings. In practice, no one enforces this] |  |
| 10 | Exhaustive analysis of docx files against 21.801 | Issues would be discovered that are currently hidden. | This would have a very large impact on ways of working since authors would need to strictly use MS Word according to rules. |  |
| 11 | Externalization of APIs and data structures - APIs and data structures could be stored externally to the specification. | Standard text editors would take on the role of enhancing readability of APIs and data structures by providing colorization, indentation, and syntax highlighting. | The specifications would be more difficult to search. Field descriptions and procedures related to the APIs and data structures would be stored separately, requiring more careful checking. Requires code to be separate from CRs, essentially. This reduces CR specification cohesiveness. |  |
| 12 | Single representation of APIs and data structures - APIs and data structures could be limited to a single representation. | The need for specifying which version of an API or data structure is authoritative would be eliminated. | Some WGs include additional information, e.g., a tabular form of APIs and data structures and would need to devise a new way of capturing the additional information. |  |
| 13 | [New tool] Automatic syntax checking - Syntax checking Reference checking, e.g., checking that a datatype of a parameter exists as an intrinsic to the language or as defined elsewhere in the API or data structure specifications. | The volume of CRs with syntax and consistency errors would be reduced. | Many errors cannot be identified without checking across all content associated with the entire target specification, or across multiple specifications. |  |
| 14 | Include the specification number and clause number in references – When available, include enough information to find the information referenced. |  |  |  |
| 15 | Use hyperlinks in references – manual process | Reader could immediately navigate to a reference without having to search each document. | The specifications are stored as zip files on the 3GPP server and in the 3GPP portal. If links were provided in the specifications, they would be to zip files, which would have to be downloaded, extracted and opened. |  |
| 16 | [New tool] Script to create, validate, and automate verification of cross-references between specifications | Completeness and accuracy of references can be improved (if a solution is feasible). | If a solution is feasible, it will require extensive parsing of many specifications which may take time and may turn out to be prone to errors. Validation will be complex. |  |
| 17 | Store figure source files - For editable figures, store at least the source file, named in accordance with the figure number, alongside the specification. | - Standalone editors could be used to modify figures - The figure source would never be lost. - Because captions and headings are never reused, file naming consistency should be feasible. | - Any time a figure is edited, an extra step needs to be taken to ensure that the figure source file is updated. - Errors could occur in naming of the source files. - This disaggregation of content removes one benefit of the use of MS Word, namely that a single file contains all content in a simple way. |  |
| 18 | Store images as vector graphics | Scalable Vector Graphics (SVG) are scalable without distortion and are smaller in file size. |  |  |
| 19 | Support a limited number of supported formats which work across WGs and platforms | Limited changes to the current way of working. |  |  |
| 20 | Provide comments on CRs in a separate file instead of using bubble comments | Using a separate WORD document for collecting comments is a widely used technique in WGs, and allows responding to comments efficiently | When using a draft FTP folder, comments from multiple delegates may collide in time and separate branches of the same file are created, which are then cumbersome to merge. Delegates have to carefully provide the reference to the clause and line of the spec they are commenting on. |  |
| 21 | Use NWM to collect comments | Using NWM for collecting comments is a technique used by some TSGs/WGs. | Using NWM avoids collision of comments, but NWM is not convenient for commenting on equations or figures. Delegates have to carefully provide the reference to the clause and line of the spec they are commenting on. |  |
| 22 | [New tool] Extract text under review and use Git to manage reviews | This would help reviewing text-based large parts of specs where typically a large number of comments are received during the review of a draft TS. Delegates would be able to see spec text and provide a comment directly over that spec text without using another file. | This requires separating (during review time) some parts of the specs which are only text-based for separate commenting, since text-based file formats may not be suitable to parts of specs that contains other types of objects (equations, figures, etc). |  |
| 23 | Use FTP to download latest inputs and upload new inputs | It is based on existing tools. | Simultaneous editing and updating is not possible nor traceable. Coordination is very difficult. |  |
| 24 | Split the CR for review into multiple files – this can include splitting the ASN.1 from a spec like 38.331 for review. | It is based on existing tools and does help speed up work and reduces some contention. | Other problems are created, as changes to different parts of the spec can diverge. This approach does not scale to large files. |  |
| 25 | Split large specifications into smaller parts | Faster opening | Decreased locality of content, more difficult to maintain, read, etc. |  |
| 26 | Open specification and change to draft mode | - Loading and editing times could be reduced. - Change marks are still visible in draft mode | - It is more difficult to check bubble comments in draft mode - Some document editing tools have been known to crash prior to being able to enter draft mode. - Need to ensure correct insertion of figures in WORD so that they are still visible in draft view. |  |
| 27 | Produce 3GPP PDF version of the specification after each plenary | - Quicker access to a version of the specification which isn’t impacted by the slowness of some WYSIWYG editors. - Processing for converting all WORD docs into PDF is done only once for each spec version, | - Cannot be used for producing CRs. - Need to ensure that conversion does not lose any information. - Requires additional storage on FTP server. |  |
| 28 | Make all specs available in HTML | Faster opening, faster search. Processing for converting all WORD docs into HTML is done only once for each spec version, presumably by MCC. | Cannot be used for producing CRs. No navigation panel in html. Need to ensure that conversion does not lose any information. Requires additional storage on FTP server. |  |
| 29 | Mandate the numbering of requirements (PR and CPR) - In some WGs, requirement numbering is done already. Maintenance of numbering is done the normal way - through careful alignment of provisions whenever they are introduced or corrected. | It is possible to refer to requirements without copying the text of the requirement. This reduces the risk of misalignment between specifications. |  |  |
| 30 | Include WI and meeting number with editor’s notes - For example, Editor’s Note: [FS_AIML-air_core, RAN2#129] | - It would be easier to find the delegate(s) responsible for the editor’s note - It would be easier to identify stale topics to resolve. | - Slightly more work and introduces meeting-related details into the specification, which isn’t ideal. An alternative would be to find the CR which introduced the Editor’s note. |  |

## Requirements identification

### Conference call #2 (Sep-2025)

According to the agreed pCR 6GSM-250107, the following table summarizes the identified requirements:

| # | Requirement | Description | Applicable to objective 2 | Applicable to objective 3 |
| --- | --- | --- | --- | --- |
| 1 | Global availability | There shall be no geographic limitations on availability and usability of tools and formats. | Yes | Yes |
| 2 | Cost and licensing | Tools shall be available at reasonable cost. Ideally, the use of open source and free tools (under reasonable licensing terms) should be prioritized, if possible. | Yes | Yes |
| 3 | Security and Availability | Tools shall be secure i.e. robust to manipulation of source code and shall be systematically available (i.e. usable) to avoid 3GPP coming to a halt. | Yes | Yes |
| 4 | Cross platform | It shall be possible to view and edit the specifications, including figures, on all major computer platforms (not necessarily including mobile platforms). Ideally, online variants of these tools should be available (a.k.a. web-GUIs ). | Yes | Yes |
| 5 | Simplicity and learning curve | Tools shall be reasonably easy to use and should not cause significant change of the existing working procedures and/or drafting rules. | Yes | Yes |
| 6 | Limited number of tools and formats. | Specifications shall only require a limited number of approved formats and recommended tools. | Yes | Yes |
| 7 | One reference | It should be possible to produce representations of the specifications in various formats suitable for different situations. However, at the same time there shall be only one reference. The representation (i.e. reference output) should have consistent appearance to specifications of past generations. | Yes | Yes |
| 8 | Fast opening, navigating and editing | Specifications shall be fast (no longer than 30 seconds on a reasonably modern computer even for large and complex specifications) to open. Furthermore, there shall be no noticeable lag when searching, navigating or editing the specifications. The tool shall be stable (no crashes) and shall not require splitting large documents into multiple files. | Yes | Yes |
| 9 | CR implementation | It shall be possible to quickly and easily (ideally automatically) implement a CR. This applies to both MCC and delegates (e.g. CR/TR rapporteurs). Furthermore, it should be possible to automatically detect CR conflicts (e.g. detect if multiple CRs propose changes to text in the same clauses, or the same text, or if a definition is changed for a term that is used in other CRs, etc). | Yes | Yes |
| 10 | CR Traceability | Specifications format and tools shall support an easy way for implementors to track introduction of a new feature and corrections to the existing ones. Tools used for handling CRs (submission, revisions, agreement, rejection, etc) should allow that all decisions be traceable along with the reasons for decision. | Yes | Yes |
| 11 | Feature Traceability | The tools shall allow to document features and capabilities in a format that can be automatically parsed. It should be possible (if feasible) to associate a feature with the relevant text in all specifications which are related to the feature. | Yes | Yes |
| 12 | Cross-referencing functionality to track and point to exact section in the other specification | It shall be possible to cross reference multiple specifications including (sub-)clauses. Such cross referencing shall be easy to navigate. (NOTE 1) | Yes | Yes |
| 13 | Tracking of Editor’s notes | It shall be possible to automatically track Editor’s Notes (EN) addition/removal/resolution, if ENs are used as an issue raising/resolving mechanism. | Yes | Yes |
| 14 | References to discussion papers | It shall be possible to reference discussion papers (for background information) in CR cover page (or its equivalent in the new tool/format). | Yes | Yes |
| 15 | Automation | Specification format shall enable automation, e.g. to: - Ensure consistent use of styles - Extract ASN.1 (if in the new format ASN.1 isn’t stored separately) | Yes | Yes |
| 16 | Specification browsing | It shall be possible to search and navigate within and across specs. It shall be possible to select the exact release and version of a TR or TS to access. | Yes | Yes |
| 17 | CR drafting during and in between meetings | It shall be possible to handle the process of drafting a CR involving a large number of contributors who provide a large number of comments and propose a large number of changes. CR handling needs to work for normal meetings, where only author can provide revisions, and before and after meetings for collaborative work between source companies preparing input documents. (NOTE 2) | Yes | Yes |
| 18 | Collaboration on CR drafting between companies | Sharing of draft CRs within and between the source companies for preparing input documents shall be possible. | Yes | Yes |
| 19 | Consistent access to CRs and specifications | The tools should allow 3GPP delegates to access CRs and specifications in a similar way as today during pre-meeting, post-meeting and online meeting. | Yes | Yes |
| 20 | Visual representation of changes | Tools shall allow visual representation of the changes in a CR and between the original spec and the revision. The author, i.e., at least the source company, of proposed changes should be visible. The effects or changes imposed by a CR should be easily readable and navigable such that all the changes can be viewed in the context of the clause affected. | Yes | Yes |
| 21 | Supported styles | Tools shall support the following styles: - Headings – Heading 1, Heading 2, up to Heading 6 - Hyperlinks – Internal and external - Lists (bulleted) - Lists (hierarchical) - Lists (numbered) (NOTE 3) - Tables (simple cell contents; allows different justification {left, center, right}) - Table (column header contents) - Table of contents - Text bolding (or equivalent highlighting) - Text italicization (or equivalent highlighting) - Text subscript - Text superscript - code (in the case code is not stored separately in its native format) - symbols (non-alpha-numeric characters), non-breaking spaces and hyphens - Notes - Editor's Notes - Table footers - Table headers - References - Equations Ideally, there should be a way to preclude usage of styles not listed above. In particular, complex tables shall be avoided. | Yes | Yes |
| 22 | Supported objects | Tools shall support: - Figures (e.g. block diagrams, call flow diagrams, images) - Equations (inline and block). | Yes | Yes |
| 23 | Seeing output while editing | The tool shall support the ability to view the content of (part of a specification) in the form in which it will be published (as a specification), including tables and diagrams while and after editing. | No | Yes |
| 24 | Offline work | The tool shall support the ability to read and edit specifications and CRs and to verify CRs offline, in particular to allow private editing before submission. | Yes | Yes |
| 25 | Meeting decisions and reports (meeting minutes) | Tools shall support tracking decisions of the following: CR, draft CR, pCR, TS, TR with Tdoc numbers in meeting reports. | Yes | Yes |
| 26 | Public access to specifications and CRs | Tools shall allow easy access to CRs and Specifications to the larger community, including those who do not actively participate in 3GPP. The specifications shall be convertible to PDF for publishing. | Yes | Yes |
| 27 | Fault tolerance | Tools and working methods should have reasonable fault tolerance against minor human errors. Corner cases should be manageable. | Yes | No |
| 28 | Consistent appearance | New specifications (for 6G) need to have a consistent appearance and quality, similar to previous generations. | Yes | Yes |
| 29 | Supported style functions | A delegate, MCC rapporteur shall be able to highlight text, to draw attention to it, even though highlighting is not allowed in the drafting rules and cannot be in an approved CR. A delegate, MCC rapporteur shall be able to remove all formatting from text. A delegate, MCC rapporteur shall be able to view non-printing characters (non-breaking spaces, tabs, new lines, page breaks, etc.) if these are added to the content of the CR. | Yes | Yes |
| 30 | Comments | It shall be possible to associate a comment with specific provisions (e.g. a paragraph, single word, figure, etc.) of any CR. This comment includes comment text, the identity of the commenter and the time in which the comment was provided. It shall be possible to remove or 'disassociate' comments with text in a CR. | Yes | Yes |
| 31 | CR format | CRs using the new specification format for 6G should support visual emphasis of modified sections, be compatible with file formats acceptable for submission to 3GPP meetings, and allow for seamless transfer of the proposed changes into discussion papers as Text Proposals | Yes | Yes |
| 32 | CR cover pages | The tool shall support CR cover page (or its equivalent in the new tool) checking (e.g. no dates in the future, CRs to specifications that don't exist, the other specs affected tick boxes are neither yes nor no, and so on). The tool shall also support automatic generation of CR cover pages (where possible). The tool shall also support reviewing, editing and commenting on CR cover pages during CR review process. | Yes | Yes |
| 33 | Code | The tool and formats shall support code (e.g. ASN.1). If code is in the same document as the CR/TS, there should be a means to extract the code portions from the technical specification so that the code can be used as input to machine processing (e.g. an interpreter, compiler, etc.) Code that is part of a CR/TS should be able to be checked for syntax errors, and compile time errors. Code that is part of a CR should be able to be checked in combination with other code provided in the specification, to determine if there are redundancies, mismatches, etc. | Yes | Yes |
| 34 | Support review of figures | Every change to a figure shall result in a 'change marking' visible, indicating what changed and who made the change. (NOTE 4) | Yes | Yes |
| 35 | Support consistent common configuration | It shall be possible to enforce simple and consistent configuration for users, so that errors arising while creating, editing and cut-n-pasting due to inconsistencies are rare or impossible. (NOTE 5) | Yes | Yes |
| 36 | CRs comprising multiple files | The tools shall support (if needed) CRs comprising multiple files in simple and consistent order (i.e. as separate documents stored in a mandatory way for the associated CR or specification.) | Yes | Yes |
| 37 | CR database | Changes to CR documents (including headers and content) shall be automatically captured in CR databases to ensure the correct correspondence between an approved and implemented CR and the CR database. | Yes | Yes |
| 38 | Support for gathering input on a CR | The tool shall support gathering input on a CR or its content for highly active topics involving many participants, without divergence in the process. Use of tools to collect comments and feedback shall enable orderly capture of responses, so that the order, source and input from each reviewer is captured without ambiguity or loss. This facility shall scale up to 100s of active reviewers posting 1000s of comments a day. (NOTE 6) | Yes | Yes |
| 39 | Simplicity to follow drafting rules | The tool shall enable editors not to be concerned with styles, but rather with the technical content and its hierarchy in the document. | Yes | Yes |
| 40 | Support for visualization different parts of a document concurrently on the same screen | The tool shall enable visualizing different (incl. disjoint) parts of the same document concurrently on the same screen similar to Split View in MS Word. | Yes | Yes |
| 41 | Support for visualizing different documents concurrently on the same screen | The tool should enable visualizing different documents concurrently on the same screen. | Yes | Yes |
| 42 | Support for personal scripts | The tool should enable delegate, consumer, MCC/Rapporteur to develop and use their own scripts to manipulate a document locally or together with other documents (similar to MS Office Macros). | Yes | Yes |
| 43 | Language proofing | The format shall allow checking a document for errors in spelling, grammar, and punctuation, and ensuring it adheres to the rules of the English language. | Yes | Yes |
| NOTE 1: For example, conformance test specifications use heavy cross-referencing between the test spec (e.g. 38.521-1) and associated details in test point analysis (TR 38.905), MU/TT (TR 38.903), UE/UE connection diagrams (TS 38.508-1) which are critical for implementation of the test however difficult to maintain with manual effort. NOTE 2: The tool should improve upon the process used today. NOTE 3: Lists can be indented to different levels, e.g. to allow sub-bullets). NOTE 4: The granularity of the change marking is at least 'the figure has changed.' Greater granularity than that, e.g. highlighting changes in text in a figure, is considered Very Nice To Have. NOTE 5: This addresses the problem with current use of MS Word where the configuration of the tool is frequently inconsistent and hard to control, leading to poorer CR and specification quality. NOTE 6: The comments and related information that are gathered can be removed or are not intrinsically part of the CR or specification. |  |  |  |  |