3GPP TSG-RAN WG4 Meeting #117												R4-2520215

Dallas, USA, 17 – 21 November, 2025

Agenda item:			6.9.2.1

Source:	Huawei, HiSilicon

Title:	on TRP/TRS post measurement campaign data processing

Document for:	approval

# Introduction

The WF in RAN4 #116bis [1] has a remaining issue on post measurement campaign data processing. This contribution addresses the issue in more details.

Issue 3-1-7: Processing of measurement data pool for Rel-19 requirements discussion

Agreements:

The data pool will be refined based on one of the following options:

- Option 1: Remove 1 best and 1 worst performance data

- Option 2: Remove the data based on the CDF, FFS the criteria

# Discussion

Option 1 is simply an arbitrary choice and has neither logical nor statistical foundation for it. As a result, option 1 should be discarded.

Option 2 should only be applied to faulty data, e.g. from a non-functional device or from a functional device but test systems cannot correctly measure it performance, e.g. Tx antenna switching for TRP.

There are a few methods in statistics to identify outliers. One of them is to assume a normal distribution for the data set and use multiples of standard deviation to identify outliers. However, TRP and TRS measurement data do not follow normal distribution and such an approach would be an approximation.

Six sigma rule is well known in most industries and could be used here too. Sigma represents standard deviation in a normal distribution. Because six sigma is still within the data set. An outlier would be some distance away from the data set boundary. In other words, a margin would be needed to ensure good data points are not excluded by this rule. An additional two sigma should be sufficient to avoid removing a good data point by mistake.

Proposal 1: a data point from measurement campaign can be excluded from final analysis if either its TRP or TRS value is eight standard deviations away from the data set average.

# Conclusion

This contribution make the following proposal.

Proposal 1: a data point from measurement campaign can be excluded from final analysis if either its TRP or TRS value is eight standard deviations away from the data set average.

# Reference

- R4-2514868 WF on TRP_TRS_MIMO_OTA_Ph3