3GPP TSG RAN WG1 Meeting #117	 								                         R4-2520218

Dallas, Texas, U.S. , Nov 17th – 21th , 2025

Agenda Item:	8.6

Source:	Amazon

Title:	Consideration on 6GR RRM on TN and NTN

Document for:	Discussion and Decision

# Introduction

The 6G Radio Study Item was approved at the RAN#108 meeting (RP-251881) [1]. The main objectives are:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

This document provides an overview of 6GR NTN RRM requirements.

# Discussion

NTN RRM Requirements

A group of cells on the ground may be served by a single beam of a satellite and experience discontinuous transmission and reception (DTX/DRX) as the beam periodically “hops” across its coverage area in a TDM manner. This may be required if, for example, the total number of cells to be served is larger than the total number of supported beams per satellite across the system. An example is shown in the figure below, where an 200ms beam hopping configuration periodicity is configured for 4 cells, with each using a 50ms beam-hopping cycle duration:

Figure 1. Example of Beam Hopping

The frequency of the beam hopping across cells may be dependent on multiple factors such as the geographic area being served as well as the traffic load distribution. As part of the 6G study item, beam hopping for NTN should be considered in the overall design of reference signals and DL and UL control and data channels and its impact on user experience KPIs (e.g. throughput and latency) evaluated for both active and inactive users. In addition, performance requirements for active and idle users should take into account beam hopping as well as in the design of RRM measurements and procedures for mobility, cell (re)selection, and radio-link monitoring.

Proposal-1: 6GR performance requirements for active and idle users should take into account beam hopping as well as in the design of RRM measurements and procedures for mobility, cell (re)selection, and radio-link monitoring.

## Mobility and NTN/TN Interworking

The 6G study item should consider different mobility scenarios for NTN, including intra- and inter-frequency handovers, mobility between NTN and TN networks, and even the possibility of multi-connectivity between NTN and TN networks for the same device. It is desirable to harmonize NTN and TN mobility measurements and procedures where possible to simplify UE and network implementations, including the use of measurement reports, time/location-based triggers, and cell-selection criteria.

However, it is important to consider necessary enhancements for NTN UEs as well as the impact on measurement and performance requirements due to the specific challenges of NTN networks, such as very large propagation delays, Doppler variations across satellites, and discontinuous coverage, while also taking into account the signaling of assistance information (e.g. satellite ephemeris, propagation delay, timing offset parameters) in dedicated system broadcast messages which can be used for handovers, intra-RAT and inter-RAT cell selection (e.g. to/from 5G or 6G), or mobility to/from NTN and TN networks.

Proposal-2: The 6G study should consider necessary enhancements for NTN UEs as well as the impact on measurement and performance requirements due to the specific challenges of NTN networks, such as very large propagation delays, Doppler variations across satellites, and discontinuous coverage.

# Conclusion

In this contribution, we propose the following:

Proposal-1: 6GR performance requirements for active and idle users should take into account beam hopping as well as in the design of RRM measurements and procedures for mobility, cell (re)selection, and radio-link monitoring.

Proposal-2: The 6G study should consider necessary enhancements for NTN UEs as well as the impact on measurement and performance requirements due to the specific challenges of NTN networks, such as very large propagation delays, Doppler variations across satellites, and discontinuous coverage.