3GPP TSG-RAN4 Meeting #117	R4-2520219

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 3055 | rev | - | Current version: | 19.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_CADC_SUL_R19-Core) CR to 38.101-1-j31 to change MSD for band n41 10MHz CBW and to remove redundant dual-UL IMD MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Skyworks Solutions, Inc., Murata Manufacturing Co, Qualcomm France, T-Mobile USA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_SUL_R19-Core HPUE_NR_FR1_bands_R19-Core |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Refer to agreed WF R4-2514596 and discussion paper R4-2514497. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrections of dual-UL IMD MSD requirements due to band n41 10MHz CBW. Removal of redundant dual-UL IMD MSD requirements and - Table 7.3A.5-1, Note 21 is added: “NOTE 21: This band is subject to IMD4 also which MSD is not specified.” - Table 7.3A.5-1a, Note 18 is added: “NOTE 18: This band is subject to IMD4 also which MSD is not specified.” - Table 7.3A.5-1b, Note 4 is added: “NOTE 4: This band is subject to IMD4 also which MSD is not specified.” - The following redundant MSD requirements are not removed due to regional frequency restrictions: - in Table 7.3A.5-2: n2 IMD4 CA_n2-n66-n77, n25 IMD4 CA_n25-n66-n77, n77 IMD5 CA_n26-n70-n77 in Table 7.3A.5-2a: n25 IMD4 CA_n25-n66-n77. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MSD requirements remained specified for the band n41 optional 5MHz CBW. Redundant dual-UL MSD requirements remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.4, 7.3A.5. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

Table 7.3A.4-4a-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n25 | n41 | 5 | 15 | 6 | 10 | 2.213 3.214 | NOTE 8 | UL4/DL3 |
| n25 | n713 | 5 | 15 | 25 | 5 | 29.513 34.514 | NOTE 7 | UL1/DL3 |
| n25 | n713 | 5 | 15 | 25 | 20 | 18.213 23.314 | NOTE 7 | UL1/DL3 |
| n39 | n41 | 5 | 15 | 6 | 10 | 8.6 | NOTE 8 | UL4/DL3 |
| n39 | n41 | 5 | 15 | 6 | 100 | 4.9 | NOTE 8 | UL4/DL3 |
| n40 | n28 | 10 | 15 | 25 | 5 | 40.8 | NOTE 7 | UL1/DL3 |
| n40 | n28 | 10 | 15 | 25 | 20 | 33.3 | NOTE 7 | UL1/DL3 |
| n41 | n186 | 10 | 15 | 25 | 5 | 29.3 | NOTE 7 | UL1/DL3 |
| n41 | n186 | 10 | 15 | 25 | 15 | 24.3 | NOTE 7 | UL1/DL3 |
| n41 | n39 | 10 | 15 | 8 | 5 | 6.1 | NOTE 9 | UL3/DL4 |
| n41 | n39 | 10 | 15 | 8 | 40 | 1.5 | NOTE 9 | UL3/DL4 |
| n41 | n77 | 10 | 15 | 6 | 10 | 11.6 | NOTE 8 | UL4/DL3 |
| n41 | n77 | 10 | 15 | 6 | 100 | 3.6 | NOTE 8 | UL4/DL3 |
| n41 | n78 | 10 | 15 | 6 | 10 | 12.1 | NOTE 8 | UL4/DL3 |
| n41 | n78 | 10 | 15 | 6 | 100 | 3.8 | NOTE 8 | UL4/DL3 |
| n77 | n2 | 10 | 15 | 25 | 5 | 9.2 | NOTE 4 | UL1/DL2 |
| n77 | n2 | 10 | 15 | 25 | 20 | 4.4 | NOTE 4 | UL1/DL2 |
| n77 | n3 | 10 | 15 | 25 | 5 | 8.1 | NOTE 4 | UL1/DL2 |
| n77 | n3 | 10 | 15 | 25 | 40 | 0.8 | NOTE 4 | UL1/DL2 |
| n77 | n5 | 10 | 15 | 25 | 5 | 8.1 | NOTE 5 | UL1/DL4 |
| n77 | n5 | 10 | 15 | 25 | 20 | 1.5 | NOTE 5 | UL1/DL4 |
| n77 | n7 | 10 | 15 | 12 | 5 | 17.7 | NOTE 3 | UL2/DL3 |
| n77 | n7 | 10 | 15 | 12 | 50 | 3.6 | NOTE 3 | UL2/DL3 |
| n77 | n8 | 10 | 15 | 25 | 5 | 8.1 | NOTE 5 | UL1/DL4 |
| n77 | n8 | 10 | 15 | 25 | 20 | 1.5 | NOTE 5 | UL1/DL4 |
| n77 | n12 | 10 | 15 | 25 | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | n12 | 10 | 15 | 25 | 15 | 21 | NOTE 1 | UL1/DL5 |
| n77 | n13 | 10 | 15 | 25 | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | n13 | 10 | 15 | 25 | 10 | 30.8 | NOTE 1 | UL1/DL5 |
| n77 | n14 | 10 | 15 | 25 | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | n14 | 10 | 15 | 25 | 10 | 30.8 | NOTE 1 | UL1/DL5 |
| n77 | n25 | 10 | 15 | 25 | 5 | 7.9 | NOTE 4 | UL1/DL2 |
| n77 | n25 | 10 | 15 | 25 | 40 | 0.5 | NOTE 4 | UL1/DL2 |
| n77 | n28 | 10 | 15 | 25 | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | n28 | 10 | 15 | 25 | 30 | 14.1 | NOTE 1 | UL1/DL5 |
| n772 | n29 | 10 | 15 | 25 | 5 | 34 | NOTE 1 | UL1/DL5 |
| n772 | n29 | 10 | 15 | 25 | 10 | 30.8 | NOTE 1 | UL1/DL5 |
| n77 | n30 | 10 | 15 | 12 | 5 | 13.2 | NOTE 3 | UL2/DL3 |
| n77 | n30 | 10 | 15 | 12 | 10 | 10.2 | NOTE 3 | UL2/DL3 |
| n77 | n40 | 10 | 15 | 12 | 10 | 14.5 | NOTE 3 | UL2/DL3 |
| n77 | n40 | 10 | 15 | 12 | 100 | 5.5 | NOTE 3 | UL2/DL3 |
| n77 | n41 | 10 | 15 | 12 | 10 | 14.5 | NOTE 3 | UL2/DL3 |
| n77 | n41 | 10 | 15 | 12 | 100 | 5.5 | NOTE 3 | UL2/DL3 |
| n772 | n70 | N/A | N/A | N/A | N/A | N/A | NOTE 4 | UL1/DL2 |
| n77 | n85 | 10 | 15 | 25 | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | n85 | 10 | 15 | 25 | 15 | 21 | NOTE 1 | UL1/DL5 |
| n78 | n3 | 10 | 15 | 25 | 5 | 8.1 | NOTE 4 | UL1/DL2 |
| n78 | n3 | 10 | 15 | 25 | 40 | 0.8 | NOTE 4 | UL1/DL2 |
| n78 | n5 | 10 | 15 | 25 | 5 | 8.1 | NOTE 5 | UL1/DL4 |
| n78 | n5 | 10 | 15 | 25 | 20 | 1.5 | NOTE 5 | UL1/DL4 |
| n78 | n8 | 10 | 15 | 25 | 5 | 8.1 | NOTE 5 | UL1/DL4 |
| n78 | n8 | 10 | 15 | 25 | 20 | 1.5 | NOTE 5 | UL1/DL4 |
| n78 | n26 | 10 | 15 | 25 | 5 | 8.1 | NOTE 5 | UL1/DL4 |
| n78 | n28 | 10 | 15 | 25 | 5 | 34 | NOTE 1 | UL1/DL5 |
| n78 | n28 | 10 | 15 | 25 | 30 | 11.3 | NOTE 1 | UL1/DL5 |
| n78 | n40 | 10 | 15 | 12 | 10 | 14.5 | NOTE 3 | UL2/DL3 |
| n78 | n40 | 10 | 15 | 12 | 100 | 5.5 | NOTE 3 | UL2/DL3 |
| n79 | n8 | 10 | 15 | 25 | 5 | 28.0 | NOTE 1 | UL1/DL5 |
| NOTE 1: The requirements should be verified for DL NR-ARFCN of the victim (lower) band (superscript LB) such that and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with fULHB the UL carrier frequency and BWChannelHB the channel bandwidth configured in the higher band, both in MHz. NOTE 2: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 3: The requirements should be verified for DL NR-ARFCN of the Victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with fULHB the UL carrier frequency and BWChannelHB the channel bandwidth configured in the higher band, both in MHz. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 6: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel's transmission bandwidth of an aggressor (higher) band. NOTE 7: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 8: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 9: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 10: Void NOTE 11: Void. NOTE 12: The requirements should be verified using RBstart = floor((NRB-LCRB)/2), where floor(x) is the greatest integer less than or equal to x, and where the UL parameters NRB and LCRB are respectively, the transmission bandwidth configuration and the number of RB’s for the specified UL band channel bandwidth and the UL band subcarrier spacing. NOTE 13: Applicable to UE’s supporting PC2 with 1Tx NOTE 14: Applicable to UE’s supporting PC2 with 2Tx |  |  |  |  |  |  |  |  |

Table 7.3A.4-4a-2: Void

==============Next change==============

### 7.3A.5	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA

For inter-band carrier aggregation with uplink assigned to two CCs from up to two UL NR bands and three CCs from two UL NR bands given in Table 7.3A.5-1, Table 7.3A.5-1a, Table 7.3A.5-1b, Table 7.3A.5-2, Table 7.3A.5-2a and Table 7.3A.5-2b the reference sensitivity is defined only for the specific uplink and downlink test points specified in Table 7.3A.5-1, Table 7.3A.5-1a, Table 7.3A.5-1b, Table 7.3A.5-, Table 7.3A.5-2a and Table 7.3A.5-2b. For these test points the reference sensitivity requirement specified in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-2 and Table 7.3.2-2a  are relaxed by the amount of the corresponding parameter MSD given in Table 7.3A.5-1, Table 7.3A.5-1a, Table 7.3A.5-1b, Table 7.3A.5-2, Table 7.3A.5-2a and Table 7.3A.5-2b.

Table 7.3A.5-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n3 | n1 | 1950 | 5 | 25 | 2140 | 23 | FDD | IMD3 |
|  | n3 | 1760 | 5 | 25 | 1855 | N/A | TDD | N/A |
| CA_n1-n8 | n1 | 1965 | 5 | 25 | 2155 | 6.0 | FDD | IMD4 |
|  | n8 | 887.5 | 5 | 25 | 932.5 | N/A | FDD | N/A |
| CA_n1-n46 | n1 | 1950 | 5 | 25 | 2140 | 5 | FDD | IMD5 |
|  | n46 | 5660 | 20 | 100 | 5660 | N/A | TDD | N/A |
| CA_n1-n71 | n1 | 1958 | 5 | 25 | 2148 | N/A | FDD | N/A |
|  | n71 | 668 | 5 | 25 | 622 | 15.1 | FDD | IMD3 |
| CA_n1-n77 | n1 | 1950 | 5 | 25 | 2140 | 29.8 | FDD | IMD24,21 |
|  | n77 | 4090 | 10 | 50 | 4090 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2130 | 17 | FDD | IMD516 |
|  | n7712 | 3310 | 10 | 1 (RBSTART=25) | 3310 | N/A | TDD | N/A |
|  |  | 3900 | 10 | 1 (RBSTART=25) | 3900 |  |  |  |
| CA_n1-n78 | n1 | 1950 | 5 | 25 | 2140 | 8.0 | FDD | IMD4 |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2167.5 | 1.7 | FDD | IMD717 |
|  | n7812 | 3305 | 10 | 1 (RBSTART=0) | 3305 | N/A | TDD | N/A |
|  |  | 3675 | 10 | 1 (RBSTART=44) | 3675 |  |  |  |
| CA_n1-n102 | n1 | 1922.5 | 5 | 25 | 2112.5 | 13 | FDD | IMD3 |
|  | n102 | 5957.5 | 20 | 100 | 5957.5 | N/A | TDD | N/A |
| CA_n1-n105 | n1 | 1958 | 5 | 25 | 2148 | N/A | FDD | N/A |
|  | n105 | 673 | 5 | 25 | 622 | 15.1 | FDD | IMD3 |
| CA_n2-n48 | n2 | 1852.5 | 5 | 25 | 1932.5 | 12 | FDD | IMD4 |
|  | n48 | 3625 | 20 | 100 | 3625 | N/A | TDD | N/A |
| CA_n2-n66 | n2 | 1855 | 5 | 25 | 1935 | 20 | FDD | IMD34 |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | FDD | N/A |
| CA_n2-n77 | n2 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD221 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n2 | 1885 | 5 | 25 | 1965 | 5 | FDD | IMD5 |
|  | n77 | 3810 | 10 | 50 | 3810 | N/A | TDD | N/A |
|  | n2 | N/A | 5 | N/A | 1987.5 | 2.7 | FDD | IMD7 |
|  | n7712 | 3455 | 10 | 1 (RBSTART=10) | 3455 | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 (RBSTART=0) | 3945 |  |  |  |
| CA_n2-n78 | n2 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD24 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
| CA_n3-n5 | n3 | 1771 | 10 | 50 | 1866 | 4 | FDD | IMD4 |
|  | n5 | 838 | 5 | 25 | 883 | N/A | FDD | N/A |
|  | n3 | 1721 | 10 | 50 | 1816 | N/A | FDD | N/A |
|  | n5 | 838 | 5 | 25 | 883 | 24 | FDD | IMD23 |
| CA_n3-n7 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n7 | 2535 | 10 | 50 | 2655 | 10.2 | FDD | IMD4 |
| CA_n3-n8 | n3 | 1755 | 10 | 50 | 1850 | N/A | FDD | N/A |
|  | n8 | 900 | 5 | 25 | 945 | 8 | FDD | IMD44 |
|  | n3 | 1747.5 | 10 | 50 | 1842.5 | 6.4 | FDD | IMD5 |
|  | n8 | 897.5 | 5 | 25 | 942.5 | N/A | FDD | N/A |
| CA_n3-n18 | n3 | 1731 | 5 | 25 | 1826 | 4 | FDD | IMD4 |
|  | n18 | 818 | 5 | 25 | 863 | N/A | FDD | N/A |
| CA_n3-n20 | n3 | 1775 | 5 | 25 | 1870 | 4 | FDD | IMD4 |
|  | n20 | 840 | 5 | 25 | 799 | N/A | FDD | N/A |
|  | n3 | 1735 | 5 | 25 | 1830 | N/A | FDD | N/A |
|  | n20 | 847 | 5 | 25 | 806 | 9 | FDD | IMD4 |
| CA_n3-n26 | n3 | 1771 | 5 | 25 | 1866 | 4 | FDD | IMD4 |
|  | n26 | 838 | 5 | 25 | 883 | N/A | FDD | N/A |
|  | n3 | 1721 | 5 | 25 | 1816 | N/A | FDD | N/A |
|  | n26 | 838 | 5 | 25 | 883 | 26 | FDD | IMD24 |
| CA_n3-n38 | n3 | 1713 | 5 | 25 | 1808 | 8.2 | FDD | IMD4 |
|  | n38 | 2617 | 5 | 25 | 2617 | N/A | TDD | N/A |
| CA_n3-n41 | n3 | 1740 | 5 | 25 | 1835 | 8.2 | FDD | IMD4 |
|  | n41 | 2657.5 | 10 | 50 | 2657.5 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1877.5 | N/A | FDD | IMD914 |
|  | n41 | 2545 | 60 | 1 (RBSTART= 0) | 2545 | N/A | TDD | N/A |
|  |  | 2625 | 100 | 1 (RBSTART= 272) | 2625 |  |  |  |
|  | n3 | 1747.5 | 5 | 25 (RBSTART= 0) | 1842.5 | 15.3 | FDD | IMD3 |
|  | n41 | 2560 | 60 | 1 (RBSTART= 30) | 2560 | N/A | TDD | N/A |
|  |  | 2620 | 60 | 1 (RBSTART= 127) | 2620 |  |  |  |
| CA_n3-n77 | n3 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD24,21 |
|  | n77 | 3575 | 10 | 50 | 3575 | N/A | TDD | N/A |
|  | n3 | N/A | N/A | N/A | N/A | N/A | FDD | IMD57 |
|  | n7712 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1877.5 | 2.2 | FDD | IMD7 |
|  | n7712 | 3427.5 | 10 | 1 (RBSTART=10) | 3427.5 | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 (RBSTART=0) | 3945 |  |  |  |
| CA_n3-n78 | n3 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD24,21 |
|  | n78 | 3575 | 10 | 25 | 3575 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1877.5 | 2.2 | FDD | IMD7 |
|  | n7812 | 3305 | 10 | 1 (RBSTART=3) | 3305 | N/A | TDD | N/A |
|  |  | 3780 | 10 | 1 (RBSTART=0) | 3780 |  |  |  |
| CA_n3-n104 | n3 | 1750 | 5 | 25 | 1845 | 9.0 | FDD | IMD4 |
|  | n104 | 7095 | 20 | 50 | 7095 | N/A | TDD | N/A |
| CA_n5-n7 | n5 | 834 | 5 | 25 | 879 | 12 | FDD | IMD34 |
|  | n7 | 2547 | 10 | 50 | 2667 | N/A | FDD | N/A |
| CA_n5-n12 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | N/A18 | FDD | N/A |
|  |  | 838.9 | 10 | 10 (RBSTART=36) | 883.9 |  |  |  |
|  | n12 | N/A | 5 | N/A | 743.5 | 20.8 | FDD | IMD11 |
| CA_n5-n13 | n5 | 828 | 5 | 25 | 873 | 25 | FDD | IMD3 |
|  | n13 | 783 | 5 | 25 | 752 | N/A | FDD | N/A |
| CA_n5-n14 | n5 | 836 | 5 | 25 | 881 | 25 | FDD | IMD34 |
|  | n14 | 791 | 5 | 25 | 761 | N/A | FDD | N/A |
|  | n5 | 826.5 | 5 | 25 | 871.5 | N/A | FDD | N/A |
|  | n14 | 795.5 | 5 | 25 | 765.5 | 25 | FDD | IMD3 |
|  | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | N/A18 | FDD | N/A |
|  |  | 838.9 | 10 | 10 (RBSTART=28) | 883.9 |  |  |  |
|  | n14 | N/A | 5 | N/A | 765.5 | 26.2 | FDD | IMD9 |
| CA_n5-n29 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | N/A18 | FDD | N/A |
|  |  | 838.9 | 10 | 10 (RBSTART=36) | 883.9 |  |  |  |
|  | n29 | N/A | 5 | N/A | 725.5 | 16.0 | FDD | IMD13 |
| CA_n5_n41 | n5 | 839 | 5 | 25 | 884 | 15.6 | FDD | IMD33 |
|  | n41 | 2562 | 10 | 50 | 2562 | N/A | TDD | N/A |
| CA_n5-n66 | n5 | 838 | 5 | 25 | 883 | 30 | FDD | IMD24 |
|  | n66 | 1721 | 5 | 25 | 2121 | N/A | FDD | N/A |
| CA_n5-n77 | n5 | N/A | N/A | N/A | N/A | N/A | FDD | IMD27 |
|  | n7712 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n5 | 844 | 5 | 25 | 889 | 8.3 | FDD | IMD413 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |
|  | n5 | 829 | 5 | 25 | 874 | 5.5 | FDD | IMD513 |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | TDD | N/A |
|  | n5 | N/A | 5 | N/A | 880 | 8.6 | FDD | IMD4 |
|  | n7712 | 3410 | 10 | 1 (RBSTART=25) | 3410 | N/A | TDD | N/A |
|  |  | 3850 | 10 | 1 (RBSTART=25) | 3850 |  |  |  |
| CA_n5-n78 | n5 | 844 | 5 | 25 | 889 | 8.3 | FDD | IMD4 |
|  | n78 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |
|  | n5 | N/A | 5 | N/A | 880 | 8.6 | FDD | IMD417 |
|  | n7812 | 3340 | 10 | 1 (RBSTART=25) | 3340 | N/A | TDD | N/A |
|  |  | 3780 | 10 | 1 (RBSTART=25) | 3780 |  |  |  |
| CA_n7-n20 | n7 | 2512 | 5 | 50 | 2632 | N/A | FDD | N/A |
|  | n20 | 851 | 5 | 25 | 810 | 12 | FDD | IMD3 |
| CA_n7-n26 | n7 | 2556 | 5 | 25 | 2676 | N/A | FDD | N/A |
|  | n26 | 837 | 5 | 25 | 882 | 16.0 | FDD | IMD34 |
|  | n7 | 2567.5 | 5 | 25 | 2687.5 | 2.5 | FDD | IMD5 |
|  | n26 | 816.5 | 5 | 25 | 861.5 | N/A | FDD | N/A |
| CA_n7-n40 | n7 | 2510 | 5 | 25 | 2630 | 23 | FDD | IMD3 |
|  | n40 | 2390 | 5 | 25 | 2390 | N/A | TDD | N/A |
| CA_n7-n46 | n7 | 2550 | 10 | 50 | 2670 | 26.8 | FDD | IMD24 |
|  | n46 | 5220 | 20 | 100 | 5220 | N/A | TDD | N/A |
| CA_n7-n66 | n7 | 2535 | 10 | 50 | 2655 | 15 | FDD | IMD4 |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |
| CA_n7-n77 | n7 | 2540 | 5 | 25 | 2660 | 7.1 | FDD | IMD4 |
|  | n77 | 3870 | 10 | 50 | 3870 | N/A | TDD | N/A |
|  | n7 | N/A | 5 | N/A | 2687.5 | 15 | FDD | IMD5 |
|  | n7712 | 3455 | 10 | 1 (RBSTART=0) | 3455 | N/A | TDD | N/A |
|  |  | 3835 | 10 | 1 (RBSTART=7) | 3835 |  |  |  |
| CA_n7-n78 | n7 | N/A | 5 | N/A | 2650 | 15 | FDD | IMD5 |
|  | n7812 | 3350 | 10 | 1 (RBSTART=25) | 3350 | N/A | TDD | N/A |
|  |  | 3700 | 10 | 1 (RBSTART=25) | 3700 |  |  |  |
| CA_n8-n20 | n8 | 890.5 | 5 | 25 | 935.5 | N/A | FDD | N/A |
|  | n20 | 849.5 | 5 | 25 | 808.5 | 25 | FDD | IMD34 |
|  | n8 | 892.5 | 5 | 25 | 937.5 | 25 | FDD | IMD34 |
|  | n20 | 847.5 | 5 | 25 | 806.5 | N/A | FDD | N/A |
| CA_n8-n41 | n8 | 882.5 | 5 | 25 | 927.5 | 12.1 | FDD | IMD34 |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
| CA_n8-n77 | n8 | 897.5 | 5 | 25 | 942.5 | 8.3 | FDD | IMD4 |
|  | n77 | 3635 | 10 | 50 | 3635 | N/A | TDD | N/A |
| CA_n8-n78 | n8 | 897.5 | 5 | 25 | 942.5 | 8.3 | FDD | IMD4 |
|  | n78 | 3635 | 10 | 50 | 3635 | N/A | TDD | N/A |
| CA_n8-n79 | n8 | 897.5 | 5 | 25 | 942.5 | 4.8 | FDD | IMD5 |
|  | n79 | 4532.5 | 40 | 216 | 4532.5 | N/A | TDD | N/A |
| CA_n12-n66 | n12 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
|  | n66 | 1765 | 5 | 25 | 2115 | 5.0 | FDD | IMD4 |
| CA_n12-n77 | n12 | 702 | 5 | 20 | 732 | 5.5 | FDD | IMD5 |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | TDD | N/A |
| CA_n12-n78 | n12 | 710 | 5 | 25 | 740 | 5.5 | FDD | IMD5 |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | TDD | N/A |
| CA_n13-n77 | n13 | 782 | 5 | 20 | 751 | 5.5 | FDD | IMD5 |
|  | n77 | 3880 | 10 | 50 | 3880 | N/A | TDD | N/A |
| CA_n14-n77 | n14 | 793 | 5 | 20 | 763 | 5.5 | FDD | IMD5 |
|  | n77 | 3935 | 10 | 50 | 3935 | N/A | TDD | N/A |
| CA_n18-n41 | n18 | 820 | 5 | 25 | 865 | 14.1 | FDD | IMD3 |
|  | n41 | 2505 | 10 | 50 | 2505 | N/A | TDD | N/A |
| CA_n18-n77 | n18 | 827.5 | 5 | 25 | 872.5 | 8.4 | FDD | IMD48 |
|  | n77 | 3355 | 10 | 50 | 3355 | N/A | TDD | N/A |
|  | n18 | 817.5 | 5 | 25 | 862.5 | 4.5 | FDD | IMD58 |
|  | n77 | 4130 | 10 | 50 | 4130 | N/A | TDD | N/A |
|  | n18 | N/A | N/A | N/A | N/A | N/A | FDD | IMD27 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n18 | N/A | 5 | N/A | 870 | 8.6 | FDD | IMD4 |
|  | n77 | 3480 | 10 | 1 (RBSTART=25) | 3480 | N/A | TDD | N/A |
|  |  | 3915 | 10 | 1 (RBSTART=25) | 3800 |  |  |  |
| CA_n18-n78 | n18 | 827.5 | 5 | 25 | 872.5 | 8.3 | FDD | IMD49 |
|  | n78 | 3355 | 10 | 50 | 3355 | N/A | TDD | N/A |
| CA_n20-n41 | n20 | 851 | 5 | 25 | 810 | 12.1 | FDD | IMD34 |
|  | n41 | 2512 | 10 | 50 | N/A | N/A | TDD | N/A |
| CA_n20-n77 | n20 | 850 | 5 | 25 | 809 | 11 | FDD | IMD4 |
|  | n77 | 3359 | 10 | 50 | 3359 | N/A | TDD | N/A |
|  | n20 | 840 | 5 | 25 | 799 | 6.5 | FDD | IMD5 |
|  | n77 | 4159 | 10 | 50 | 4159 | N/A | TDD | N/A |
| CA_n20-n78 | n20 | 850 | 5 | 25 | 809 | 11 | FDD | IMD4 |
|  | n78 | 3359 | 10 | 50 | 3359 | N/A | TDD | N/A |
|  | n20 | N/A | 5 | N/A | 800 | 8.6 | FDD | IMD415 |
|  | n7812 | 3350 | 10 | 1 (RBSTART=25) | 3350 | N/A | TDD | N/A |
|  |  | 3750 | 10 | 1 (RBSTART=25) | 3750 |  |  |  |
| CA_n24-n77 | n24 | N/A | N/A | N/A | N/A | N/A | FDD | IMD410 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
| CA_n25-n41 | n25 | N/A | 5 | N/A | 1992.5 | 8.5 | FDD | IMD7 |
|  | n41 | 2545 | 90 | 1 (RBSTART=0) | 2545 | N/A | TDD | N/A |
|  |  | 2640 | 100 | 1 (RBSTART=221) | 2640 |  |  |  |
|  | n25 | 1860 | 5 | 25 | 1940 | 15.3 | FDD | IMD3 |
|  | n41 | 2501 | 10 | 1 (RBstart = 25) | 2501 | N/A | TDD | N/A |
|  |  | 2556 | 100 | 1 (RBstart = 208) | 2556 |  |  |  |
| CA_n25-n48 | n25 | 1852.5 | 5 | 25 | 1932.5 | 12 | FDD | IMD4 |
|  | n48 | 3625 | 20 | 100 | 3625 | N/A | TDD | N/A |
| CA_n25-n66 | n25 | 1855 | 5 | 25 | 1935 | 20 | FDD | IMD3 |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | FDD | N/A |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | FDD | N/A |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | 23 | FDD | IMD34 |
| CA_n25-n77 | n25 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD221 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n25 | 1885 | 5 | 25 | 1965 | 5 | FDD | IMD5 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n25 | N/A | 5 | N/A | 1987.5 | 2.7 | FDD | IMD7 |
|  | n7712 | 3455 | 10 | 1 (RBSTART=10) | 3455 | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 (RBSTART=0) | 3945 |  |  |  |
| CA_n25-n78 | n25 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD24 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n25 | N/A | 5 | N/A | 1980 | 2.7 | FDD | IMD7 |
|  | n7812 | 3315 | 10 | 1 (RBSTART=7) | 3315 | N/A | TDD | N/A |
|  |  | 3760 | 10 | 1 (RBSTART=0) | 3760 |  |  |  |
| CA_n26-n66 | n26 | 838 | 5 | 25 | 883 | 30 | FDD | IMD24 |
|  | n66 | 1721 | 5 | 25 | 2121 | N/A | FDD | N/A |
| CA_n26-n70 | n26 | 831 | 5 | 25 | 876 | 30 | FDD | IMD24 |
|  | n70 | 1707.5 | 5 | 25 | 2007.5 | N/A | FDD | N/A |
| CA_n26-n77 | n26 | N/A | N/A | N/A | N/A | N/A | FDD | IMD413 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n26 | N/A | N/A | N/A | N/A | N/A | FDD | IMD513 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
| CA_n26-n78 | n26 | 836.5 | 5 | 25 | 881.5 | 11.1 | FDD | IMD4 |
|  | n78 | 3391 | 10 | 50 | 3391 | N/A | TDD | N/A |
|  | n26 | 824 | 5 | 12 (RBSTART=0) | 869 | N/A | TDD | N/A |
|  |  | 839 | 5 | 12 (RBSTART=0) | 884 |  |  |  |
|  | n78 | N/A | 10 | N/A | 3336 | 11.1 | FDD | IMD4 |
| CA_n28-n50 | n28 | 730 | 10 | 50 | 775 | 15.3 | FDD | IMD24,21 |
|  | n50 | 1500 | 10 | 50 | 1500 | N/A | TDD | N/A |
| CA_n28-n74 | n28 | 705.5 | 5 | 25 | 760.5 | 24.6 | FDD | IMD24,21 |
|  | n74 | 1466 | 5 | 25 | 1514 | N/A | FDD | N/A |
|  | n28 | 709 | 5 | 25 | 764 | N/A | FDD | N/A |
|  | n74 | 1466 | 5 | 25 | 1514 | 14.6 | FDD | IMD4 |
|  | n28 | 735.5 | 5 | 25 | 790.5 | N/A | FDD | N/A |
|  | n74 | 1450.4 | 5 | 25 | 1498.4 | 2.5 | FDD | IMD5 |
| CA_n28-n77 | n28 | N/A | N/A | N/A | N/A | N/A | FDD | IMD27 |
|  | n7712 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n28 | 705.5 | 5 | 25 | 760.5 | 5.5 | FDD | IMD5 |
|  | n77 | 3582.5 | 10 | 50 | 3582.5 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 780 | 8.5 | FDD | IMD415 |
|  | n7712 | 3510 | 10 | 1 (RBSTART=25) | 3510 | N/A | TDD | N/A |
|  |  | 3900 | 10 | 1 (RBSTART=25) | 3900 |  |  |  |
|  | n28 | N/A | 5 | N/A | 705.5 | 8.6 | FDD | IMD4 |
|  | n7712 | 3455 | 10 | 1 (RBSTART=17) | 3455 | N/A | TDD | N/A |
|  |  | 3805 | 10 | 1 (RBSTART=0) | 3805 |  |  |  |
| CA_n28-n78 | n28 | 705.5 | 5 | 25 | 760.5 | 5.5 | FDD | IMD5 |
|  | n78 | 3582.5 | 10 | 50 | 3582.5 | N/A | TDD | N/A |
| CA_n29-n77 | n29 | N/A | 5 | N/A | 720 | 8.6 | SDL | IMD415 |
|  | n7712 | 3540 | 10 | 1 (RBSTART=25) | 3410 | N/A | TDD | N/A |
|  |  | 3900 | 10 | 1 (RBSTART=25) | 3850 |  |  |  |
| CA_n30-n77 | n30 | 2310 | 5 | 25 | 2355 | 8.0 | FDD | IMD4 |
|  | n77 | 3487.5 | 10 | 50 | 3487.5 | N/A | TDD | N/A |
|  | n30 | N/A | 5 | N/A | 2352.5 | 3.2 | FDD | IMD7 |
|  | n7712 | 3455 | 10 | 1 (RBSTART=17) | 3455 | N/A | TDD | N/A |
|  |  | 3825 | 10 | 1 (RBSTART=0) | 3825 |  |  |  |
| CA_n40-n41 | n40 | N/A | 5 | N/A | 2397.5 | 41.7 | TDD | IMD314,16,19 |
|  | n41 | 2565 | 100 | 1 (RBSTART=0) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 1 (RBSTART=52) | 2644.8 | N/A | TDD | N/A |
|  | n40 | N/A | 5 | N/A | 2397.5 | 35.5 | TDD | IMD314,16,19,20 |
|  | n41 | 2565 | 100 | 270 (RBSTART=3) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 162 (RBSTART=0) | 2644.8 | N/A | TDD | N/A |
| CA_n41-n66 | n41 | 2545 | 90 | 1 (RBSTART=0) | 2545 | N/A | TDD | N/A |
|  |  | 2640 | 100 | 1 (RBSTART=171) | 2640 |  |  |  |
|  | n66 | N/A | 5 | N/A | 2197.5 | 32.5 | FDD | IMD5 |
| CA_n41-n71 | n41 | 2614 | 10 | 50 | 2614 | N/A | TDD | N/A |
|  | n71 | 665 | 5 | 25 | 619 | 9.5 | FDD | IMD4 |
| CA_n41-n77 | n41 | 2545 | 60 | 1 (RBSTART=0) | 2545 | N/A | TDD | N/A |
|  |  | 2625 | 100 | 1 (RBSTART=272) | 2625 |  |  |  |
|  | n77 | N/A | 10 | N/A | 3305 | 2.7 | TDD | IMD9 |
|  | n41 | N/A | 10 | N/A | 2565 | 17 | TDD | IMD516 |
|  | n7712 | 3485 | 10 | 1 (RBSTART=25) | 3485 | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 (RBSTART=25) | 3945 |  |  |  |
| CA_n41-n78 | n41 | N/A | 10 | N/A | 2500 | 2.7 | TDD | IMD9 |
|  | n78 | 3350 | 100 | 1 (RBSTART=0) | N/A | N/A | TDD | N/A |
|  |  | 3450 |  | 1 (RBSTART=272) |  |  |  |  |
| CA_n41-n79 | n41 | 2565 | 100 | 1 (RBSTART=89) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 1 (RBSTART=93) | 2644.8 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4995 | 10.4 | TDD | IMD414,15 |
|  | n41 | 2565 | 100 | 270 (RBSTART=3) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 162 (RBSTART=0) | 2644.8 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4995 | 6 | TDD | IMD414,15,20 |
|  | n4112 | 2565 | 100 | 270 (RBSTART=3) | 2565 | N/A | TDD | N/A |
|  |  | 2655 | 40 | 100 (RBSTART=6) | 2655 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4995 | 5.3 | TDD | IMD414,15,20 |
| CA_n46-n77 | n46 | N/A | 20 | N/A | 5155 | N/A | TDD | IMD5 |
|  | n7712 | 3385 | 10 | 1 (RBSTART= 25) | 3385 | N/A | TDD | N/A |
|  |  | 3975 | 10 | 1 (RBSTART= 25) | 3975 |  |  |  |
|  | n46 | N/A | 20 | N/A | 5660 | N/A | TDD | IMD6 |
|  | n7712 | 3310 | 10 | 1 (RBSTART= 25) | 3310 | N/A | TDD | N/A |
|  |  | 3790 | 10 | 1 (RBSTART= 25) | 3790 |  |  |  |
|  | n46 | N/A | 20 | N/A | 5230 | N/A | TDD | IMD7 |
|  | n7712 | 3310 | 10 | 1 (RBSTART= 25) | 3310 | N/A | TDD | N/A |
|  |  | 3790 | 10 | 1 (RBSTART= 25) | 3790 |  |  |  |
| CA_n46-n78 | n46 | N/A | 20 | N/A | 5660 | N/A | TDD | IMD6 |
|  | n7812 | 3310 | 10 | 1 (RBSTART= 25) | 3310 | N/A | TDD | N/A |
|  |  | 3790 | 10 | 1 (RBSTART= 25) | 3790 |  |  |  |
|  | n46 | N/A | 20 | N/A | 5230 | N/A | TDD | IMD7 |
|  | n7812 | 3310 | 10 | 1 (RBSTART= 25) | 3310 | N/A | TDD | N/A |
|  |  | 3790 | 10 | 1 (RBSTART= 25) | 3790 |  |  |  |
| CA_n48-n66 | n48 | 3660 | 5 | 25 | 3660 | N/A | TDD | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 | 5.0 | FDD | IMD5 |
| CA_n48-n70 | n48 | 3695 | 10 | 50 | 3695 | N/A | TDD | N/A |
|  | n70 | 1697.5 | 25/15 | 25 | 1997.5 | 26 | FDD | IMD24 |
| CA_n66-n71 | n66 | 1750 | 5 | 25 | 2150 | 5 | FDD | IMD4 |
|  | n71 | 675 | 5 | 25 | 629 | N/A | FDD | N/A |
| CA_n66-n77 | n66 | 1775 | 5 | 25 | 2175 | 31 | FDD | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | TDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 5.0 | FDD | IMD5 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
|  | n66 | N/A | 5 | N/A | 2197.5 | 15 | FDD | IMD513 |
|  | n7712 | 3305 | 10 | 1 (RBSTART=0) | 3305 | N/A | TDD | N/A |
|  |  | 3855 | 10 | 1 (RBSTART=8) | 3855 |  |  |  |
|  | n66 | 1730 | 5 | 25 | 2130 | 1.7 | FDD | IMD7 |
|  | n7712 | 3455 | 10 | 1 (RBSTART=10) | 3455 | N/A | TDD | N/A |
|  |  | 3875 | 10 | 1 (RBSTART=0) | 3875 |  |  |  |
| CA_n66-n78 | n66 | 1730 | 5 | 25 | 2130 | 5.0 | FDD | IMD5 |
|  | n78 | 3660 | 10 | 50 | 3660 | N/A | TDD | N/A |
|  | n66 | N/A | 5 | 25 | 2150 | 1.7 | FDD | IMD7 |
|  | n7812 | 3350 | 10 | 1 (RBSTART=7) | 3350 | N/A | TDD | N/A |
|  |  | 3750 | 10 | 1 (RBSTART=0) | 3750 |  |  |  |
| CA_n66-n85 | n66 | 1770 | 5 | 25 | 2138 | 5 | FDD | IMD4 |
|  | n85 | 701 | 5 | 25 | 731 | N/A | FDD | N/A |
| CA_n67-n78 | n67 | N/A | 5 | N/A | 748 | 8.6 | SDL | IMD415 |
|  | n7812 | 3376 | 10 | 1 (RBSTART=25) | 3376 | N/A | TDD | N/A |
|  |  | 3750 | 10 | 1 (RBSTART=25) | 3750 |  |  |  |
| CA_n70-n71 | n70 | 1697.5 | 5 | 25 | 1997.5 | 5 | FDD | IMD4 |
|  | n71 | 695.5 | 5 | 25 | 649.5 | N/A | FDD | N/A |
| CA_n70-n77 | n70 | 1702.5 | 5 | 25 | 2002.5 | 31 | FDD | IMD2 |
|  | n77 | 3705 | 10 | 50 | 3705 | N/A | TDD | N/A |
|  | n70 | 1697.5 | 5 | 25 | 1997.5 | 5.0 | FDD | IMD5 |
|  | n77 | 3545 | 10 | 50 | 3545 | N/A | TDD | N/A |
| CA_n70-n78 | n70 | 1705 | 5 | 25 | 2005 | 31 | FDD | IMD2 |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
|  | n70 | 1705 | 5 | 25 | 2005 | 5.0 | FDD | IMD5 |
|  | n78 | 3560 | 10 | 50 | 3560 | N/A | TDD | N/A |
| CA_n71-n77 | n71 | 671 | 5 | 25 | 625 | 5.5 | FDD | IMD513 |
|  | n77 | 3309 | 10 | 50 | 3309 | N/A | TDD | N/A |
|  | n71 | N/A | 5 | N/A | 640 | 8.6 | FDD | IMD4 |
|  | n7712 | 3480 | 10 | 1 (RBSTART=25) | 3480 | N/A | TDD | N/A |
|  |  | 3800 | 10 | 1 (RBSTART=25) | 3800 |  |  |  |
| CA_n71-n78 | n71 | 681.5 | 5 | 25 | 635.5 | 5.5 | FDD | IMD5 |
|  | n78 | 3361.5 | 10 | 50 | 3361.5 | N/A | TDD | N/A |
| CA_n74-n77 | n74 | N/A | 5 | N/A | 1500 | 4.6 | FDD | IMD6 |
|  | n7712 | 3505 | 10 | 1(Rbstart=26) | 3505 | N/A | TDD | N/A |
|  |  | 4005 | 10 | 1(Rbstart=26) | 4005 |  |  |  |
|  | n74 | N/A | 5 | N/A | 1505 | 1.7 | FDD | IMD7 |
|  | n7712 | 3305 | 10 | 1(Rbstart=26) | 3305 | N/A | TDD | N/A |
|  |  | 3905 | 10 | 1(Rbstart=26) | 3905 |  |  |  |
| CA_n77-n85 | n77 | 3540 | 10 | 50 | 3540 | N/A | TDD | N/A |
|  | n85 | 702 | 5 | 20 | 732 | 5.5 | FDD | IMD5 |
| CA_n78-n102 | n7812 | 3320 | 10 | 1 (RBSTART=25) | 3320 | N/A | TDD | N/A |
|  |  | 3680 | 10 | 1 (RBSTART=25) | 3680 |  |  |  |
|  | n102 | 6280 | 20 | 100 | 6280 | N/A | TDD | IMD44 |
| CA_n78-n105 | n78 | 3361.5 | 10 | 50 | 3361.5 | N/A | TDD | N/A |
|  | n105 | 682.5 | 5 | 25 | 631.5 | 5.5 | FDD | IMD5 |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,f,c) as defined in clause 6.2A.4 NOTE 2: RBSTART = 0, 15 kHz SCS is assumed. NOTE 3: No requirements apply when there is at least one individual RE within the intermodulation generated by the dual uplink is within the downlink transmission bandwidth of the FDD band. The reference sensitivity should only be verified when this is not the case (the requirements specified in clause 7.3 apply). NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: Void. NOTE 6: Void. NOTE 7: In current release the maximum separation bandwidth class is 600MHz, therefore, no IMD MSD requirement apply for this CA configuration when two uplink sub blocks are assigned within CA_77(2A). NOTE8: For a UE which supports this band combination only when the Band n77 frequency range restriction of 3400 – 4100 MHz applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 9: For a UE which supports this band combination only when the Band n78 frequency range restriction of 3400 – 3800 MHz, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 10: There is no IMD4 product in band n24 downlink for n77 operating in 3450 – 3980 MHz and n24 uplink restricted to between 1627.5 – 1637.5 MHz and between 1646.5 – 1656.5 MHz. NOTE 11: Void. NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE 13: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 14: Applicable when n41 spectrum is restricted to 2515-2675MHz NOTE 15: This band is subject to IMD6 also which MSD is not specified NOTE 16: This band is subject to IMD7 also which MSD is not specified. NOTE 17: For a UE which supports this band combination only when the Band n78 frequency range restriction of 3400 – 3800 MHz or 3300 – 3600 MHz applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 18: This component carrier is affected by IMD due to CA_n5B for which the MSD is not specified. NOTE 19: This band is subject to IMD5, IMD9, IMD11 and IMD13 also which MSD are not specified. NOTE 20: This second test point is optional NOTE 21: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

Table 7.3A.5-1a: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n3 | n1 | 1950 | 5 | 25 | 2140 | 32 | FDD | IMD3 |
|  | n3 | 1760 | 5 | 25 | 1855 | N/A | FDD | N/A |
| CA_n1-n77 | n1 | 1950 | 5 | 25 | 2140 | 35.8 | FDD | IMD24,18 |
|  | n77 | 4090 | 10 | 50 | 4090 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2130 | 31 | FDD | IMD515 |
|  | n7712 | 3310 | 10 | 1 (RBSTART=25) | 3310 | N/A | TDD | N/A |
|  |  | 3900 | 10 | 1 (RBSTART=25) | 3900 | N/A | TDD | N/A |
| CA_n1-n78 | n1 | 1950 | 5 | 25 | 2140 | 17.8 | FDD | IMD4 |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 13.6 | FDD | IMD7 |
|  | n7812 | 3305 | 10 | 1 (RBSTART=0) | 3305 | N/A | TDD | N/A |
|  |  | 3675 | 10 | 1 (RBSTART=44) | 3675 |  |  |  |
| CA_n2-n77 | n2 | 1855 | 5 | 25 | 1935 | 32.10 | FDD | IMD218 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n2 | 1885 | 5 | 25 | 1965 | 20.0 | FDD | IMD5 |
|  | n77 | 3810 | 10 | 50 | 3810 | N/A | TDD | N/A |
| CA_n3-n7 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n7 | 2535 | 10 | 50 | 2655 | 21.8 | FDD | IMD4 |
| CA_n3-n20 | n3 | 1775 | 5 | 25 | 1870 | 14 | FDD | IMD4 |
|  | n20 | 840 | 5 | 25 | 799 | N/A | FDD | N/A |
|  | n3 | 1735 | 5 | 25 | 1830 | N/A | FDD | N/A |
|  | n20 | 847 | 5 | 25 | 806 | 20.4 | FDD | IMD4 |
| CA_n3-n41 | n3 | 1740 | 5 | 25 | 1835 | 18.4 | FDD | IMD4 |
|  | n41 | 2657.5 | 10 | 50 | 2657.5 | N/A | TDD | N/A |
|  | n3 | 1747.5 | 5 | 25 (RBSTART= 0) | 1842.5 | 23.3 | FDD | IMD3 |
|  | n41 | 2560 | 60 | 1 (RBSTART= 30) | 2560 | N/A | TDD | N/A |
|  |  | 2620 | 60 | 1 (RBSTART= 127) | 2620 |  |  |  |
|  | n3 | N/A | 5 | N/A | 1877.5 | N/A17 | FDD | IMD9 |
|  | n41 | 2545 | 60 | 1 (RBSTART= 0) | 2545 | N/A | TDD | N/A |
|  |  | 2625 | 100 | 1 (RBSTART= 272) | 2625 |  |  |  |
| CA_n3-n77 | n3 | 1740 | 5 | 25 | 1835 | 31.9 | FDD | IMD24,18 |
|  | n77 | 3575 | 10 | 50 | 3575 | N/A | TDD | N/A |
|  | n3 | N/A | N/A | N/A | N/A | N/A6 | FDD | IMD5 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1877.5 | 13.6 | FDD | IMD7 |
|  | n7712 | 3427.5 | 10 | 1 (RBstart=10) | 3427.5 | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 (RBstart=0) | 3945 | N/A | TDD | N/A |
| CA_n3-n78 | n3 | 1740 | 5 | 25 | 1835 | 31.9 | FDD | IMD24,18 |
|  | n78 | 3435 | 10 | 50 | 3435 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1877.5 | 13.6 | FDD | IMD7 |
|  | n7812 | 3305 | 10 | 1 (RBSTART=3) | 3305 | N/A | TDD | N/A |
|  |  | 3780 | 10 | 1 (RBSTART=0) | 3780 |  |  |  |
| CA_n5-n77 | n5 | 844 | 5 | 25 | 889 | 18.6 | FDD | IMD44,13 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |
|  | n5 | N/A | 5 | N/A | 880 | 18.5 | FDD | IMD4 |
|  | n7712 | 3410 | 10 | 1 (RBSTART=25) | 3410 | N/A | TDD | N/A |
|  |  | 3850 | 10 | 1 (RBSTART=25) | 3850 | N/A | TDD | N/A |
| CA_n5-n78 | n5 | 844 | 5 | 25 | 889 | 18.6 | FDD | IMD4 |
|  | n78 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |
| CA_n5-n784 | n5 | N/A | 5 | N/A | 880 | 18.5 | FDD | IMD4 |
|  | n7812 | 3340 | 10 | 1 (RBSTART=25) | 3340 | N/A | TDD | N/A |
|  |  | 3780 | 10 | 1 (RBSTART=25) | 3780 |  |  |  |
| CA_n7-n20 | n7 | 2512 | 5 | 50 | 2632 | N/A | FDD | N/A |
|  | n20 | 851 | 5 | 25 | 810 | 20.7 | FDD | IMD3 |
| CA_n7-n77 | n7 | 2540 | 5 | 25 | 2660 | 15.8 | FDD | IMD4 |
|  | n77 | 3870 | 10 | 50 | 3870 | N/A | TDD | N/A |
|  | n7 | N/A | 5 | N/A | 2687.5 | 29.9 | FDD | IMD515 |
|  | n7712 | 3455 | 10 | 1 (RBSTART=0) | 3455 | N/A | TDD | N/A |
|  |  | 3835 | 10 | 1 (RBSTART=7) | 3835 | N/A | TDD | N/A |
| CA_n7-n78 | n7 | N/A | 5 | N/A | 2650 | 29.9 | FDD | IMD515 |
|  | n7812 | 3350 | 10 | 1 (RBSTART=25) | 3350 | N/A | TDD | N/A |
|  |  | 3700 | 10 | 1 (RBSTART=25) | 3700 | N/A | TDD | N/A |
| CA_n8-n41 | n8 | 882.5 | 5 | 25 | 927.5 | 21.1 | FDD | IMD34 |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
| CA_n8-n77 | n8 | 897.5 | 5 | 25 | 942.5 | 15.5 | FDD | IMD4 |
|  | n77 | 3635 | 10 | 50 | 3635 | N/A | TDD | N/A |
| CA_n8-n78 | n8 | 897.5 | 5 | 25 | 942.5 | 15.5 | FDD | IMD4 |
|  | n78 | 3635 | 10 | 50 | 3635 | N/A | TDD | N/A |
| CA_n8-n79 | n8 | 897.5 | 5 | 25 | 942.5 | 21.5 | FDD | IMD5 |
|  | n79 | 4532.5 | 40 | 216 | 4532.5 | N/A | TDD | N/A |
| CA_n12-n77 | n12 | 702 | 5 | 20 | 732 | 11.7 | FDD | IMD5 |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | TDD | N/A |
| CA_n13-n77 | n13 | N/A | 5 | N/A | 750 | 8.6 | FDD | IMD415 |
|  | n7712 | 3510 | 10 | 1 (RBSTART=25) | 3510 | N/A | TDD | N/A |
|  |  | 3885 | 10 | 1 (RBSTART=25) | 3885 |  |  |  |
|  | n13 | 781 | 5 | 25 | 750 | 18.5 | FDD | IMD414 |
|  | n7712 | 3510 | 10 | 1 (RBSTART=25) | 3510 | N/A | TDD | N/A |
|  |  | 3885 | 10 | 1 (RBSTART=25) | 3885 | N/A | TDD | N/A |
|  | n13 | 782 | 5 | 20 | 751 | 20.5 | FDD | IMD5 |
|  | n77 | 3880 | 10 | 50 | 3880 | N/A | TDD | N/A |
| CA_n14-n77 | n14 | 795.5 | 5 | 15 | 765.5 | 11.7 | FDD | IMD5 |
|  | n77 | 3947.5 | 10 | 50 | 3947.5 | N/A | TDD | N/A |
| CA_n18-n41 | n18 | 820 | 5 | 25 | 865 | 23.1 | FDD | IMD3 |
|  | n41 | 2505 | 10 | 50 | 2505 | N/A | TDD | N/A |
| CA_n18-n77 | n18 | 827.5 | 5 | 25 | 872.5 | 17.5 | FDD | IMD48 |
|  | n77 | 3355 | 10 | 50 | 3355 | N/A | TDD | N/A |
|  | n18 | 817.5 | 5 | 25 | 862.5 | 10.5 | FDD | IMD58 |
|  | n77 | 4130 | 10 | 50 | 4130 | N/A | TDD | N/A |
|  | n18 | N/A | N/A | N/A | N/A | N/A | FDD | IMD27 |
|  | n7712 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n18 | N/A | 5 | N/A | 870 | 8.6 | FDD | IMD4 |
|  | n7712 | 3480 3915 | 10 10 | 1 (RBSTART=25) 1 (RBSTART=25) | 3480 3800 | N/A | TDD | N/A |
| CA_n25-n41 | n25 | N/A | 5 | N/A | 1992.5 | 8.5 | FDD | IMD7 |
|  | n41 | 2545 | 90 | 1 (RBstart=0) | 2545 | N/A | TDD | N/A |
|  |  | 2640 | 100 | 1 (RBstart=221) | 2640 |  |  |  |
|  | n25 | 1860 | 5 | 25 | 1940 | 23.3 | FDD | IMD3 |
|  | n41 | 2501 | 10 | 1 (RBstart = 25) | 2501 | N/A | TDD | N/A |
|  |  | 2556 | 100 | 1 (RBstart = 208) | 2556 |  |  |  |
| CA_n25-n66 | n66 | 1775 | 5 | 25 | 2175 | N/A | FDD | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 | 29 | FDD | IMD3 |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | 32 | FDD | IMD34 |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | FDD | N/A |
| CA_n25-n77 | n25 | 1855 | 5 | 25 | 1935 | 32.1 | FDD | IMD24,18 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n25 | N/A | 5 | N/A | 1987.5 | 13.6 | FDD | IMD7 |
|  | n7712 | 3455 | 10 | 1 (RBSTART=10) | 3455 | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 (RBSTART=0) | 3945 | N/A | TDD | N/A |
| CA_n25-n78 | n25 | 1855 | 5 | 25 | 1935 | 32.10 | FDD | IMD24 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n25 | N/A | 5 | N/A | 1980 | 13.6 | FDD | IMD7 |
|  | n7812 | 3315 | 10 | 1 (RBSTART=7) | 3315 | N/A | TDD | N/A |
|  |  | 3760 | 10 | 1 (RBSTART=0) | 3760 |  |  |  |
| CA_n26-n78 | n26 | 836.5 | 5 | 25 | 881.5 | 23.8 | FDD | IMD4 |
|  | n78 | 3391 | 10 | 50 | 3391 | N/A | TDD | N/A |
| CA_n28-n77 | n28 | N/A | N/A | N/A | N/A | N/A | FDD | IMD27 |
|  | n7712 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n28 | 705.5 | 5 | 25 | 760.5 | 19.2 | FDD | IMD5 |
|  | n77 | 3582.5 | 10 | 50 | 3582.5 | N/A | TDD | N/A |
|  | n28 | 725 | 5 | 25 | 780 | 18.5 | FDD | IMD414 |
|  | n7712 | 3510 | 10 | 1 (RBSTART=25) | 3510 | N/A | TDD | N/A |
|  |  | 3900 | 10 | 1 (RBSTART=25) | 3900 | N/A | TDD | N/A |
| CA_n28-n78 | n28 | N/A | 5 | N/A | 780 | 18.5 | FDD | IMD414 |
|  | n7812 | 3310 | 10 | 1 (RBSTART=7) | 3310 | N/A | TDD | N/A |
|  |  | 3700 | 10 | 1 (RBSTART=0) | 3700 |  |  |  |
|  | n28 | 705.5 | 5 | 25 | 760.5 | 19.2 | FDD | IMD5 |
|  | n78 | 3582.5 | 10 | 50 | 3582.5 | N/A | TDD | N/A |
| CA_n30-n77 | n30 | 2310 | 5 | 25 | 2355 | 17.6 | FDD | IMD4 |
|  | n77 | 3487.5 | 10 | 50 | 3487.5 | N/A | TDD | N/A |
| CA_n41-n66 | n41 | 2545 | 90 | 1 (RBstart=0) | 2545 | N/A | TDD | N/A |
|  |  | 2640 | 100 | 1 (RBstart=171) | 2640 |  |  |  |
|  | n66 | N/A | 5 | N/A | 2197.5 | 32.5 | FDD | IMD5 |
| CA_n41-n71 | n41 | 2614 | 10 | 50 | 2614 | N/A | TDD | N/A |
|  | n71 | 665 | 5 | 25 | 619 | 21.5 | FDD | IMD4 |
| CA_n41-n77 | n41 | 2545 | 60 | 1 (RBstart=0) | 2545 | N/A | TDD | N/A |
|  |  | 2625 | 100 | 1 (RBstart=272) | 2625 |  |  |  |
|  | n77 | N/A | 10 | N/A | 3305 | 2.7 | TDD | IMD9 |
|  | n41 | N/A | 10 | N/A | 2565 | 32 | TDD | IMD516 |
|  | n7712 | 3485 | 10 | 1 (RBSTART=25) | 3485 | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 (RBSTART=25) | 3945 |  |  |  |
| CA_n66-n71 | n66 | 1750 | 5 | 25 | 2150 | 15.5 | FDD | IMD4 |
|  | n71 | 675 | 5 | 25 | 629 | N/A | FDD | N/A |
| CA_n66-n77 | n66 | 1775 | 5 | 25 | 2175 | 34.33 | FDD | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | TDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 11.27 | FDD | IMD5 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
|  | n66 | N/A | 5 | N/A | 2197.5 | 31 | FDD | IMD515 |
|  | n7712 | 3305 | 10 | 1 (RBstart=0) | 3305 | N/A | TDD | N/A |
|  |  | 3855 | 10 | 1 (RBstart=8) | 3855 | N/A | TDD | N/A |
| CA_n66-n78 | n66 | 1760 | 5 | 25 | 2160 | 11.27 | FDD | IMD5 |
|  | n78 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
|  | n66 | N/A | 5 | 25 | 2150 | 13.6 | FDD | IMD7 |
|  | n7812 | 3350 | 10 | 1 (RBSTART=7) | 3350 | N/A | TDD | N/A |
|  |  | 3750 | 10 | 1 (RBSTART=0) | 3750 |  |  |  |
| CA_n66-n85 | n66 | 1770 | 5 | 25 | 2138 | 15.5 | FDD | IMD4 |
|  | n85 | 701 | 5 | 25 | 731 | N/A | FDD | N/A |
| CA_n70-n77 | n70 | 1702.5 | 5 | 25 | 2002.5 | 37 | FDD | IMD2 |
|  | n77 | 3705 | 10 | 50 | 3705 | N/A | TDD | N/A |
|  | n70 | 1697.5 | 5 | 25 | 1997.5 | 18.4 | FDD | IMD5 |
|  | n77 | 3545 | 10 | 50 | 3545 | N/A | TDD | N/A |
| CA_n71-n77 | n71 | 681.5 | 5 | 25 | 635.5 | 11.4 | FDD | IMD513 |
|  | n77 | 3361.5 | 10 | 50 | 3361.5 | N/A | TDD | N/A |
|  | n71 | N/A | 5 | N/A | 640 | 18.5 | FDD | IMD414 |
|  | n7712 | 3480 | 10 | 1 (RBSTART=25) | 3480 | N/A | TDD | N/A |
|  |  | 3800 | 10 | 1 (RBSTART=25) | 3800 |  |  |  |
| CA_n71-n78 | n71 | 681.5 | 5 | 25 | 635.5 | 11.4 | FDD | IMD5 |
|  | n78 | 3361.5 | 10 | 50 | 3361.5 | N/A | TDD | N/A |
| CA_n74-n77 | n74 | N/A | 5 | N/A | 1500 | 4.6 | FDD | IMD6 |
|  | n7712 | 3505 4005 | 10 10 | 1(Rbstart=26) 1(Rbstart=26) | 3505 4005 | N/A | TDD | N/A |
|  | n74 | N/A | 5 | N/A | 1505 | 1.7 | FDD | IMD7 |
|  | n7712 | 3305 3905 | 10 10 | 1(Rbstart=26) 1(Rbstart=26) | 3305 3905 | N/A | TDD | N/A |
| CA_n77-n85 | n77 | 3540 | 10 | 50 | 3540 | N/A | TDD | N/A |
|  | n85 | 702 | 5 | 20 | 732 | 11.7 | FDD | IMD5 |
| NOTE 1: Both of the transmitters shall be set min(+23 dBm, PCMAX_L,f,c) as defined in clause 6.2A.4 NOTE 2: RBSTART = 0, 15 kHz SCS is assumed. NOTE 3: No requirements apply when there is at least one individual RE within the intermodulation generated by the dual uplink is within the downlink transmission bandwidth of the FDD band. The reference sensitivity should only be verified when this is not the case (the requirements specified in clause 7.3 apply). NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: Void. NOTE 6: Void. NOTE 7: In current release the maximum separation bandwidth class is 600MHz, therefore, no IMD2 MSD requirement apply for this CA configuration when two uplink sub blocks are assigned within CA_77(2A). NOTE8: For a UE which supports this band combination only when the Band n77 frequency range restriction of 3400 – 4100 MHz applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 9: Void. NOTE 10: Void. NOTE 11: Void. NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE 13: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 14: This band is subject to IMD6 also which MSD is not specified. NOTE 15: This band is subject to IMD7 also which MSD is not specified. NOTE 16: Void. NOTE 17: Applicable when n41 spectrum is restricted to 2515-2675MHz NOTE 18: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

Table 7.3A.5-1b: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC1.5 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n78 | n1 | 1950 | 5 | 25 | 2140 | 26.4 | FDD | IMD4 |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
| CA_n2-n77 | n2 | 1855 | 5 | 25 | 1935 | 35.2 | FDD | IMD24 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n2 | 1885 | 5 | 25 | 1965 | 28.0 | FDD | IMD5 |
|  | n77 | 3810 | 10 | 50 | 3810 | N/A | TDD | N/A |
| CA_n3-n78 | n3 | 1750 | 5 | 25 | 1845 | 35.2 | FDD | IMD24 |
|  | n78 | 3595 | 10 | 50 | 3595 | N/A | TDD | N/A |
| CA_n5-n772 | n5 | 844 | 5 | 25 | 889 | 25.5 | FDD | IMD4 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |
|  | n5 | 826.5 | 5 | 25 | 871.5 | 24.3 | FDD | IMD5 |
|  | n77 | 4177.5 | 10 | 50 | 4177.5 | N/A | TDD | N/A |
| CA_n7-n77 | n7 | 2540 | 5 | 25 | 2660 | 29.7 | FDD | IMD4 |
|  | n77 | 3870 | 10 | 50 | 3870 | N/A | TDD | N/A |
| CA_n7-n78 | n7 | 2520 | 5 | 25 | 2640 | 29.7 | FDD | IMD4 |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | TDD | N/A |
| CA_n25-n41 | n25 | N/A | 5 | N/A | 1992.5 | 8.5 | FDD | IMD7 |
|  | n41 | 2545 | 90 | 1 (RBstart=0) | 2545 | N/A | TDD | N/A |
|  |  | 2640 | 100 | 1 (RBstart=221) | 2640 |  |  |  |
|  | n25 | 1860 | 5 | 25 | 1940 | 30.3 | FDD | IMD3 |
|  | n41 | 2501 | 10 | 1 (RBstart = 25) | 2501 | N/A | TDD | N/A |
|  |  | 2556 | 100 | 1 (RBstart = 208) | 2556 |  |  |  |
| CA_n25-n77 | n25 | 1855 | 5 | 25 | 1935 | 37.5 | FDD | IMD24 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n25 | 1885 | 5 | 25 | 1965 | 29.8 | FDD | IMD5 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
| CA_n26-n78 | n26 | 836.5 | 5 | 25 | 881.5 | 25.5 | FDD | IMD4 |
|  | n78 | 3391 | 10 | 50 | 3391 | N/A | TDD | N/A |
| CA_n28-n78 | n28 | 705.5 | 5 | 25 | 760.5 | 24.3 | FDD | IMD5 |
|  | n78 | 3582.5 | 10 | 50 | 3582.5 | N/A | TDD | N/A |
| CA_n41-n66 | n41 | 2545 | 90 | 1 (RBstart=0) | 2545 | N/A | TDD | N/A |
|  |  | 2640 | 100 | 1 (RBstart=171) | 2640 |  |  |  |
|  | n66 | N/A | 5 | N/A | 2197.5 | 32.5 | FDD | IMD5 |
| CA_n41-n71 | n41 | 2614 | 10 | 50 | 2614 | N/A | TDD | N/A |
|  | n71 | 665 | 5 | 25 | 619 | 33.5 | FDD | IMD4 |
| CA_n41-n77 | n41 | 2545 | 60 | 1 (RBstart=0) | 2545 | N/A | TDD | N/A |
|  |  | 2625 | 100 | 1 (RBstart=272) | 2625 |  |  |  |
|  | n77 | N/A | 10 | N/A | 3305 | 2.7 | TDD | IMD9 |
| CA_n66-n77 | n66 | 1775 | 5 | 25 | 2175 | 40.0 | FDD | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | TDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 26.0 | FDD | IMD5 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
| CA_n70-n77 | n70 | 1702.5 | 5 | 25 | 2002.5 | 41.8 | FDD | IMD2 |
|  | n77 | 3705 | 10 | 50 | 3705 | N/A | TDD | N/A |
|  | n70 | 1697.5 | 5 | 25 | 1997.5 | 27.9 | FDD | IMD5 |
|  | n77 | 3545 | 10 | 50 | 3545 | N/A | TDD | N/A |
| CA_n71-n77 | n71 | 681.5 | 5 | 25 | 635.5 | 16.0 | FDD | IMD53 |
|  | n77 | 3361.5 | 10 | 50 | 3361.5 | N/A | TDD | N/A |
| CA_n77-n85 | n77 | 3540 | 10 | 50 | 3540 | N/A | TDD | N/A |
|  | n85 | 702 | 5 | 20 | 732 | 16.2 | FDD | IMD5 |
| NOTE 1: This band combination is specified for inter-band UL CA with UL MIMO or Tx diversity capabilites, and the transmitter shall be set at min (+23 dBm, PCMAX_L,f,c) for the band with 1Tx antenna connector as defined in clause 6.2A.4, and set at min (+27.8 dBm, PCMAX_L,f,c) for the band with 2Tx antenna connectors as defined in clause 6.2H.3 or 6.2L.3.4. NOTE 2: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 3: In current release the maximum separation bandwidth class is 600MHz, therefore, no IMD2 MSD requirement apply for this CA configuration when two uplink sub blocks are assigned within CA_77(2A). NOTE 4: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

Table 7.3A.5-2: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n3-n28 | n1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1818.5 | 4.0 | FDD | IMD5 |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |
|  | n1 | N/A | 5 | N/A | 2139 | 11.0 | FDD | IMD4 |
| CA_n1-n3-n40 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1735 | 5 | 25 | 1830 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2380 | 8.0 | TDD | IMD5 |
| CA_n1-n3-n41 | n1 | 1977.5 | 5 | 25 | 2167.5 | N/A | FDD | N/A |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2507.5 | 5.0 | TDD | IMD5 |
| CA_n1-n3-n71 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1855 | 4 | FDD | IMD5 |
|  | n71 | 695 | 5 | 25 | 649 | N/A | FDD | N/A |
|  | n1 | N/A | 5 | N/A | 2160 | 5 | FDD | IMD4 |
|  | n3 | 1775 | 5 | 25 | 1870 | N/A | FDD | N/A |
|  | n71 | 695 | 5 | 25 | 649 | N/A | FDD | N/A |
| CA_n1-n3-n75 | n1 | 1960 | 5 | 25 | 2150 | N/A | FDD | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n75 | N/A | 5 | N/A | 1480 | 24.2 | SDL | IMD34 |
| CA_n1-n3-n77 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3700 | 28.4 | TDD | IMD22 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1807.5 | 31.5 | FDD | IMD21,2 |
|  | n77 | 3757.5 | 10 | 50 | 3757.5 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 31.0 | FDD | IMD21 |
|  | n3 | 1775 | 5 | 25 | 1870 | N/A | FDD | N/A |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | TDD | N/A |
| CA_n1-n3-n78 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A |  | N/A |
|  | n78 | N/A | 10 | N/A | 3700 | 28.4 | TDD | IMD22 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1830 | 27.9 |  | IMD2 |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | TDD | N/A |
|  | n1 | 1925 | 5 | 25 | 2115 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | 25 | 1875 | N/A | 15.3 | IMD3 |
|  | n78 | 3300 | 100 | 1 (RBstart=203) | 3300 | N/A | TDD | N/A |
|  |  | 3400 | 100 | 1 (RBstart=67) | 3400 |  |  |  |
| CA_n1-n3-n79 | n1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n79 | N/A | 40 | N/A | 4950 | 4.7 | TDD | IMD5 |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |
|  | n79 | 4860 | 40 | 216 | 4860 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 3.6 | FDD | IMD5 |
| CA_n1-n3-n105 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1855 | 4 | FDD | IMD5 |
|  | n105 | 695 | 5 | 25 | 644 | N/A | FDD | N/A |
|  | n1 | N/A | 5 | N/A | 2160 | 5 | FDD | IMD4 |
|  | n3 | 1775 | 5 | 25 | 1870 | N/A | FDD | N/A |
|  | n105 | 695 | 5 | 25 | 644 | N/A | FDD | N/A |
| CA_n1-n5-n7 | n1 | 1968 | 5 | 25 | 2158 | N/A | FDD | N/A |
|  | n7 | 2512 | 10 | 50 | 2632 | N/A | FDD | N/A |
|  | n5 | N/A | 5 | N/A | 880 | 1.0 | FDD | IMD5 |
| CA_n1-n5-n28 | n1 | N/A | 5 | N/A | 2123 | 4 | FDD | IMD5 |
|  | n5 | 829 | 5 | 25 | 874 | N/A | FDD | N/A |
|  | n28 | 738 | 5 | 25 | 793 | N/A | FDD | N/A |
|  | n1 | 1965 | 5 | 25 | 2155 | N/A | FDD | N/A |
|  | n5 | N/A | 5 | N/A | 875 | 4.6 | FDD | IMD5 |
|  | n28 | 710 | 5 | 25 | 765 | N/A | FDD | N/A |
| CA_n1-n5-n40 | n1 | N/A | 5 | N/A | 2144 | 4.0 | FDD | IMD5 |
|  | n5 | 832 | 5 | 25 | 877 | N/A | FDD | N/A |
|  | n40 | 2320 | 5 | 25 | 2320 | N/A | TDD | N/A |
|  | n1 | 1945 | 5 | 25 | 2135 | N/A | FDD | N/A |
|  | n5 | N/A | 5 | N/A | 880 | 8.0 | FDD | IMD4 |
|  | n40 | 2385 | 5 | 25 | 2385 | N/A | TDD | N/A |
|  | n1 | 1977.5 | 5 | 25 | 2167.5 | N/A | FDD | N/A |
|  | n5 | 826.5 | 5 | 25 | 871.5 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2305 | 9.0 | TDD | IMD4 |
| CA_n1-n5-n78 | n1 | N/A | 5 | N/A | 2122 | 18.1 | FDD | IMD3 |
|  | n5 | 829 | 5 | 25 | 874 | N/A | FDD | N/A |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | TDD | N/A |
|  | n1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |
|  | n5 | N/A | 5 | N/A | 885 | 3.1 | FDD | IMD5 |
|  | n78 | 3405 | 10 | 50 | 3405 | N/A | TDD | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3610 | 15.7 | TDD | IMD3 |
| CA_n1-n5-n79 | n1 | N/A | 5 | N/A | 2160 | 1.2 | FDD | IMD4 |
|  | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n79 | 4650 | 40 | 100 | 4650 | N/A | TDD | N/A |
|  | n1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n5 | N/A | 5 | N/A | 890 | 15.2 | FDD | IMD3 |
|  | n79 | 4750 | 40 | 100 | 4750 | N/A | TDD | N/A |
|  | n1 | 1923 | 5 | 25 | 2113 | N/A | FDD | N/A |
|  | n5 | N/A | 5 | N/A | 879 | 10.3 | FDD | IMD4 |
|  | n79 | 4890 | 40 | 100 | 4890 | N/A | TDD | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n5 | 845 | 5 | 25 | 890 | N/A | FDD | N/A |
|  | n79 | N/A | 40 | N/A | 4785 | 14.9 | TDD | IMD3 |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n5 | 830 | 5 | 25 | 874 | N/A | FDD | N/A |
|  | n79 | N/A | 40 | N/A | 4430 | 9.4 | TDD | IMD4 |
| CA_n1-n5-n105 | n1 | 1925 | 5 | 25 | 2115 | N/A | FDD | N/A |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n105 | N/A | 5 | N/A | 614.5 | 9.4 | FDD | IMD4 |
|  | n1 | 1929 | 5 | 25 | 2119 | N/A | FDD | N/A |
|  | n5 | N/A | 5 | N/A | 873 | 4.6 | FDD | IMD5 |
|  | n105 | 700.5 | 5 | 25 | 649.5 | N/A | FDD | NA |
|  | n1 | N/A | 5 | N/A | 2162 | 16 | FDD | IMD3 |
|  | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n105 | 635 | 5 | 25 | 615 | N/A | FDD | NA |
| CA_n1-n7-n8 | n1 | 1977.5 | 5 | 25 | 2167.5 | N/A | FDD | N/A |
|  | n7 | 2502.5 | 5 | 25 | 2622.5 | N/A | FDD | N/A |
|  | n8 | N/A | 5 | N/A | 927.5 | 1.0 | FDD | IMD5 |
| CA_n1-n7-n20 | n1 | 1940 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n7 | 2510 | 10 | 50 | 2630 | N/A | FDD | N/A |
|  | n20 | N/A | 5 | N/A | 800 | 4.5 | FDD | IMD5 |
| CA_n1-n7-n26 | n1 | 1965 | 5 | 25 | 2155 | N/A | FDD | N/A |
|  | n7 | 2510 | 10 | 50 | 2630 | N/A | FDD | N/A |
|  | n26 | N/A | 5 | N/A | 875 | 3.5 | FDD | IMD5 |
| CA_n1-n7-n28 | n1 | 1935 | 5 | 25 | 2125 | N/A | FDD | N/A |
|  | n7 | N/A | 10 | N/A | 2653 | 30.0 | FDD | IMD2 |
|  | n28 | 718 | 5 | 25 | 773 | N/A | FDD | N/A |
|  | n1 | 1935 | 5 | 25 | 2125 | N/A | FDD | N/A |
|  | n7 | 2510 | 10 | 50 | 2630 | N/A | FDD | N/A |
|  | n28 | N/A | 10 | N/A | 785 | 4.5 | FDD | IMD5 |
| CA_n1-n7-n40 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n7 | N/A | 5 | N/A | 2630 | 23 | FDD | IMD3 |
|  | n40 | 2390 | 5 | 25 | 2390 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2120 | 16.4 | FDD | IMD3 |
|  | n7 | 2530 | 5 | 25 | 2650 | N/A | FDD | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | TDD | N/A |
| CA_n1-n7-n67 | n1 | 1948 | 5 | 25 | 2138 | N/A | FDD | N/A |
|  | n7 | 2548 | 5 | 25 | 2668 | N/A | FDD | N/A |
|  | n67 | N/A | 5 | N/A | 748 | 3.3 | SDL | IMD5 |
| CA_n1-n7-n75 | n1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |
|  | n7 | 2510 | 5 | 25 | 2630 | N/A | FDD | N/A |
|  | n75 | N/A | 5 | N/A | 1450 | 13.2 | SDL | IMD3 |
| CA_n1-n7-n78 | n1 | 1977.5 | 5 | 25 | 2167.5 | N/A | FDD | N/A |
|  | n7 | N/A | 5 | N/A | 2627.5 | 9.1 | FDD | IMD4 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 8.7 | FDD | IMD4 |
|  | n7 | 2510 | 10 | 50 | 2630 | N/A | FDD | N/A |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | TDD | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n7 | 2520 | 5 | 25 | 2640 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3390 | 10.1 | TDD | IMD4 |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3770 | 1.3 | TDD | IMD5 |
| CA_n1-n7-n105 | n1 | 1935 | 5 | 25 | 2125 | N/A | FDD | N/A |
|  | n7 | 2565 | 10 | 50 | 2685 | N/A | FDD | N/A |
|  | n105 | N/A | 5 | N/A | 630 | 28.7 | FDD | IMD2 |
|  | n1 | 1925 | 5 | 25 | 2115 | N/A | FDD | N/A |
|  | n7 | 2565 | 10 | 50 | 2565 | N/A | FDD | N/A |
|  | n105 | N/A | 5 | N/A | 645 | 1 | FDD | IMD5 |
|  | n1 | 1968.5 | 5 | 25 | 2158.5 | N/A | FDD | N/A |
|  | n7 | N/A | 5 | N/A | 2634.5 | 30 | FDD | IMD22 |
|  | n105 | 666 | 5 | 25 | 615 | N/A | FDD | N/A |
| CA_n1-n8-n40 | n1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n8 | N/A | 5 | N/A | 930 | 8.0 | FDD | IMD4 |
|  | n40 | 2395 | 5 | 25 | 2395 | N/A | TDD | N/A |
| CA_n1-n8-n41 | n1 | 1977.5 | 5 | 25 | 2167.5 | N/A | FDD | N/A |
|  | n8 | N/A | 5 | N/A | 927.5 | 1.0 | FDD | IMD5 |
|  | n41 | 2502.5 | 10 | 50 | 2502.5 | N/A | TDD | N/A |
| CA_n1-n8-n78 | n1 | 1945 | 5 | 25 | 2135 | N/A | FDD | N/A |
|  | n8 | 900 | 5 | 25 | 945 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3745 | 14.9 | TDD | IMD3 |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n8 | N/A | 5 | N/A | 940 | 3.3 | FDD | IMD5 |
|  | n78 | 3380 | 10 | 50 | 3380 | N/A | TDD | N/A |
| CA_n1-n18-n28 | n1 | 1965 | 5 | 25 | 2155 | N/A | FDD | N/A |
|  | n28 | 708 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n18 | N/A | 5 | N/A | 867 | 4.6 | FDD | IMD5 |
|  | n18 | 825 | 5 | 25 | 870 | N/A | FDD | N/A |
|  | n28 | 738 | 5 | 25 | 793 | N/A | FDD | N/A |
|  | n1 | N/A | 5 | N/A | 2127 | 4 | FDD | IMD5 |
| CA_n1-n18-n41 | n1 | 1960 | 5 | 25 | 2150 | N/A | FDD | N/A |
|  | n41 | 2505 | 10 | 50 | 2505 | N/A | TDD | N/A |
|  | n18 | N/A | 5 | N/A | 870 | 3.3 | FDD | IMD5 |
| CA_n1-n18-n77 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n18 | 825 | 5 | 25 | 870 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3600 | 15.7 | TDD | IMD31 |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 3390 | 10 | 50 | 3390 | N/A | TDD | N/A |
|  | n18 | N/A | 5 | N/A | 870 | 3.5 | FDD | IMD5 |
|  | n1 | N/A | 5 | N/A | 2120 | 16.4 | FDD | IMD3 |
|  | n18 | 825 | 5 | 25 | 870 | N/A | FDD | N/A |
|  | n77 | 3770 | 10 | 50 | 3770 | N/A | TDD | N/A |
| CA_n1-n20-n41 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n20 | N/A | 5 | N/A | 810 | 3.3 | FDD | IMD5 |
|  | n41 | 2550 | 10 | 50 | 2550 | N/A | TDD | N/A |
| CA_n1-n20-n71 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n20 | 859 | 5 | 25 | 818 | N/A | FDD | N/A |
|  | n71 | N/A | 5 | N/A | 627 | 9.4 | FDD | IMD4 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n20 | N/A | 5 | N/A | 806 | 4.6 | FDD | IMD5 |
|  | n71 | 689 | 5 | 25 | 643 | N/A | FDD | N/A |
|  | n1 | N/A | 5 | N/A | 2167 | 16 | FDD | IMD3 |
|  | n20 | 835 | 5 | 25 | 794 | N/A | FDD | N/A |
|  | n71 | 666 | 5 | 25 | 620 | N/A | FDD | N/A |
| CA_n1-n20-n77 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3644 | 15.7 | TDD | IMD31 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n20 | N/A | 5 | N/A | 806 | 3.5 | FDD | IMD5 |
|  | n77 | 3328 | 10 | 50 | 3328 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 16.4 | FDD | IMD3 |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n77 | 3834 | 10 | 50 | 3834 | N/A | TDD | N/A |
| CA_n1-n20-n78 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3644 | 15.7 | TDD | IMD3 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n20 | N/A | 5 | N/A | 806 | 3.1 | FDD | IMD5 |
|  | n78 | 3328 | 10 | 50 | 3328 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2120 | 18.1 | FDD | IMD3 |
|  | n20 | 835 | 5 | 25 | 794 | N/A | FDD | N/A |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
| CA_n1-n26-n78 | n1 | N/A | 5 | N/A | 2122 | 18.1 | FDD | IMD3 |
|  | n26 | 829 | 5 | 25 | 874 | N/A | FDD | N/A |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | TDD | N/A |
|  | n1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |
|  | n26 | N/A | 5 | N/A | 885 | 3.1 | FDD | IMD5 |
|  | n78 | 3405 | 10 | 50 | 3405 | N/A | TDD | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n26 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3610 | 15.7 | TDD | IMD3 |
| CA_n1-n28-n40 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 780 | 8 | FDD | IMD4 |
|  | n40 | 2340 | 5 | 25 | 2340 | N/A | TDD | N/A |
|  | n1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n28 | 735 | 5 | 25 | 790 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2390 | 9.0 | TDD | IMD4 |
| CA_n1-n28-n41 | n1 | 1935 | 5 | 25 | 2125 | N/A | FDD | N/A |
|  | n28 | 718 | 5 | 25 | 773 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2653 | 30.1 | TDD | IMD22 |
|  | n1 | 1923 | 5 | 25 | 2113 | N/A | FDD | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 762 | 29.3 | FDD | IMD21 |
| CA_n1-n28-n46 | n1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |
|  | n28 | 710 | 5 | 25 | 765 | N/A | FDD | N/A |
|  | n46 | N/A | 20 | N/A | 5215 | N/A | TDD | IMD4 |
|  | n1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 765 | 10.5 | FDD | IMD4 |
|  | n46 | 5160 | 20 | 100 | 5160 | N/A | TDD | N/A |
| CA_n1-n28-n77 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n28 | 733 | 5 | 25 | 788 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3416 | 15.7 | TDD | IMD32 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 790 | 4.2 | FDD | IMD5 |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n77 | 3630 | 10 | 50 | 3630 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2150 | 15.7 | FDD | IMD3 |
| CA_n1-n28-n78 | n1 | N/A | 5 | N/A | 2150 | 15.7 | FDD | IMD3 |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n78 | 3630 | 10 | 50 | 3630 | N/A | TDD | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 794 | 4.2 | FDD | IMD5 |
|  | n78 | 3352 | 10 | 50 | 3352 | N/A | TDD | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n28 | 733 | 5 | 25 | 788 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3416 | 15.7 | TDD | IMD3 |
| CA_n1-n28-n79 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n28 | 730 | 5 | 25 | 785 | N/A | FDD | N/A |
|  | n79 | N/A | 40 | N/A | 4630 | 14.9 | TDD | IMD31 |
|  | n1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n79 | 4648 | 40 | 216 | 4648 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 788 | 15.2 | FDD | IMD32 |
|  | n28 | 745.5 | 5 | 25 | 800.5 | N/A | FDD | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2167.5 | 1.2 | FDD | IMD41 |
| CA_n1-n28-n102 | n1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n28 | 706 | 5 | 25 | 761 | N/A | FDD | N/A |
|  | n102 | N/A | 40 | N/A | 5978 | N/A12 | TDD | IMD5 |
| CA_n1-n40-n41 | n1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |
|  | n40 | 2305 | 5 | 25 | 2305 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2635 | 21 | TDD | IMD3 |
|  | n1 | N/A | 5 | N/A | 2130 | 23.7 | FDD | IMD31 |
|  | n40 | 2335 | 5 | 25 | 2335 | N/A | TDD | N/A |
|  | n41 | 2540 | 10 | 50 | 2540 | N/A | TDD | N/A |
| CA_n1-n40-n77 | n1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3480 | 9.8 | TDD | IMD41 |
|  | n1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2340 | 10.6 | TDD | IMD41 |
|  | n77 | 3450 | 10 | 50 | 3450 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 9.1 | FDD | IMD4 |
|  | n40 | 2380 | 5 | 25 | 2380 | N/A | TDD | N/A |
|  | n77 | 3450 | 10 | 50 | 3450 | N/A | TDD | N/A |
| CA_n1-n40-n78 | n1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | TDD | N/A |
|  | n78 | N/A | 10 | N/A | 3480 | 9.8 | TDD | IMD41 |
|  | n1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2340 | 10.6 | TDD | IMD4 |
|  | n78 | 3450 | 10 | 50 | 3450 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 9.1 | FDD | IMD4 |
|  | n40 | 2380 | 5 | 25 | 2380 | N/A | TDD | N/A |
|  | n78 | 3450 | 10 | 50 | 3450 | N/A | TDD | N/A |
| CA_n1-n40-n79 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4960 | 19 | TDD | IMD4 |
|  | n1 | N/A | 5 | N/A | 2150 | 29.4 | FDD | IMD22 |
|  | n40 | 2380 | 5 | 25 | 2380 | N/A | TDD | N/A |
|  | n79 | 4530 | 10 | 50 | 4530 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2150 | 0 | FDD | IMD5 |
|  | n40 | 2350 | 5 | 25 | 2350 | N/A | TDD | N/A |
|  | n79 | 4600 | 10 | 50 | 4600 | N/A | TDD | N/A |
| CA_n1-n40-n105 | n1 | 1977 | 5 | 25 | 2167 | N/A | FDD | N/A |
|  | n40 | 2305 | 10 | 50 | 2305 | N/A | TDD | N/A |
|  | n105 | N/A | 5 | N/A | 649 | 1dB | FDD | IMD4 |
| CA_n1-n41-n77 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n41 | 2650 | 10 | 50 | 2650 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3330 | 19.6 | TDD | IMD31,2 |
|  | n1 | 1975 | 5 | 10 | 2165 | N/A | FDD | N/A |
|  | n77 | 3410 | 10 | 50 | 3410 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2515 | 11.5 | TDD | IMD41 |
|  | n41 | 2640 | 10 | 50 | 2640 | N/A | TDD | N/A |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 9.3 | FDD | IMD4 |
| CA_n1-n41-n78 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n41 | 2650 | 10 | 50 | 2650 | N/A | TDD | N/A |
|  | n78 | N/A | 10 | N/A | 3330 | 19.6 | TDD | IMD31,2 |
|  | n1 | 1975 | 5 | 10 | 2165 | N/A | FDD | N/A |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2515 | 11.5 | TDD | IMD4 |
|  | n1 | N/A | 5 | N/A | 2140 | 9.3 | FDD | IMD4 |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
|  | n41 | 2640 | 10 | 50 | 2640 | N/A | TDD | N/A |
| CA_n1-n41-n79 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n41 | 2530 | 10 | 50 | 2530 | N/A | TDD | N/A |
|  | n79 | N/A | 40 | N/A | 4500 | 19.0 | TDD | IMD21 |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n79 | 4500 | 40 | 216 | 4500 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2530 | 29.4 | TDD | IMD21 |
|  | n41 | 2530 | 10 | 50 | 2530 | N/A | TDD | N/A |
|  | n79 | 4690 | 40 | 216 | 4690 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2160 | 29.9 | FDD | IMD21 |
| CA_n1-n46-n78 | n1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n46 | 5430 | 20 | 100 | 5430 | N/A | TDD | N/A |
|  | n78 | N/A | 10 | N/A | 3500 | 29 | TDD | IMD2 |
|  | n1 | N/A | 5 | N/A | 2130 | 30 | FDD | IMD2 |
|  | n46 | 5630 | 20 | 100 | 5630 | N/A | TDD | N/A |
|  | n78 | 3500 | 10 | 50 | 3500 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2120 | 15 | FDD | IMD3 |
|  | n46 | 5160 | 20 | 100 | 5160 | N/A | TDD | N/A |
|  | n78 | 3640 | 10 | 50 | 3640 | N/A | TDD | N/A |
|  | n1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n46 | N/A | 20 | N/A | 5430 | N/A | TDD | IMD2 |
|  | n78 | 3500 | 10 | 50 | 3500 | N/A | TDD | N/A |
|  | n1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n46 | N/A | 20 | N/A | 5250 | N/A | TDD | IMD3 |
|  | n78 | 3590 | 10 | 50 | 3590 | N/A | TDD | N/A |
| CA_n1-n67-n78 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n67 | N/A | 5 | N/A | 748 | 3.5 | SDL | IMD5 |
|  | n78 | 3329 | 10 | 50 | 3329 | N/A | TDD | N/A |
| CA_n1-n71-n77 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n71 | N/A | 5 | N/A | 635 | 15.2 | FDD | IMD3 |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n71 | 686 | 5 | 25 | 640 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3342 | 15.7 | TDD | IMD3 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n71 | 680 | 5 | 25 | 634 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3992 | 9.4 | TDD | IMD4 |
|  | n1 | N/A | 5 | N/A | 2160 | 15.7 | FDD | IMD3 |
|  | n71 | 686 | 5 | 25 | 640 | N/A | FDD | N/A |
|  | n77 | 3532 | 10 | 50 | 3532 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2141 | 10.1 | FDD | IMD4 |
|  | n71 | 680 | 5 | 25 | 634 | N/A | FDD | N/A |
|  | n77 | 4181 | 10 | 50 | 4181 | N/A | TDD | N/A |
| CA_n1-n71-n78 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n71 | N/A | 5 | N/A | 635 | 15.2 | FDD | IMD3 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n71 | 686 | 5 | 25 | 640 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3342 | 15.7 | TDD | IMD3 |
|  | n1 | N/A | 5 | N/A | 2160 | 15.7 | FDD | IMD3 |
|  | n71 | 686 | 5 | 25 | 640 | N/A | FDD | N/A |
|  | n78 | 3532 | 10 | 50 | 3532 | N/A | TDD | N/A |
| CA_n1-n75-n78 | n1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n75 | N/A | 5 | N/A | 1470 | 30.4 | SDL | IMD2 |
|  | n78 | 3400 | 10 | 52 | 3400 | N/A | TDD | N/A |
| CA_n1-n77-n79 | n1 | N/A | 5 | N/A | 2140 | 15.6 | FDD | IMD31,2 |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n79 | 4660 | 40 | 216 | 4660 | N/A | TDD | N/A |
| CA_n1-n78-n79 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | TDD | N/A |
|  | n79 | N/A | 40 | N/A | 4870 | 15.9 | TDD | IMD31,3 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3490 | 4.6 | TDD | IMD53 |
|  | n79 | 4670 | 40 | 216 | 4670 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 15.6 | FDD | IMD31,2 |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n79 | 4660 | 40 | 216 | 4660 | N/A | TDD | N/A |
| CA_n1-n78-n102 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n78 | 3320 | 5 | 25 | 3320 | N/A | TDD | N/A |
|  | n102 | N/A | 40 | N/A | 6020 | N/A12 | TDD | IMD5 |
|  | n1 | N/A | 5 | N/A | 2155 | 29.9 | FDD | IMD21 |
|  | n78 | 3790 | 5 | 25 | 3790 | N/A | TDD | N/A |
|  | n102 | 5945 | 40 | 216 | 5945 | N/A | TDD | N/A |
| CA_n1-n78-n105 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n105 | N/A | 5 | N/A | 635 | 15.2 | FDD | IMD3 |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3342 | 15.7 | TDD | IMD3 |
|  | n105 | 686 | 5 | 25 | 635 | N/A | FDD | N/A |
|  | n1 | N/A | 5 | N/A | 2160 | 15.7 | FDD | IMD3 |
|  | n78 | 3532 | 10 | 50 | 3532 | N/A | TDD | N/A |
|  | n105 | 686 | 5 | 25 | 635 | N/A | FDD | N/A |
| CA_n2-n5-n30 | n2 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
|  | n5 | N/A | 5 | N/A | 880 | 9.7 | FDD | IMD4 |
|  | n30 | 2310 | 10 | 50 | 2355 | N/A | FDD | N/A |
| CA_n2-n5-n41 | n2 | 1855 | 10 | 50 | 1935 | N/A | FDD | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2685 | 30.0 | TDD | IMD2 |
| CA_n2-n5-n48 | n2 | N/A | 5 | N/A | 1962 | 15.6 | FDD | IMD3 |
|  | n5 | 839 | 5 | 25 | 884 | N/A | FDD | N/A |
|  | n48 | 3640 | 10 | 50 | 3640 | N/A | TDD | N/A |
|  | n2 | 1905 | 5 | 25 | 1985 | N/A | FDD | N/A |
|  | n5 | 844 | 5 | 25 | 889 | N/A | FDD | N/A |
|  | n48 | N/A | 10 | 50 | 3593 | 16.6 | TDD | IMD3 |
| CA_n2-n5-n66 | n2 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2140 | 7.2 | FDD | IMD4 |
| CA_n2-n5-n77 | n2 | 1907.5 | 5 | 25 | 1987.5 | N/A | FDD | N/A |
|  | n5 | N/A | 5 | N/A | 887.5 | 3.8 | FDD | IMD55 |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n2 | N/A | 5 | N/A | 1987 | 16.5 | FDD | IMD35 |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n77 | 3680 | 10 | 50 | 3680 | N/A | TDD | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3540 | 16.0 | TDD | IMD31 |
| CA_n2-n12-n30 | n2 | 1885 | 5 | 25 | 1965 | N/A | FDD | N/A |
|  | n12 | 708.5 | 5 | 25 | 738.5 | N/A | FDD | N/A |
|  | n30 | N/A | 5 | N/A | 2353 | 12.0 | FDD | IMD4 |
| CA_n2-n12-n71 | n2 | 1907.5 | 5 | 25 | 1987.5 | N/A | FDD | N/A |
|  | n12 | N/A | 5 | N/A | 743.5 | 4.2 | FDD | IMD5 |
|  | n71 | 665.5 | 5 | 25 | 649.5 | N/A | FDD | N/A |
| CA_n2-n12-n77 | n2 | N/A | 5 | N/A | 1960 | 16.5 | FDD | IMD32,5 |
|  | n12 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
|  | n77 | 3375 | 10 | 50 | 3375 | N/A | TDD | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n12 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3315 | 16.0 | TDD | IMD31,2,5 |
| CA_n2-n14-n66 | n2 | 1874 | 5 | 25 | 1954 | N/A | FDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2162 | 7.6 | FDD | IMD4 |
|  | n2 | N/A | 5 | N/A | 1954 | 7.2 | FDD | IMD4 |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | FDD | N/A |
| CA_n2-n14-n77 | n2 | N/A | 5 | N/A | 1954 | 16.5 | FDD | IMD3 |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | TDD | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3466 | 16.0 | TDD | IMD31 |
| CA_n2-n30-n77 | n2 | N/A | 5 | N/A | 1986 | 8.6 | FDD | IMD45 |
|  | n30 | 2312 | 5 | 25 | 2357 | N/A | FDD | N/A |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n2 | 1905 | 5 | 25 | 1985 | N/A | FDD | N/A |
|  | n30 | N/A | 5 | N/A | 2354 | 10.6 | FDD | IMD45 |
|  | n77 | 3361 | 10 | 50 | 3361 | N/A | TDD | N/A |
|  | n2 | 1860 | 5 | 25 | 1940 | N/A | FDD | N/A |
|  | n30 | N/A | 5 | N/A | 2354 | 3.4 | FDD | IMD5 |
|  | n77 | 3967 | 10 | 50 | 3967 | N/A | TDD | N/A |
|  | n2 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 4180 | 29.4 | TDD | IMD22,5 |
| CA_n2-n48-n66 | n2 | 1855 | 5 | 25 | 1935 | N/A | FDD | N/A |
|  | n48 | N/A | 10 | 50 | 3625 | 32.0 | TDD | IMD2 |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | FDD | N/A |
|  | n2 | 1905 | 5 | 25 | 1985 | N/A | FDD | N/A |
|  | n48 | 3560 | 10 | 50 | 3560 | N/A | TDD | N/A |
|  | n66 | N/A | 5 | N/A | 2155 | 12.1 | FDD | IMD4 |
|  | n2 | N/A | 5 | N/A | 1960 | 28.3 | FDD | IMD21 |
|  | n48 | 3695 | 10 | 50 | 3695 | N/A | TDD | N/A |
|  | n66 | 1735 | 5 | 25 | 2135 | N/A | FDD | N/A |
| CA_n2-n66-n77 | n2 | N/A | 5 | 25 | 1950 | 15.3 | FDD | IMD3 |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | FDD | N/A |
|  | n77 | 3300 | 100 | 1 (RBstart=203) | 3300 | N/A | TDD | N/A |
|  |  | 3400 | 100 | 1 (RBstart=67) | 3400 |  |  |  |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3620 | 29.4 | TDD | IMD22,5 |
|  | n2 | 1855 | 5 | 25 | 1935 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2115 | 29.2 | FDD | IMD22 |
|  | n77 | 3970 | 10 | 50 | 3970 | N/A | TDD | N/A |
|  | n2 | 1885 | 5 | 25 | 1965 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2175 | 4.0 | FDD | IMD5 |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | TDD | N/A |
|  | n2 | N/A | 5 | N/A | 1960 | 32.1 | FDD | IMD2 |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
|  | n2 | N/A | 5 | N/A | 1960 | 9.1 | FDD | IMD45 |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | FDD | N/A |
|  | n77 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
|  | n2 | N/A | 5 | N/A | 1960 | 2.1 | FDD | IMD55 |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | N/A | TDD | N/A |
| CA_n3-n5-n7 | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n5 | 845 | 5 | 25 | 890 | N/A | FDD | N/A |
|  | n7 | N/A | 10 | N/A | 2625 | 30.0 | FDD | IMD24 |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n5 | N/A | 5 | N/A | 880 | 19.0 | FDD | IMD3 |
|  | n7 | 2560 | 10 | 50 | 2680 | N/A | FDD | N/A |
| CA_n3-n5-n28 | n3 | N/A | 5 | N/A | 1829.5 | 8.7 | FDD | IMD4 |
|  | n5 | 845 | 5 | 25 | 890 | N/A | FDD | N/A |
|  | n28 | 705.5 | 5 | 25 | 760.5 | N/A | FDD | N/A |
|  | n3 | 1713 | 5 | 25 | 1808 | N/A | FDD | N/A |
|  | n5 | 827 | 5 | 25 | 872 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 768 | 9.4 | FDD | IMD4 |
| CA_n3-n5-n78 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n5 | 839 | 5 | 25 | 884 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3408 | 16.1 | TDD | IMD31 |
|  | n3 | N/A | 5 | N/A | 1862 | 15.7 | FDD | IMD3 |
|  | n5 | 839 | 5 | 25 | 884 | N/A | FDD | N/A |
|  | n78 | 3540 | 10 | 50 | 3540 | N/A | TDD | N/A |
| CA_n3-n5-n79 | n3 | N/A | 5 | N/A | 1877.5 | 8.8 | FDD | IMD4 |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n5 | N/A | 5 | N/A | 860 | 15.3 | FDD | IMD3 |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n5 | N/A | 5 | N/A | 890 | 10.3 | FDD | IMD4 |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |
|  | n3 | 1782.5 | 5 | 25 | 1875.5 | N/A | FDD | N/A |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n79 | N/A | 40 | N/A | 4420 | 15.7 | TDD | IMD3 |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n5 | 846 | 5 | 25 | 891 | N/A | FDD | N/A |
|  | n79 | N/A | 40 | N/A | 4494 | 9.4 | TDD | IMD4 |
| CA_n3-n7-n8 | n3 | 1735 | 5 | 25 | 1830 | N/A | FDD | N/A |
|  | n7 | 2530 | 10 | 50 | 2650 | N/A | FDD | N/A |
|  | n8 | N/A | 5 | N/A | 940 | 18.0 | FDD | IMD3 |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n7 | N/A | 10 | N/A | 2670 | 29.0 | FDD | IMD211 |
|  | n8 | 890 | 5 | 25 | 935 | N/A | FDD | N/A |
| CA_n3-n7-n20 | n3 | 1747 | 5 | 25 | 1842 | N/A | FDD | N/A |
|  | n7 | 2543 | 10 | 50 | 2663 | N/A | FDD | N/A |
|  | n20 | N/A | 5 | N/A | 796 | 20.0 | FDD | IMD2 |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n7 | N/A | 10 | N/A | 2625 | 29.0 | FDD | IMD2 |
|  | n20 | 845 | 5 | 25 | 804 | N/A | FDD | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |
|  | n7 | N/A | 5 | 25 | N/A | 17.0 | FDD | IMD3 |
|  | n20 | 835 | 5 | 25 | 794 | N/A | FDD | N/A |
| CA_n3-n7-n26 | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n7 | 2560 | 10 | 50 | 2680 | N/A | FDD | N/A |
|  | n26 | N/A | 5 | N/A | 880 | 17.5 | FDD | IMD3 |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n7 | N/A | 10 | N/A | 2625 | 29.0 | FDD | IMD24 |
|  | n26 | 845 | 5 | 25 | 890 | N/A | FDD | N/A |
| CA_n3-n7-n28 | n3 | 1747 | 5 | 25 | 1842 | N/A | FDD | N/A |
|  | n7 | 2543 | 5 | 25 | 2663 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 796 | 20.0 | FDD | IMD2 |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | FDD | N/A |
|  | n7 | N/A | 5 | N/A | 2682 | 17.0 | FDD | IMD3 |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1832.5 | 26 | FDD | IMD2 |
|  | n7 | 2543 | 5 | 25 | 2663 | N/A | FDD | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |
| CA_n3-n7-n40 | n3 | 1770 | 10 | 50 | 1865 | 4.5 | FDD | IMD5 |
|  | n7 | 2525 | 10 | 50 | 2645 | N/A | FDD | N/A |
|  | n40 | 2305 | 10 | 50 | 2305 | N/A | TDD | N/A |
| CA_n3-n7-n67 | n3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n7 | 2520 | 5 | 25 | 2640 | N/A | FDD | N/A |
|  | n67 | N/A | 5 | N/A | 750 | 20 | SDL | IMD2 |
| CA_n3-n7-n75 | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n7 | 2505 | 5 | 25 | 2625 | N/A | FDD | N/A |
|  | n75 | N/A | 5 | N/A | 1450 | 10.4 | SDL | IMD4 |
| CA_n3-n7-n77 | n3 | N/A | 5 | N/A | 1820 | 17.6 | FDD | IMD3 |
|  | n7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n77 | 3310 | 10 | 50 | 3310 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1820 | 8.6 | FDD | IMD4 |
|  | n7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n77 | 3475 | 10 | 50 | 3475 | N/A | TDD | N/A |
|  | n3 | 1715 | 5 | 25 | 1810 | N/A | FDD | N/A |
|  | n7 | N/A | 5 | N/A | 2685 | 3.4 | FDD | IMD5 |
|  | n77 | 4175 | 10 | 50 | 4175 | N/A | TDD | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3390 | 16.1 | TDD | IMD31 |
| CA_n3-n7-n78 | n3 | N/A | 5 | N/A | 1820 | 17.6 | FDD | IMD3 |
|  | n7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1820 | 8.6 | FDD | IMD4 |
|  | n7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n78 | 3475 | 10 | 50 | 3475 | N/A | TDD | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3390 | 16.1 | TDD | IMD3 |
| CA_n3-n7-n105 | n3 | N/A | 5 | N/A | 1875 | 16.5 | FDD | IMD2 |
|  | n7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n105 | 675 | 5 | 25 | 624 | N/A | FDD | N/A |
| CA_n3-n8-n40 | n3 | N/A | 5 | N/A | 1874 | 4 | FDD | IMD5 |
|  | n8 | 912 | 5 | 25 | 957 | N/A | FDD | N/A |
|  | n40 | 2305 | 5 | 25 | 2305 | N/A | TDD | N/A |
| CA_n3-n8-n41 | n3 | 1722.5 | 5 | 25 | 1817.5 | N/A | FDD | N/A |
|  | n8 | 887.5 | 5 | 25 | 932.5 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2610 | 28.0 | FDD | IMD24 |
|  | n3 | 1725 | 5 | 25 | 1820 | N/A | FDD | N/A |
|  | n8 | N/A | 5 | N/A | 945 | 26.0 | FDD | IMD24 |
|  | n41 | 2516 | 10 | 50 | 2516 | N/A | FDD | N/A |
| CA_n3-n8-n78 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3550 | 16.1 | TDD | IMD31 |
|  | n3 | N/A | 5 | N/A | 1820 | 15.7 | FDD | IMD3 |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n78 | 3640 | 10 | 50 | 3640 | N/A | TDD | N/A |
| CA_n3-n8-n79 | n3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n8 | 885 | 5 | 25 | 930 | N/A | FDD | N/A |
|  | n79 | N/A | 40 | N/A | 4425 | 15.7 | TDD | IMD32 |
|  | n3 | 1755 | 5 | 25 | 1850 | N/A | FDD | N/A |
|  | n8 | N/A | 5 | N/A | 955 | 15.3 | FDD | IMD3 |
|  | n79 | 4465 | 40 | 216 | 4465 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1850 | 8.8 | FDD | IMD4 |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n79 | 4580 | 40 | 216 | 4580 | N/A | FDD | N/A |
| CA_n3-n18-n28 | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 770 | 9.4 | FDD | IMD4 |
|  | n18 | 827.5 | 5 | 25 | 872.5 | N/A | FDD | N/A |
| CA_n3-n18-n41 | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2540 | N/A | TDD | IMD2 |
|  | n3 | 1725 | 5 | 25 | 1820 | N/A | FDD | N/A |
|  | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2630 | 16.0 | TDD | IMD3 |
|  | n3 | 1765 | 5 | 25 | 1860 | N/A | FDD | N/A |
|  | n18 | N/A | 5 | N/A | 865 | 28.9 | FDD | IMD2 |
|  | n41 | 2630 | 10 | 50 | 2630 | N/A | TDD | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n18 | N/A | 5 | N/A | 862.5 | 17.5 | FDD | IMD3 |
|  | n41 | 2597.5 | 10 | 50 | 2597.5 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1850 | 28.8 | FDD | IMD2 |
|  | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n41 | 2670 | 10 | 50 | 2670 | N/A | TDD | N/A |
| CA_n3-n18-n77 | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3410 | 16.3 | TDD | IMD31,2 |
|  | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1865 | 15.7 | FDD | IMD3 |
|  | n77 | 3505 | 10 | 50 | 3505 | N/A | TDD | N/A |
| CA_n3-n20-n28 | n3 | N/A | 5 | N/A | 1828 | 9.4 | FDD | IMD4 |
|  | n20 | 852 | 5 | 25 | 811 | N/A | FDD | N/A |
|  | n28 | 728 | 5 | 25 | 783 | N/A | FDD | N/A |
|  | n3 | 1748 | 5 | 25 | 1843 | N/A | FDD | N/A |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 793 | 9.4 | FDD | IMD4 |
| CA_n3-n20-n41 | n3 | 1735 | 5 | 25 | 1830 | N/A | FDD | N/A |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2623 | 29 | TDD | IMD2 |
|  | n3 | 1747 | 5 | 25 | 1842 | N/A | FDD | N/A |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2594 | 16 | TDD | IMD3 |
|  | n3 | 1747 | 5 | 25 | 1842 | N/A | FDD | N/A |
|  | n20 | N/A | 5 | N/A | 806 | 28.9 | FDD | IMD2 |
|  | n41 | 2553 | 10 | 50 | 2553 | N/A | TDD | N/A |
|  | n3 | 1715 | 5 | 25 | 1810 | N/A | FDD | N/A |
|  | n20 | N/A | 5 | N/A | 806 | 19 | FDD | IMD3 |
|  | n41 | 2624 | 10 | 50 | 2624 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1831 | 28.9 | FDD | IMD2 |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n41 | 2678 | 10 | 50 | 2678 | N/A | TDD | N/A |
| CA_n3-n20-n67 | n3 | 1775 | 5 | 25 | 1870 | N/A | FDD | N/A |
|  | n20 | 840 | 5 | 25 | 799 | N/A | FDD | N/A |
|  | n67 | N/A | 5 | N/A | 745 | 9.4 | FDD | IMD4 |
| CA_n3-n20-n71 | n3 | N/A | 5 | N/A | 1843 | 9.4 | FDD | IMD4 |
|  | n20 | 841 | 5 | 25 | 800 | N/A | FDD | N/A |
|  | n71 | 680 | 5 | 25 | 634 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1841 | 5.9 | FDD | IMD5 |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n71 | 672 | 5 | 25 | 626 | N/A | FDD | N/A |
| CA_n3-n20-n77 | n3 | 1747 | 5 | 25 | 1842 | N/A | FDD | N/A |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3441 | 16.3 | TDD | IMD31 |
|  | n3 | N/A | 5 | N/A | 1842 | 15.7 | FDD | IMD3 |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n77 | 3536 | 10 | 50 | 3536 | N/A | TDD | N/A |
| CA_n3-n20-n78 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n20 | 845 | 5 | 25 | 804 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3420 | 16.1 | TDD | IMD31 |
|  | n3 | N/A | 5 | N/A | 1820 | 17.3 | FDD | IMD3 |
|  | n20 | 845 | 5 | 25 | 804 | N/A | FDD | N/A |
|  | n78 | 3510 | 10 | 50 | 3510 | N/A | TDD | N/A |
| CA_n3-n26-n78 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n26 | 839 | 5 | 25 | 884 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3408 | 16.1 | TDD | IMD31 |
|  | n3 | N/A | 5 | N/A | 1862 | 15.7 | FDD | IMD3 |
|  | n26 | 839 | 5 | 25 | 884 | N/A | FDD | N/A |
|  | n78 | 3540 | 10 | 50 | 3540 | N/A | TDD | N/A |
| CA_n3-n28-n41 | n3 | 1782.5 | 5 | 25 | 1877.5 | N/A | FDD | N/A |
|  | n28 | 745.5 | 5 | 25 | 800.5 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2528 | 25.4 | TDD | IMD2 |
|  | n3 | 1715 | 5 | 25 | 1810 | N/A | FDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2687 | 13.9 | TDD | IMD3 |
|  | n3 | 1760 | 5 | 25 | 1855 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 790 | 24.5 | FDD | IMD24 |
|  | n41 | 2550 | 10 | 50 | 2550 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1832.5 | 26.0 | FDD | IMD2 |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |
|  | n41 | 2543 | 10 | 50 | 2543 | N/A | TDD | N/A |
| CA_n3-n28-n77 | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n28 | 733 | 5 | 25 | 788 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 4173 | 15.9 | TDD | IMD3 |
|  | n28 | 735 | 5 | 25 | 790 | N/A | FDD | N/A |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1850 | 17.0 | FDD | IMD3 |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | FDD | N/A |
|  | n77 | 4195 | 10 | 50 | 4195 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 770 | 15.3 | FDD | IMD3 |
| CA_n3-n28-n78 | n28 | 735 | 5 | 25 | 790 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3320 | N/A | TDD | IMD31 |
|  | n3 | 1755 | 5 | 25 | 1850 | 17.3 | FDD | N/A |
| CA_n3-n28-n79 | n3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n28 | 725 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n79 | N/A | 40 | N/A | 4585 | 9.4 | TDD | IMD41 |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n79 | 4530 | 40 | 216 | 4530 | N/A | TDD | N/A |
|  | n28 | 725 | 5 | 25 | 780 | 10.3 | FDD | IMD4 |
|  | n28 | 725 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | TDD | N/A |
|  | n3 | 1775 | 5 | 25 | 1870 | 5.7 | FDD | IMD5 |
| CA_n3-n34-n41 | n3 | 1742.5 | 5 | 25 | 1837.5 | N/A | FDD | N/A |
|  | n34 | 2020 | 5 | 25 | 2020 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2575 | 0.8 | TDD | IMD5 |
| CA_n3-n34-n79 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n34 | 2020 | 5 | 25 | 2020 | N/A | TDD | N/A |
|  | n79 | N/A | 40 | N/A | 4900 | 3.2 | TDD | IMD5 |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n34 | 4900 | 40 | 216 | 4900 | N/A | TDD | N/A |
|  | n79 | N/A | 5 | N/A | 2020 | 9.7 | TDD | IMD5 |
| CA_n3-40-n41 | n3 | N/A | 5 | N/A | 1842.5 | 1.0 | FDD | IMD5 |
|  | n40 | 2347.5 | 5 | 25 | 2347.5 | N/A | TDD | N/A |
|  | n41 | 2600 | 10 | 50 | 2600 | N/A | TDD | N/A |
| CA_n3-n40-n77 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n40 | 2320 | 5 | 25 | 2320 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 4050 | 19.0 | TDD | IMD21 |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2310 | 29.4 | TDD | IMD21 |
|  | n77 | 4030 | 10 | 50 | 4030 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1820 | 29.9 | FDD | IMD22 |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | TDD | N/A |
|  | n77 | 4130 | 10 | 50 | 4130 | N/A | TDD | N/A |
| CA_n3-n40-n78 | n3 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n40 | 2350 | 5 | 25 | 2350 | N/A | TDD | N/A |
|  | n78 | N/A | 10 | N/A | 3440 | 16.8 | TDD | IMD4 |
|  | n3 | 1925 | 5 | 25 | 2115 | N/A | FDD | N/A |
|  | n40 | 2395 | 5 | 25 | 2395 | N/A | TDD | N/A |
|  | n78 | N/A | 10 | N/A | 3335 | 0.8 | TDD | IMD5 |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2360 | 5.3 | TDD | IMD5 |
|  | n78 | 3760 | 10 | 50 | 3760 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1860 | 14 | FDD | IMD4 |
|  | n40 | 2390 | 5 | 25 | 2390 | N/A | TDD | N/A |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
| CA_n3-n40-n79 | n3 | 1782.5 | 5 | 25 | 1877.5 | N/A | FDD | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | TDD | N/A |
|  | n79 | N/A | 40 | N/A | 4820 | 3.3 | TDD | IMD5 |
|  | n3 | 1782.5 | 5 | 25 | 1877.5 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2310 | 5.9 | TDD | IMD5 |
|  | n79 | 4820 | 40 | 216 | 4820 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1850 | 4.7 | FDD | IMD5 |
|  | n40 | 2350 | 5 | 25 | 2350 | N/A | TDD | N/A |
|  | n79 | 4450 | 40 | 216 | 4450 | N/A | TDD | N/A |
| CA_n3-n40-n105 | n3 | 1745 | 5 | 25 | 1840 | N/A | FDD | N/A |
|  | n40 | 2380 | 10 | 50 | 2380 | N/A | TDD | N/A |
|  | n105 | N/A | 5 | N/A | 635 | 26.0 | FDD | IMD24 |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n40 | N/A | 10 | N/A | 2388 | 26.0 | TDD | IMD2 |
|  | n105 | 668 | 5 | 25 | 617 | N/A | FDD | N/A |
| CA_n3-n41-n71 | n3 | N/A | 5 | N/A | 1875 | 16.5 | FDD | IMD2 |
|  | n41 | 2550 | 10 | 50 | 2550 | N/A | FDD | N/A |
|  | n71 | 675 | 5 | 25 | 629 | N/A | FDD | N/A |
| CA_n3-n41-n77 | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2640 | 4.3 | TDD | IMD5 |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1840 | 14.9 | FDD | IMD3 |
|  | n41 | 2620 | 10 | 50 | 2620 | N/A | TDD | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n41 | 2580 | 10 | 50 | 2580 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3440 | 15.3 | TDD | IMD31 |
| CA_n3-n41-n78 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n41 | 2560 | 10 | 50 | 2560 | N/A | TDD | N/A |
|  | n78 | N/A | 10 | N/A | 3390 | 15.3 | TDD | IMD3 |
|  | n3 | N/A | 5 | N/A | 1840 | 14.9 | TDD | IMD3 |
|  | n41 | 2620 | 10 | 50 | 2620 | N/A | FDD | N/A |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
| CA_n3-n41-n79 | n3 | N/A | 5 | N/A | 1850 | 29.4 | FDD | IMD21 |
|  | n41 | 2570 | 10 | 50 | 2570 | N/A | TDD | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2670 | 30.2 | TDD | IMD21 |
|  | n79 | 4440 | 40 | 216 | 4440 | N/A | TDD | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n41 | 2670 | 10 | 50 | 2670 | N/A | TDD | N/A |
|  | n79 | N/A | 40 | N/A | 4440 | 30.8 | TDD | IMD21 |
| CA_n3-n67-n78 | n3 | N/A | 5 | N/A | 1877.5 | 2.2 | FDD | IMD7 |
|  | n67 | N/A | 5 | N/A | N/A | N/A | SDL | N/A |
|  | n7810 | 3305 | 10 | 1 (RBSTART=25) | 3305 | N/A | TDD | N/A |
|  |  | 3780 | 10 | 1 (RBSTART=25) | 3780 |  |  |  |
| CA_n3-n71-n77 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n71 | 680 | 5 | 25 | 634 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 4140 | 15.9 | TDD | IMD31 |
|  | n3 | 1747 | 5 | 25 | 1842 | N/A | FDD | N/A |
|  | n71 | 680 | 5 | 25 | 634 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3787 | 10.1 | TDD | IMD4 |
|  | n3 | 1748 | 5 | 25 | 1843 | N/A | FDD | N/A |
|  | n71 | N/A | 5 | N/A | 632 | 15.3 | FDD | IMD3 |
|  | n77 | 4128 | 10 | 50 | 4128 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1843 | 12.5 | FDD | IMD4 |
|  | n71 | 680 | 5 | 25 | 634 | N/A | FDD | N/A |
|  | n77 | 3883 | 10 | 50 | 3883 | N/A | TDD | N/A |
| CA_n3-n71-n78 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n71 | 670 | 5 | 25 | 624 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3740 | 17.3 | TDD | IMD41 |
| CA_n3-n75-n78 | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n75 | N/A | 5 | N/A | 1450 | 10.8 | SDL | IMD4 |
|  | n78 | 3700 | 10 | 50 | 3700 | N/A | TDD | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n75 | N/A | 5 | N/A | 1490 | 1.0 | SDL | IMD5 |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
| CA_n3-n77-n79 | n77 | 3350 | 10 | 50 | 3350 | N/A | FDD | N/A |
|  | n79 | 4840 | 40 | 216 | 4840 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 15.7 | TDD | IMD31, 2 |
| CA_n3-n78-n79 | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n78 | 3320 | 10 | 52 | 3320 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4860 | 13.1 | TDD | IMD3 |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3480 | 1 | TDD | IMD5 |
|  | n79 | 4410 | 10 | 52 | 4410 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 15.7 | FDD | IMD31 |
|  | n78 | 3350 | 10 | 52 | 3350 | N/A | TDD | N/A |
|  | n79 | 4840 | 10 | 52 | 4840 | N/A | TDD | N/A |
|  | n3 | 1745 | 5 | 25 | 1840 | 3 | FDD | IMD4 |
|  | n78 | 3780 | 10 | 52 | 3780 | N/A | TDD | N/A |
|  | n79 | 4700 | 10 | 52 | 4700 | N/A | TDD | N/A |
| CA_n3-n78-n105 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3740 | 17.3 | TDD | IMD44 |
|  | n105 | 670 | 5 | 25 | 619 | N/A | FDD | N/A |
| CA_n5-n7-n25 | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n7 | N/A | 5 | N/A | 2685 | 30.0 | FDD | IMD2 |
|  | n25 | 1855 | 5 | 25 | 1935 | N/A | FDD | N/A |
| CA_n5-n7-n66 | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n7 | N/A | 5 | N/A | 2624 | 29.0 | FDD | IMD2 |
|  | n66 | 1777.5 | 5 | 25 | 2177.5 | N/A | FDD | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n7 | N/A | 5 | N/A | 2670 | 13 | FDD | IMD3 |
|  | n66 | 1750 | 5 | 25 | 2150 | N/A | FDD | N/A |
|  | n5 | N/A | 5 | N/A | 880 | 13 | FDD | IMD3 |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A | FDD | N/A |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
| CA_n5-n7-n77 | n5 | 844 | 5 | 25 | 889 | N/A | FDD | N/A |
|  | n7 | N/A | 5 | N/A | 2645 | 30.1 | FDD | IMD2 |
|  | n77 | 3489 | 10 | 50 | 3489 | N/A | TDD | N/A |
|  | n5 | N/A | 5 | N/A | 879 | 30.2 | FDD | IMD21, 4 |
|  | n7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n77 | 3429 | 10 | 50 | 3429 | N/A | TDD | N/A |
|  | n5 | 827 | 5 | 25 | 852 | N/A | FDD | N/A |
|  | n7 | 2503 | 5 | 25 | 2623 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3330 | 30.2 | TDD | IMD22, 4 |
| CA_n5-n7-n78 | n5 | N/A | 5 | N/A | 879 | 30.2 | FDD | IMD2 |
|  | n7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n78 | 3429 | 10 | 50 | 3429 | N/A | TDD | N/A |
|  | n5 | N/A | 5 | N/A | 875 | 3.3 | FDD | IMD5 |
|  | n7 | 2525 | 5 | 25 | 2645 | N/A | FDD | N/A |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
|  | n5 | 844 | 5 | 25 | 889 | N/A | FDD | N/A |
|  | n7 | N/A | 5 | N/A | 2645 | 30.1 | FDD | IMD2 |
|  | n78 | 3489 | 10 | 50 | 3489 | N/A | TDD | N/A |
|  | n5 | 835 | 5 | 25 | 880 | N/A | FDD | N/A |
|  | n7 | 2540 | 5 | 25 | 2660 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3375 | 29.7 | TDD | IMD22 |
| CA_n5-n7-n105 | n5 | 834.5 | 5 | 25 | 879.5 | N/A | FDD | N/A |
|  | n7 | N/A | 5 | N/A | 2655 | 6.5 | FDD | IMD5 |
|  | n105 | 683 | 5 | 25 | 632 | N/A | FDD | N/A |
| CA_n5-n12-n77 | n5 | N/A | 5 | N/A | 880 | 3.9 | FDD | IMD5 |
|  | n12 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
|  | n5 | 835 | 5 | 25 | 880 | N/A | FDD | N/A |
|  | n12 | N/A | 5 | N/A | 740 | 4.4 | FDD | IMD55 |
|  | n77 | 4080 | 10 | 50 | 4080 | N/A | TDD | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n12 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3905 | 4.4 | TDD | IMD5 |
| CA_n5-n14-n77 | n5 | N/A | 5 | N/A | 880 | 3.9 | FDD | IMD55 |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n77 | 4052 | 10 | 50 | 4052 | N/A | TDD | N/A |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n14 | N/A | 5 | N/A | 765.5 | 11.6 | FDD | IMD41,5 |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n5 | 840 | 5 | 25 | 885 | N/A | FDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3313 | 10.3 | TDD | IMD41,5 |
| CA_n5-n25-n41 | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2685 | 28.0 | TDD | IMD2 |
| CA_n5-n25-n66 | n5 | 834 | 5 | 25 | 879 | N/A | FDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2132 | 7.2 | FDD | IMD4 |
| CA_n5-n25-n77 | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3540 | 16.0 | TDD | IMD3 |
|  | n5 | N/A | 5 | N/A | 889 | 3.8 | FDD | IMD55 |
|  | n25 | 1907 | 5 | 25 | 1987 | N/A | FDD | N/A |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n25 | N/A | 5 | N/A | 1987 | 16.5 | FDD | IMD3 |
|  | n77 | 3680 | 10 | 25 | 3680 | N/A | TDD | N/A |
| CA_n5-n25-n78 | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3560 | 16.1 | TDD | IMD3 |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n25 | N/A | 5 | N/A | 1987 | 16.5 | FDD | IMD3 |
|  | n78 | 3680 | 10 | 50 | 3680 | N/A | TDD | N/A |
|  | n5 | N/A | 5 | N/A | 887.5 | 3.8 | FDD | IMD5 |
|  | n25 | 1907.5 | 5 | 25 | 1987.5 | N/A | FDD | N/A |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
| CA_n5-n28-n78 | n5 | N/A | 5 | N/A | 874 | 3.8 | FDD | IMD5 |
|  | n28 | 723 | 5 | 25 | 778 | N/A | FDD | N/A |
|  | n78 | 3766 | 10 | 50 | 3756 | N/A | TDD | N/A |
|  | n5 | 844 | 5 | 25 | 889 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 778 | 11.6 | FDD | IMD4 |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | TDD | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n28 | 707 | 5 | 25 | 762 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3781 | 4.0 | TDD | IMD5 |
| CA_n5-n28-n105 | n5 | 845 | 5 | 25 | 890 | N/A | FDD | N/A |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n105 | 686 | 5 | 25 | 635 | 25.0 | FDD | IMD3 |
| CA_n5-n29-n66 | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n29 | N/A | 5 | N/A | 720 | 9.4 | SDL | IMD4 |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | FDD | N/A |
| CA_n5-n29-n77 | n5 | 845 | 5 | 25 | 890 | N/A | FDD | N/A |
|  | n29 | N/A | 5 | N/A | 720 | 4.4 | SDL | IMD55 |
|  | n77 | 4100 | 10 | 50 | 4100 | N/A | TDD | N/A |
| CA_n5-n30-n66 | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n30 | 2307.5 | 5 | 25 | 2352.5 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2125 | 4 | FDD | IMD5 |
| CA_n5-n30-n77 | n5 | N/A | 5 | N/A | 880 | 15.2 | FDD | IMD31 |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | 3740 | 10 | 50 | 3740 | N/A | TDD | N/A |
|  | n5 | 835 | 5 | 25 | 880 | N/A | FDD | N/A |
|  | n30 | N/A | 5 | N/A | 2355 | 13.2 | FDD | IMD35 |
|  | n77 | 4025 | 10 | 50 | 4025 | N/A | TDD | N/A |
|  | n5 | 840 | 5 | 25 | 885 | N/A | FDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3780 | 16.1 | TDD | IMD3 |
| CA_n5-n40-n78 | n5 | N/A | 5 | N/A | 880 | 15.2 | FDD | IMD3 |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | TDD | N/A |
|  | n78 | 3740 | 10 | 50 | 3740 | N/A | TDD | N/A |
|  | n5 | 840 | 5 | 25 | 885 | N/A | FDD | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | TDD | N/A |
|  | n78 | N/A | 10 | N/A | 3780 | 16.1 | TDD | IMD3 |
| CA_n5-n40-n105 | n5 | 836.5 | 5 | 25 | 881.5 | N/A | FDD | N/A |
|  | n40 | 2305 | 5 | 25 | 2305 | N/A | TDD | N/A |
|  | n105 | N/A | 5 | N/A | 632 | 17 | FDD | IMD3 |
|  | n5 | 836.5 | 5 | 25 | 881.5 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2356 | 18.8 | TDD | IMD3 |
|  | n105 | 683 | 5 | 25 | 632 | N/A | FDD | NA |
| CA_n5-n41-n66 | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2624 | 29.0 | TDD | IMD24 |
|  | n66 | 1777.5 | 5 | 25 | 2177.5 | N/A | FDD | N/A |
|  | n5 | N/A | 5 | N/A | 875 | 28.9 | FDD | IMD24 |
|  | n41 | 2640 | 10 | 50 | 2640 | N/A | TDD | N/A |
|  | n66 | 1765 | 5 | 25 | 2165 | N/A | FDD | N/A |
| CA_n5-n41-n77 | n5 | 835 | 5 | 25 | 880 | N/A | FDD | N/A |
|  | n41 | 2540 | 10 | 50 | 2540 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3375 | 28.2 | TDD | IMD22 |
|  | n5 | 840 | 5 | 25 | 885 | N/A | FDD | N/A |
|  | n41 | 2510 | 10 | 50 | 2510 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 4180 | 14.6 | TDD | IMD3 |
|  | n5 | 844 | 5 | 25 | 889 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2645 | 28.1 | TDD | IMD2 |
|  | n77 | 3489 | 10 | 50 | 3489 | N/A | TDD | N/A |
|  | n5 | 835 | 5 | 25 | 880 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2510 | 11.2 | FDD | IMD3 |
|  | n77 | 4180 | 10 | 50 | 4180 | N/A | TDD | N/A |
|  | n5 | N/A | 5 | N/A | 879 | 28.7 | FDD | IMD2 |
|  | n41 | 2550 | 10 | 50 | 2550 | N/A | TDD | N/A |
|  | n77 | 3429 | 10 | 50 | 3429 | N/A | TDD | N/A |
|  | n5 | N/A | 5 | N/A | 880 | 13.7 | FDD | IMD31 |
|  | n41 | 2510 | 10 | 50 | 2510 | N/A | TDD | N/A |
|  | n77 | 4140 | 10 | 50 | 4140 | N/A | TDD | N/A |
| CA_n5-n48-n66 | n5 | 829 | 5 | 25 | 874 | N/A | FDD | N/A |
|  | n48 | N/A | 10 | N/A | 3622 | 3.6 | TDD | IMD5 |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
| CA_n5-n66-n77 | n5 | 845 | 5 | 25 | 890 | N/A | FDD | N/A |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3465 | 16.1 | TDD | IMD31 |
|  | n5 | 826.5 | 5 | 25 | 871.5 | N/A | FDD | N/A |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 4192 | 8.2 | TDD | IMD45 |
|  | n5 | 826.5 | 5 | 25 | 871.5 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2142 | 13.2 | FDD | IMD3 |
|  | n77 | 3795 | 10 | 50 | 3795 | N/A | TDD | N/A |
| CA_n5-n66-n78 | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3380 | 16.1 | TDD | IMD3 |
|  | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2120 | 13.2 | FDD | IMD3 |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | TDD | N/A |
| CA_n5-n78-n79 | n5 | 846 | 5 | 25 | 891 | N/A | FDD | N/A |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n79 | N/A | 40 | N/A | 4636 | 26.2 | TDD | IMD2 |
|  | n5 | 827 | 5 | 25 | 872 | N/A | FDD | N/A |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n79 | N/A | 40 | N/A | 4959 | 22 | TDD | IMD3 |
|  | n5 | 827 | 5 | 25 | 872 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3593 | 26.9 | TDD | IMD2 |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |
|  | n5 | 827 | 5 | 25 | 872 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3326 | 17 | TDD | IMD3 |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | TDD | N/A |
|  | n5 | N/A | 5 | N/A | 880 | 16.2 | FDD | IMD2 |
|  | n78 | 3550 | 10 | 50 | 3550 | N/A | TDD | N/A |
|  | n79 | 4430 | 40 | 216 | 4430 | N/A | TDD | N/A |
|  | n5 | N/A | 5 | N/A | 875 | 3 | FDD | IMD5 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n79 | 4520 | 40 | 216 | 4520 | N/A | TDD | N/A |
| CA_n5-n78-n105 | n5 | 836.5 | 5 | 25 | 881.5 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3568.5 | 4 | TDD | IMD5 |
|  | n105 | 683 | 5 | 25 | 632 | N/A | FDD | N/A |
|  | n5 | N/A | 5 | N/A | 881.5 | 3.8 | FDD | IMD5 |
|  | n78 | 3613.5 | 10 | 52 | 3613.5 | N/A | TDD | N/A |
|  | n105 | 683 | 5 | 25 | 632 | N/A | FDD | N/A |
| CA_n7-n8-n40 | n7 | 2530 | 5 | 25 | 2650 | N/A | FDD | N/A |
|  | n8 | 905 | 5 | 25 | 950 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2345 | 3.0 | TDD | IMD5 |
| CA_n7-n8-n78 | n7 | 2555 | 5 | 25 | 2675 | N/A | FDD | N/A |
|  | n8 | 900 | 5 | 25 | 945 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3455 | 28.5 | TDD | IMD22 |
|  | n7 | 2555 | 5 | 25 | 2675 | N/A | FDD | N/A |
|  | n8 | N/A | 5 | N/A | 945 | 29.7 | FDD | IMD2 |
|  | n78 | 3500 | 10 | 50 | 3500 | N/A | TDD | N/A |
|  | n7 | 2520 | 5 | 25 | 2640 | N/A | FDD | N/A |
|  | n8 | N/A | 5 | N/A | 940 | 3.1 | FDD | IMD5 |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | TDD | N/A |
|  | n7 | N/A | 5 | N/A | 2650 | 28 | FDD | IMD2 |
|  | n8 | 895 | 5 | 25 | 940 | N/A | FDD | N/A |
|  | n78 | 3545 | 10 | 50 | 3545 | N/A | TDD | N/A |
| CA_n7-n20-n67 | n7 | 2565 | 10 | 50 | 2685 | N/A | TDD | N/A |
|  | n20 | 834.5 | 5 | 25 | 793.5 | N/A | FDD | N/A |
|  | n67 | N/A | 5 | N/A | 773 | 3.9 | FDD | IMD5 |
| CA_n7-n20-n78 | n7 | 2560 | 5 | 25 | 2680 | N/A | FDD | N/A |
|  | n20 | N/A | 5 | N/A | 810 | 30.5 | FDD | IMD21 |
|  | n78 | 3370 | 10 | 50 | 3370 | N/A | TDD | N/A |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A | FDD | N/A |
|  | n20 | N/A | 5 | N/A | 810 | 3.0 | FDD | IMD5 |
|  | n78 | 3435 | 10 | 50 | 3435 | N/A | TDD | N/A |
|  | n7 | N/A | 5 | N/A | 2675 | 30.8 | FDD | IMD2 |
|  | n20 | 845 | 5 | 25 | 804 | N/A | FDD | N/A |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | TDD | N/A |
|  | n7 | 2540 | 5 | 25 | 2660 | N/A | FDD | N/A |
|  | n20 | 835 | 5 | 25 | 794 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3375 | 29.7 | TDD | IMD22 |
| CA_n7-n25-n29 | n7 | 2567.5 | 5 | 25 | 2687.5 | N/A | FDD | N/A |
|  | n25 | 1852.5 | 5 | 25 | 1932.5 | N/A | FDD | N/A |
|  | n29 | N/A | 5 | N/A | 719.5 | 26 | SDL | IMD2 |
|  | n7 | 2503 | 5 | 25 | 2623 | N/A | FDD | N/A |
|  | n25 | 1910 | 5 | 25 | 1990 | N/A | FDD | N/A |
|  | n29 | N/A | 5 | N/A | 724 | 4.5 | SDL | IMD5 |
| CA_n7-n25-n77 | n7 | N/A | 5 | N/A | 2640 | 5.3 | FDD | IMD5 |
|  | n25 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
|  | n77 | 4125 | 10 | 50 | 4125 | N/A | TDD | N/A |
|  | n7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n25 | N/A | 5 | N/A | 1950 | 8.6 | FDD | IMD4 |
|  | n77 | 3525 | 10 | 50 | 3525 | N/A | TDD | N/A |
|  | n7 | 2520 | 5 | 25 | 2640 | N/A | FDD | N/A |
|  | n25 | 1905 | 5 | 25 | 1985 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3750 | 4.5 | TDD | IMD5 |
| CA_n7-n25-n78 | n7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n25 | N/A | 5 | N/A | 1950 | 8.6 | FDD | IMD4 |
|  | n78 | 3525 | 10 | 50 | 3525 | N/A | TDD | N/A |
|  | n7 | 2520 | 5 | 25 | 2640 | N/A | FDD | N/A |
|  | n25 | 1905 | 5 | 25 | 1985 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3750 | 4.5 | TDD | IMD5 |
| CA_n7-n26-n78 | n7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n26 | N/A | 5 | N/A | 879 | 30.2 | FDD | IMD2 |
|  | n78 | 3429 | 10 | 50 | 3429 | N/A | TDD | N/A |
|  | n7 | 2525 | 5 | 25 | 2645 | N/A | FDD | N/A |
|  | n26 | N/A | 5 | N/A | 875 | 3.3 | FDD | IMD5 |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
|  | n7 | N/A | 5 | N/A | 2645 | 30.1 | FDD | IMD2 |
|  | n26 | 844 | 5 | 25 | 889 | N/A | FDD | N/A |
|  | n78 | 3489 | 10 | 50 | 3489 | N/A | TDD | N/A |
|  | n7 | 2540 | 5 | 25 | 2660 | N/A | FDD | N/A |
|  | n26 | 835 | 5 | 25 | 880 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3375 | 29.7 | TDD | IMD2 |
| CA_n7-n28-n40 | n7 | N/A | 5 | N/A | 2630 | 5.9 | FDD | IMD5 |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | TDD | N/A |
| CA_n7-n28-n78 | n7 | 2567.5 | 5 | 25 | 2687.5 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 782.5 | 28.8 | FDD | IMD2 |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
|  | n7 | 2567.5 | 5 | 25 | 2687.5 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 782.5 | 3.0 | FDD | IMD5 |
|  | n78 | 3460 | 10 | 50 | 3460 | N/A | TDD | N/A |
|  | n7 | N/A | 5 | N/A | 2650 | 30.5 | FDD | IMD2 |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | N/A | TDD | N/A |
|  | n7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n28 | 745 | 5 | 25 | 800 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3310 | 29.7 | TDD | IMD22 |
| CA_n7-n29-n66 | n7 | 2502.5 | 5 | 25 | 2622.5 | N/A | FDD | N/A |
|  | n29 | N/A | 5 | N/A | 725 | 31 | SDL | IMD2 |
|  | n66 | 1777.5 | 5 | 25 | 2197.5 | N/A | FDD | N/A |
| CA_n7-n29-n77 | n7 | 2540 | 5 | 25 | 2660 | N/A | FDD | N/A |
|  | n29 | N/A | 5 | N/A | 720 | 3.0 | SDL | IMD5 |
|  | n77 | 3450 | 10 | 50 | 3450 | N/A | TDD | N/A |
| CA_n7-n40-n78 | n7 | N/A | 5 | N/A | 2630 | 10.1 | FDD | IMD4 |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | TDD | N/A |
|  | n78 | 3625 | 10 | 50 | 3625 | N/A | TDD | N/A |
|  | n7 | 2510 | 5 | 25 | 2630 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2310 | 8.7 | TDD | IMD4 |
|  | n78 | 3785 | 10 | 50 | 3785 | N/A | TDD | N/A |
| CA_n7-n40-n79 | n7 | 2520 | 5 | 25 | 2640 | N/A | FDD | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4830 | 30.5 | TDD | IMD22 |
|  | n7 | 2530 | 5 | 25 | 2650 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2320 | 25 | TDD | IMD2 |
|  | n79 | 4850 | 10 | 50 | 4850 | N/A | FDD | N/A |
|  | n7 | 2510 | 5 | 25 | 2630 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2370 | 6 | TDD | IMD5 |
|  | n79 | 4950 | 10 | 50 | 4950 | N/A | FDD | N/A |
|  | n7 | N/A | 5 | N/A | 2630 | 29.4 | FDD | IMD2 |
|  | n40 | 2320 | 5 | 25 | 2320 | N/A | TDD | N/A |
|  | n79 | 4950 | 10 | 50 | 4950 | N/A | TDD | N/A |
|  | n7 | N/A | 5 | N/A | 2650 | 3 | FDD | IMD5 |
|  | n40 | 2350 | 5 | 25 | 2350 | N/A | TDD | N/A |
|  | n79 | 4850 | 10 | 50 | 4850 | N/A | TDD | N/A |
| CA_n7-n40-n105 | n7 | N/A | 5 | N/A | 2655 | 5.9 | FDD | IMD5 |
|  | n40 | 2352 | 5 | 25 | 2352 | N/A | TDD | N/A |
|  | n105 | 683 | 5 | 25 | 632 | N/A | FDD | N/A |
| CA_n7-n46-n78 | n7 | 2530 | 5 | 25 | 2650 | N/A | FDD | N/A |
|  | n46 | 5840 | 20 | 100 | 5840 | N/A | TDD | N/A |
|  | n78 | N/A | 10 | N/A | 3310 | 29.7 | TDD | IMD21 |
|  | n7 | 2530 | 5 | 25 | 2650 | N/A | FDD | N/A |
|  | n46 | N/A | 20 | N/A | 5840 | 25.2 | TDD | IMD21 |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | TDD | N/A |
| CA_n7-n66-n77 | n7 | 2560 | 5 | 25 | 2680 | N/A | FDD | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3390 | 16.1 | TDD | IMD31 |
|  | n7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2150 | 8.7 | FDD | IMD4 |
|  | n77 | 3625 | 10 | 50 | 3625 | N/A | TDD | N/A |
|  | n7 | N/A | 5 | N/A | 2640 | 3.4 | FDD | IMD5 |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | TDD | N/A |
| CA_n7-n66-n78 | n7 | 2560 | 5 | 25 | 2680 | N/A | FDD | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3390 | 16.1 | TDD | IMD3 |
|  | n7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2150 | 8.7 | FDD | IMD4 |
|  | n78 | 3625 | 10 | 50 | 3625 | N/A | TDD | N/A |
| CA_n7-n67-n78 | n7 | 2562 | 5 | 25 | 2682 | N/A | FDD | N/A |
|  | n67 | N/A | 5 | N/A | 748 | 28.8 | SDL | IMD21 |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | TDD | N/A |
| CA_n7-n71-n77 | n7 | 2505 | 5 | 25 | 2625 | N/A | FDD | N/A |
|  | n71 | 666 | 5 | 25 | 620 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3837 | 16.0 | TDD | IMD3 |
|  | n7 | N/A | 5 | N/A | 2670 | 29.6 | FDD | IMD2 |
|  | n71 | 680 | 5 | 25 | 634 | N/A | FDD | N/A |
|  | n77 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
| CA_n7-n75-n78 | n7 | 2510 | 5 | 25 | 2630 | N/A | FDD | N/A |
|  | n75 | N/A | 5 | N/A | 1470 | 16 | SDL | IMD3 |
|  | n78 | 3550 | 10 | 50 | 3550 | N/A | TDD | N/A |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A | FDD | N/A |
|  | n75 | N/A | 5 | N/A | 1500 | 10.5 | SDL | IMD4 |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | TDD | N/A |
| CA_n7-n78-n79 | n7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n78 | 3500 | 10 | 50 | 3500 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4450 | 20.6 | TDD | IMD31 |
|  | n7 | 2567.5 | 5 | 25 | 2687.5 | N/A | FDD | N/A |
|  | n78 | 3302.5 | 10 | 50 | 3302.5 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4400 | 14 | TDD | IMD4 |
|  | n7 | 2525 | 5 | 25 | 2645 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3760 | 6.4 | TDD | IMD4 |
|  | n79 | 4405 | 10 | 50 | 4405 | N/A | TDD | N/A |
|  | n7 | N/A | 5 | N/A | 1860 | 19 | FDD | IMD3 |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
|  | n79 | 4780 | 10 | 50 | 4780 | N/A | TDD | N/A |
|  | n7 | N/A | 5 | N/A | 2660 | 8 | FDD | IMD4 |
|  | n78 | 3500 | 10 | 50 | 3500 | N/A | TDD | N/A |
|  | n79 | 4830 | 10 | 50 | 4830 | N/A | TDD | N/A |
| CA_n7-n78-n102 | n7 | 2560 | 5 | 25 | 2680 | N/A | FDD | N/A |
|  | n78 | 3420 | 5 | 25 | 3420 | N/A | TDD | N/A |
|  | n102 | N/A | 40 | N/A | 5980 | N/A12 | TDD | IMD21 |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A | FDD | N/A |
|  | n78 | N/A | 5 | N/A | 3420 | 29.6 | TDD | IMD21 |
|  | n102 | 5980 | 40 | 216 | 5980 | N/A | TDD | N/A |
|  | n7 | N/A | 5 | N/A | 2680 | 29.6 | FDD | IMD21 |
|  | n78 | 3320 | 5 | 25 | 3320 | N/A | TDD | N/A |
|  | n102 | 6000 | 40 | 216 | 6000 | N/A | TDD | N/A |
| CA_n7-n78-n105 | n7 | 2555 | 5 | 25 | 2675 | N/A | FDD | N/A |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | TDD | N/A |
|  | n105 | N/A | 5 | N/A | 625 | 3.9 | FDD | IMD5 |
|  | n7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3714 | 9.7 | TDD | IMD4 |
|  | n105 | 693 | 5 | 25 | 642 | N/A | FDD | N/A |
|  | n7 | N/A | 5 | N/A | 2625 | 28.7 | FDD | IMD2 |
|  | n78 | 3308 | 10 | 50 | 3308 | N/A | TDD | N/A |
|  | n105 | 683 | 5 | 25 | 632 | N/A | FDD | N/A |
| CA_n8-n20-n28 | n8 | N/A | 5 | 25 | 951.5 | 24.3 | FDD | IMD3 |
|  | n20 | 834.5 | 5 | 25 | 793.5 | N/A | FDD | N/A |
|  | n28 | 717.5 | 5 | 25 | 772.5 | N/A | FDD | N/A |
|  | n8 | 887.5 | 5 | 25 | 932.5 | N/A | FDD | N/A |
|  | n20 | 834.5 | 5 | 25 | 793.5 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | 25 | 781.5 | 24 | FDD | IMD3 |
| CA_n8-n28-n40 | n8 | N/A | 5 | N/A | 928 | 17.0 | FDD | IMD3 |
|  | n28 | 706 | 5 | 25 | 761 | N/A | FDD | N/A |
|  | n40 | 2340 | 5 | 25 | 2340 | N/A | TDD | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n28 | 706 | 5 | 25 | 761 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2322 | 18.8 | TDD | IMD3 |
| CA_n8-n28-n77 | n8 | N/A | 5 | N/A | 928 | 3.8 | FDD | IMD5 |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | TDD | N/A |
|  | n8 | 883 | 5 | 25 | 928 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 761 | 11.6 | FDD | IMD4 |
|  | n77 | 3410 | 10 | 50 | 3410 | N/A | TDD | N/A |
|  | n8 | 883 | 5 | 25 | 928 | N/A | FDD | N/A |
|  | n28 | 745 | 5 | 25 | 800 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3394 | 10.3 | TDD | IMD4 |
|  | n8 | 883 | 5 | 25 | 928 | N/A | FDD | N/A |
|  | n28 | 745 | 5 | 25 | 800 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3863 | 4.0 | TDD | IMD5 |
| CA_n8-n39-n40 | n8 | N/A | 5 | N/A | 940 | 8.6 | FDD | IMD4 |
|  | n39 | 1920 | 5 | 25 | 1920 | N/A | TDD | N/A |
|  | n40 | 2370 | 5 | 25 | 2370 | N/A | TDD | N/A |
|  | n8 | N/A | 5 | N/A | 950 | 4.3 | FDD | IMD5 |
|  | n39 | 1910 | 5 | 25 | 1910 | N/A | TDD | N/A |
|  | n40 | 2390 | 5 | 25 | 2390 | N/A | TDD | N/A |
|  | n8 | 905 | 5 | 25 | 950 | N/A | FDD | N/A |
|  | n39 | N/A | 5 | N/A | 1905 | 3.5 | TDD | IMD5 |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | TDD | N/A |
| CA_n8-n39-n79 | n8 | 900 | 5 | 25 | 945 | N/A | FDD | N/A |
|  | n39 | 1890 | 10 | 50 | 1890 | N/A | TDD | N/A |
|  | n79 | N/A | 40 | N/A | 4680 | 15.9 | TDD | IMD3 |
|  | n8 | 890 | 5 | 25 | 935 | N/A | FDD | N/A |
|  | n39 | 1890 | 10 | 50 | 1890 | N/A | TDD | N/A |
|  | n79 | N/A | 40 | N/A | 4560 | 12.1 | TDD | IMD4 |
|  | n8 | 897.5 | 5 | 25 | 942.5 | N/A | FDD | N/A |
|  | n39 | N/A | 10 | N/A | 1907.5 | 13.8 | TDD | IMD4 |
|  | n79 | 4600 | 40 | 216 | 4600 | N/A | TDD | N/A |
|  | n8 | N/A | 5 | N/A | 940 | 15.1 | FDD | IMD3 |
|  | n39 | 1900 | 10 | 50 | 1900 | N/A | TDD | N/A |
|  | n79 | 4740 | 40 | 216 | 4740 | N/A | TDD | N/A |
|  | n8 | N/A | 5 | N/A | 940 | 7.1 | FDD | IMD4 |
|  | n39 | 1900 | 10 | 50 | 1900 | N/A | TDD | N/A |
|  | n79 | 4750 | 40 | 216 | 4750 | N/A | TDD | N/A |
| CA_n8-n40-n41 | n8 | 895 | 5 | 25 | 940 | N/A | FDD | N/A |
|  | n40 | 2355 | 5 | 25 | 2355 | 4.9 | TDD | IMD5 |
|  | n41 | N/A | 10 | N/A | 2520 | N/A | TDD | N/A |
| CA_n8-n40-n77 | n8 | N/A | 5 | N/A | 950 | 30.5 | FDD | IMD21 |
|  | n40 | 2380 | 5 | 25 | 2380 | N/A | TDD | N/A |
|  | n77 | 3330 | 10 | 50 | 3330 | N/A | TDD | N/A |
|  | n8 | N/A | 5 | N/A | 935 | 19.8 | FDD | IMD31 |
|  | n40 | 2320 | 5 | 25 | 2320 | N/A | TDD | N/A |
|  | n77 | 3705 | 10 | 50 | 3705 | N/A | TDD | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2395 | 28 | TDD | IMD2 |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2330 | 13.2 | TDD | IMD3 |
|  | n77 | 4150 | 10 | 50 | 4150 | N/A | TDD | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n40 | 2395 | 5 | 25 | 2395 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3305 | 28.8 | TDD | IMD2 |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 4130 | 16.1 | TDD | IMD3 |
| CA_n8-n40-n78 | n8 | N/A | 5 | N/A | 950 | 30.5 | FDD | IMD2 |
|  | n40 | 2380 | 5 | 25 | 2380 | N/A | TDD | N/A |
|  | n78 | 3330 | 10 | 50 | 3330 | N/A | TDD | N/A |
|  | n8 | N/A | 5 | N/A | 935 | 19.8 | FDD | IMD3 |
|  | n40 | 2320 | 5 | 25 | 2320 | N/A | TDD | N/A |
|  | n78 | 3705 | 10 | 50 | 3705 | N/A | TDD | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2395 | 28 | TDD | IMD2 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n40 | 2395 | 5 | 25 | 2395 | N/A | TDD | N/A |
|  | n78 | N/A | 10 | N/A | 3305 | 28.8 | TDD | IMD24 |
|  | n8 | 890 | 5 | 25 | 935 | N/A | FDD | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | TDD | N/A |
|  | n78 | N/A | 10 | N/A | 3730 | 16.8 | TDD | IMD3 |
| CA_n8-n40-n79 | n8 | 885 | 5 | 25 | 930 | N/A | FDD | N/A |
|  | n40 | 2305 | 5 | 25 | 2305 | N/A | TDD | N/A |
|  | n79 | N/A | 40 | N/A | 4960 | 10.7 | TDD | IMD4 |
|  | n8 | 885 | 5 | 25 | 930 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2305 | 9.2 | TDD | IMD4 |
|  | n79 | 4960 | 40 | 216 | 4960 | N/A | TDD | N/A |
| CA_n8-n41-n78 | n8 | N/A | 10 | N/A | 950 | 29.1 | FDD | IMD2 |
|  | n41 | 2630 | 10 | 50 | N/A | N/A | TDD | N/A |
|  | n78 | 3580 | 10 | 50 | N/A | N/A | TDD | N/A |
|  | n8 | N/A | 10 | N/A | 940 | 4.0 | FDD | IMD5 |
|  | n41 | 2680 | 10 | 50 | N/A | N/A | TDD | N/A |
|  | n78 | 3550 | 10 | 50 | N/A | N/A | TDD | N/A |
|  | n8 | 895 | 5 | 25 | 940 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2650 | 28.0 | TDD | IMD2 |
|  | n78 | 3545 | 10 | 50 | N/A | N/A | TDD | N/A |
|  | n8 | 900 | 5 | 25 | 945 | N/A | FDD | N/A |
|  | n41 | 2555 | 10 | 50 | N/A | N/A | TDD | N/A |
|  | n78 | N/A | 10 | N/A | 3455 | 28.5 | TDD | IMD22 |
| CA_n8-n41-n79 | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n41 | 2650 | 10 | 50 | 2650 | N/A | TDD | N/A |
|  | n79 | 4470 | 10 | 50 | 4470 | 16.3 | TDD | IMD3 |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n41 | 2650 | 10 | 50 | 2650 | 15.5 | TDD | IMD3 |
|  | n79 | 4470 | 10 | 50 | 4470 | N/A | TDD | N/A |
|  | n8 | 895 | 5 | 25 | 940 | 11.8 | FDD | IMD31 |
|  | n41 | 2680 | 10 | 50 | 2680 | N/A | TDD | N/A |
|  | n79 | 4420 | 10 | 50 | 4420 | N/A | TDD | N/A |
| CA_n8-n78-n79 | n8 | 900 | 5 | 25 | 945 | N/A | FDD | N/A |
|  | n78 | 3700 | 10 | 52 | 3700 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4600 | 25 | TDD | IMD22 |
|  | n8 | 900 | 5 | 25 | 945 | N/A | FDD | N/A |
|  | n78 | 3700 | 10 | 52 | 3700 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4700 | 2 | TDD | IMD5 |
|  | n8 | 827 | 5 | 25 | 872 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3593 | 25 | TDD | IMD2 |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |
|  | n8 | N/A | 5 | N/A | 930 | 20.3 | FDD | IMD2 |
|  | n78 | 3600 | 10 | 50 | 3600 | N/A | TDD | N/A |
|  | n79 | 4530 | 40 | 216 | 4530 | N/A | TDD | N/A |
|  | n8 | N/A | 5 | N/A | 940 | 3 | FDD | IMD5 |
|  | n78 | 3600 | 10 | 50 | 3600 | N/A | TDD | N/A |
|  | n79 | 4930 | 40 | 216 | 4930 | N/A | TDD | N/A |
| CA_n12-n30-n77 | n12 | N/A | 5 | N/A | 740 | 15.2 | FDD | IMD31 |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | 3880 | 10 | 50 | 3880 | N/A | TDD | N/A |
|  | n12 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
|  | n30 | N/A | 5 | N/A | 2355 | 13.2 | FDD | IMD3 |
|  | n77 | 3770 | 10 | 50 | 3770 | N/A | TDD | N/A |
|  | n12 | 707 | 5 | 25 | 737 | N/A | FDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3913 | 16.0 | TDD | IMD3 |
| CA_n12-n66-n77 | n12 | N/A | 5 | N/A | 740 | 15.2 | FDD | IMD35 |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n77 | 4180 | 10 | 50 | 4180 | N/A | TDD | N/A |
|  | n12 | 707 | 5 | 25 | 737 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2126 | 13.2 | FDD | IMD3 |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | TDD | N/A |
|  | n12 | 704 | 5 | 25 | 734 | N/A | FDD | N/A |
|  | n66 | 1723 | 5 | 25 | 2123 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 4150 | 16.0 | TDD | IMD31,2,5 |
| CA_n12-n71-n77 | n12 | N/A | 5 | N/A | 732 | 4.4 | FDD | IMD5 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3504 | 10 | 50 | 3504 | N/A | TDD | N/A |
|  | n12 | 711 | 5 | 25 | 741 | N/A | FDD | N/A |
|  | n71 | N/A | 5 | N/A | 646 | 3.9 | FDD | IMD5 |
|  | n77 | 3490 | 10 | 50 | 3490 | N/A | TDD | N/A |
| CA_n13-n25-n66 | n13 | 782 | 5 | 25 | 751 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2156 | 7.2 | FDD | IMD4 |
|  | n25 | 1860 | 5 | 25 | 1940 | N/A | FDD | N/A |
|  | n13 | 780 | 10 | 50 | 749 | N/A | FDD | N/A |
|  | n25 | N/A | 5 | N/A | 1940 | 6.2 | FDD | IMD4 |
|  | n66 | 1750 | 5 | 25 | 2150 | N/A | FDD | N/A |
| CA_n13-n25-n77 | n13 | 782 | 5 | 25 | 751 | N/A | FDD | N/A |
|  | n25 | 1896 | 5 | 25 | 1976 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3460 | 17.3 | TDD | IMD31,2 |
|  | n13 | 782 | 5 | 25 | 751 | N/A | FDD | N/A |
|  | n25 | N/A | 5 | N/A | 1960 | 16.0 | FDD | IMD3 |
|  | n77 | 3524 | 10 | 50 | 3524 | N/A | TDD | N/A |
| CA_n13-n66-n77 | n13 | 782 | 5 | 25 | 751 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2146 | 17.1 | FDD | IMD3 |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
|  | n13 | N/A | 5 | N/A | 750 | 15.2 | FDD | IMD35 |
|  | n66 | 1710 | 5 | 25 | 2110 | N/A | FDD | N/A |
|  | n77 | 4170 | 10 | 50 | 4170 | N/A | TDD | N/A |
|  | n13 | 782 | 5 | 25 | 751 | N/A | FDD | N/A |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3334 | 16.3 | TDD | IMD31,2,5 |
| CA_n14-n30-n77 | n14 | N/A | 5 | N/A | 763 | 15.2 | FDD | IMD31 |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | 3857 | 10 | 50 | 3857 | N/A | TDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n30 | N/A | 5 | N/A | 2355 | 13.2 | FDD | IMD3 |
|  | n77 | 3941 | 10 | 50 | 3941 | N/A | TDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3896 | 16.0 | TDD | IMD3 |
| CA_n14-n66-n77 | n14 | N/A | 5 | N/A | 763 | 15.2 | FDD | IMD35 |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | N/A | FDD | N/A |
|  | n77 | 4188 | 10 | 50 | 4188 | N/A | TDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2155 | 13.2 | FDD | IMD3 |
|  | n77 | 3741 | 10 | 50 | 3741 | N/A | TDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n66 | 1755 | 5 | 25 | 2155 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3341 | 16.0 | TDD | IMD31,2,5 |
| CA_n18-n28-n41 | n18 | 825 | 5 | 25 | 870 | N/A | FDD | N/A |
|  | n28 | 738 | 5 | 25 | 793 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2562 | 4.4 | TDD | IMD5 |
|  | n18 | 825 | 5 | 25 | 870 | N/A | FDD | N/A |
|  | n41 | 2505 | 10 | 50 | 2505 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 795 | 3.9 | FDD | IMD5 |
| CA_n18-n28-n77 | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n28 | 710 | 5 | 25 | 765 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3770 | 4.0 | TDD | IMD5 |
|  | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 778 | 4.4 | FDD | IMD5 |
|  | n77 | 4058 | 10 | 50 | 4058 | N/A | TDD | N/A |
|  | n18 | N/A | 5 | N/A | 865 | 3.9 | FDD | IMD5 |
|  | n28 | 723 | 5 | 25 | 778 | N/A | FDD | N/A |
|  | n77 | 3757 | 10 | 50 | 3757 | N/A | TDD | N/A |
| CA_n18-n41-n77 | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n41 | 2630 | 10 | 50 | 2630 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3450 | 28.6 | TDD | IMD22,4 |
|  | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2630 | 26.5 | TDD | IMD24 |
|  | n77 | 3450 | 10 | 50 | 3450 | N/A | TDD | N/A |
|  | n18 | N/A | 5 | N/A | 870 | 29.3 | FDD | IMD21,4 |
|  | n41 | 2590 | 10 | 50 | 2590 | N/A | TDD | N/A |
|  | n77 | 3460 | 10 | 50 | 3460 | N/A | TDD | N/A |
| CA_n20-n41-n71 | n20 | 834 | 5 | 25 | 793 | N/A | FDD | N/A |
|  | n41 | 2686 | 10 | 25 | 2686 | N/A | TDD | N/A |
|  | n71 | N/A | 5 | N/A | 650 | 5.9 | FDD | IMD5 |
|  | n20 | 835 | 5 | 25 | 794 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2660 | 6.5 | TDD | IMD5 |
|  | n71 | 680 | 5 | 25 | 634 | N/A | FDD | N/A |
| CA_n20-n41-n77 | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n41 | 2593 | 10 | 50 | 2593 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3440 | 28.2 | TDD | IMD22 |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n41 | 2506 | 10 | 50 | 2506 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 4165 | 14.6 | TDD | IMD3 |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2593 | 28.1 | TDD | IMD2 |
|  | n77 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
|  | n20 | 837 | 5 | 25 | 796 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2516 | 11.2 | TDD | IMD3 |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | TDD | N/A |
|  | n20 | N/A | 5 | N/A | 806 | 28.7 | FDD | IMD2 |
|  | n41 | 2594 | 10 | 50 | 2594 | N/A | TDD | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n20 | N/A | 5 | N/A | 817 | 13.7 | FDD | IMD3 |
|  | n41 | 2501 | 10 | 50 | 2501 | N/A | TDD | N/A |
|  | n77 | 4185 | 10 | 50 | 4185 | N/A | TDD | N/A |
| CA_n20-n41-n78 | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n41 | 2593 | 10 | 50 | 2593 | N/A | TDD | N/A |
|  | n78 | N/A | 10 | N/A | 3440 | 28.8 | TDD | IMD22 |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2593 | 29.8 | TDD | IMD2 |
|  | n78 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
|  | n20 | N/A | 5 | N/A | 806 | 30.8 | FDD | IMD2 |
|  | n41 | 2593 | 10 | 50 | 2593 | N/A | TDD | N/A |
|  | n78 | 3399 | 10 | 50 | 3399 | N/A | TDD | N/A |
|  | n20 | N/A | 5 | N/A | 806 | 3.3 | FDD | IMD5 |
|  | n41 | 2635 | 10 | 50 | 2635 | N/A | TDD | N/A |
|  | n78 | 3549.5 | 10 | 50 | 3549.5 | N/A | TDD | N/A |
| CA_n20-n67-n78 | n20 | 855 | 5 | 25 | 814 | N/A | FDD | N/A |
|  | n67 | N/A | 5 | N/A | 755 | 11.6 | FDD | IMD4 |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
| CA_n20-n71-n78 | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n71 | 680 | 5 | 25 | 634 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3567 | 4 | TDD | IMD5 |
|  | n20 | N/A | 5 | N/A | 808 | 3.8 | FDD | IMD5 |
|  | n71 | 680 | 5 | 25 | 634 | N/A | FDD | N/A |
|  | n78 | 3528 | 10 | 50 | 3528 | N/A | TDD | N/A |
| CA_n24-n41-n48 | n24 | 1649 | 5 | 25 | 1528.5 | N/A | FDD | N/A |
|  | n41 | 2610 | 10 | 50 | 2610 | N/A | TDD | N/A |
|  | n48 | N/A | 10 | N/A | 3571 | 15.3 | TDD | IMD3 |
|  | n24 | 1630 | 5 | 25 | 1528.5 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2501 | 4.3 | TDD | IMD5 |
|  | n48 | 3695 | 10 | 50 | 3695 | N/A | TDD | N/A |
|  | n24 | N/A | 5 | N/A | 1530 | 14.9 | FDD | IMD3 |
|  | n41 | 2592.5 | 10 | 50 | 2592.5 | N/A | TDD | N/A |
|  | n48 | 3655 | 10 | 50 | 3655 | N/A | TDD | N/A |
| CA_n24-n41-n77 | n24 | 1630 | 5 | 25 | 1528.5 | N/A | FDD | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3740 | 15.3 | TDD | IMD31,5 |
|  | n24 | 1630 | 5 | 25 | 1528.5 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2620 | 4.3 | TDD | IMD55 |
|  | n77 | 3755 | 10 | 50 | 3755 | N/A | TDD | N/A |
|  | n24 | N/A | 5 | N/A | 1528.5 | 14.9 | FDD | IMD32,5 |
|  | n41 | 2501 | 10 | 50 | 2501 | N/A | TDD | N/A |
|  | n77 | 3473.5 | 10 | 50 | 3473.5 | N/A | TDD | N/A |
| CA_n25-n38-n78 | n25 | N/A | 5 | N/A | 1932.5 | 16.4 | FDD | IMD3 |
|  | n38 | 2617.5 | 5 | 25 | 2617.5 | N/A | TDD | N/A |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n25 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
|  | n38 | 2610 | 5 | 25 | 2610 | N/A | TDD | N/A |
|  | n78 | N/A | 10 | N/A | 3350 | 14.8 | TDD | IMD3 |
|  | n25 | N/A | 5 | N/A | 1960 | 8.6 | TDD | IMD4 |
|  | n38 | 2570 | 5 | 25 | 2570 | N/A | FDD | N/A |
|  | n78 | 3550 | 10 | 50 | 3550 | N/A | TDD | N/A |
| CA_n25-n41-n66 | n25 | N/A | 5 | N/A | 1940 | 11.0 | FDD | IMD4 |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
|  | n66 | 1715 | 5 | 25 | 2115 | N/A | FDD | N/A |
|  | n25 | N/A | 5 | 25 | 1950 | 15.3 | FDD | IMD3 |
|  | n41 | 2546 | 100 | 1 (RBstart=24) | 2546 | N/A | TDD | N/A |
|  |  | 2641 | 90 | 1 (RBstart=232) | 2641 |  |  |  |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | FDD | N/A |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | FDD | N/A |
|  | n41 | 2546 | 100 | 1 (RBstart=260) | 2546 | N/A | TDD | N/A |
|  |  | 2641 | 90 | 1 (RBstart=121) | 2641 |  |  |  |
|  | n66 | N/A | 5 | 25 | 1962.5 | 15.3 | FDD | IMD3 |
| CA_n25-n41-n77 | n25 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
|  | n41 | 2670 | 10 | 50 | 2670 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3470 | 13.3 | TDD | IMD31 |
|  | n25 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2640 | 4.3 | TDD | IMD55 |
|  | n77 | 4125 | 10 | 50 | 4125 | N/A | TDD | N/A |
|  | n25 | N/A | 5 | N/A | 1950 | 16.1 | FDD | IMD35 |
|  | n41 | 2675 | 10 | 50 | 2675 | N/A | TDD | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n25 | N/A | 5 | N/A | 1950 | 7.6 | FDD | IMD4 |
|  | n41 | 2550 | 10 | 50 | 2550 | N/A | TDD | N/A |
|  | n77 | 3525 | 10 | 50 | 3525 | N/A | TDD | N/A |
| CA_n25-n41-n78 | n25 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
|  | n41 | 2670 | 10 | 50 | 2670 | N/A | TDD | N/A |
|  | n78 | N/A | 10 | N/A | 3470 | 13.3 | TDD | IMD31 |
|  | n25 | N/A | 5 | N/A | 1950 | 16.1 | FDD | IMD3 |
|  | n41 | 2675 | 10 | 50 | 2675 | N/A | TDD | N/A |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n25 | N/A | 5 | N/A | 1950 | 7.6 | FDD | IMD4 |
|  | n41 | 2550 | 10 | 50 | 2550 | N/A | TDD | N/A |
|  | n78 | 3525 | 10 | 50 | 3525 | N/A | TDD | N/A |
| CA_n25-n41-n85 | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n41 | 2638 | 10 | 50 | 2638 | N/A | TDD | N/A |
|  | n85 | N/A | 5 | N/A | 738 | 28.7 | FDD | IMD24 |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2608 | 26.7 | TDD | IMD2 |
|  | n85 | 708 | 5 | 25 | 738 | N/A | FDD | N/A |
|  | n25 | N/A | 5 | N/A | 1952 | 26 | FDD | IMD2 |
|  | n41 | 2660 | 10 | 50 | 2660 | N/A | TDD | N/A |
|  | n85 | 708 | 5 | 50 | 738 | N/A | FDD | N/A |
| CA_n25-n48-n66 | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n48 | 3540 | 10 | 50 | 3540 | N/A | TDD | N/A |
|  | n66 | N/A | 5 | N/A | 2160 | 10.4 | FDD | IMD4 |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n48 | N/A | 10 | N/A | 3620 | 29.4 | TDD | IMD2 |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n25 | N/A | 5 | N/A | 1960 | 32.1 | FDD | IMD21 |
|  | n48 | 3700 | 10 | 50 | 3700 | N/A | TDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
| CA_n25-n66-n77 | n25 | 1855 | 5 | 25 | 1935 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2115 | 29.2 | FDD | IMD22 |
|  | n77 | 3970 | 10 | 50 | 3970 | N/A | TDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2160 | 4.0 | FDD | IMD5 |
|  | n77 | 3930 | 10 | 50 | 3930 | N/A | TDD | N/A |
|  | n25 | N/A | 5 | N/A | 1960 | 32.1 | FDD | IMD2 |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
|  | n25 | N/A | 5 | N/A | 1960 | 9.1 | FDD | IMD45 |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | FDD | N/A |
|  | n77 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
|  | n25 | N/A | 5 | N/A | 1960 | 2.1 | FDD | IMD55 |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3620 | 29.4 | TDD | IMD22,5 |
| CA_n25-n66-n78 | n25 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3620 | 29.4 | TDD | IMD2 |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2150 | 10.4 | FDD | IMD4 |
|  | n78 | 3550 | 10 | 50 | 3550 | N/A | TDD | N/A |
|  | n25 | N/A | 5 | N/A | 1960 | 32.1 | FDD | IMD21,2 |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n78 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
| CA_n25-n66-n85 | n25 | N/A | 5 | N/A | 1992.5 | 11.0 | FDD | IMD4 |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | N/A | FDD | N/A |
|  | n85 | 713.5 | 5 | 25 | 743.5 | N/A | FDD | N/A |
| CA_n25-n71-n77 | n25 | 1907.5 | 5 | 25 | 1987.5 | N/A | FDD | N/A |
|  | n71 | 695.5 | 5 | 25 | 649.5 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3305 | 8.0 | TDD | IMD31,2,5 |
|  | n25 | N/A | 5 | N/A | 1954 | 16.5 | FDD | IMD32,5 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3340 | 10 | 50 | 3340 | N/A | TDD | N/A |
| CA_n25-n71-n78 | n25 | 1907.5 | 5 | 25 | 1987.5 | N/A | FDD | N/A |
|  | n71 | 695.5 | 5 | 25 | 649.5 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3305 | 8.0 | TDD | IMD3 |
|  | n25 | N/A | 5 | N/A | 1954 | 16.5 | FDD | IMD3 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n78 | 3340 | 10 | 50 | 3340 | N/A | TDD | N/A |
| CA_n25-n71-n85 | n25 | 1912.5 | 5 | 25 | 1992,5 | N/A | FDD | N/A |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | FDD | N/A |
|  | n85 | N/A | 5 | 25 | 743.5 | 4.2 | FDD | IMD5 |
| CA_n25-n77-n85 | n25 | N/A | 5 | N/A | 1960 | 16.5 | FDD | IMD32 |
|  | n77 | 3375 | 10 | 50 | 3375 | N/A | TDD | N/A |
|  | n85 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3315 | 16.0 | TDD | IMD31,2 |
|  | n85 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
| CA_n26-n29-n66 | n26 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n29 | N/A | 5 | N/A | 720 | 9.4 | SDL | IMD4 |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | FDD | N/A |
| CA_n26-n48-n66 | n26 | 829 | 5 | 25 | 874 | N/A | FDD | N/A |
|  | n48 | N/A | 10 | N/A | 3622 | 3.6 | TDD | IMD5 |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
| CA_n26-n48-n70 | n26 | 826.5 | 5 | 25 | 871.5 | N/A | FDD | N/A |
|  | n48 | 3653 | 10 | 50 | 3653 | N/A | TDD | N/A |
|  | n70 | N/A | 5 | N/A | 2000 | 13.2 | FDD | IMD3 |
| CA_n26-n66-n71 | n26 | N/A | 5 | N/A | 892 | 4.2 | FDD | IMD5 |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | FDD | N/A |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | FDD | N/A |
| CA_n26-n66-n77 | n26 | 845 | 5 | 25 | 890 | N/A | FDD | N/A |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3465 | 16.1 | TDD | IMD31 |
|  | n26 | 826.5 | 5 | 25 | 871.5 | N/A | FDD | N/A |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 4192 | 8.2 | TDD | IMD4 |
|  | n26 | 826.5 | 5 | 25 | 871.5 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2142 | 13.2 | FDD | IMD3 |
|  | n77 | 3795 | 10 | 50 | 3795 | N/A | TDD | N/A |
| CA_n26-n70-n77 | n26 | 845 | 5 | 25 | 890 | N/A | FDD | N/A |
|  | n70 | 1700 | 5 | 25 | 2000 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3390 | 16.1 | TDD | IMD35 |
|  | n26 | 826.5 | 5 | 25 | 871.5 | N/A | FDD | N/A |
|  | n70 | 1700 | 5 | 25 | 2000 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 4179.5 | 8.2 | TDD | IMD45 |
|  | n26 | 835 | 5 | 25 | 880 | N/A | FDD | N/A |
|  | n70 | 1700 | 5 | 25 | 2000 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3430 | 3.3 | TDD | IMD55 |
|  | n26 | 826.5 | 5 | 25 | 871.5 | N/A | FDD | N/A |
|  | n70 | N/A | 5 | N/A | 2000 | 13.2 | FDD | IMD35 |
|  | n77 | 3653 | 10 | 50 | 3653 | N/A | TDD | N/A |
| CA_n28-n39-n40 | n28 | N/A | 5 | N/A | 790 | 8.6 | FDD | IMD4 |
|  | n39 | 1915 | 5 | 25 | 1915 | N/A | FDD | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | FDD | N/A |
| CA_n28-n39-n41 | n28 | N/A | 5 | N/A | 762 | 29.3 | FDD | IMD2 |
|  | n39 | 1923 | 5 | 25 | 1923 | N/A | TDD | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
| CA_n28-n39-n79 | n28 | 715 | 5 | 25 | 770 | N/A | FDD | N/A |
|  | n39 | 1902.5 | 5 | 25 | 1902.5 | N/A | TDD | N/A |
|  | n79 | 4520 | 40 | 216 | 4520 | 6.7 | TDD | IMD3 |
|  | n28 | 727.5 | 5 | 25 | 782.5 | N/A | FDD | N/A |
|  | n39 | 1902.5 | 5 | 25 | 1902.5 | N/A | TDD | N/A |
|  | n79 | 4980 | 40 | 216 | 4980 | 4.0 | TDD | IMD41 |
|  | n28 | 715.5 | 5 | 25 | 770.5 | N/A | FDD | N/A |
|  | n39 | 1898 | 5 | 25 | 1898 | 5.7 | TDD | IMD5 |
|  | n79 | 4760 | 40 | 216 | 4760 | N/A | TDD | N/A |
|  | n28 | 730 | 5 | 25 | 785 | 15.6 | FDD | IMD3 |
|  | n39 | 1887.5 | 5 | 25 | 1887.5 | N/A | TDD | N/A |
|  | n79 | 4560 | 40 | 216 | 4560 | N/A | TDD | N/A |
|  | n28 | 725 | 5 | 25 | 780 | 8.5 | FDD | IMD4 |
|  | n39 | 1900 | 5 | 25 | 1900 | N/A | TDD | N/A |
|  | n79 | 4920 | 40 | 216 | 4920 | N/A | TDD | N/A |
| CA_n28-n40-n41 | n28 | N/A | 5 | N/A | 765 | 7.6 | FDD | IMD4 |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | TDD | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
|  | n28 | 740 | 5 | 25 | 795 | N/A | TDD | N/A |
|  | n40 | 2380 | 5 | 25 | 2380 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2540 | 11.4 | TDD | IMD5 |
| CA_n28-n40-n71 | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n40 | 2350 | 5 | 25 | 2350 | N/A | TDD | N/A |
|  | n71 | N/A | 5 | N/A | 622 | 3.9 | FDD | IMD5 |
| CA_n28-n40-n77 | n28 | N/A | 5 | N/A | 800.5 | 11 | FDD | IMD31 |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | TDD | N/A |
|  | n77 | 3795 | 10 | 50 | 3795 | N/A | TDD | N/A |
|  | n28 | 708 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3726 | 26.7 | TDD | IMD32 |
|  | n28 | 708 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2390 | 12.3 | TDD | IMD3 |
|  | n77 | 3806 | 10 | 50 | 3806 | N/A | TDD | N/A |
| CA_n28-n40-n78 | n28 | N/A | 5 | N/A | 800.5 | 11 | FDD | IMD3 |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | TDD | N/A |
|  | n78 | 3795 | 10 | 50 | 3795 | N/A | N/A | N/A |
|  | n28 | 708 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | TDD | N/A |
|  | n78 | N/A | 10 | N/A | 3726 | 26.7 | TDD | IMD32 |
|  | n28 | 708 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2379 | 12.3 | TDD | IMD3 |
|  | n78 | 3795 | 10 | 50 | 3795 | N/A | TDD | N/A |
| CA_n28-n40-n79 | n28 | 730 | 5 | 25 | 785 | N/A | FDD | N/A |
|  | n40 | 2350 | 5 | 25 | 2350 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4540 | 10.7 | TDD | IMD4 |
|  | n28 | 720 | 5 | 25 | 775 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2340 | 9.2 | TDD | IMD4 |
|  | n79 | 4500 | 10 | 50 | 4500 | N/A | TDD | N/A |
| CA_n28-n41-n74 | n28 | N/A | 5 | N/A | 782 | 3.5 | FDD | IMD5 |
|  | n41 | 2569 | 10 | 50 | 2569 | N/A | TDD | N/A |
|  | n74 | 1452 | 5 | 25 | 1500 | N/A | FDD | N/A |
| CA_n28-n41-n77 | n28 | N/A | 5 | N/A | 798 | 29.3 | FDD | IMD24 |
|  | n41 | 2642 | 10 | 50 | 2642 | N/A | TDD | N/A |
|  | n77 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 782.5 | 3.0 | FDD | IMD5 |
|  | n41 | 2567.5 | 10 | 50 | 2567.5 | N/A | TDD | N/A |
|  | n77 | 3460 | 10 | 50 | 3460 | N/A | TDD | N/A |
|  | n28 | 738 | 5 | 25 | 793 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2642 | 27.5 | TDD | IMD2 |
|  | n77 | 3380 | 10 | 50 | 3380 | N/A | TDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n41 | 2580 | 10 | 50 | 2580 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3323 | 26.7 | TDD | IMD24 |
| CA_n28-n41-n78 | n28 | 738 | 5 | 25 | 793 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2642 | 27.5 | TDD | IMD2 |
|  | n78 | 3380 | 10 | 50 | 3380 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 798 | 29.3 | FDD | IMD21 |
|  | n41 | 2642 | 10 | 50 | 2642 | N/A | TDD | N/A |
|  | n78 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n41 | 2580 | 10 | 50 | 2580 | N/A | TDD | N/A |
|  | n78 | N/A | 10 | N/A | 3323 | 28.2 | TDD | IMD22 |
| CA_n28-n41-n79 | n28 | N/A | 5 | N/A | 780 | 13.0 | FDD | IMD31 |
|  | n41 | 2600 | 10 | 50 | 2600 | N/A | TDD | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |
|  | n28 | 720 | 5 | 25 | 775 | N/A | FDD | N/A |
|  | n41 | 2600 | 10 | 50 | 2600 | N/A | TDD | N/A |
|  | n79 | N/A | 40 | N/A | 4480 | 10.1 | TDD | IMD32 |
|  | n28 | 735 | 5 | 25 | 790 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2645 | 10.4 | TDD | IMD4 |
|  | n79 | 4850 | 40 | 216 | 4850 | N/A | TDD | N/A |
| CA_n28-n46-n78 | n28 | 710 | 5 | 25 | 765 | N/A | FDD | N/A |
|  | n46 | 5170 | 20 | 100 | 5170 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3750 | 17 | TDD | IMD31 |
|  | n28 | N/A | 5 | N/A | 780 | 16 | FDD | IMD3 |
|  | n46 | 5900 | 20 | 100 | 5900 | N/A | FDD | N/A |
|  | n78 | 3340 | 10 | 50 | 3340 | N/A | TDD | N/A |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n46 | N/A | 20 | N/A | 5900 | 22 | TDD | IMD31,2 |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
| CA_n28-n71-n77 | n28 | 713 | 5 | 25 | 768 | N/A | FDD | N/A |
|  | n71 | N/A | 5 | N/A | 628 | 4.7 | FDD | IMD5 |
|  | n77 | 3480 | 10 | 50 | 3480 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 778 | 4.3 | FDD | IMD5 |
|  | n71 | 688 | 5 | 25 | 642 | N/A | FDD | N/A |
|  | n77 | 3530 | 10 | 50 | 3530 | N/A | TDD | N/A |
| CA_n28-n74-n77 | n28 | 730 | 5 | 25 | 785 | N/A | FDD | N/A |
|  | n74 | N/A | 5 | 25 | 1500 | 9.0 | FDD | IMD4 |
|  | n77 | 3690 | 10 | 50 | 3690 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | 25 | 785 | 18.7 | FDD | IMD3 |
|  | n74 | 1452 | 5 | 25 | 1500 | N/A | FDD | N/A |
|  | n77 | 3689 | 10 | 50 | 3689 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | 25 | 785 | 13.2 | FDD | IMD4 |
|  | n74 | 1458 | 5 | 25 | 1506 | N/A | FDD | N/A |
|  | n77 | 3589 | 10 | 50 | 3589 | N/A | TDD | N/A |
| CA_n28-n77-n79 | n77 | 3620 | 10 | 50 | 3620 | N/A | TDD | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |
|  | n28 | 745 | 5 | 25 | 800 | 16.2 | FDD | IMD21,2 |
| CA_n28-n78-n79 | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n78 | 3700 | 10 | 50 | 3700 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4440 | 26.2 | TDD | IMD2 |
|  | n28 | 708 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4721 | 5.1 | TDD | IMD31 |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3700 | 26.9 | TDD | IMD23 |
|  | n79 | 4440 | 10 | 50 | 4440 | N/A | TDD | N/A |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3310 | 4.6 | TDD | IMD3 |
|  | n79 | 4790 | 10 | 50 | 4790 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 800 | 16.2 | FDD | IMD21 |
|  | n78 | 3620 | 10 | 50 | 3620 | N/A | TDD | N/A |
|  | n79 | 4420 | 10 | 50 | 4420 | N/A | TDD | N/A |
| CA_n28-n78-n102 | n28 | 710 | 5 | 25 | 765 | N/A | FDD | N/A |
|  | n78 | 3380 | 5 | 25 | 3380 | N/A | TDD | N/A |
|  | n102 | N/A | 40 | N/A | 6050 | 22 | TDD | IMD31,2 |
|  | n28 | 730 | 5 | 25 | 785 | N/A | FDD | N/A |
|  | n78 | N/A | 5 | N/A | 3755 | 10.3 | TDD | IMD41 |
|  | n102 | 5945 | 40 | 216 | 5945 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 775 | 16 | FDD | IMD31,2 |
|  | n78 | 3395 | 5 | 25 | 3395 | N/A | TDD | N/A |
|  | n102 | 6015 | 40 | 216 | 6015 | N/A | TDD | N/A |
| CA_n29-n30-n66 | n29 | N/A | 5 | N/A | 719.5 | 4.5 | SDL | IMD5 |
|  | n30 | 2307.5 | 5 | 25 | 2352.5 | N/A | FDD | N/A |
|  | n66 | 1777.5 | 5 | 25 | 2177.5 | N/A | FDD | N/A |
| CA_n29-n30-n77 | n29 | N/A | 5 | N/A | 722 | 15.2 | SDL | IMD31 |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | 3898 | 10 | 50 | 3898 | N/A | TDD | N/A |
| CA_n29-n66-n77 | n29 | N/A | 5 | N/A | 722 | 15.2 | SDL | IMD35 |
|  | n66 | 1734 | 5 | 25 | 2134 | N/A | FDD | N/A |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | TDD | N/A |
| CA_n30-n66-n77 | n30 | N/A | 5 | N/A | 2355 | 29.2 | FDD | IMD25 |
|  | n66 | 1745 | 5 | 25 | 2145 | N/A | FDD | N/A |
|  | n77 | 4100 | 10 | 50 | 4100 | N/A | TDD | N/A |
|  | n30 | N/A | 5 | N/A | 2355 | 3.4 | FDD | IMD5 |
|  | n66 | 1735 | 5 | 25 | 2135 | N/A | FDD | N/A |
|  | n77 | 3780 | 10 | 50 | 3780 | N/A | TDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2160 | 8.7 | FDD | IMD45 |
|  | n77 | 3390 | 10 | 50 | 3390 | N/A | TDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n66 | 1745 | 5 | 25 | 2145 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 4055 | 28.4 | TDD | IMD21,5 |
| CA_n34-n39-n40 | n34 | 2022.5 | 5 | 25 | 2022.5 | N/A | TDD | N/A |
|  | n39 | 1882.5 | 5 | 25 | 1882.5 | N/A | TDD | N/A |
|  | n40 | N/A | 5 | N/A | 2302.5 | 2.4 | TDD | IMD5 |
| CA_n34-n40-n41 | n34 | N/A | 5 | N/A | 2015 | 18.3 | TDD | IMD31 |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | TDD | N/A |
|  | n41 | 2590 | 10 | 50 | 2590 | N/A | TDD | N/A |
|  | n34 | 2020 | 5 | 25 | 2020 | N/A | TDD | N/A |
|  | n40 | 2320 | 5 | 25 | 2320 | N/A | TDD | N/A |
|  | n41 | 2620 | 10 | 50 | 2620 | 16.5 | TDD | IMD3 |
| CA_n34-n41-n79 | n34 | 2020 | 5 | 25 | 2020 | N/A | TDD | N/A |
|  | n41 | 2660 | 10 | 50 | 2660 | N/A | TDD | N/A |
|  | n79 | N/A | 40 | 216 | 4680 | 17.8 | TDD | IMD2 |
|  | n34 | 2020 | 5 | 25 | 2020 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | 50 | 2550 | 25.2 | TDD | IMD2 |
|  | n79 | 4570 | 40 | 216 | 4570 | N/A | TDD | N/A |
|  | n34 | N/A | 5 | 25 | 2015 | 27.1 | TDD | IMD2 |
|  | n41 | 2585 | 10 | 50 | 2585 | N/A | TDD | N/A |
|  | n79 | 4600 | 40 | 216 | 4600 | N/A | TDD | N/A |
|  | n34 | N/A | 5 | 25 | 2015 | 6.5 | TDD | IMD5 |
|  | n41 | 2515 | 10 | 50 | 2515 | N/A | TDD | N/A |
|  | n79 | 4780 | 40 | 216 | 4780 | N/A | TDD | N/A |
| CA_n38-n66-n78 | n38 | 2550 | 5 | 25 | 2550 | N/A | TDD | N/A |
|  | n66 | N/A | 5 | N/A | 2150 | 8.7 | FDD | IMD4 |
|  | n78 | 3625 | 10 | 50 | 3625 | N/A | TDD | N/A |
|  | n38 | 2610 | 5 | 25 | 2610 | N/A | TDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3460 | 15.0 | TDD | IMD3 |
| CA_n39-n40-n41 | n39 | 1917.5 | 5 | 25 | 1917.5 | N/A | TDD | N/A |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2685 | 30.3 | TDD | IMD3 |
|  | n39 | N/A | 5 | N/A | 1915 | 27.4 | TDD | IMD31 |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | TDD | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
| CA_n39-n40-n79 | n39 | 1917.5 | 5 | 25 | 1917.5 | N/A | TDD | N/A |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | TDD | N/A |
|  | n79 | N/A | 40 | N/A | 4980 | 5.8 | TDD | IMD4 |
| CA_n39-n41-n79 | n39 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n41 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n79 | N/A | N/A | N/A | N/A | N/A | TDD | IMD29 |
|  | n39 | 1900 | 5 | 25 | 1900 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2650 | 29.8 | TDD | IMD21 |
|  | n79 | 4550 | 10 | 50 | 4550 | N/A | TDD | N/A |
|  | n39 | N/A | 5 | N/A | 1900 | 28.9 | TDD | IMD21 |
|  | n41 | 2650 | 10 | 50 | 2650 | N/A | TDD | N/A |
|  | n79 | 4550 | 10 | 50 | 4550 | N/A | TDD | N/A |
| CA_n40-n41-n79 | n40 | 2340 | 5 | 25 | 2340 | N/A | TDD | N/A |
|  | n41 | 2600 | 10 | 50 | 2600 | N/A | TDD | N/A |
|  | n79 | N/A | 40 | N/A | 4940 | 30.5 | TDD | IMD2 |
|  | n40 | N/A | 5 | N/A | 2335 | 31 | TDD | IMD21 |
|  | n41 | 2545 | 10 | 50 | 2545 | N/A | TDD | N/A |
|  | n79 | 4880 | 40 | 216 | 4880 | N/A | TDD | N/A |
|  | n40 | 2340 | 5 | 25 | 2340 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2600 | 29.4 | TDD | IMD21 |
|  | n79 | 4880 | 40 | 216 | 4940 | N/A | TDD | N/A |
| CA_n40-n71-n77 | n40 | 2320 | 5 | 25 | 2320 | N/A | TDD | N/A |
|  | n71 | N/A | 5 | N/A | 630 | 18.7 | FDD | IMD32 |
|  | n77 | 3580 | 10 | 50 | 3580 | N/A | TDD | N/A |
|  | n40 | 2330 | 5 | 25 | 2330 | N/A | TDD | N/A |
|  | n71 | 688 | 5 | 25 | 642 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3706 | 16.3 | TDD | IMD31 |
|  | n40 | N/A | 5 | N/A | 2354 | 15.5 | TDD | IMD3 |
|  | n71 | 688 | 5 | 25 | 642 | N/A | FDD | N/A |
|  | n77 | 3730 | 10 | 50 | 3530 | N/A | TDD | N/A |
| CA_n40-n78-n79 | n40 | 2350 | 5 | 25 | 2350 | N/A | TDD | N/A |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4450 | 23 | TDD | IMD3 |
|  | n40 | N/A | 5 | N/A | 2350 | 12 | TDD | IMD31 |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n79 | 4450 | 10 | 50 | 4450 | N/A | TDD | N/A |
|  | n40 | N/A | 5 | N/A | 2320 | 6 | TDD | IMD4 |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n79 | 4560 | 10 | 50 | 4560 | N/A | TDD | N/A |
| CA_n40-n78-n105 | n40 | 2310 | 5 | 25 | 2310 | N/A | TDD | N/A |
|  | n78 | 3789 | 10 | 50 | 3789 | N/A | TDD | N/A |
|  | n105 | N/A | 5 | N/A | 648 | 3.3 | FDD | IMD5 |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | TDD | N/A |
|  | n78 | N/A | 10 | N/A | 3708 | 16 | TDD | IMD3 |
|  | n105 | 699 | 5 | 25 | 648 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2310 | 15.7 | TDD | IMD3 |
|  | n78 | 3708 | 10 | 50 | 3708 | N/A | TDD | N/A |
|  | n105 | 699 | 5 | 25 | 648 | N/A | FDD | N/A |
| CA_n41-n66-n77 | n41 | 2600 | 10 | 50 | 2600 | N/A | TDD | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3470 | 14.6 | TDD | IMD31,2 |
|  | n41 | N/A | 10 | N/A | 2670 | 4.2 | TDD | IMD55 |
|  | n66 | 1715 | 5 | 25 | 2115 | N/A | FDD | N/A |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | TDD | N/A |
|  | n41 | 2640 | 10 | 50 | 2640 | N/A | TDD | N/A |
|  | n66 | N/A | 5 | N/A | 2160 | 8.0 | FDD | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
| CA_n41-n66-n78 | n41 | 2560 | 10 | 50 | 2560 | N/A | TDD | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3390 | 14.6 | TDD | IMD31 |
|  | n41 | 2530 | 10 | 50 | 2530 | N/A | TDD | N/A |
|  | n66 | N/A | 5 | N/A | 2160 | 8.0 | FDD | IMD4 |
|  | n78 | 3610 | 10 | 50 | 3610 | N/A | TDD | N/A |
| CA_n41-n66-n85 | n41 | 2501 | 10 | 50 | 2501 | N/A | TDD | N/A |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | FDD | N/A |
|  | n85 | N/A | 5 | N/A | 731 | 29.5 | FDD | IMD21 |
| CA_n41-n70-n78 | n41 | 2655 | 10 | 50 | 2655 | N/A | TDD | N/A |
|  | n70 | N/A | 5 | N/A | 2000 | 17.6 | FDD | IMD3 |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | TDD | N/A |
|  | n41 | 2565 | 10 | 50 | 2565 | N/A | TDD | N/A |
|  | n70 | N/A | 5 | N/A | 2000 | 8.6 | FDD | IMD4 |
|  | n78 | 3565 | 10 | 50 | 3565 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2480 | 5.3 | TDD | IMD5 |
|  | n70 | 1700 | 5 | 25 | 2000 | N/A | FDD | N/A |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n41 | 2545 | 10 | 50 | 2545 | N/A | FDD | N/A |
|  | n70 | 1700 | 5 | 25 | 2000 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3390 | 16.1 | TDD | IMD3 |
| CA_n41-n71-n77 | n41 | 2615 | 10 | 50 | 2615 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3308 | 27.7 | TDD | IMD21,2,5 |
|  | n41 | 2564 | 10 | 50 | 2564 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3950 | 14.8 | TDD | IMD31 |
|  | n41 | N/A | 10 | N/A | 2615 | 26.7 | TDD | IMD25 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3308 | 10 | 50 | 3308 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2564 | 13.5 | TDD | IMD3 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | TDD | N/A |
|  | n41 | 2680 | 10 | 50 | 2680 | N/A | TDD | N/A |
|  | n71 | N/A | 5 | N/A | 640 | 29.3 | FDD | IMD25 |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
| CA_n41-n71-n78 | n41 | 2615 | 10 | 50 | 2615 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3308 | 27.6 | TDD | IMD21,2 |
|  | n41 | N/A | 10 | N/A | 2615 | 28.7 | TDD | IMD2 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n78 | 3308 | 10 | 50 | 3308 | N/A | TDD | N/A |
|  | 41 | 2642 | 10 | 50 | 2642 | N/A | TDD | N/A |
|  | n71 | N/A | 5 | N/A | 798 | 29.3 | FDD | IMD2 |
|  | n78 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
| CA_n41-n74-n77 | n41 | 2650 | 10 | 50 | 2650 | N/A | TDD | N/A |
|  | n74 | N/A | 5 | N/A | 1500 | 29.2 | FDD | IMD22 |
|  | n77 | 4150 | 10 | 50 | 4150 | NA | TDD | N/A |
|  | n41 | 2525 | 10 | 50 | 2525 | N/A | TDD | N/A |
|  | n74 | N/A | 5 | N/A | 1500 | 15 | FDD | IMD3 |
|  | n77 | 3550 | 10 | 50 | 3550 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2650 | 28 | TDD | IMD2 |
|  | n74 | 1450 | 5 | 25 | 1500 | N/A | FDD | N/A |
|  | n77 | 4100 | 10 | 50 | 4150 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2518 | 3 | TDD | IMD5 |
|  | n74 | 1454 | 5 | 25 | 1502 | N/A | FDD | N/A |
|  | n77 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
|  | n41 | 2650 | 10 | 50 | 2650 | N/A | TDD | N/A |
|  | n74 | 1450 | 5 | 25 | 1500 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 4100 | 27 | TDD | IMD2 |
|  | n41 | 2525 | 10 | 50 | 2525 | N/A | TDD | N/A |
|  | n74 | 1450 | 5 | 25 | 1498 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3600 | 14 | TDD | IMD31 |
| CA_n41-n77-n79 | n77 | 3600 | 10 | 50 | 3600 | N/A | TDD | N/A |
|  | n79 | 4600 | 40 | 216 | 4600 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2600 | 10.7 | TDD | IMD31,2 |
| CA_n41-n77-n85 | n41 | 2687 | 10 | 50 | 2687 | N/A | TDD | N/A |
|  | n77 | 3420 | 10 | 50 | 3420 | N/A | TDD | N/A |
|  | n85 | N/A | 5 | N/A | 733 | 29.3 | FDD | IMD25 |
|  | n41 | N/A | 10 | N/A | 2 619 | 27.5 | TDD | IMD24.5 |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
|  | n85 | 701 | 5 | 25 | 731 | N/A | FDD | N/A |
|  | n41 | 2680 | 10 | 50 | 2680 | N/A | TDD | N/A |
|  | n77 | 3393 | 10 | N/A | 3393 | 26.7 | TDD | IMD24,5 |
|  | n85 | 713 | 5 | 25 | 743 | N/A | FDD | N/A |
| CA_n46-n78-n102 | n46 | 5315 | 20 | 100 | 5315 | N/A | TDD | N/A |
|  | n78 | 3770 | 10 | 50 | 3770 | N/A | TDD | N/A |
|  | n102 | N/A | 40 | N/A | 5995 | N/A12 | TDD | IMD4 |
|  | n46 | N/A | 10 | N/A | 5530 | N/A12 | TDD | IMD4 |
|  | n78 | 3550 | 10 | 50 | 3550 | N/A | TDD | N/A |
|  | n102 | 6315 | 40 | 216 | 6315 | N/A | TDD | N/A |
| CA_n48-n66-n70 | n48 | 3625 | 10 | 50 | 3625 | N/A | TDD | N/A |
|  | n66 | N/A | 5 | N/A | 2142.5 | 2.8 | FDD | IMD5 |
|  | n70 | 1702.5 | 5 | 25 | 2002.5 | N/A | FDD | N/A |
|  | n48 | 3645 | 10 | 50 | 3645 | N/A | TDD | N/A |
|  | n66 | 1762.5 | 5 | 25 | 2162.5 | N/A | FDD | N/A |
|  | n70 | N/A | 5 | N/A | 2002.5 | 3.1 | FDD | IMD5 |
| CA_n48-n66-n71 | n48 | 3552.5 | 10 | 50 | 3552.5 | N/A | TDD | N/A |
|  | n66 | N/A | 5 | N/A | 2161.5 | 14.4 | FDD | IMD3 |
|  | n71 | 695.5 | 5 | 25 | 649.5 | N/A | FDD | N/A |
|  | n48 | N/A | 10 | N/A | 3695 | 5.2 | TDD | IMD4 |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | N/A | FDD | N/A |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | FDD | N/A |
| CA_n48-n70-n71 | n48 | N/A | 10 | N/A | 3694 | 9 | TDD | IMD41 |
|  | n70 | 1697.5 | 5 | 25 | 1997.5 | N/A | FDD | N/A |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | FDD | N/A |
| CA_n48-n71-n77 | n48 | N/A | N/A | N/A | N/A | N/A | FDD | N/A |
|  | n71 | N/A | N/A | N/A | N/A | N/A | FDD | N/A |
|  | n77 | N/A | N/A | N/A | N/A | N/A | FDD | IMD25 |
|  | n48 | N/A | N/A | N/A | N/A | N/A | FDD | IMD25 |
|  | n71 | N/A | N/A | N/A | N/A | N/A | FDD | N/A |
|  | n77 | N/A | N/A | N/A | N/A | N/A | FDD | N/A |
| CA_n66-n70-n77 | n66 | 1757.5 | 5 | 25 | 2157.5 | N/A | FDD | N/A |
|  | n70 | N/A | 5 | N/A | 2007.5 | 32.1 | FDD | IMD22,1 |
|  | n77 | 3765 | 10 | 50 | 3765 | N/A | TDD | N/A |
|  | n66 | N/A | 5 | N/A | 2162.5 | 29.2 | FDD | IMD21 |
|  | n70 | 1702.5 | 5 | 25 | 2002.5 | N/A | FDD | N/A |
|  | n77 | 3865 | 10 | 50 | 3865 | N/A | TDD | N/A |
| CA_n66-n70-n78 | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n70 | N/A | 5 | N/A | 2000 | 32.1 | FDD | IMD22 |
|  | n78 | 3760 | 10 | 50 | 3760 | N/A | TDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n70 | N/A | 5 | N/A | 2000 | 2.1 | FDD | IMD5 |
|  | n78 | 3640 | 10 | 50 | 3640 | N/A | TDD | N/A |
|  | n66 | N/A | 5 | N/A | 2160 | 5.0 | FDD | IMD5 |
|  | n70 | 1700 | 5 | 25 | 2000 | N/A | FDD | N/A |
|  | n78 | 3630 | 10 | 50 | 3630 | N/A | TDD | N/A |
| CA_n66-n71-n77 | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n71 | 668 | 5 | 25 | 622 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 4108 | 15.9 | TDD | IMD31,2,5 |
|  | n66 | N/A | 5 | N/A | 2150 | 15.5 | FDD | IMD32 |
|  | n71 | 690 | 5 | 25 | 644 | N/A | FDD | N/A |
|  | n77 | 3530 | 10 | 50 | 3530 | N/A | TDD | N/A |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n71 | N/A | 5 | N/A | 640 | 15.3 | FDD | IMD35 |
|  | n77 | 4080 | 10 | 50 | 4080 | N/A | TDD | N/A |
| CA_n66-n71-n78 | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n71 | 668 | 5 | 25 | 622 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3724 | 9 | TDD | IMD41 |
|  | n66 | N/A | 5 | N/A | 2160 | 15.5 | FDD | IMD3 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n78 | 3546 | 10 | 50 | 3546 | N/A | TDD | N/A |
| CA_n66-n77-n85 | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n77 | 4180 | 10 | 50 | 4180 | N/A | TDD | N/A |
|  | n85 | N/A | 5 | N/A | 740 | 23.5 | FDD | IMD35 |
|  | n66 | N/A | 5 | N/A | 2124 | 21.4 | FDD | IMD3 |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | TDD | N/A |
|  | n85 | 708 | 5 | 25 | 738 | N/A | FDD | N/A |
| CA_n70-n71-n77 | n70 | N/A | N/A | N/A | N/A | N/A | FDD | N/A |
|  | n71 | N/A | N/A | N/A | N/A | N/A | FDD | N/A |
|  | n77 | N/A | N/A | N/A | N/A | N/A | TDD | IMD31,5 |
|  | n70 | 1702.5 | 5 | 25 | 2002.5 | N/A | FDD | N/A |
|  | n71 | 680.5 | 5 | 25 | 834.5 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3745 | 8.2 | TDD | IMD4 |
|  | n70 | N/A | N/A | N/A | N/A | N/A | FDD | IMD35 |
|  | n71 | N/A | N/A | N/A | N/A | N/A | FDD | N/A |
|  | n77 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n70 | N/A | N/A | N/A | N/A | N/A | FDD | IMD45 |
|  | n71 | N/A | N/A | N/A | N/A | N/A | FDD | N/A |
|  | n77 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE 4: This band is subject to IMD3 also which MSD is not specified. NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 6: Void. NOTE 7: Void. NOTE 8: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,f,c) as defined in clause 6.2A.4 NOTE 9: There is no IMD2 product in band n79 downlink for n79 operating in 4800 – 5000 MHz frequency range. NOTE 10: This band supports intra-band non-contiguous uplink configuration. NOTE 11: This MSD requirement applies with both IMD2 and IMD3 products should be generated. NOTE 12: This is a share spectrum access band, hence no MSD is defined. NOTE 13: This band is also subject to a near missed IMD2 that is not specified and is not applicable for band n77 spectrum ranges of 3450-3550MHz and 3700-3980MHz. |  |  |  |  |  |  |  |  |

Table 7.3A.5-2a: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n3-n77 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1807.5 | 37.5 | FDD | IMD21,2 |
|  | n77 | 3757.5 | 10 | 50 | 3757.5 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 37.0 | FDD | IMD21 |
|  | n3 | 1775 | 5 | 25 | 1870 | N/A | FDD | N/A |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | TDD | N/A |
| CA_n1-n3-n28 | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |
|  | n1 | N/A | 5 | N/A | 2139 | 22.7 | FDD | IMD4 |
| CA_n1-n3-n78 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1830 | 33.9 | FDD | IMD2 |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | TDD | N/A |
| CA_n1-n3-n79 | n3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |
|  | n79 | 4860 | 40 | 216 | 4860 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 18.7 | FDD | IMD5 |
| CA_n1-n7-n20 | n1 | 1940 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n7 | 2510 | 10 | 50 | 2630 | N/A | FDD | N/A |
|  | n20 | N/A | 5 | N/A | 800 | 17.7 | FDD | IMD5 |
| CA_n1-n7-n28 | n1 | 1935 | 5 | 25 | 2125 | N/A | FDD | N/A |
|  | n7 | 2510 | 10 | 50 | 2630 | N/A | FDD | N/A |
|  | n28 | N/A | 10 | N/A | 785 | 17,7 | FDD | IMD5 |
| CA_n1-n7-n78 | n1 | 1977.5 | 5 | 25 | 2167.5 | N/A | FDD | N/A |
|  | n7 | N/A | 5 | N/A | 2627.5 | 20.6 | FDD | IMD4 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 20.1 | FDD | IMD4 |
|  | n7 | 2510 | 10 | 50 | 2630 | N/A | FDD | N/A |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | TDD | N/A |
| CA_n1-n18-n41 | n1 | 1960 | 5 | 25 | 2150 | N/A | FDD | N/A |
|  | n41 | 2505 | 10 | 50 | 2505 | N/A | TDD | N/A |
|  | n18 | N/A | 5 | N/A | 870 | 12.4 | FDD | IMD5 |
| CA_n1-n18-n77 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 3390 | 10 | 50 | 3390 | N/A | TDD | N/A |
|  | n18 | N/A | 5 | N/A | 870 | 18.6 | FDD | IMD5 |
|  | n1 | N/A | 5 | N/A | 2120 | 25.4 | FDD | IMD3 |
|  | n18 | 825 | 5 | 25 | 870 | N/A | FDD | N/A |
|  | n77 | 3770 | 10 | 50 | 3770 | N/A | TDD | N/A |
| CA_n1-n26-n78 | n1 | N/A | 5 | N/A | 2122 | 27.0 | FDD | IMD3 |
|  | n26 | 829 | 5 | 25 | 874 | N/A | FDD | N/A |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | TDD | N/A |
|  | n1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |
|  | n26 | N/A | 5 | N/A | 885 | 15.3 | FDD | IMD5 |
|  | n78 | 3405 | 10 | 50 | 3405 | N/A | TDD | N/A |
| CA_n1-n28-n41 | n1 | 1923 | 5 | 25 | 2113 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 762 | 36.6 | FDD | IMD21 |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
| CA_n1-n28-n77 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 790 | 16.5 | FDD | IMD5 |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n77 | 3630 | 10 | 50 | 3630 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2150 | 24.7 | FDD | IMD3 |
| CA_n1-n28-n78 | n1 | N/A | 5 | N/A | 2150 | 24.6 | FDD | IMD3 |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n78 | 3630 | 10 | 50 | 3630 | N/A | TDD | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 794 | 17.2 | FDD | IMD5 |
|  | n78 | 3352 | 10 | 50 | 3352 | N/A | TDD | N/A |
| CA_n1-n28-n79 | n1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n79 | 4648 | 40 | 216 | 4648 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 788 | 24.2 | FDD | IMD32 |
|  | n28 | 745.5 | 5 | 25 | 800.5 | N/A | FDD | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2167.5 | 13.4 | FDD | IMD41 |
| CA_n1-n77-n79 | n1 | 1950 | 5 | 25 | 2140 | 24.6 | FDD | IMD31,2 |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n79 | 4660 | 40 | 216 | 4660 | N/A | TDD | N/A |
| CA_n1-n41-n77 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n41 | 2650 | 10 | 50 | 2650 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3330 | 28.2 | TDD | IMD31,2 |
|  | n1 | 1975 | 5 | 10 | 2165 | N/A | FDD | N/A |
|  | n77 | 3410 | 10 | 50 | 3410 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2515 | 22.0 | TDD | IMD41 |
|  | n41 | 2640 | 10 | 50 | 2640 | N/A | TDD | N/A |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 21.4 | FDD | IMD4 |
| CA_n1-n78-n79 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | TDD | N/A |
|  | n79 | N/A | 40 | N/A | 4870 | 24.9 | TDD | IMD31,3 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3490 | 17 | TDD | IMD53 |
|  | n79 | 4670 | 40 | 216 | 4670 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 24.6 | FDD | IMD31 |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n79 | 4660 | 40 | 216 | 4660 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 3.8 | FDD | IMD41 |
|  | n78 | 3590 | 10 | 50 | 3590 | N/A | TDD | N/A |
|  | n79 | 4660 | 40 | 216 | 4660 | N/A | TDD | N/A |
| CA_n2-n5-n77 | n2 | 1907.5 | 5 | 25 | 1987.5 | N/A | FDD | N/A |
|  | n5 | N/A | 5 | N/A | 887.5 | 13.6 | FDD | IMD55 |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n2 | N/A | 5 | N/A | 1987 | 24.8 | FDD | IMD35 |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n77 | 3680 | 10 | 50 | 3680 | N/A | TDD | N/A |
| CA_n2-n12-n77 | n2 | N/A | 5 | N/A | 1960 | 24,8 | FDD | IMD32,5 |
|  | n12 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
|  | n77 | 3375 | 10 | 50 | 3375 | N/A | TDD | N/A |
| CA_n2-n14-n77 | n2 | N/A | 5 | N/A | 1954 | 24.8 | FDD | IMD3 |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | TDD | N/A |
| CA_n2-n30-n77 | n2 | N/A | 5 | N/A | 1986 | 19.3 | FDD | IMD45 |
|  | n30 | 2312 | 5 | 25 | 2357 | N/A | FDD | N/A |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n2 | 1905 | 5 | 25 | 1985 | N/A | FDD | N/A |
|  | n30 | N/A | 5 | N/A | 2354 | 22.2 | FDD | IMD45 |
|  | n77 | 3361 | 10 | 50 | 3361 | N/A | TDD | N/A |
|  | n2 | 1860 | 5 | 25 | 1940 | N/A | FDD | N/A |
|  | n30 | N/A | 5 | N/A | 2354 | 12.9 | FDD | IMD5 |
|  | n77 | 3967 | 10 | 50 | 3967 | N/A | TDD | N/A |
| CA_n2-n66-n77 | n2 | 1855 | 5 | 25 | 1935 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2115 | 34.7 | FDD | IMD21,2 |
|  | n77 | 3970 | 10 | 50 | 3970 | N/A | TDD | N/A |
|  | n2 | N/A | 5 | N/A | 1960 | 37.6 | FDD | IMD21,2 |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
| CA_n3-n7-n20 | n3 | 1747 | 5 | 25 | 1842 | N/A | FDD | N/A |
|  | n7 | 2543 | 10 | 50 | 2663 | N/A | FDD | N/A |
|  | n20 | N/A | 5 | N/A | 796 | 26.0 | FDD | IMD2 |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n7 | N/A | 10 | N/A | 2625 | 35.0 | FDD | IMD2 |
|  | n20 | 845 | 5 | 25 | 804 | N/A | FDD | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |
|  | n7 | N/A | 5 | 25 | N/A | 25.9 | FDD | IMD3 |
|  | n20 | 835 | 5 | 25 | 794 | N/A | FDD | N/A |
| CA_n3-n7-n28 | n3 | 1747 | 5 | 25 | 1842 | N/A | FDD | N/A |
|  | n7 | 2543 | 5 | 25 | 2663 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 796 | 26.0 | FDD | IMD2 |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | FDD | N/A |
|  | n7 | N/A | 5 | N/A | 2682 | 25.9 | FDD | IMD3 |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1832.5 | 32 | FDD | IMD2 |
|  | n7 | 2543 | 5 | 25 | 2663 | N/A | FDD | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |
| CA_n3-n7-n78 | n3 | N/A | 5 | N/A | 1820 | 26.6 | FDD | IMD3 |
|  | n7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1820 | 17 | FDD | IMD4 |
|  | n7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n78 | 3475 | 10 | 50 | 3475 | N/A | TDD | N/A |
| CA_n3-n18-n41 | n3 | 1765 | 5 | 25 | 1860 | N/A | FDD | N/A |
|  | n18 | N/A | 5 | N/A | 865 | 34.9 | FDD | IMD2 |
|  | n41 | 2630 | 10 | 50 | 2630 | N/A | TDD | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n18 | N/A | 5 | N/A | 862.5 | 26.5 | FDD | IMD3 |
|  | n41 | 2597.5 | 10 | 50 | 2597.5 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1850 | 34.8 | FDD | IMD2 |
|  | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n41 | 2670 | 10 | 50 | 2670 | N/A | TDD | N/A |
| CA_n3-n18-n77 | n18 | 820 | 5 | 25 | 865 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1865 | 24.2 | FDD | IMD3 |
|  | n77 | 3505 | 10 | 50 | 3505 | N/A | TDD | N/A |
| CA_n3-n26-n78 | n3 | N/A | 5 | N/A | 1862 | 24.6 | FDD | IMD3 |
|  | n26 | 839 | 5 | 25 | 884 | N/A | FDD | N/A |
|  | n78 | 3540 | 10 | 50 | 3540 | N/A | TDD | N/A |
| CA_n3-n28-n41 | n3 | 1760 | 5 | 25 | 1855 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 790 | 30.5 | FDD | IMD24 |
|  | n41 | 2550 | 10 | 50 | 2550 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1832.5 | 32 | FDD | IMD2 |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |
|  | n41 | 2543 | 10 | 50 | 2543 | N/A | TDD | N/A |
| CA_n3-n28-n77 | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 770 | 24.2 | FDD | IMD3 |
|  | n77 | 4195 | 10 | 50 | 4195 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1850 | 25.8 | FDD | IMD32 |
|  | n28 | 735 | 5 | 25 | 790 | N/A | FDD | N/A |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
| CA_n3-n28-n78 | n3 | N/A | 5 | N/A | 1850 | 29.2 | FDD | IMD3 |
|  | n28 | 735 | 5 | 25 | 790 | N/A | FDD | N/A |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
| CA_n3-n28-n79 | n3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n79 | 4530 | 40 | 216 | 4530 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 780 | 21.5 | FDD | IMD4 |
|  | n28 | 725 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1870 | 20.5 | FDD | IMD5 |
| CA_n3-n41-n77 | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n41 | 2580 | 10 | 50 | 2580 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3440 | 24.3 | TDD | IMD31 |
|  | n3 | N/A | 5 | N/A | 1840 | 23.9 | FDD | IMD32 |
|  | n41 | 2620 | 10 | 50 | 2620 | N/A | TDD | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2640 | 16.7 | TDD | IMD5 |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | TDD | N/A |
| CA_n3-n41-n79 | n3 | 1755 | 5 | 25 | 1850 | 35.4 | FDD | IMD21 |
|  | n41 | 2570 | 10 | 50 | 2570 | N/A | TDD | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n41 | 2670 | 10 | 50 | 2670 | 36.2 | TDD | IMD21 |
|  | n79 | 4440 | 40 | 216 | 4440 | N/A | TDD | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n41 | 2670 | 10 | 50 | 2670 | N/A | TDD | N/A |
|  | n79 | 4440 | 40 | 216 | 4440 | 36.8 | TDD | IMD21 |
| CA_n3-n77-n79 | n77 | 3350 | 10 | 50 | 3350 | N/A | FDD | N/A |
|  | n79 | 4840 | 40 | 216 | 4840 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 24.2 | TDD | IMD31, 2 |
| CA_n3-n78-n79 | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4860 | 22.1 | TDD | IMD3 |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3480 | 8.5 | TDD | IMD5 |
|  | n79 | 4410 | 10 | 50 | 4410 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 24.7 | FDD | IMD31 |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
|  | n79 | 4840 | 10 | 50 | 4840 | N/A | TDD | N/A |
|  | n3 | 1745 | 5 | 25 | 1840 | 11.5 | FDD | IMD4 |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | TDD | N/A |
|  | n79 | 4700 | 10 | 50 | 4700 | N/A | TDD | N/A |
| CA_n5-n7-n77 | n5 | N/A | 5 | N/A | 879 | 35.2 | FDD | IMD21,4 |
|  | n7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n77 | 3429 | 10 | 50 | 3429 | N/A | TDD | N/A |
|  | n5 | 844 | 5 | 25 | 889 | N/A | FDD | N/A |
|  | n7 | N/A | 5 | N/A | 2645 | 35.1 | FDD | IMD2 |
|  | n77 | 3489 | 10 | 50 | 3489 | N/A | TDD | N/A |
| CA_n5-n7-n78 | n5 | N/A | 5 | N/A | 879 | 35.2 | FDD | IMD21 |
|  | n7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n78 | 3429 | 10 | 50 | 3429 | N/A | TDD | N/A |
|  | n5 | 844 | 5 | 25 | 889 | N/A | FDD | N/A |
|  | n7 | N/A | 5 | N/A | 2645 | 35.1 | FDD | IMD2 |
|  | n78 | 3489 | 10 | 50 | 3489 | N/A | TDD | N/A |
| CA_n5-n12-n77 | n5 | N/A | 5 | N/A | 880 | 14.0 | FDD | IMD5 |
|  | n12 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
|  | n5 | 835 | 5 | 25 | 880 | N/A | FDD | N/A |
|  | n12 | N/A | 5 | N/A | 740 | 14.9 | FDD | IMD55 |
|  | n77 | 4080 | 10 | 50 | 4080 | N/A | TDD | N/A |
| CA_n5-n14-n77 | n5 | N/A | 5 | N/A | 880 | 14.0 | FDD | IMD55 |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n77 | 4052 | 10 | 50 | 4052 | N/A | TDD | N/A |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n14 | N/A | 5 | N/A | 765.5 | 20.3 | FDD | IMD41.5 |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
| CA_n5-n25-n77 | n5 | N/A | 5 | N/A | 889 | 13.6 | FDD | IMD55 |
|  | n25 | 1907 | 5 | 25 | 1987 | N/A | FDD | N/A |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n25 | N/A | 5 | N/A | 1987 | 24.8 | FDD | IMD3 |
|  | n77 | 3680 | 10 | 25 | 3680 | N/A | TDD | N/A |
| CA_n5-n25-n78 | n5 | N/A | 5 | N/A | 889 | 13.6 | FDD | IMD5 |
|  | n25 | 1907 | 5 | 25 | 1987 | N/A | FDD | N/A |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n25 | N/A | 5 | N/A | 1987 | 24.8 | FDD | IMD3 |
|  | n78 | 3680 | 10 | 50 | 3680 | N/A | TDD | N/A |
| CA_n5-n29-n77 | n5 | 835 | 5 | 25 | 880 | N/A | FDD | N/A |
|  | n29 | N/A | 5 | N/A | 722 | 14.9 | FDD | IMD55 |
|  | n77 | 4062 | 10 | 50 | 4062 | N/A | TDD | N/A |
| CA_n5-n30-n77 | n5 | N/A | 5 | N/A | 880 | 23.5 | FDD | IMD31 |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | 3740 | 10 | 50 | 3740 | N/A | TDD | N/A |
|  | n5 | 835 | 5 | 25 | 880 | N/A | FDD | N/A |
|  | n30 | N/A | 5 | N/A | 2355 | 21.4 | FDD | IMD35 |
|  | n77 | 4025 | 10 | 50 | 4025 | N/A | TDD | N/A |
| CA_n5-n66-n77 | n5 | 826.5 | 5 | 25 | 871.5 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2142 | 22.2 | FDD | IMD3 |
|  | n77 | 3795 | 10 | 50 | 3795 | N/A | TDD | N/A |
| CA_n7-n25-n77 | n7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n25 | N/A | 5 | N/A | 1950 | 20.0 | FDD | IMD4 |
|  | n77 | 3525 | 10 | 50 | 3525 | N/A | TDD | N/A |
|  | n7 | N/A | 5 | N/A | 2640 | 18.8 | FDD | IMD5 |
|  | n25 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
|  | n77 | 4125 | 10 | 50 | 4125 | N/A | TDD | N/A |
| CA_n7-n66-n77 | n7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2150 | 20.5 | FDD | IMD4 |
|  | n77 | 3625 | 10 | 50 | 3625 | N/A | TDD | N/A |
|  | n7 | N/A | 5 | N/A | 2640 | 18.8 | FDD | IMD5 |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | TDD | N/A |
| CA_n7-n71-n77 | n7 | N/A | 5 | N/A | 2670 | 34.6 | FDD | IMD2 |
|  | n71 | 680 | 5 | 25 | 634 | N/A | FDD | N/A |
|  | n77 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
| CA_n7-n26-n78 | n7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n26 | N/A | 5 | N/A | 879 | 36.2 | FDD | IMD2 |
|  | n78 | 3429 | 10 | 50 | 3429 | N/A | TDD | N/A |
|  | n7 | 2525 | 5 | 25 | 2645 | N/A | FDD | N/A |
|  | n26 | N/A | 5 | N/A | 875 | 15.7 | FDD | IMD5 |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
|  | n7 | N/A | 5 | N/A | 2645 | 36.1 | FDD | IMD2 |
|  | n26 | 844 | 5 | 25 | 889 | N/A | FDD | N/A |
|  | n78 | 3489 | 10 | 50 | 3489 | N/A | TDD | N/A |
| CA_n7-n28-n78 | n7 | 2567.5 | 5 | 25 | 2687.5 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 782.5 | 33.8 | FDD | IMD21 |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
|  | n7 | N/A | 5 | N/A | 2650 | 35.5 | FDD | IMD2 |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | N/A | TDD | N/A |
| CA_n7-n66-n78 | n7 | 2540 | 5 | 25 | 2660 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 20.5 | FDD | IMD4 |
|  | n78 | 3620 | 10 | 50 | 3620 | N/A | TDD | N/A |
| CA_n12-n30-n77 | n12 | N/A | 5 | N/A | 740 | 23.5 | FDD | IMD31 |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | 3880 | 10 | 50 | 3880 | N/A | TDD | N/A |
|  | n12 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
|  | n30 | N/A | 5 | N/A | 2355 | 21.4 | FDD | IMD3 |
|  | n77 | 3770 | 10 | 50 | 3770 | N/A | TDD | N/A |
| CA_n12-n66-n77 | n12 | N/A | 5 | N/A | 740 | 23.5 | FDD | IMD35 |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n77 | 4180 | 10 | 50 | 4180 | N/A | TDD | N/A |
|  | n12 | 707 | 5 | 25 | 737 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2126 | 21.4 | FDD | IMD3 |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | TDD | N/A |
| CA_n13-n25-n77 | n13 | 782 | 5 | 25 | 751 | N/A | FDD | N/A |
|  | n25 | N/A | 5 | N/A | 1960 | 24.0 | FDD | IMD3 |
|  | n77 | 3524 | 10 | 50 | 3524 | N/A | TDD | N/A |
| CA_n13-n66-n77 | n13 | 782 | 5 | 25 | 751 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2146 | 26.0 | FDD | IMD3 |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
|  | n13 | N/A | 5 | N/A | 750 | 24.1 | FDD | IMD3 |
|  | n66 | 1710 | 5 | 25 | 2110 | N/A | FDD | N/A |
|  | n77 | 4170 | 10 | 50 | 4170 | N/A | TDD | N/A |
| CA_n14-n30-n77 | n14 | N/A | 5 | N/A | 763 | 23.5 | FDD | IMD31 |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | 3857 | 10 | 50 | 3857 | N/A | TDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n30 | N/A | 5 | N/A | 2355 | 21.4 | FDD | IMD3 |
|  | n77 | 3941 | 10 | 50 | 3941 | N/A | TDD | N/A |
| CA_n14-n66-n77 | n14 | N/A | 5 | N/A | 763 | 23.5 | FDD | IMD35 |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | N/A | FDD | N/A |
|  | n77 | 4188 | 10 | 50 | 4188 | N/A | TDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2155 | 21.4 | FDD | IMD3 |
|  | n77 | 3741 | 10 | 50 | 3741 | N/A | TDD | N/A |
| CA_n18-n28-n41 | n18 | 825 | 5 | 25 | 870 | N/A | FDD | N/A |
|  | n41 | 2505 | 10 | 50 | 2505 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 795 | 16.9 | FDD | IMD5 |
| CA_n18-n28-n77 | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 778 | 17.5 | FDD | IMD5 |
|  | n77 | 4058 | 10 | 50 | 4058 | N/A | TDD | N/A |
|  | n18 | N/A | 5 | N/A | 865 | 12.1 | FDD | IMD5 |
|  | n28 | 723 | 5 | 25 | 778 | N/A | FDD | N/A |
|  | n77 | 3757 | 10 | 50 | 3757 | N/A | TDD | N/A |
| CA_n18-n41-n77 | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n41 | 2630 | 10 | 50 | 2630 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3450 | 34.6 | TDD | IMD22,4 |
|  | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2630 | 32.5 | TDD | IMD24 |
|  | n77 | 3450 | 10 | 50 | 3450 | N/A | TDD | N/A |
|  | n18 | N/A | 5 | N/A | 870 | 33.8 | FDD | IMD21,4 |
|  | n41 | 2590 | 10 | 50 | 2590 | N/A | TDD | N/A |
|  | n77 | 3460 | 10 | 50 | 3460 | N/A | TDD | N/A |
| CA_n25-n41-n66 | n25 | N/A | 5 | N/A | 1940 | 23.0 | FDD | IMD4 |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
|  | n66 | 1715 | 5 | 25 | 2115 | N/A | FDD | N/A |
| CA_n25-n41-n77 | n25 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
|  | n41 | 2670 | 10 | 50 | 2670 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3470 | 22.3 | TDD | IMD31 |
|  | n25 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2640 | 16.7 | TDD | IMD55 |
|  | n77 | 4125 | 10 | 50 | 4125 | N/A | TDD | N/A |
|  | n25 | N/A | 5 | N/A | 1950 | 25.1 | FDD | IMD35 |
|  | n41 | 2675 | 10 | 50 | 2675 | N/A | TDD | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n25 | N/A | 5 | N/A | 1950 | 18.3 | FDD | IMD4 |
|  | n41 | 2550 | 10 | 50 | 2685 | N/A | TDD | N/A |
|  | n77 | 3525 | 10 | 50 | 3525 | N/A | TDD | N/A |
| CA_n25-n66-n77 | n25 | 1855 | 5 | 25 | 1935 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2115 | 35.2 | FDD | IMD22 |
|  | n77 | 3970 | 10 | 50 | 3970 | N/A | TDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2160 | 16.4 | FDD | IMD5 |
|  | n77 | 3930 | 10 | 50 | 3930 | N/A | TDD | N/A |
|  | n25 | N/A | 5 | N/A | 1960 | 38.1 | FDD | IMD2 |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
|  | n25 | N/A | 5 | N/A | 1960 | 21.1 | FDD | IMD45 |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | FDD | N/A |
|  | n77 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
|  | n25 | N/A | 5 | N/A | 1960 | 13.0 | FDD | IMD55 |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3620 | 35.4 | TDD | IMD25 |
| CA_n25-n66-n78 | n25 | 1880 | 5 | 25 | 1960 | M/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | 21.1 | FDD | IMD4 |
|  | n78 | 3500 | 10 | 50 | 3500 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | 37.6 | FDD | IMD21,2 |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n78 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
| CA_n25-n71-n77 | n25 | 1907.5 | 5 | 25 | 1987.5 | N/A | FDD | N/A |
|  | n71 | 695.5 | 5 | 25 | 649.5 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3305 | 16.1 | TDD | IMD31,2,5 |
|  | n25 | N/A | 5 | N/A | 1954 | 25.5 | FDD | IMD32,5 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3340 | 10 | 50 | 3340 | N/A | TDD | N/A |
| CA_n28-n41-n74 | n28 | N/A | 5 | N/A | 782 | 17.5 | FDD | IMD5 |
|  | n41 | 2569 | 10 | 50 | 2569 | N/A | TDD | N/A |
|  | n74 | 1452 | 5 | 25 | 1500 | N/A | FDD | N/A |
| CA_n28-n41-n77 | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n41 | 2580 | 10 | 50 | 2580 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3323 | 32.7 | TDD | IMD22,4 |
|  | n28 | N/A | 5 | N/A | 798 | 35.3 | FDD | IMD21,4 |
|  | n41 | 2642 | 10 | 50 | 2642 | N/A | TDD | N/A |
|  | n77 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
|  | n28 | 738 | 5 | 25 | 793 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2642 | 33.5 | TDD | IMD24 |
|  | n77 | 3380 | 10 | 50 | 3380 | N/A | TDD | N/A |
| CA_n28-n41-n79 | n28 | 725 | 5 | 25 | 780 | 22 | FDD | IMD31 |
|  | n41 | 2600 | 10 | 50 | 2600 | N/A | TDD | N/A |
|  | n79 | 4600 | 40 | 216 | 4600 | N/A | TDD | N/A |
|  | n28 | 720 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n41 | 2600 | 10 | 50 | 2600 | N/A | TDD | N/A |
|  | n79 | 4480 | 40 | 216 | 4600 | 19.1 | TDD | IMD32 |
|  | n28 | 735 | 5 | 25 | 790 | N/A | FDD | N/A |
|  | n41 | 2645 | 10 | 50 | 2645 | 22.4 | TDD | IMD4 |
|  | n79 | 4850 | 40 | 216 | 4850 | N/A | TDD | N/A |
| CA_n28-n74-n77 | n28 | 730 | 5 | 25 | 785 | N/A | FDD | N/A |
|  | n74 | N/A | 5 | 25 | 1500 | 20.7 | FDD | IMD4 |
|  | n77 | 3690 | 10 | 50 | 3690 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | 25 | 785 | 27.7 | FDD | IMD3 |
|  | n74 | 1452 | 5 | 25 | 1500 | N/A | FDD | N/A |
|  | n77 | 3689 | 10 | 50 | 1818.5 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | 25 | 785 | 25.1 | FDD | IMD4 |
|  | n74 | 1458 | 5 | 25 | 1516 | N/A | FDD | N/A |
|  | n77 | 3589 | 10 | 50 | 1818.5 | N/A | FDD | N/A |
| CA_n28-n77-n79 | n77 | 3620 | 10 | 50 | 3620 | N/A | TDD | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |
|  | n28 | 745 | 5 | 25 | 800 | 22.7 | FDD | IMD21, 2 |
| CA_n28-n78-n79 | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n78 | 3700 | 10 | 50 | 3700 | N/A | TDD | N/A |
|  | n79 | N/A | 40 | N/A | 4440 | 32.2 | TDD | IMD23,4 |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
|  | n79 | N/A | 40 | N/A | 4440 | 5.7 | TDD | IMD53,4 |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3700 | 32.9 | TDD | IMD23 |
|  | n79 | 4440 | 40 | 216 | 4440 | N/A | TDD | N/A |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3420 | 16.8 | TDD | IMD33 |
|  | n79 | 4900 | 40 | 216 | 4900 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 800 | 22.2 | FDD | IMD2 |
|  | n78 | 3620 | 10 | 50 | 3620 | N/A | TDD | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 790 | 5.7 | FDD | IMD5 |
|  | n78 | 3530 | 10 | 50 | 3530 | N/A | TDD | N/A |
|  | n79 | 4900 | 40 | 216 | 4900 | N/A | TDD | N/A |
| CA_n29-n30-n77 | n29 | N/A | 5 | N/A | 722 | 23.5 | FDD | IMD31 |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | 3898 | 10 | 50 | 3898 | N/A | TDD | N/A |
| CA_n29-n66-n77 | n29 | N/A | 5 | N/A | 722 | 23.5 | FDD | IMD35 |
|  | n66 | 1734 | 5 | 25 | 2134 | N/A | FDD | N/A |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | TDD | N/A |
| CA_n30-n66-n77 | n30 | N/A | 5 | N/A | 2355 | 34.2 | FDD | IMD25 |
|  | n66 | 1745 | 5 | 25 | 2145 | N/A | FDD | N/A |
|  | n77 | 4100 | 10 | 50 | 4100 | N/A | TDD | N/A |
|  | n30 | N/A | 5 | N/A | 2355 | 12.9 | FDD | IMD5 |
|  | n66 | 1735 | 5 | 25 | 2135 | N/A | FDD | N/A |
|  | n77 | 3780 | 10 | 50 | 3780 | N/A | TDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2160 | 19.2 | FDD | IMD45 |
|  | n77 | 3390 | 10 | 50 | 3390 | N/A | TDD | N/A |
| CA_n41-n66-n77 | n41 | 2600 | 10 | 50 | 2600 | N/A | TDD | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3470 | 23.6 | TDD | IMD31,2 |
|  | n41 | N/A | 10 | N/A | 2670 | 16.6 | TDD | IMD55 |
|  | n66 | 1715 | 5 | 25 | 2115 | N/A | FDD | N/A |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | TDD | N/A |
|  | n41 | 2640 | 10 | 50 | 2640 | N/A | TDD | N/A |
|  | n66 | N/A | 5 | N/A | 2160 | 19.0 | FDD | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
| CA_n41-n71-n77 | n41 | 2615 | 10 | 50 | 2615 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3308 | 33.7 | TDD | IMD21,2,5 |
|  | n41 | 2564 | 10 | 50 | 2564 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3950 | 23.8 | TDD | IMD31 |
|  | n41 | N/A | 10 | N/A | 2615 | 32.7 | TDD | IMD25 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3308 | 10 | 50 | 3308 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2564 | 22.5 | TDD | IMD3 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | TDD | N/A |
|  | n41 | 2680 | 10 | 50 | 2680 | N/A | TDD | N/A |
|  | n71 | N/A | 5 | N/A | 640 | 35.3 | FDD | IMD25 |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
| CA_n41-n74-n77 | n41 | 2650 | 10 | 50 | 2650 | N/A | TDD | N/A |
|  | n74 | N/A | 5 | N/A | 1500 | 35.2 | FDD | IMD22 |
|  | n77 | 4150 | 10 | 50 | 4150 | NA | TDD | N/A |
|  | n41 | 2525 | 10 | 50 | 2525 | N/A | TDD | N/A |
|  | n74 | N/A | 5 | N/A | 1500 | 24 | FDD | IMD3 |
|  | n77 | 3550 | 10 | 50 | 3550 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2650 | 34 | TDD | IMD2 |
|  | n74 | 1450 | 5 | 25 | 1500 | N/A | FDD | N/A |
|  | n77 | 4100 | 10 | 50 | 4150 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2518 | 13.9 | TDD | IMD5 |
|  | n74 | 1454 | 5 | 25 | 1502 | N/A | FDD | N/A |
|  | n77 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
|  | n41 | 2650 | 10 | 50 | 2650 | N/A | TDD | N/A |
|  | n74 | 1450 | 5 | 25 | 1500 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 4100 | 33 | TDD | IMD2 |
|  | n41 | 2525 | 10 | 50 | 2525 | N/A | TDD | N/A |
|  | n74 | 1450 | 5 | 25 | 1498 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3600 | 23 | TDD | IMD31 |
| CA_n66-n71-n77 | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n71 | 668 | 5 | 25 | 622 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 4108 | 24.9 | TDD | IMD31,2,5 |
|  | n66 | N/A | 5 | N/A | 2150 | 24.5 | FDD | IMD32 |
|  | n71 | 690 | 5 | 25 | 644 | N/A | FDD | N/A |
|  | n77 | 3530 | 10 | 50 | 3530 | N/A | TDD | N/A |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n71 | N/A | 5 | N/A | 640 | 24.3 | FDD | IMD35 |
|  | n77 | 4080 | 10 | 50 | 4080 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE 4: This band is subject to IMD3 also which MSD is not specified. NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 6: Both of the transmitters shall be set min(+23 dBm, PCMAX_L,f,c) as defined in clause 6.2A.4 NOTE 7: This band supports intra-band non-contiguous uplink configuration. NOTE 8: In Japan, n77 band is restricted to 3400 – 4100 MHz frequency range, and there are no valid MSD test points when using this restricted frequency range |  |  |  |  |  |  |  |  |

Table 7.3A.5-2b: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC1.5 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) |  | Duplex mode |  |  |
| CA_n1-n3-n78 | n1 | 1950 | 5 | 25 | 2140 | N/A |  | FDD |  | N/A |
|  | n3 | N/A | 5 | N/A | 1830 | 38.7 |  | FDD |  | IMD2 |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A |  | TDD |  | N/A |
| CA_n1-n7-n78 | n1 | 1977.5 | 5 | 25 | 2167.5 | N/A |  | FDD |  | N/A |
|  | n7 | N/A | 5 | N/A | 2627.5 | 25.2 |  | FDD |  | IMD4 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A |  | TDD |  | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 29.5 | FDD |  | IMD4 |  |
|  | n7 | 2510 | 10 | 50 | 2630 | N/A | FDD |  | N/A |  |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | TDD |  | N/A |  |
| CA_n1-n26-n78 | n1 | N/A | 5 | N/A | 2122 | 31.9 |  | FDD |  | IMD3 |
|  | n26 | 829 | 5 | 25 | 874 | N/A |  | FDD |  | N/A |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A |  | TDD |  | N/A |
|  | n1 | 1975 | 5 | 25 | 2165 | N/A |  | FDD |  | N/A |
|  | n26 | N/A | 5 | N/A | 885 | 23.9 |  | FDD |  | IMD5 |
|  | n78 | 3405 | 10 | 50 | 3405 | N/A |  | TDD |  | N/A |
| CA_n1-n28-n78 | n1 | N/A | 5 | N/A | 2150 | 29.4 |  | FDD |  | IMD3 |
|  | n28 | 740 | 5 | 25 | 795 | N/A |  | FDD |  | N/A |
|  | n78 | 3630 | 10 | 50 | 3630 | N/A |  | TDD |  | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A |  | FDD |  | N/A |
|  | n28 | N/A | 5 | N/A | 794 | 21.3 |  | FDD |  | IMD5 |
|  | n78 | 3352 | 10 | 50 | 3352 | N/A |  | TDD |  | N/A |
| CA_n3-n7-n78 | n3 | N/A | 5 | N/A | 1820 | 31.4 | FDD |  | IMD3 |  |
|  | n7 | 2565 | 5 | 25 | 2685 | N/A | FDD |  | N/A |  |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | TDD |  | N/A |  |
|  | n3 | N/A | 5 | N/A | 1820 | 29.4 | FDD |  | IMD4 |  |
|  | n7 | 2565 | 5 | 25 | 2685 | N/A | FDD |  | N/A |  |
|  | n78 | 3475 | 10 | 50 | 3475 | N/A | TDD |  | N/A |  |
| CA_n3-n26-n78 | n3 | N/A | 5 | N/A | 1862 | 29.4 |  | FDD |  | IMD3 |
|  | n26 | 839 | 5 | 25 | 884 | N/A |  | FDD |  | N/A |
|  | n78 | 3540 | 10 | 50 | 3540 | N/A |  | TDD |  | N/A |
| CA_n3-n28-n78 | n3 | N/A | 5 | N/A | 1850 | 31 |  | FDD |  | IMD3 |
|  | n28 | 735 | 5 | 25 | 790 | N/A |  | FDD |  | N/A |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A |  | TDD |  | N/A |
| CA_n7-n26-n78 | n7 | 2550 | 5 | 25 | 2670 | N/A |  | FDD |  | N/A |
|  | n26 | N/A | 5 | N/A | 879 | 41 |  | FDD |  | IMD2 |
|  | n78 | 3429 | 10 | 50 | 3429 | N/A |  | TDD |  | N/A |
|  | n7 | 2525 | 5 | 25 | 2645 | N/A |  | FDD |  | N/A |
|  | n26 | N/A | 5 | N/A | 875 | 24.3 |  | FDD |  | IMD5 |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A |  | TDD |  | N/A |
|  | n7 | N/A | 5 | N/A | 2645 | 40.9 | FDD |  | IMD2 |  |
|  | n26 | 844 | 5 | 25 | 889 | N/A | FDD |  | N/A |  |
|  | n78 | 3489 | 10 | 50 | 3489 | N/A | TDD |  | N/A |  |
| CA_n7-n28-n78 | n7 | 2567.5 | 5 | 25 | 2687.5 | N/A | FDD |  | N/A |  |
|  | n28 | N/A | 5 | N/A | 782.5 | 39.6 | FDD |  | IMD21 |  |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | TDD |  | N/A |  |
|  | n7 | N/A | 5 | N/A | 2650 | 41.3 |  | FDD |  | IMD2 |
|  | n28 | 740 | 5 | 25 | 795 | N/A |  | FDD |  | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | N/A |  | TDD |  | N/A |
| CA_n25-n41-n66 | n25 | N/A | 5 | N/A | 1940 | 35.0 |  | FDD |  | IMD4 |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A |  | TDD |  | N/A |
|  | n66 | 1715 | 5 | 25 | 2115 | N/A |  | FDD |  | N/A |
| CA_n25-n41-n77 | n25 | 1870 | 5 | 25 | 1950 | N/A |  | FDD |  | N/A |
|  | n41 | 2670 | 10 | 50 | 2670 | N/A |  | TDD |  | N/A |
|  | n77 | N/A | 10 | N/A | 3470 | 31.3 |  | TDD |  | IMD31 |
|  | n25 | 1870 | 5 | 25 | 1950 | N/A |  | FDD |  | N/A |
|  | n41 | N/A | 10 | N/A | 2640 | 31.5 |  | TDD |  | IMD55 |
|  | n77 | 4125 | 10 | 50 | 4125 | N/A |  | TDD |  | N/A |
|  | n25 | N/A | 5 | N/A | 1950 | 34.1 |  | FDD |  | IMD35 |
|  | n41 | 2675 | 10 | 50 | 2675 | N/A |  | TDD |  | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A |  | TDD |  | N/A |
|  | n25 | N/A | 5 | N/A | 1950 | 30.1 |  | FDD |  | IMD4 |
|  | n41 | 2550 | 10 | 50 | 2550 | N/A |  | TDD |  | N/A |
|  | n77 | 3525 | 10 | 50 | 3525 | N/A |  | TDD |  | N/A |
| CA_n25-n66-n77 | n25 | 1855 | 5 | 25 | 1935 | N/A | FDD |  | N/A |  |
|  | n66 | N/A | 5 | N/A | 2115 | 41.2 | FDD |  | IMD22 |  |
|  | n77 | 3970 | 10 | 50 | 3970 | N/A | TDD |  | N/A |  |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD |  | N/A |  |
|  | n66 | N/A | 5 | N/A | 2160 | 31.2 | FDD |  | IMD5 |  |
|  | n77 | 3930 | 10 | 50 | 3930 | N/A | TDD |  | N/A |  |
|  | n25 | N/A | 5 | N/A | 1960 | 44.1 | FDD |  | IMD22,5 |  |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD |  | N/A |  |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD |  | N/A |  |
|  | n25 | N/A | 5 | N/A | 1960 | 27.6 | FDD |  | IMD55 |  |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD |  | N/A |  |
|  | n77 | 3620 | 10 | 50 | 3620 | N/A | TDD |  | N/A |  |
| CA_n25-n71-n77 | n25 | N/A | 5 | N/A | 1954 | 34.5 | FDD |  | IMD32,5 |  |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD |  | N/A |  |
|  | n77 | 3340 | 10 | 50 | 3340 | N/A | TDD |  | N/A |  |
| CA_n41-n66-n77 | n41 | 2600 | 10 | 50 | 2600 | N/A | TDD |  | N/A |  |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | FDD |  | N/A |  |
|  | n77 | N/A | 10 | N/A | 3470 | 32.6 | TDD |  | IMD31,2 |  |
|  | n41 | N/A | 10 | N/A | 2670 | 31.4 | TDD |  | IMD55 |  |
|  | n66 | 1715 | 5 | 25 | 2115 | N/A | FDD |  | N/A |  |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | TDD |  | N/A |  |
|  | n41 | 2640 | 10 | 50 | 2640 | N/A | TDD |  | N/A |  |
|  | n66 | N/A | 5 | N/A | 2160 | 30.8 | FDD |  | IMD4 |  |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD |  | N/A |  |
| CA_n41-n71-n77 | n41 | 2615 | 10 | 50 | 2615 | N/A |  | TDD |  | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A |  | FDD |  | N/A |
|  | n77 | N/A | 10 | N/A | 3308 | 39.7 |  | TDD |  | IMD21,2,5 |
|  | n41 | 2564 | 10 | 50 | 2564 | N/A |  | TDD |  | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A |  | FDD |  | N/A |
|  | n77 | N/A | 10 | N/A | 3950 | 32.8 |  | TDD |  | IMD31 |
|  | n41 | N/A | 10 | N/A | 2615 | 38.7 |  | TDD |  | IMD25 |
|  | n71 | 693 | 5 | 25 | 647 | N/A |  | FDD |  | N/A |
|  | n77 | 3308 | 10 | 50 | 3308 | N/A |  | TDD |  | N/A |
|  | n41 | N/A | 10 | N/A | 2564 | 31.5 |  | TDD |  | IMD3 |
|  | n71 | 693 | 5 | 25 | 647 | N/A |  | FDD |  | N/A |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A |  | TDD |  | N/A |
|  | 41 | 2680 | 10 | 50 | 2680 | N/A |  | TDD |  | N/A |
|  | n71 | N/A | 5 | N/A | 640 | 41.3 |  | FDD |  | IMD25 |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A |  | TDD |  | N/A |
| CA_n66-n71-n77 | n66 | N/A | 5 | N/A | 2150 | 33.5 |  | FDD |  | IMD32 |
|  | n71 | 690 | 5 | 25 | 644 | N/A |  | FDD |  | N/A |
|  | n77 | 3530 | 10 | 50 | 3530 | N/A |  | TDD |  | N/A |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A |  | FDD |  | N/A |
|  | n71 | N/A | 5 | N/A | 640 | 33.3 |  | FDD |  | IMD35 |
|  | n77 | 4080 | 10 | 50 | 4080 | N/A |  | TDD |  | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE 4: This band is subject to IMD3 also which MSD is not specified. NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped NOTE 6: This band combination is specified with support of two band inter-band UL CA with UL MIMO or Tx diversity capabilities, and the transmitter shall be set at min (+23 dBm, PCMAX_L,f,c) for the UL band with single Tx antenna connector as defined in clause 6.2A.4, and set at min (+27.8 dBm, PCMAX_L,f,c) for the UL band with 2Tx antenna connectors as defined in clause 6.2H.3.4 or 6.2L.3.4. |  |  |  |  |  |  |  |  |  |  |

==============End of change==============