3GPP TSG-RAN WG4 # 117	R4-2520232

Dallas, USA, 17th November 2025 – 21st November 2025

Agenda item:	7.5.4

Source:	MediaTek Inc.

Title:	Discussion on performance requirements for 6Rx UE with interference

Document for:	Discussion

# Introduction

This contribution provides our views on performance requirements for 6Rx UE with interference focusing on open issues listed in the last WF [1].

# Discussion

In this section, we discuss the open issues in several subsections.

## 2.1 General

For information, in the Rel-19 work item NR_ENDC_RF_Ph4-Perf, RAN4 agreed to define a single test for handheld and FWA devices.

Issue 2-2-1: Test cases and MIMO correlation for 4 MIMO layers for FWA and Handheld UE - Agreement - One single test case that is applicable for both FWA and Handheld UE with α = 0 and [β = 0.07179]

Coverage of UE types - Observation: - It is clearly stated in the approved WID that the 6Rx requirements with interference will cover both FWA and handheld UEs. - Proposals: - Proposal 1: FFS whether to define one set or two sets of requirements for handheld and FWA UEs until there is clear agreement in Rel-19. - Proposal 2: Define only a single requirement for both handheld and FWA devices. - Proposal 3: Define separate requirements for handheld and FWA devices

We consider it adequate to follow the principle in the 6Rx requirements to define a single test for handheld and FWA devices.

Observation #1: Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’ agreed to define a single test for handheld and FWA devices.

Proposal #1: Define only a single requirement for both handheld and FWA devices as in Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’.

Antenna correlation - Candidate options: - Proposal 1: Follow the decision in the Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’. - Proposal 2: Evaluate the performance with α = 0 and β = 0.07179 for handheld UE and low correlation for FWA - Proposal 3: For initial simulation alignment, cover both ULA Low and ULA “non-Low” as per outcome of NR_ENDC_RF_Ph4_Demod_6Rx.

We consider it adequate to follow the principle in the 6Rx requirements and use antenna correlation α = 0 and β = 0.07179 as a starting point. We can further evaluate whether Rx correlation causes any issues in the requirements with interference. If Rx correlation β = 0.07179 causes testability issues with interference, we are open to reconsidering the use of low correlation.

Proposal #2: Use antenna correlation α = 0 and β = 0.07179 as in Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’ as a starting point.

## 2.2 PDSCH requirements with inter-cell interference

Propagation condition - Candidate options: - Option 1: TDLA30-10 - Option 2: Consider TDLC300-100 and TDLA30-10 for handheld UE. For FWA devices, consider TDLA30-10 only. - Option 3: TDLC300-100 for 2 interfering cells (HomNet) and TDLA30-10 for 1 interfering cell (HetNet)

We consider it adequate to define the inter‑cell interference requirements using only the TDLA30‑10 propagation conditions.

Proposal #3: Use only TDLA30-10 propagation condition.

MCS - Candidate options: - Option 1: MCS 17 for HomNet and MCS 13 for HetNet - Option 2: MCS 17 - Option 3: MCS 17 for rank 2 and MCS 13 for rank 4

We consider it adequate to follow the principle in the 8Rx requirements to test HomNet scenario with rank 2 and MCS 17 and HetNet scenario with rank 4 and MCS 13.

Proposal #4: Use 2 interfering cells (HomNet) for rank 2.

Proposal #5: Use 1 interfering cells (HetNet) for rank 4.

Proposal #6: Use MCS 17 for rank 2.

Proposal #7: Use MCS 13 for rank 4.

## 2.3 PDSCH requirements with intra-cell inter-user interference

Propagation condition - Candidate options: - Option 1: TDLA30-10 - Option 2: Consider TDLC300-100 and TDLA30-10 for handheld UE. For FWA devices, consider TDLA30-10 only. - Option 3: TDLC300-100 with rank 1+1 and TDLA30-10 with rank 2+2

We consider that low mobility scenarios are more suitable for intra-cell inter-user interference scenarios.

Proposal #8: Use only TDLA30-10 propagation condition.

Precoding method for the co-scheduled UE - Candidate options: - Option 1: Random precoding with rank 1+1 and orthogonal precoding for rank 2+2 tests - Option 2: Orthogonal precoding for rank 2+2 tests

We consider it adequate to follow the principle in the 8Rx requirements and define the requirements for rank 2+2 only.

Proposal #9: Orthogonal precoding for rank 2+2 tests.

MCS for the target UE - Candidate options: - Option 1: MCS 17 (Table 1) for 2+2 case, and MCS19 (Table 1) for 1+1 case - Option 2: MCS 17 (Table 1) for 2+2 case - Option 3: Cover MCS 13 and MCS 17 for evaluation

We prefer to evaluate both the MCS 13 and MCS 17 options for the 2+2 case before making a final decision.

Proposal #10: Cover MCS 13 and MCS 17 for evaluation.

## 2.4 CQI reporting requirements with inter-cell interference

For information, in the Rel‑19 work item NR_ENDC_RF_Ph4‑Perf, RAN4 agreed to define CQI requirements for a single test under AWGN propagation conditions.

Issue 4-1-1: CQI test for 6Rx - Agreements: - Define CQI requirements with rank 4 under static channel - Propagation channel: AWGN - Antenna: 4x6 with static channel specified in Annex B.1 - RAN4 to only define one CQI reporting requirement applying to both handheld UEs and FWA UEs without additional applicability rules.

Whether to introduce CQI reporting requirements with inter-cell interference - Candidate options: - Option 1: No matter how RAN4 will design the CQI reporting tests for basic 6Rx, there is necessity to define CQI reporting requirements with inter-cell interference. - Option 2: Don’t define CQI reporting requirements with inter-cell interference for 6Rx

We recognize that there is a single CQI requirement defined for 6Rx in the Rel‑19 work item NR_ENDC_RF_Ph4‑Perf. We also recognize that CQI reporting requirements with inter‑cell interference are defined for 4Rx and 8Rx. However, we are not fully convinced that CQI reporting requirements with inter‑cell interference are necessary for 6Rx.

Observation #2: Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’ agreed to define single CQI requirements with rank 4 under static channel without additional applicability rules.

Proposal #11: Not define CQI reporting requirements with inter-cell interference.

## 2.5 Applicability and release independent

For information, in the Rel-19 work item NR_ENDC_RF_Ph4-Perf, RAN4 agreed to define CQI requirements for a single test under AWGN propagation conditions.

Issue 1-1: Test applicability rule for 6Rx 4 layers performance requirements - Agreement: - Introduce the test applicability rules that 4 MIMO layers performance requirements will be applicable for all 6Rx UEs that supporting maxNumberMIMO-LayersPDSCH = 4 or maxNumberMIMO-LayersPDSCH-v1900 = 6.

Test applicability for 6Rx requirements with interference - Candidate options: - Option 1: 6Rx requirements with interference should be mandatory for all 6Rx UEs without additional test applicability rule - Option 2: Define a test applicability rule to skip 2/4Rx requirements if 6Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not Release independent for 6Rx requirements with interference - Candidate options: - Option 1: Follow the same conclusion in Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’ - Option 2: All new 6Rx interference requirements introduced shall be release independent from Rel-17

We consider it adequate to follow the principle in the 6Rx requirements.

Observation #3: Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’ agreed to introduce the test applicability rules that 4 MIMO layers performance requirements will be applicable for all 6Rx UEs that supporting maxNumberMIMO-LayersPDSCH = 4 or maxNumberMIMO-LayersPDSCH-v1900 = 6.

Proposal #12: 6Rx requirements with interference can follow the same applicability and release independent rules defined in Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’.

# Summary

In this paper we provided our views to performance requirements for 6Rx UE with interference. The following observations and proposals are made.

Observation #1: Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’ agreed to define a single test for handheld and FWA devices.

Proposal #1: Define only a single requirement for both handheld and FWA devices as in Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’.

Proposal #2: Use antenna correlation α = 0 and β = 0.07179 as in Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’ as a starting point.

Proposal #3: Use only TDLA30-10 propagation condition.

Proposal #4: Use 2 interfering cells (HomNet) for rank 2.

Proposal #5: Use 1 interfering cells (HetNet) for rank 4.

Proposal #6: Use MCS 17 for rank 2.

Proposal #7: Use MCS 13 for rank 4.

Proposal #8: Use only TDLA30-10 propagation condition.

Proposal #9: Orthogonal precoding for rank 2+2 tests.

Proposal #10: Cover MCS 13 and MCS 17 for evaluation.

Observation #2: Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’ agreed to define single CQI requirements with rank 4 under static channel without additional applicability rules.

Proposal #11: Not define CQI reporting requirements with inter-cell interference.

Observation #3: Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’ agreed to introduce the test applicability rules that 4 MIMO layers performance requirements will be applicable for all 6Rx UEs that supporting maxNumberMIMO-LayersPDSCH = 4 or maxNumberMIMO-LayersPDSCH-v1900 = 6.

Proposal #12: 6Rx requirements with interference can follow the same applicability and release independent rules defined in Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’.

# References

[1] R4-2514823, “WF on NR_demod_Ph6_Ph5”, China Telecom