3GPP TSG-RAN WG4 # 117	R4-2520233

Dallas, USA, 17th November 2025 – 21st November 2025

Agenda item:	7.9.3

Source:	MediaTek Inc.

Title:	Discussion on training and interoperability for two-sided AIML models

Document for:	Discussion

# Introduction

This contribution provides our views on open issues to work item of Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2 [1].

# Discussion

Newly introduced work item of AI/ML for NR air interface Phase 2 provides the normative support for the CSI feedback enhancement (two-sided model) use case as well as standards-based UE data collection. Work item objectives for RAN4 are defined as follows. [2]

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

Recommendations for RAN4#117 The following are not agreements, but are recommendations for contributions and discussions at RAN4#117: - Proposals for assumptions for the dataset including how to cover scalability, channel models, number of samples - Proposals for how to evaluate the dataset including potential metrics to compare different dataset contributions - Proposals on model hyperparameters, considering in particular complexity, considering also quantization size - Proposals and reasoning for scalability solutions - Proposals and reasoning for training parameters - Proposals for performance metrics for evaluating the different potential model structures - Proposals for how to capture the model and the dataset in 3GPP (format, how to store etc.)

In the following sections, we present our views on the recommended open issues, along with several observations and proposals.

## 2.1 Discussion of dataset content

One of the main objectives of this work item is to define standardized dataset that is used for training the reference encoder and the test decoder.

First, we need to define the dataset content. In the Rel-19 RAN4 study, the dataset consisted of eigenvectors of the strongest layer. The existing PMI reporting requirements favor Rank 2 configurations, as this is usually a good indicator of PMI feedback quality. Therefore, if the dataset content is agreed to remain eigenvectors, it should be extended to cover the two strongest layers. Because an eigenvector representation omits some channel information, there have been proposals to use raw channel data in the datasets. Nevertheless, we prefer to retain an eigenvector-based dataset unless there is strong justification for collecting full raw channel data instead.

Observation #1: Raw channel data depends on the Tx and Rx dimension.

Observation #2: Eigenvector data depends on the Tx and the number of layers.

Proposal #1: Define the dataset content as eigenvectors up to Rank 2.

Second, we need to define the scenarios to be covered in the datasets. To test generalization, we need to define different antenna array shapes (N1, N2) for different tests. We propose maintaining separate datasets for different configurations to maximize the implementation’s ability to generalize. We propose considering 8 to 32 port Tx configurations for the datasets and excluding configurations with more than 32 ports, at least for now. We also propose reusing the Rx antenna configuration assumption from the Rel‑19 SI. Furthermore, we need to confirm whether it is sufficient to focus on only one duplex mode and one carrier frequency.

Observation #3: Different configurations may affect performance, e.g., different antenna array shape (N1, N2).

Observation #4: In the Rel-19 SI, the dataset was generated with a single configuration:

- 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ (N1=8, N2=2)

Proposal #2: Reuse the Rel-19 SI assumption for the Rx antenna configuration (4 Rx only).

Proposal #3: Consider generating datasets for several different Tx antenna configurations up to 32 ports:

- 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ (N1=8, N2=2)

- 32 ports: (8,8,2,1,1,4,4), (dH,dV) = (0.5, 0.8)λ (N1=4, N2=4)

- 16 ports: (8,8,2,1,1,1,8), (dH,dV) = (0.5, 0.8)λ (N1=8, N2=1)

- 16 ports: (8,8,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ (N1=4, N2=2)

- 8 ports: (8,8,2,1,1,1,4), (dH,dV) = (0.5, 0.8)λ (N1=4, N2=1)

- 8 ports: (8,8,2,1,1,2,2), (dH,dV) = (0.5, 0.8)λ (N1=2, N2=2)

Proposal #4: Do not consider datasets with more than 32 ports or keep this FFS.

Proposal #5: Keep different datasets separate to allow all generalization options.

Proposal #6: Confirm the duplex mode and carrier frequency assumptions for the dataset.

- FDD (15kHz SCS) in 2GHz fc and/or TDD (30kHz SCS) in 4GHz fc

When training the reference encoder and the test decoder, we propose combining all datasets for training. We also propose agreeing on the total target size of each dataset and the suggested contribution size from each company.

Proposal #7. Define minimum total dataset target size as 500,000 samples.

Proposal #8. Define minimum dataset target size from each company as 100,000 samples.

Lastly, we need to define the methodology for generating the dataset. In the Rel-19 SI, it was shown that performance depends strongly on whether the dataset is created from SLS or LLS data. SLS-based datasets tend to generalize well and can be assumed to cover LLS scenarios adequately, but not vice versa. Therefore, we propose generating the dataset solely using SLS configured with the CDL channel model.

Observation #5: In the Rel-19 SI, it was shown that performance depends strongly on whether the dataset is created from SLS or LLS data.

Proposal #9: Use SLS with the CDL channel model to generate the dataset.

## 2.2 Discussion of dataset evaluation

We need to define methodologies for verifying dataset correctness. In the Rel-19 study, datasets provided by some companies were found to differ significantly from most other datasets. We propose three metrics as potential verification measures.

Observation #6: In the Rel-19 SI, datasets provided by some companies were found to differ significantly from most other datasets.

Proposal #10: Study and agree on dataset-related metrics and statistics to ensure dataset correctness.

Proposal #11: Study the following metrics: power spectral entropy (PSE), frequency coherence (FC), and spatial coherence (SP).

## 2.3 Discussion of hyperparameters, complexity and quantization

One of the main objectives of this work item is to define standardized reference encoder and test decoder models.

The Rel-19 study item did not consider an implementation-friendly model-parameter format; it used full precision. However, when designing the reference encoder and decoder models, it is essential to ensure that the chosen parameter format is practical for real-world implementation. Specifically, while full precision is necessary during model training to achieve optimal performance, a quantized format should be adopted for the final deployed model to balance compression performance and memory requirements. Therefore, the format of the 3GPP reference encoder’s and test decoder’s model parameters must be studied and agreed. This decision will have clear implications for both compression efficiency and resource utilization.

Observation #7: The quantization of model parameters directly impacts compression performance and memory requirements.

Proposal #12: Study and agree on the appropriate format for the model parameters of the reference encoder and test decoder, distinguishing between training (full precision) and deployment (quantized format).

Proposal #13: 8-bit precision parameters be used for the reference encoder for deployment, with 16-bit precision as a fallback option if necessary.

Proposal #14: 8-bit precision parameters be used for the test decoder for deployment, with 16-bit precision as a fallback option if necessary.

Before we consider model backbones, we first need to understand the constraints. In practical implementations, FLOPs and memory budgets may differ significantly between UE and TE, and these should be considered separately. First, we need to agree on the upper bounds for FLOPs and parameter count for the reference encoder and the test decoder, respectively. With these constraints agreed, we can proceed to design and agree on the reference encoder and test decoder backbones.

Observation #8: Practical FLOPs and memory budgets may differ significantly between UE and TE and should be considered separately.

Observation #9: Backbones with the same complexity and memory footprint can achieve different levels of compression performance.

Proposal #15: As a first step, study and agree on implementation-friendly target complexity and memory budgets for the reference encoder and test decoder backbones.

Proposal #16: Define an upper bound of 8 million FLOPs per inference for the reference encoder.

Proposal #17: Define an upper bound of 500,000 parameters for the reference encoder.

Proposal #18: Ask feedback from TE vendors to define the upper bounds on FLOPs and parameter count.

Proposal #19: As a second step, agree on the reference encoder and test decoder backbones, considering the agreed target complexity and memory from the first step.

We have performed a quick complexity analysis in terms of FLOPS and the number of parameters for the Transformer reference encoder model under different parameter assumptions. The assumed parameters are listed in Table 2.3‑1, and the complexity analysis is shown in Table 2.3‑2, where configurations exceeding our proposed complexity thresholds are marked in red.

Table 2.3-1. Transformer Reference Encoder complexity analysis parameters.

| n_token | d_token | d_model | N_TF | d_feedforward | d_head | N_TF |
| --- | --- | --- | --- | --- | --- | --- |
| 13 | 64 | {32, 64, 128} | {2, 4, 8} | 256 | 8 | 4 |

Table 2.3-2. Transformer Reference Encoder complexity analysis with different parameters.

| (d_model, N_TF) | (32, 2) | (32, 4) | (32, 8) | (64, 2) | (64, 4) | (64, 8) | (128, 2) | (128, 4) | (128, 8) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| kFLOPS | 425.18 | 809.98 | 1580 | 1480 | 2890 | 5690 | 5520 | 10880 | 21590 |
| kParams | 40.83 | 66.24 | 117.06 | 130.78 | 230.75 | 430.69 | 458.14 | 854.69 | 1650 |

## 2.4 Discussion of scalability solutions

To support multiple configurations, either multiple models must be defined for different configurations, or scalability mechanisms must be defined within a single model. Methods such as padding/truncation and adaptation layers for handling different input/output dimensions were studied by RAN1 in the Rel-18 AI/ML for Air Interface SI. It was shown that the padding/truncation approach offers advantages in model size and FLOPs compared with the adaptation-layer approach, with comparable performance. Therefore, we propose using padding/truncation as the scalability mechanism for handling different input/output dimensions. Furthermore, various solutions for handling different latent-message sizes (CSI payload) were studied by RAN1 in the Rel-18 AI/ML for Air Interface SI, without a clear recommended solution. Therefore, we need to discuss and analyze which solution should be adopted by RAN4. The supported scalability should cover all agreed test-case configurations. Our intention is to define a single scalable reference encoder and a single scalable test decoder.

Observation #10: The padding/truncation approach has advantages in terms of model size and FLOPs compared to adaptation layer approach with comparable performance.

Observation #11: Various solutions for handling of different latent message sizes (CSI payload) has been studied by RAN1 in Rel-18 AI/ML for Air Interface SI.

Proposal #20: Define a reference encoder model which achieves scalability through zero-padding approach.

Proposal #21: Define a test decoder model which achieves scalability through truncation approach.

Proposal #22: First analyze and agree on what solution to adopt for latent message scalability.

Proposal #23: Define a reference encoder and a test decoder which supports the agreed method of handling various latent message sizes that covers all the agreed test case configurations which affects payload size.

## 2.5 Discussion of training parameters

For the training parameters, we consider that following the study item assumptions for hyperparameters is still adequate and provides a good starting point for the work item. For the final training of the reference encoder and test decoder, we propose using a combined dataset that covers all agreed test case configurations.

Proposal #24: Train reference encoder and test decoder using a combined dataset that covers all the agreed test case configurations.

Proposal #25: Follow study item assumptions for hyperparameters as a starting point

- Ratio of samples of total samples for training, validation, and testing: 80% / 10% / 10%

- Optimizer: Adam

- Learning rate: 0.0001

- Batch size: 256

- Max number of epochs: 150

- Stopping criteria: No validation loss improvement over 25 epochs

- Loss function: 1 - SGCS

- Drop out: No drop out

## 2.6 Discussion of performance metrics for model structures

We consider that reusing the SGCS per layer metric from the Rel‑19 SI is still adequate. We may need an additional metric to evaluate performance relative to model complexity.

Proposal #26: Define SGCS per layer as performance metric for model structures.

Proposal #27: Define some metric normalized to model complexity if several complexities are studied.

## 2.7 Discussion of how to capture the model and the dataset

We consider that reusing the model and dataset formats from the Rel‑19 SI is still adequate. If backwards compatibility becomes an issue with new tool versions, the old models and datasets should be converted to the new formats, and correct functionality must be confirmed for the converted models and datasets. For the model definition, we also suggest using a flowchart representation.

Proposal #28: For datasets, use NumPy standard binary file format (.npy).

Proposal #29: For models, use PyTorch (.pt) and ONNX (.onnx).

Proposal #30: For models, include a flowchart representation.

Proposal #31: Guarantee backwards compatibility by converting models and datasets to new tool versions as necessary.

# Summary

In this paper we provided our views to training and interoperability for two-sided AIML models topics. The following observations and proposals are made.

Observation #1: Raw channel data depends on the Tx and Rx dimension.

Observation #2: Eigenvector data depends on the Tx and the number of layers.

Proposal #1: Define the dataset content as eigenvectors up to Rank 2.

Observation #3: Different configurations may affect performance, e.g., different antenna array shape (N1, N2).

Observation #4: In the Rel-19 SI, the dataset was generated with a single configuration:

- 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ (N1=8, N2=2)

Proposal #2: Reuse the Rel-19 SI assumption for the Rx antenna configuration (4 Rx only).

Proposal #3: Consider generating datasets for several different Tx antenna configurations up to 32 ports:

- 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ (N1=8, N2=2)

- 32 ports: (8,8,2,1,1,4,4), (dH,dV) = (0.5, 0.8)λ (N1=4, N2=4)

- 16 ports: (8,8,2,1,1,1,8), (dH,dV) = (0.5, 0.8)λ (N1=8, N2=1)

- 16 ports: (8,8,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ (N1=4, N2=2)

- 8 ports: (8,8,2,1,1,1,4), (dH,dV) = (0.5, 0.8)λ (N1=4, N2=1)

- 8 ports: (8,8,2,1,1,2,2), (dH,dV) = (0.5, 0.8)λ (N1=2, N2=2)

Proposal #4: Do not consider datasets with more than 32 ports or keep this FFS.

Proposal #5: Keep different datasets separate to allow all generalization options.

Proposal #6: Confirm the duplex mode and carrier frequency assumptions for the dataset.

- FDD (15kHz SCS) in 2GHz fc and/or TDD (30kHz SCS) in 4GHz fc

Proposal #7. Define minimum total dataset target size as 500,000 samples.

Proposal #8. Define minimum dataset target size from each company as 100,000 samples.

Observation #5: In the Rel-19 SI, it was shown that performance depends strongly on whether the dataset is created from SLS or LLS data.

Proposal #9: Use SLS with the CDL channel model to generate the dataset.

Observation #6: In the Rel-19 SI, datasets provided by some companies were found to differ significantly from most other datasets.

Proposal #10: Study and agree on dataset-related metrics and statistics to ensure dataset correctness.

Proposal #11: Study the following metrics: power spectral entropy (PSE), frequency coherence (FC), and spatial coherence (SP).

Observation #7: The quantization of model parameters directly impacts compression performance and memory requirements.

Proposal #12: Study and agree on the appropriate format for the model parameters of the reference encoder and test decoder, distinguishing between training (full precision) and deployment (quantized format).

Proposal #13: 8-bit precision parameters be used for the reference encoder for deployment, with 16-bit precision as a fallback option if necessary.

Proposal #14: 8-bit precision parameters be used for the test decoder for deployment, with 16-bit precision as a fallback option if necessary.

Observation #8: Practical FLOPs and memory budgets may differ significantly between UE and TE and should be considered separately.

Observation #9: Backbones with the same complexity and memory footprint can achieve different levels of compression performance.

Proposal #15: As a first step, study and agree on implementation-friendly target complexity and memory budgets for the reference encoder and test decoder backbones.

Proposal #16: Define an upper bound of 8 million FLOPs per inference for the reference encoder.

Proposal #17: Define an upper bound of 500,000 parameters for the reference encoder.

Proposal #18: Ask feedback from TE vendors to define the upper bounds on FLOPs and parameter count.

Proposal #19: As a second step, agree on the reference encoder and test decoder backbones, considering the agreed target complexity and memory from the first step.

Observation #10: The padding/truncation approach has advantages in terms of model size and FLOPs compared to adaptation layer approach with comparable performance.

Observation #11: Various solutions for handling of different latent message sizes (CSI payload) has been studied by RAN1 in Rel-18 AI/ML for Air Interface SI.

Proposal #20: Define a reference encoder model which achieves scalability through zero-padding approach.

Proposal #21: Define a test decoder model which achieves scalability through truncation approach.

Proposal #22: First analyze and agree on what solution to adopt for latent message scalability.

Proposal #23: Define a reference encoder and a test decoder which supports the agreed method of handling various latent message sizes that covers all the agreed test case configurations which affects payload size.

Proposal #24: Train reference encoder and test decoder using a combined dataset that covers all the agreed test case configurations.

Proposal #25: Follow study item assumptions for hyperparameters as a starting point

- Ratio of samples of total samples for training, validation, and testing: 80% / 10% / 10%

- Optimizer: Adam

- Learning rate: 0.0001

- Batch size: 256

- Max number of epochs: 150

- Stopping criteria: No validation loss improvement over 25 epochs

- Loss function: 1 - SGCS

- Drop out: No drop out

Proposal #26: Define SGCS per layer as performance metric for model structures.

Proposal #27: Define some metric normalized to model complexity if several complexities are studied.

Proposal #28: For datasets, use NumPy standard binary file format (.npy).

Proposal #29: For models, use PyTorch (.pt) and ONNX (.onnx).

Proposal #30: For models, include a flowchart representation.

Proposal #31: Guarantee backwards compatibility by converting models and datasets to new tool versions as necessary.

# References

[1] R4-2514614, “WF on R20 NR_AIML_air_Ph2”, Ericsson

[2] RP-252445, “New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2”, Qualcomm