3GPP TSG-RAN WG4 # 117	R4-2520234

Dallas, USA, 17th November 2025 – 21st November 2025

Agenda item:	6.11.3

Source:	MediaTek Inc.

Title:	Discussion on CSI reporting requirement and testing framework for CSI prediction

Document for:	Discussion

# Introduction

In the previous RAN4#116bis meeting, the CSI reporting requirements and the testing framework for CSI prediction were discussed. The WF and topic summary documents contain several open issues for further work [1][2]. In this document, we discuss the remaining open issues and provide several observations and proposals.

# Discussion

In the latest WF and topic summary documents, open issue remain regarding performance monitoring.

Issue 1-5: Performance monitoring accuracy metric - Proposals - Option 1: RAN4 could consider defining the report delay requirement based on the ratio of SGCS reports during the whole testing time, and to check whether the ratio can be larger than pre-defined threshold Y with X% of test time, where the SGCS reporting is based on RAN1 definition. - Option 2: Introduce accuracy requirements for SGCS1 and SGCS2 separately - SGCS 1 is calculated based on predicted CSI for one inference reporting, and ground truth CSI. - SGCS 2 is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance. - Option 3: Ensure network trust in UE-reported SGCS and define LCM criteria by (i) enabling the test equipment (TE) to independently recompute SGCS1/SGCS2 and (ii) defining absolute and relative SGCS performance thresholds as triggers for LCM actions. - Option 4: Study the feasibility to set the CSI-PAI reporting requirements with that the SGCS1 should be more than X1 in [90]% of the test time with SNR=Y1 (dB), e.g., X1=0.85 and Y1=10dB. - Study the feasibility to set the CSI-PAI reporting requirements with that the radio SGCS1/SGCS2 should be more than X2 in [90]% of the test time with SNR=Y2 (dB), e.g., X1=1.2 and Y1=10dB. - Option 5: RAN4 should define reporting accuracy requirements for CSI prediction performance monitoring using a test framework that evaluates the stability of reported values in a fixed environment. - Option 6: RAN4 selects the statistics of SGCS1, defined based on predicted CSI for inference reporting and ground truth CSI, as the metric to evaluate UE’s performance monitoring for CSI prediction. UE would pass a test if its reported SGCS1, averaged across many occasions, exceed a threshold. - RAN4 investigates UE’s reported SGCS1 during performance monitoring for CSI prediction across various channels, e.g., TDL-A, TDL-B and TDL-C, and selects the worst-case scenario to define the threshold for requirements. - Option 7: other proposals

In the PMI requirements study, we observed a Doppler sweet spot that enables the largest performance gains from precoding prediction. When Doppler is too low, there is little opportunity for gain; when Doppler is too high, the channel is under sampled and accurate prediction cannot be achieved.

Observation #1: The performance gains of precoding prediction depend strongly on Doppler.

Because measuring the absolute accuracy of performance monitoring is challenging, we consider that the most practical approach is to evaluate the monitoring under conditions where the highest gains are expected and under conditions where no gain (or even a loss) is expected, and to verify that the performance monitoring metrics reflect these outcomes.

However, there is now strong time pressure to complete the work item, and no proposals or alignment simulations have been provided to identify suggested use cases for verifying performance monitoring. Therefore, we propose testing performance monitoring using the same test cases used to verify prediction performance in the PMI requirements. This approach covers scenarios where prediction gains are assumed and allows verification that the monitoring metrics confirm these gains.

Proposal #1: Evaluate the performance‑monitoring metric under the same test conditions used to CSI prediction PMI requirements.

In Figures 1 and 2, we present CSI‑PAI distributions and CDFs of per‑layer SGCS1 and SGCS2 measured under the agreed 2Rx FDD and 4Rx TDD test conditions. The legends also include the mean and median quantized SGCS1 and SGCS2 values. In Table 1, we present the proportions of the per‑layer condition SGCS1 > SGCS2.

We consider that the sample‑and‑hold metric (SGCS2) does not require explicit testing but can be used for comparison with the prediction metric (SGCS1), as shown in Table 1. We are also open to considering an additional test metric for SGCS1 only, using either mean, median, or CDF‑based test criteria.

Proposal #2: Use proportion of per layer SGCS1 > SGCS2 as a test metric.

Proposal #3: Consider an additional test metric for SGCS1 only, using either mean, median, or CDF‑based test criteria.

Figure 1: CSI-PAI SGCS distributions and CDFs at 90% throughput (16TX, rank 2, MCS-17/19 in FDD).
Mean and median quantized SGCS values shown in the legend.

Figure 2: CSI-PAI SGCS distributions and CDFs at 90% throughput (16TX, rank 2, MCS-17/19 in TDD).
Mean and median quantized SGCS values shown in the legend.

Table 1: Proportion of quantized SGCS1_k > SGCS2_k for AI/ML CSI prediction in TDLA30-20, rank 2.

| Test | Testing SNR [dB] | Proportion of SGCS1_k > SGCS2_k |
| --- | --- | --- |
| FDD 16TX MCS-17 | 7 | [0.983, 0.994] |
| FDD 16TX MCS-19 | 9 | [0.980, 0.993] |
| TDD 16TX MCS-17 | 4 | [0.970, 0.984] |
| TDD 16TX MCS-19 | 5 | [0.978, 0.987] |

# Conclusion

In this contribution we provide our views on the CSI reporting requirement and testing framework for CSI prediction issues. The following observations and proposals are made:

Observation #1: The performance gains of precoding prediction depend strongly on Doppler.

Proposal #1: Evaluate the performance-monitoring metric under the same test conditions used to CSI prediction PMI requirements.

Proposal #2: Use proportion of per layer SGCS1 > SGCS2 as a test metric.

Proposal #3: Consider an additional test metric for SGCS1 only, using either mean, median, or CDF-based test criteria.

# Reference

- R4-2514631, “WF on [116bis][112] R19 AI for air interface”, Qualcomm inc.

- R4-2514519, “Topic Summary for [116bis][112] R19 AI for air interface”, Qualcomm inc.