3GPP TSG-RAN WG4 # 117	R4-2520237

Dallas, USA, 17th November 2025 – 21st November 2025

Agenda item:	6.12.3

Source:	MediaTek Inc.

Title:	Simulation results of NR MIMO Phase 5 demodulation and CSI requirements

Document for:	Discussion

# Introduction

In RAN4#116bis, RAN4 agreed basic parameters for simulations of typeI-SinglePanel-r19 and eTypeII-r19 codebooks with 64 ports [1]. In this contribution, we provide our simulation results for NR MIMO Phase 5 demodulation and CSI requirements.

# Simulation results

In this section, we present the simulation results for the 64TX Type I‑Single Panel‑r19 and 64TX eType II‑r19 codebooks.

## 2.1 Single PMI with 64TX Type I-Single Panel-r19 codebook

In this section, we present the simulation setup for NR MIMO Phase 5 demodulation and CSI requirements. At RAN4#115, it was agreed that the general simulation configuration should follow the existing 38.101‑4 Single PMI with 32TX Type I‑Single Panel codebook requirement configuration, as specified in Sections 6.3.3.1.4 and 6.3.3.2.4 [2]. The test parameters were further refined during the RAN4#116 and RAN4#116bis meetings. For FDD Single PMI with 64TX Type I‑Single Panel‑r19 codebook, the final parameters assumed in the simulations are listed in Table 2.1-1, and the corresponding parameters for TDD are provided in Table 2.1-2.

Note that the CQI/RI/PMI delays are determined according to the number of CSI‑RS ports and CSI processing Capability 2. In FDD, the PMI delay is configured as 7 slots and 11 ms, while in TDD the PMI delay is configured as 11 slots and 9 ms.

Table 2.1-1: Test parameters of FDD Single PMI with 64TX TypeI-SinglePanel-r19 codebook tests
(baseline Table 6.3.3.1.4-1 in [2])

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |
| Subcarrier spacing |  | kHz | 15 |  |
| Duplex Mode |  |  | FDD |  |
| Propagation channel |  |  | TDLA30-5 |  |
| Antenna configuration |  |  | High XP 64 x numRX (N1,N2) = (8,4) |  |
| Beamforming Model |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,(5) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 5,(7) |  |
|  | CSI-RS periodicity and offset | slot | 5/1 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource ID |  | Resource #1 | Resource #2 |
|  | CSI-RS resource Type |  | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 32 | 32 |
|  | CDM Type |  | CDM4 (FD2, TD2) | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17,(2, 4, 6, 8) | Row 17,(2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | Row 17,(2, 9) | Row 17,(5, 12) |
|  | CSI-RS interval and offset | slot | Not configured | Not configured |
|  | aperiodicTriggeringOffset |  | 0 | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |  |
|  | CSI-IM RE pattern |  | Pattern 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (5,7) |  |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |  |
| ReportConfigType |  |  | Aperiodic |  |
| CQI-table |  |  | Table 1 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| pmi-FormatIndicator |  |  | Wideband |  |
| Sub-band Size |  | RB | 8 |  |
| csi-ReportingBand |  |  | 1111111 |  |
| CSI-Report interval and offset |  | slot | Not configured |  |
| Aperiodic Report Slot Offset |  |  | 7 |  |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |
| reportTriggerSize |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel-r19 |  |
|  | Codebook Mode |  | 1 |  |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (8,4) |  |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |  |
|  | CodebookSubsetRestriction |  | 0x FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF |  |
|  | RI Restriction |  | 00000010 |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 11 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |
| Measurement channel |  |  | R.PDSCH.1-6.3 FDD (Modified to MCS22) |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |  |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-7), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |

Table 2.1-2: Test parameters of TDD Single PMI with 64TX TypeI-SinglePanel-r19 codebook tests
(baseline Table 6.3.3.2.4-1 in [2])

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |  |
| Subcarrier spacing |  | kHz | 30 |  |
| Duplex Mode |  |  | TDD |  |
| TDD DL-UL configurations |  |  | FR1.30-1 as specified in Annex A |  |
| Propagation channel |  |  | TDLA30-5 |  |
| Antenna configuration |  |  | High XP 64 x numRX (N1,N2) = (8,4) |  |
| Beamforming Model |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,(5) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 5,(7) |  |
|  | CSI-RS periodicity and offset | slot | 20/6 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource ID |  | Resource #1 | Resource #2 |
|  | CSI-RS resource Type |  | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 32 | 32 |
|  | CDM Type |  | CDM4 (FD2, TD2) | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17,(2, 4, 6, 8) | Row 17,(2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 17,(2, 9) | Row 17,(5, 12) |
|  | CSI-RS interval and offset | slot | Not configured | Not configured |
|  | aperiodicTriggeringOffset |  | 0 | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |  |
|  | CSI-IM RE pattern |  | Pattern 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (5,7) |  |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |  |
| ReportConfigType |  |  | Aperiodic |  |
| CQI-table |  |  | Table 1 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannnelMeasurements |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| pmi-FormatIndicator |  |  | Wideband |  |
| Sub-band Size |  | RB | 16 |  |
| csi-ReportingBand |  |  | 1111111 |  |
| CSI-Report interval and offset |  | slot | Not configured |  |
| Aperiodic Report Slot Offset |  |  | 12 |  |
| CSI request |  |  | 1 in slots i, where mod(i, 20) = 6, otherwise it is equal to 0 |  |
| reportTriggerSize |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel-r19 |  |
|  | Codebook Mode |  | 1 |  |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (8,4) |  |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |  |
|  | CodebookSubsetRestriction |  | 0x FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF |  |
|  | RI Restriction |  | 00000010 |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 9 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |
| Measurement channel |  |  | Custom MCS22 |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |  |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-12), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |

In the following Figures 2.1-1 and Tables 2.1-3, 2.1-4, 2.1-5, and 2.1-6 simulation results of FDD 4RX, FDD 2RX, TDD 4RX, and TDD 2RX are presented for typeI-SinglePanel-r19 codebook with Scheme A.

Figure 2.1-1: Simulation results of FDD and TDD Single PMI with 64TX TypeI-SinglePanel-r19 codebook tests.

Table 2.1-3: Simulation results of FDD 4Rx Single PMI with 64TX TypeI-SinglePanel-r19 codebook tests

| FDD 4Rx | MCS-22 – 4Rx, 64Tx (8x4) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SNR (90%) 3.11 dB | SNR offset, X [dB] |  |  |  |  |  |
|  | +0 | +1 | +2 | +3 | +4 | +5 |
| Gamma | 28.51 | 20.76 | 15.36 | 11.71 | 9.21 | 7.53 |

Table 2.1-4: Simulation results of FDD 2Rx Single PMI with 64TX TypeI-SinglePanel-r19 codebook tests

| FDD 2Rx | MCS-22 – 2Rx, 64Tx (8x4) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SNR (90%) 9.62 dB | SNR offset, X [dB] |  |  |  |  |  |
|  | +0 | +1 | +2 | +3 | +4 | +5 |
| Gamma | 15.92 | 12.25 | 9.52 | 7.75 | 6.36 | 5.33 |

Table 2.1-5: Simulation results of TDD 4Rx Single PMI with 64TX TypeI-SinglePanel-r19 codebook tests

| TDD 4Rx | MCS-22 – 4Rx, 64Tx (8x4) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SNR (90%) 2.93 dB | SNR offset, X [dB] |  |  |  |  |  |
|  | +0 | +1 | +2 | +3 | +4 | +5 |
| Gamma | 30.22 | 22.13 | 16.76 | 12.69 | 10.02 | 7.88 |

Table 2.1-6: Simulation results of TDD 2Rx Single PMI with 64TX TypeI-SinglePanel-r19 codebook tests

| TDD 2Rx | MCS-22 – 2Rx, 64Tx (8x4) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SNR (90%) 9.64 dB | SNR offset, X [dB] |  |  |  |  |  |
|  | +0 | +1 | +2 | +3 | +4 | +5 |
| Gamma | 16.24 | 11.84 | 9.41 | 7.49 | 6.04 | 5.03 |

## 2.2 Multiple PMI with 64TX eTypeII-r19 codebook

In this section, we present the simulation setup for NR MIMO Phase 5 demodulation and CSI requirements. At RAN4#116bis, it was agreed to define requirements for Multiple PMI with 64TX eType II‑r19 codebook. The final parameters assumed for FDD in the simulations are listed in Table 2.2-1, and the corresponding parameters for TDD are provided in Table 2.2-2.

Note that the CQI/RI/PMI delays are determined according to the number of CSI-RS ports and CSI processing Capability 2. In FDD, the PMI delay is configured as 7 slots and 11 ms, while in TDD the PMI delay is configured as 11 slots and 9 ms.

Table 2.2-1: Test parameters of FDD Single PMI with 64TX eTypeII-r19 codebook tests

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |
| Subcarrier spacing |  | kHz | 15 |  |
| Duplex Mode |  |  | FDD |  |
| Propagation channel |  |  | TDLA30-5 |  |
| Antenna configuration |  |  | High XP 64 x numRX (N1,N2) = (8,4) |  |
| Beamforming Model |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,(5) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 5,(7) |  |
|  | CSI-RS periodicity and offset | slot | 5/1 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource ID |  | Resource #1 | Resource #2 |
|  | CSI-RS resource Type |  | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 32 | 32 |
|  | CDM Type |  | CDM4 (FD2, TD2) | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17,(2, 4, 6, 8) | Row 17,(2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | Row 17,(2, 9) | Row 17,(5, 12) |
|  | CSI-RS interval and offset | slot | Not configured | Not configured |
|  | aperiodicTriggeringOffset |  | 0 | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |  |
|  | CSI-IM RE pattern |  | Pattern 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (5,7) |  |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |  |
| ReportConfigType |  |  | Aperiodic |  |
| CQI-table |  |  | Table 1 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| pmi-FormatIndicator |  |  | Not configured |  |
| Sub-band Size |  | RB | 4 |  |
| csi-ReportingBand |  |  | 1111111111111 |  |
| CSI-Report interval and offset |  | slot | Not configured |  |
| Aperiodic Report Slot Offset |  |  | 7 |  |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |
| reportTriggerSize |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Codebook configuration | Codebook Type |  | eTypeII-r19 |  |
|  | paramCombination-r16 |  | 6 (L=4, pν=1/2, β=1/2) |  |
|  | R(numberOfPMISubbandsPerCQISubband-r16) |  | 1 |  |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (8,4) |  |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |  |
|  | CodebookSubsetRestriction |  | 0x FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF |  |
|  | RI Restriction |  | 00000010 |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 11 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |
| Measurement channel |  |  | R.PDSCH.1-6.3 FDD (Modified to MCS22) |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with i1 wideband granularity and i2 subband granularity |  |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. The random precoder generation shall follow 'typeI-SinglePanel' codebook configuration as specified in table 6.3.3.1.3-1. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-7), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the dual-cluster beam directions shall be used as specified in Annex B.2.3.2.3A. The value of relative power ratio (p) shall be fixed as 1 during the test. |  |  |  |  |

Table 2.2-2: Test parameters of TDD Single PMI with 64TX eTypeII-r19 codebook tests

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |  |
| Subcarrier spacing |  | kHz | 30 |  |
| Duplex Mode |  |  | TDD |  |
| TDD DL-UL configurations |  |  | FR1.30-1 as specified in Annex A |  |
| Propagation channel |  |  | TDLA30-5 |  |
| Antenna configuration |  |  | High XP 64 x numRX (N1,N2) = (8,4) |  |
| Beamforming Model |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,(5) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 5,(7) |  |
|  | CSI-RS periodicity and offset | slot | 20/6 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource ID |  | Resource #1 | Resource #2 |
|  | CSI-RS resource Type |  | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 32 | 32 |
|  | CDM Type |  | CDM4 (FD2, TD2) | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17,(2, 4, 6, 8) | Row 17,(2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 17,(2, 9) | Row 17,(5, 12) |
|  | CSI-RS interval and offset | slot | Not configured | Not configured |
|  | aperiodicTriggeringOffset |  | 0 | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |  |
|  | CSI-IM RE pattern |  | Pattern 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (5,7) |  |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |  |
| ReportConfigType |  |  | Aperiodic |  |
| CQI-table |  |  | Table 1 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannnelMeasurements |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| pmi-FormatIndicator |  |  | Not configured |  |
| Sub-band Size |  | RB | 8 |  |
| csi-ReportingBand |  |  | 11111111111111 |  |
| CSI-Report interval and offset |  | slot | Not configured |  |
| Aperiodic Report Slot Offset |  |  | 12 |  |
| CSI request |  |  | 1 in slots i, where mod(i, 20) = 6, otherwise it is equal to 0 |  |
| reportTriggerSize |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Codebook configuration | Codebook Type |  | eTypeII-r19 |  |
|  | paramCombination-r16 |  | 6 (L=4, pν=1/2, β=1/2) |  |
|  | R(numberOfPMISubbandsPerCQISubband-r16) |  | 1 |  |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (8,4) |  |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |  |
|  | CodebookSubsetRestriction |  | 0x FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF |  |
|  | RI Restriction |  | 00000010 |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 9 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |
| Measurement channel |  |  | Custom MCS22 |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with i1 wideband granularity and i2 subband granularity |  |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity) with equal probability of each applicable i1, i2 combination. The random precoder generation shall follow 'typeI-SinglePanel' codebook configuration as specified in table 6.3.3.2.3-1. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-12), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). Note 3: Randomization of the dual-cluster beam directions shall be used as specified in Annex B.2.3.2.3A. The value of relative power ratio (p) shall be fixed as 1 during the test. |  |  |  |  |

In the following Figures 2.2-1 and Tables 2.2-3, 2.2-4, 2.2-5, and 2.2-6 simulation results of FDD 4RX, FDD 2RX, TDD 4RX, and TDD 2RX are presented for eTypeII-r19 codebook.

Figure 2.2-1: Simulation results of FDD and TDD Single PMI with 64TX eTypeII-r19 codebook tests.

Table 2.2-3: Simulation results of FDD 4Rx Single PMI with 64TX eTypeII-r19 codebook tests

| FDD 4Rx | MCS-22 – 4Rx, 64Tx (8x4) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SNR (90%) 1.76 dB | SNR offset, X [dB] |  |  |  |  |  |
|  | +0 | +1 | +2 | +3 | +4 | +5 |
| Gamma | 29.65 | 19.19 | 13.98 | 10.09 | 7.64 | 5.98 |

Table 2.2-4: Simulation results of FDD 2Rx Single PMI with 64TX eTypeII-r19 codebook tests

| FDD 2Rx | MCS-22 – 2Rx, 64Tx (8x4) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SNR (90%) 4.67 dB | SNR offset, X [dB] |  |  |  |  |  |
|  | +0 | +1 | +2 | +3 | +4 | +5 |
| Gamma | 57.66 | 35.88 | 23.53 | 16.17 | 11.52 | 8.47 |

Table 2.2-5: Simulation results of TDD 4Rx Single PMI with 64TX eTypeII-r19 codebook tests

| TDD 4Rx | MCS-22 – 4Rx, 64Tx (8x4) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SNR (90%) 2.77 dB | SNR offset, X [dB] |  |  |  |  |  |
|  | +0 | +1 | +2 | +3 | +4 | +5 |
| Gamma | 19.23 | 13.64 | 9.95 | 7.41 | 5.92 | 4.84 |

Table 2.2-6: Simulation results of TDD 2Rx Single PMI with 64TX eTypeII-r19 codebook tests

| TDD 2Rx | MCS-22 – 2Rx, 64Tx (8x4) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SNR (90%) 6.52 dB | SNR offset, X [dB] |  |  |  |  |  |
|  | +0 | +1 | +2 | +3 | +4 | +5 |
| Gamma | 26.83 | 17.36 | 12.37 | 9.14 | 6.88 | 5.48 |

# Conclusion

In this paper we provided our simulation setup and results for NR MIMO Phase 5 demodulation and CSI requirements.

# Reference

- R4-2514806, WF on NR_MIMO_demod, Samsung

- 3GPP TS 38.101-4 V19.0.0 (2025-09), User Equipment (UE) radio transmission and reception, Part 4: Performance requirements