3GPP TSG-RAN WG4 Meeting #117	R4-2520250

Dallas, United States, 17th – 21st November 2025

Source:	Nokia

Title:	On Performance and CSI reporting requirements for 6Rx UE with interference

Agenda item:	7.5.4

Document for:	Discussion

Introduction

In the last RAN4 meeting several agreements were made for enabling companies to provide simulation results. In this contribution we cover the remaining open topics and provide our observations and proposals.

Discussion

General

Coverage of UE types

In RAN4#116bis it was discussed if a single or multiple requirements are to be defined for FWA and handheld UEs (see [1]):

Coverage of UE types - Observation: - It is clearly stated in the approved WID that the 6Rx requirements with interference will cover both FWA and handheld UEs. - Proposals: - Proposal 1: FFS whether to define one set or two sets of requirements for handheld and FWA UEs until there is clear agreement in Rel-19. - Proposal 2: Define only a single requirement for both handheld and FWA devices. - Proposal 3: Define separate requirements for handheld and FWA devices

As can be seen from proposal 1 above, one option is to wait for the agreement from Rel-19. In the WF from Rel-19 [2] the following agreement can be found for a 4 layer UE:

Issue 2-2-1: Test cases and MIMO correlation for 4 MIMO layers for FWA and Handheld UE - Agreement - One single test case that is applicable for both FWA and Handheld UE with α = 0 and [β = 0.07179]

Since it was agreed to have only one single test case applicable for both FWA and Handheld UE for 4 layers, we see it reasonable to follow the same agreement also for interference cases.

In Rel-19 6Rx PDSCH requirements, it was agreed in RAN4#116bis to define test case which is applicable for both FWA and handheld UE for each layer configuration.

Requirements defined in this work item shall be applicable for both handheld and FWA devices

Antenna correlation

In RAN4#116bis the used antenna correlation was discussed (see [1]).

Antenna correlation - Candidate options: - Proposal 1: Follow the decision in the Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’. - Proposal 2: Evaluate the performance with α = 0 and β = 0.07179 for handheld UE and low correlation for FWA - Proposal 3: For initial simulation alignment, cover both ULA Low and ULA “non-Low” as per outcome of NR_ENDC_RF_Ph4_Demod_6Rx.

The same issue has also been discussed in Rel-19 6Rx where it was decided for 4 layers to use (see [2] section 2.4):

Propagation channel; 4 Layers: TDLA30-10 ULA correlation channel with α = 0 and β = 0.07179 6 Layers: TDLA30-10 ULA Low.

Since it is not clear at this point if the same correlation will be feasible for all cases discussed in this WI, we find it reasonable to cover both ULA correlation channel with α = 0 and β = 0.07179 and ULA Low in the initial analysis.

In Rel-19 6Rx discussion the following correlation has been selected: ULA correlation channel with α = 0 and β = 0.07179 for rank 4.

In the Appendix we have provided our initial simulation results for inter-cell interference using the proposed MCS13 and MCS17 as well as the discussed antenna correlations. All simulations are done using TDLA30-10.
When comparing to the results from 8Rx we see an expected increase in SNR values when using ULA Low. For “ULA [MedC] (α = 0 and β = 0.07179)” we see reasonable SNR values up to rank 2 for requirement definition, however for rank 4 the SNR is increased to above 23dB.

Based on our initial simulation results it is feasible to define requirements for rank 1,2 and 4 using ULA-Low. For “ULA [MedC] (α = 0 and β = 0.07179)” we see SNR above 23dB for Rank 4 hence it should be considered if it is only feasible to define requirements with inter-cell interference for rank 4 with ULA-Low.

Further evaluate the performance with the following correlations:
- ULA Low
- ULA using α = 0 and β = 0.07179

PDSCH requirements with inter cell interference

Rank for the serving cell

In RAN4#116bis it was agreed to cover rank 1,2 and 4 for initial evaluation purpose (see [1]):

Rank for the serving cell - Agreement: - For initial evaluation purpose, cover rank 1,2 and 4 And make further selection based on simulation.

Our initial simulation results in Appendix show it feasible to define requirements for rank 1,2 and 4 with the currently discussed propagation channels and correlation. Only potential issue at this point is using “ULA [MedC] (α = 0 and β = 0.07179)” for rank 4.

Propagation condition

In RAN4#116bis the propagation condition was discussed with two propagation channels suggested for further study (see [1]):

Propagation condition - Candidate options: - Option 1: TDLA30-10 - Option 2: Consider TDLC300-100 and TDLA30-10 for handheld UE. For FWA devices, consider TDLA30-10 only. - Option 3: TDLC300-100 for 2 interfering cells (HomNet) and TDLA30-10 for 1 interfering cell (HetNet)

At this point in time, it is not clear if there will be one or two sets of requirements, hence we see it reasonable to cover both TDLA30-10 as well as TDLC300-100 for initial simulation alignment. We are open to consider down selection based on number of interference cells to reduce simulation load if needed.

For PDSCH with inter-cell interference, cover the following propagation conditions for initial simulation alignment:
- TDLA30-10
- TDLC300-100
Further discuss if both propagation conditions are required for simulation alignment for all discussed testcases.

MCS

In RAN4#116bis the MCS was discussed with two options suggested for further study (see [1]):

MCS - Candidate options: - Option 1: MCS 17 for HomNet and MCS 13 for HetNet - Option 2: MCS 17 - Option 3: MCS 17 for rank 2 and MCS 13 for rank 4

At this point in time, it is not clear which MCS would be feasible for requirement definition. It has been suggested (option 1) to link different MCS to HomNet and HetNet. Both proposed MCS13 and MCS17 shall be taken into account for simulation alignment, however we are open to link dedicated MCS to number of interference cells as well as rank if that will reduce the simulation load.

Cover the following MCS for initial simulation alignment:
- MCS 13
- MCS 17
Further discuss if both MCS13 and MCS17 are required for initial simulation alignment for all considered test cases.

PDSCH requirements with intra cell inter user interference

Propagation condition

In RAN4#116bis the propagation condition was discussed with two propagation channels suggested for further study (see [1]):

Propagation condition - Candidate options: - Option 1: TDLA30-10 - Option 2: Consider TDLC300-100 and TDLA30-10 for handheld UE. For FWA devices, consider TDLA30-10 only. - Option 3: TDLC300-100 with rank 1+1 and TDLA30-10 with rank 2+2

At this point in time, it is not clear if there will be one or two sets of requirements, hence we see it reasonable to cover both TDLA30-10 as well as TDLC300-100 for initial simulation alignment. We are open to consider down selection based on number of interference cells to reduce simulation load if needed.

For PDSCH requirements with intra-cell inter-user interference, cover the following propagation conditions for initial simulation alignment:
- TDLA30-10
- TDLC300-100
Further discuss if both propagation conditions are required for simulation alignment for all considered rank combinations.

Precoding method for the co-scheduled UE

In RAN4#116bis the precoding method for the co-scheduled UE was discussed (see [1]):

Precoding method for the co-scheduled UE - Candidate options: - Option 1: Random precoding with rank 1+1 and orthogonal precoding for rank 2+2 tests - Option 2: Orthogonal precoding for rank 2+2 tests

In previous topics covering intra-cell inter-user interference random precoding has been used with rank 1+1 and orthogonal precoding has been used for rank 2+2 test cases. We see this as a good starting point for simulation alignment. In addition, the two options from RAN4#116bis propose orthogonal precoding for rank 2+2 test cases.

For precoding for the co-scheduled UE in intra-cell inter-user interference cases use the following as starting point for alignment:
- rank 1+1: Random precoding
- rank 2+2: Orthogonal precoding.

MCS for the target UE

In RAN4#116bis the MCS was discussed with three options suggested for further study (see [1]):

MCS for the target UE - Candidate options: - Option 1: MCS 17 (Table 1) for 2+2 case, and MCS19 (Table 1) for 1+1 case - Option 2: MCS 17 (Table 1) for 2+2 case - Option 3: Cover MCS 13 and MCS 17 for evaluation

Based on above options option 1 is a superset of all 3 as it covers MCS17 for rank 2+2 and MCS19 for rank 1+1 which at the same time secures coverage of both MCS13 and MCS17 for evaluation.

Cover the following MCS for initial simulation alignment:
- Rank 1+1: MCS 13
- Rank 2+2: MCS 17

CQI reporting requirements with inter-cell interference

Whether to introduce CQI reporting requirements with inter-cell interference

In RAN4#116bis the question on introducing CQI reporting requirements with inter-cell interference was not further discussed as the WID dictated to follow the pending agreement in Rel-19 6Rx. The following is the options from RAN4#116bis (see [1]):

Whether to introduce CQI reporting requirements with inter-cell interference - Candidate options: - Option 1: No matter how RAN4 will design the CQI reporting tests for basic 6Rx, there is necessity to define CQI reporting requirements with inter-cell interference. - Option 2: Don’t define CQI reporting requirements with inter-cell interference for 6Rx

From the Rel-19 6Rx WF the following agreement can be found (see [2]):

Issue 4-1-1: CQI test for 6Rx - Agreements: - Define CQI requirements with rank 4 under static channel - Propagation channel: AWGN - Antenna: 4x6 with static channel specified in Annex B.1 - RAN4 to only define one CQI reporting requirement applying to both handheld UEs and FWA UEs without additional applicability rules.

As it can be seen, it has been agreed to introduced CQI requirements with rank 4 under static channel for 6Rx, which according to the WID means, that it shall also be considered in Rel-20 to introduce CQI reporting requirements with inter-cell interference.

RAN4 has decided to introduce CQI requirements for 6Rx in Rel-19.

Define CQI requirements with inter-cell interference for rank 2.

Interference modelling and test parameters for CQI reporting requirements with inter-cell interference (if introduced)

Interference modelling and test parameters for CQI reporting requirements with inter-cell interference (if introduced) - Candidate options: - Option 1: RAN4 to reuse only Rel-17 interference models for 6Rx devices. - Option 2: Use the configuration provided in below table as starting point Parameter; Value. SCS/CBW; FDD: 15kHz/10MHz TDD: 30kHz/40MHz, 7D1S2U(S=6D+4G+4U). Rank for serving cell; Rank 2. Propagation conditions; TDLA30-5 for the serving cell AWGN for the interference cell. Antenna and MIMO correlation for serving cell; 2T6R with ULA Low 2T6R with ULA “non-Low” as per outcome of NR_ENDC_RF_Ph4_Demod_6Rx.. Antenna configuration for the interference cell:; 1T6R. Interference power level; INR = 10.04dB for the serving cell.. Other parameters; For other parameters re-use the same configuration as used in Rel-17 requirements..

Based on the agreement in Rel-19 for 6Rx to use ULA with α = 0 and β = 0.07179 the same can be used for CQI with interference, hence we propose the following configuration as starting point:

For CQI with interference, use the following configuration as starting point for initial alignment:

| Parameter | Value |
| --- | --- |
| SCS/CBW | TDD: 30kHz/40MHz, 7D1S2U(S=6D+4G+4U) |
| Rank for serving cell | Rank 2 |
| Propagation conditions | TDLA30-5 for the serving cell AWGN for the interference cell |
| Antenna and MIMO correlation for serving cell | ULA with α = 0 and β = 0.07179. |
| Antenna configuration for the interference cell: | 1T6R |
| Interference power level | INR = 10.04dB for the serving cell. |
| Other parameters | For other parameters re-use the same configuration as used in Rel-17 requirements. |

Test metric for CQI reporting requirements with inter-cell interference (if introduced)

The following was proposed in RAN4#116bis for CQI test metric (see [1]):

Test metric for CQI reporting requirements with inter-cell interference (if introduced) - Candidate options: - Option 1: Reuse the same test metric for Rel-17 2Rx, 4Rx and 8Rx (Nokia) a) the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥  b) when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to X%..

Since the same test metric has been used for 2Rx/4Rx and 8Rx CQI interference it can also be used for 6Rx CQI interference cases.

Reuse the same test metric for Rel-17 2Rx, 4Rx and 8Rx

a) the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥  b) when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to X%.

Applicability and release independent

Test applicability for 6Rx requirements with interference

In RAN4#116bis the question of test applicability was raised but not further discussed (see [1]):

Test applicability for 6Rx requirements with interference - Candidate options: - Option 1: 6Rx requirements with interference should be mandatory for all 6Rx UEs without additional test applicability rule - Option 2: Define a test applicability rule to skip 2/4Rx requirements if 6Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not

As we expect the target UE will maximum have rank 4, we do not see any reason to introduce additional test applicability rules.

6Rx requirements with interference should be mandatory for all 6Rx UEs without additional test applicability rule

Release independent for 6Rx requirements with interference

In RAN4#116bis the question of release independency for 6Rx requirements with interference was raised but not further discussed (see [1]):

Release independent for 6Rx requirements with interference - Candidate options: - Option 1: Follow the same conclusion in Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’ - Option 2: All new 6Rx interference requirements introduced shall be release independent from Rel-17

For 8Rx with interference requirements introduced in Rel-19, it was agreed that all requirements was to be release independent from Rel-17, hence the same shall apply for 6Rx with interference requirements.

The agreement from 8Rx with interference to have release independence from Rel-17 can also apply for 6Rx with interference requirements.

: All new 6Rx interference requirements introduced shall be release independent from Rel-17.

Conclusion

This contribution has provided Nokia’s view on the topic of 6Rx with interference.

The following Observations and Proposals were made:

General

Coverage of UE types

- In Rel-19 6Rx PDSCH requirements, it was agreed in RAN4#116bis to define test case which is applicable for both FWA and handheld UE for each layer configuration.

- Requirements defined in this work item shall be applicable for both handheld and FWA devices

Antenna correlation

In Rel-19 6Rx discussion the following correlation has been selected: ULA correlation channel with α = 0 and β = 0.07179 for rank 4.

Based on our initial simulation results it is feasible to define requirements for rank 1,2 and 4 using ULA-Low. For “ULA [MedC] (α = 0 and β = 0.07179)” we see SNR above 23dB for Rank 4 hence it should be considered if it is only feasible to define requirements with inter-cell interference for rank 4 with ULA-Low.

Further evaluate the performance with the following correlations:
- ULA Low
- ULA using α = 0 and β = 0.07179

PDSCH requirements with inter cell interference

Propagation condition

For PDSCH with inter-cell interference, cover the following propagation conditions for initial simulation alignment:
- TDLA30-10
- TDLC300-100
Further discuss if both propagation conditions are required for simulation alignment for all discussed testcases.

MCS

Cover the following MCS for initial simulation alignment:
- MCS 13
- MCS 17
Further discuss if both MCS13 and MCS17 are required for initial simulation alignment for all considered test cases.

PDSCH requirements with intra cell inter user interference

Propagation condition

For PDSCH requirements with intra-cell inter-user interference, cover the following propagation conditions for initial simulation alignment:
- TDLA30-10
- TDLC300-100
Further discuss if both propagation conditions are required for simulation alignment for all considered rank combinations.

Precoding method for the co-scheduled UE

For precoding for the co-scheduled UE in intra-cell inter-user interference cases use the following as starting point for alignment:
- rank 1+1: Random precoding
- rank 2+2: Orthogonal precoding.

MCS for the target UE

Cover the following MCS for initial simulation alignment:
- Rank 1+1: MCS 13
- Rank 2+2: MCS 17

CQI reporting requirements with inter-cell interference

Whether to introduce CQI reporting requirements with inter-cell interference

RAN4 has decided to introduce CQI requirements for 6Rx in Rel-19.

Define CQI requirements with inter-cell interference for rank 2.

Interference modelling and test parameters for CQI reporting requirements with inter-cell interference (if introduced)

For CQI with interference, use the following configuration as starting point for initial alignment:

| Parameter | Value |
| --- | --- |
| SCS/CBW | TDD: 30kHz/40MHz, 7D1S2U(S=6D+4G+4U) |
| Rank for serving cell | Rank 2 |
| Propagation conditions | TDLA30-5 for the serving cell AWGN for the interference cell |
| Antenna and MIMO correlation for serving cell | ULA with α = 0 and β = 0.07179. |
| Antenna configuration for the interference cell: | 1T6R |
| Interference power level | INR = 10.04dB for the serving cell. |
| Other parameters | For other parameters re-use the same configuration as used in Rel-17 requirements. |

Test metric for CQI reporting requirements with inter-cell interference (if introduced)

Reuse the same test metric for Rel-17 2Rx, 4Rx and 8Rx

a) the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥  b) when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to X%.

Applicability and release independent

Test applicability for 6Rx requirements with interference

6Rx requirements with interference should be mandatory for all 6Rx UEs without additional test applicability rule

Release independent for 6Rx requirements with interference

The agreement from 8Rx with interference to have release independence from Rel-17 can also apply for 6Rx with interference requirements.

: All new 6Rx interference requirements introduced shall be release independent from Rel-17.

- Appendix

- Simulation results for inter-cell interference

The following shows our initial simulation results for inter-cell interference:

| TC# | Duplex Mode and SCS | CHBW | Scenario | MIMO | Rank (target UE) | MCS (target UE) | INR1 | INR2 | Channel Model | Antenna correlation | SNR@ 70%TP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | TDD 30KHz SCS | 40 MHz | HomNet | 2T6R | 1 | 13 | 7.77 | 2.29 | TDLA30-10 | ULA Low | 3.5 |
| 2 |  |  | HetNet | 2T6R | 1 | 13 | 7.58 | N/A |  |  | 2.3 |
| 3 |  |  | HomNet | 2T6R | 1 | 17 | 7.77 | 2.29 |  |  | 6 |
| 4 |  |  | HetNet | 2T6R | 1 | 17 | 7.58 | N/A |  |  | 4.9 |
| 5 |  |  | HomNet | 2T6R | 2 | 13 | 7.77 | 2.29 |  |  | 7.2 |
| 6 |  |  | HetNet | 2T6R | 2 | 13 | 7.58 | N/A |  |  | 6.2 |
| 7 |  |  | HomNet | 2T6R | 2 | 17 | 7.77 | 2.29 |  |  | 10.2 |
| 8 |  |  | HetNet | 2T6R | 2 | 17 | 7.58 | N/A |  |  | 9.2 |
| 9 |  |  | HomNet | 4T6R | 4 | 13 | 7.77 | 2.29 |  |  | 12.7 |
| 10 |  |  | HetNet | 4T6R | 4 | 13 | 7.58 | N/A |  |  | 10.9 |
| 11 |  |  | HomNet | 4T6R | 4 | 17 | 7.77 | 2.29 |  |  | 16.6 |
| 12 |  |  | HetNet | 4T6R | 4 | 17 | 7.58 | N/A |  |  | 14.6 |
| 13 |  |  | HomNet | 2T6R | 1 | 13 | 7.77 | 2.29 |  | ULA [MedC] (α = 0 and β = 0.07179) | 7.6 |
| 14 |  |  | HetNet | 2T6R | 1 | 13 | 7.58 | N/A |  |  | 4.8 |
| 15 |  |  | HomNet | 2T6R | 1 | 17 | 7.77 | 2.29 |  |  | 10.2 |
| 16 |  |  | HetNet | 2T6R | 1 | 17 | 7.58 | N/A |  |  | 7.4 |
| 17 |  |  | HomNet | 2T6R | 2 | 13 | 7.77 | 2.29 |  |  | 14.4 |
| 18 |  |  | HetNet | 2T6R | 2 | 13 | 7.58 | N/A |  |  | 12.1 |
| 19 |  |  | HomNet | 2T6R | 2 | 17 | 7.77 | 2.29 |  |  | 17.4 |
| 20 |  |  | HetNet | 2T6R | 2 | 17 | 7.58 | N/A |  |  | 15.2 |
| 21 |  |  | HomNet | 4T6R | 4 | 13 | 7.77 | 2.29 |  |  | 23.6 |
| 22 |  |  | HetNet | 4T6R | 4 | 13 | 7.58 | N/A |  |  | 22.9 |
| 23 |  |  | HomNet | 4T6R | 4 | 17 | 7.77 | 2.29 |  |  | 28 |
| 24 |  |  | HetNet | 4T6R | 4 | 17 | 7.58 | N/A |  |  | 27.4 |

- References

- R4-2514823 - WF on NR_demod_Ph6_Ph5

- R4-2514788 - Way Forward for [116bis][215] NR_ENDC_RF_Ph4_Demod_6Rx